Initial Environmental Examination Report

Project Number: 55236-001

July 2024

Project Readiness Financing for Punjab Urban Development Projects

Updated IEE for Augmentation of Surface Water Supply for Khayaban-e-Sir Syed from Khanpur Dam and Creation of DMAs and Solarization Component under DREAMS-I

Prepared by Project Management Unit of PRF PUDP, Government of Punjab, Pakistan

This updated version of the Initial Environmental Examination Report is a document of the borrower. The views expressed herein do not necessarily represent those of ADB’s Board of Directors, Management, or staff, and may be preliminary in nature. Your attention is directed to the “terms of use” section of the ADB website.

In preparing any country program or strategy, financing any project, or by making any designation of or reference to a particular territory or geographic area in this document, the Asian Development Bank does not intend to make any judgements as to the legal or other status of any territory or area.

CURRENCY EQUIVALENTS

As of 13th July, 2024           Currency Unit – Pak Rupees (Pak Rs.)

Pak Rs 1.00 = $ 0.0036          US$1.00 = Pak Rs. 277

CONVERSIONS

|  | 1 meter = 3.28 feet |
| --- | --- |
|  | 1 hectare = 2.47 acre |

Acronyms

| ADB | Asian Development Bank |
| --- | --- |
| AIP | Access to Information Policy |
| AMSL | Above Mean Sea Level |
| AC | Asbestos Cement |
| BC | Before Construction |
| BOQ | Bill of Quantities |
| COVID-19 | Corona Virus Infection Disease-2019 |
| CSC | Construction Supervision Consultation |
| DC | During Construction |
| DMA | District Metered Area |
| DO | During Operation |
| EA | Executing Agency |
| EDCM | Engineering Design Construction Management |
| EGL | Existing Ground Level |
| EHS | Environment Health and Safety |
| EIA | Environmental Impact Assessment |
| EMP | Environmental Management Plan |
| EPA | Environmental Protection Agency |
| GFR | Gross Enrollment Rate |
| GI | Galvanized Iron |
| GST | Ground Storage Tanks |
| GoP | Government of Pakistan |
| GRM | Grievance Redress Mechanism |
| GWSS | Gravity Water Supply Scheme |
| HDPE | High Density Polyethylene |
| IA | Implementing Agency |
| IBAT | Integrated Biodiversity Assessment Tool |
| IEE | Initial Environmental Examination |
| IFC | International Finance Corporation |
| KPI | Key Performance Indicators |
| LAA | Land Acquisition Act (of 1984) |
| LARP | Land Acquisition and Resettlement Plan |
| Lea | Equivalent sound pressure level |
| MGD | Million Gallons per Day |
| MSDS | Material safety Data Sheet |
| NEP | National Environmental Policy |
| NEQS | National Environmental Quality Standards |
| NER | Net Enrolment Rate |
| NRW | Non-Revenue Water |
| OHS | Occupational Safety and Health |
| O&M | Operation and Maintenance |
| OHR | Overhead Reservoirs |
| PAP | Project Affected People |
| PC | Public Consultation |
| PEQS | Punjab Environmental Quality Standards |
| PPC | Plain Cement Concrete |
| PDD | Planning and Development Department |
| PEPA | Pakistan Environmental Protection Act 1997 |
| PEPC | Pakistan environmental protection Council |
| PEQS | Punjab Environmental Quality Standards |
| PGA | Peak Ground Acceleration |
| PMU | Program Management Unit |
| PPE | Personal protective Equipment |
| Punjab-EPA | Punjab Environmental Protection Agency |
| PV RCC | Photovoltaic Reinforced Cement Concrete |
| REA | Rapid Environmental Assessment |
| RP | Resettlement Plan |
| RWASA | Rawalpindi Water and Sanitation Agency |
| SOPs | Standard Operating Procedures |
| SS | Suspended Solids |
| TDS | Total Dissolved Solids |
| SPS | Safeguard Policy Statement |
| SSEMP | Site Specific Environmental Management Plan |
| TMA | Tehsil Municipal Administration |
| TMP | Traffic Management Plan |
| UCs | Union Councils |
| USEPA | United States Environmental Protection Agency |
| WHO | World Health Organization |
| WTP | Water Treatment Plant |

Contents

EXECUTIVE SUMMARY	i

Project Overview	i

Project Need	v

Project Benefits	v

Policy, Legal and Administrative Framework	v

Environmental Safeguards Category of the Project	v

Project proponent	vi

Project Alternatives	vi

Description of Environment	vi

Environmental Testing & monitoring	vii

Sensitive Receptors	vii

Potential Environmental Impacts and Mitigation Measures	vii

Project Impact Evaluation Matrix	vii

Environmental Management Plan	xiii

Institutional Arrangements	xiii

Environment Budget	xiii

Public Consultation and Information Disclosure	xiii

Grievance Redressal Mechanism	xiv

Conclusion and Recommendations	xiv

1.	INTRODUCTION	1

1.1	Overview of the project	1

1.2	Existing Water Supply system of KSS	4

1.3	Project Need	4

1.4	Project Benefits	7

1.5	Environmental Safeguards Category of the Project	7

1.6	Scope and Objectives of the IEE	7

1.7	Methodology of IEE Study	8

1.8	Project Proponent	9

1.9	Structure of the Report	10

1.10	IEE Team	10

1.11	Further Additions & Updating of IEE Study	11

2.	POLICY, LEGAL AND ADMINISTRATIVE FRAMEWORK	12

2.1	General	12

2.2	National Policy and Legal Framework	12

2.3	Regulations for Environmental Assessment, Pakistan EPA	12

2.4	Regulatory Clearances, Punjab EPA and Pak EPA	12

2.5	Guidelines for Environmental Assessment, Pakistan EPA	12

2.6	Punjab Environmental Quality Standards (NEQS) 2000 & 2010	13

2.7	Other Environment Related Legislations	13

2.8	ADB’s Safeguard Policy Statement (SPS), 2009	26

2.9	ADB’s Access to Information Policy (AIP) 2018	26

2.10	ADB’s Accountability Mechanism Policy 2012	26

2.11	ADB Gender and Development Policy	27

2.12	Climate Change Risk Management Framework	27

2.13	Implications of ADB’s safeguard policies on proposed project	27

2.14	IFC Environmental, Health and Safety Guidelines for Water and Sanitation	28

2.15	Comparison of International and Local Environmental Legislations	29

3.	PROJECT DESCRIPTION	34

3.1	Project Introduction	34

Existing Water Supply System of KSS	36

3.2	Components of Project	41

3.3	Creation of DMAs for KSS	41

3.4	Selection of Study Area for DMAs	44

3.5	Design Philosophy of DMAs	44

3.6	DMA Boundaries from Water GEMS Model	46

3.7	Isolation Plan of DMAs	46

3.8	Water Metering	51

3.9	Water Meters Required for KSS	51

3.10	Flow Meters for Bulk Supply Main	51

3.11	Domestic Water Meters	51

3.12	Design of Bulk Supply Mains	54

3.13	Hydraulic Model of Bulk Supply Mains	56

3.14	Excavation	56

3.15	Bedding and Laying/ Jointing	56

3.16	Testing	56

3.17	Restoration of damaged Utility Services	56

3.18	Provision of Thrust Blocks	56

3.19	Backfilling	56

3.20	Restoration of Road/ Street	56

3.21	Pipe Jacking	56

3.22	DESIGN OF DISTRIBUTION NETWORK	58

3.23	Replacement of Outlived Pipes	58

3.24	Distribution Lines for Unserved Areas	58

3.25	Restoration of Road/ Street	59

3.26	Pumping Requirement	62

3.27	Pumping at Sangjani Treatment Plant	62

3.28	Pumping Requirement for Water Supply Distribution in KSS	62

3.29	Climatic Consideration	63

3.30	Schedule of Project Implementation	63

3.31	Construction phase activities	64

3.32	Construction Machinery Requirement	66

3.33	Solar PV System Installation	66

3.34	Construction Activities for Solar PV installation	70

3.35	Operation phase of Solar installation	71

3.36	Decommissioning	72

3.37	Construction Materials Requirement	72

3.38	Operation Phase Details for Proposed project	73

3.39	Operation Equipment and Machinery	73

3.40	Manpower Requirement	73

3.41	Cost of Project	73

3.42	Electricity Required	75

3.43	Water Required for Construction and Operation	75

3.44	Generation of Solid Waste	75

3.45	Generation of Wastewater and its disposal mechanism	75

4.	ANALYSIS OF ALTERNATIVES	76

4.1	Overview	76

4.2	Design and Siting of Alternatives	76

4.3	Options for Water Meters	79

Selected Option:	81

4.4	Siting the Contractor’s Facilities at Appropriate Site, Be it Public Land or Private Land	81

4.5	Labor	81

4.6	Technology Alternatives	82

4.7	Solar PV Installation Component	83

5.	Description of Environment	85

5.1	General	85

5.2	Site Visit for baseline visit	85

5.3	Physical Environment	86

5.4	Topography	89

5.5	Land use of the Proposed Project	89

5.6	Soils	91

5.7	Geologic Setting	91

5.8	Water Resources	92

5.9	Groundwater	92

5.10	Surface Water	92

5.11	Seismicity	93

5.12	Climate	95

5.13	Temperature	95

5.14	Rainfall	96

5.15	Relative Humidity	97

5.16	Wind Speed	98

5.17	Wind Rose	99

5.18	Floods	100

5.19	Ecological Environment	103

5.20	Ecosystem	103

5.21	Protected areas	103

5.22	Flora	105

5.23	Fauna	105

5.24	Socio-economic Environment	107

5.25	Environmental Sensitive Receptors of the Proposed Project	113

5.26	Environmental Monitoring for Water Supply Works	120

5.27	Environmental Monitoring for Solar installations	131

6.	POTENTIAL ENVIRONMENTAL IMPACTS AND MITIGATION MEASURES	118

6.1	General	118

6.2	Methodology for Impact Screening	118

6.3	Potential Positive Impacts	119

6.4	Potential Adverse Impacts	120

6.5	Design/Pre-Construction Phase	120

6.6	Adverse Impacts during Planning and Design Phase	122

6.7	Adverse Impacts during Construction Phase	125

6.8	Impacts during Site Restorations	149

6.9	Anticipated Impacts during Operation Phase	150

6.10	Water System Leaks and Water Discharges during Flushing	151

6.11	Improved Drinking Water Availability	152

6.12	Improvements in Public Health	152

Solar PV Installation	152

6.13	Impacts and mitigation measures during pre-Construction phase	152

6.14	Impacts and mitigation measures during Construction Phase	156

6.15	Operation Phase	171

6.16	Waste Generation	174

6.17	Ecological Impacts	174

6.18	Occupational Health and Safety	174

6.19	Glare from solar panels might blind pilots or air traffic control towers	175

6.20	Interference with navigation and communication systems of aircrafts (electromagnetic interference)	176

6.21	Green energy-based climate friendly form of power generation	176

6.22	Decommissioning	176

6.23	Cumulative Impacts	177

6.24	Indirect and Induced Impacts	177

7.	ENVIRONMENTAL MANAGEMENT, MONITORING AND REPORTING PLAN	178

7.1	Introduction	178

7.2	Environmental Management Plan	178

7.3	Objectives of EMP	178

7.4	Environmental Management, Monitoring and Reporting	179

7.5	Inclusion of EMP in Contract documents	179

7.6	Institutional Arrangements	179

7.7	Monitoring Parameters	199

7.8	Environmental Training	204

7.9	Environmental Management Costs for Water Supply Component	206

7.10	Environmental Management Costs for Solar Installation	233

7.11	Performance Indicators	235

8.	Public Consultation and Information Disclosure	237

8.1	Identification of Stakeholders	238

8.2	Consultation Process	238

8.3	Consultation with Project Affected Peoples	239

8.4	Issues, Concerns and Findings of the Focal Group Discussion	239

8.5	Consultation with Official Stakeholders	255

8.6	Consultation Plan for Construction and Operation Phase	259

8.7	Information Disclosure	260

8.8	Consultations for Solar Installations	260

9.	GRIEVANCE REDRESSAL MECHANISM	269

9.1	General	269

10.	Conclusion and Recommendations	272

10.1	Conclusion	272

10.2	Recommendations	273

Annexures	274

Annexure-I Filled Checklist of REA	275

Annexure-II Environmental Monitoring Reports	281

Annexure-III Calibration Certificates of Monitoring Equipment	292

Annexure-IV Chance Find Procedure	298

Annexure-V Framework for Traffic Management Plan	300

Annexure-VI Framework for Resource Management Plan	309

Annexure-VII Framework for Occupational Safety and Health Management Plan	312

Annexure-VIII Framework for Emergency Response Management Plan	323

Annexure-IX Framework for Dust Management Plan	340

Annexure-X Framework for Noise and Vibration Management Plan	345

Annexure-XI Framework for Asbestos Management Plan	349

Annexure-XII Template for Site Specific EMP	352

Annexure-XIII Tool for Socio-economic Survey	360

Annexure-XIV Attendance Sheets of FGDs	364

Annexure-XV: Public Consultation Minutes for Solar Installation Development & Public Hearing Minutes	374

Annexure-XVI: Laboratory Monitoring Results for Solar Installation Site at Sanjrani Water Filtration Plant	403

List of Figures

Figure ES.1: Location of the Project	iii

Figure ES.2: Proposed solar PV installation at Sangjani WTP site	iv

Figure 1.1: Location of the project	3

Figure 1.2: Details of sub-projects under DREAMS-1 Project	2

Figure 1.3: Proposed solar PV installation at Sangjani WTP site	3

Figure 1.4: Detail of outlived distribution pipelines to be replaced	6

Figure 3.1: Urban Core Area of Rawalpindi	35

Figure 3.2: Existing Water Resources of KSS	38

Figure 3.3: Existing Water Supply System in KSS	40

Figure 3.4: Alignment of Existing Conductance Main and Proposed Conductance Main	42

Figure 3.5: A typical Thematic view of DMA	43

Figure 3.6: Proposed DMAs, / Zones in Khayaban-e-Sir Syed	45

Figure 3.7: Water supply boundaries of DMAs derived from Hydraulic Modelling	47

Figure 3.8: Isolation Map of DMA-1	48

Figure 3.9 Isolation Map of DMA-2	49

Figure 3.10 Isolation Map of DMA-3	50

Figure 3.11: Locations of Flow Meters on Bulk Supply Mains	52

Figure 3.12 Location of water meters at Feeding Mains	53

Figure 3.13 Layout of Proposed Bulk Supply Line	55

Figure 3.14 Layout of Hydraulic Model from Water GEMS	57

Figure 3.15 Outlived Distribution Pipes to be Replaced	60

Figure 3.16 Layout of Distribution Lines for Unserved Areas	61

Figure 3.17: Conceptual design of Solar PV system installation at Sangjani WTP	70

Figure 5.1 Location of proposed project marked with COI	87

Figure 5.2 Geographical Map of Rawalpindi-Islamabad	88

Figure 5.3 Land Use around the project site	89

Figure 5.4 Seismic Map of Pakistan	94

Figure 5.5 Mean Yearly Temperature	96

Figure 5.6 Mean Monthly Rainfall	97

Figure 5.7 Mean Yearly Humidity (2018-2022)	98

Figure 5.8 Mean Monthly Wind Speed	99

Figure 5.9 Wind Rose of Rawalpindi	100

Figure 5.10 Flood prone areas of Rawalpindi	102

Figure 5.11 Location of Forests near the project site	104

Figure 5.12: Photographs depicting Socio-economic Conditions of the project area	109

Figure 5.13 Demographic features of Rawalpindi	111

Figure 5.14: Environmental Sensitive Receptors	114

Figure 5.15: Detail of Environmental Sensitive Receptors	115

Figure 5.16 Locations of Environmental Monitoring Points for Water Supply Works	122

Figure 5.17: Environmental Monitoring Photo Log	130

Figure 5.18: Monitoring point for ambient Air Quality and Noise for Solar installation at Sangjani Treatment Plant	132

Figure 8.1 Photographs of Stakeholder Consultation	253

Figure 8.2 Photographs of Consultations with Institutional Stakeholders	259

Figure 9.1 Process Flow Diagram of Grievance Redressal Mechanism	271

List of Tables

Table ES.1: Screening of possible Impacts during Design/Pre- Construction phase for ‘’Water Supply Component’’	vii

Table ES.2: Screening of Possible Impacts during Construction Phase for ‘’Water Supply Component’’	viii

Table ES.3: Screening of Potential Impacts during Operation Phase for ‘’Water Supply Component’’	x

Table ES.4: Screening of possible Impacts during Design/Pre- Construction phase for ‘Solar PV Component’	x

Table ES.5: Screening of Possible Impacts during Construction Phase for ‘Solar PV Component’	xi

Table ES.6: Screening of Potential Impacts during Operation Phase for ‘Solar PV Component’	xii

Table 1.1: Project Original and revised scope	1

Table 1.2: Detail of water demand in KSS	4

Table 1.3: Water deficit for KSS	5

Table 1.4: Executing Agency Contact Details	9

Table 1.5: IEE Team	11

Table 2.1: Main Strategies/Policies Related to Environment and Relevance to the Project	14

Table 2.2: Main Legislation/Acts Related to Environment and Their Relevance to the Project	18

Table 2.4: IFC Work Environment Noise limits1.2	30

Table 2.5: Comparison of International and local Air Quality Standards*1.3	30

Table 2.6: Comparison of International and Local Noise Standards1.4	31

Table 2.7: Comparison of International and Local Water Quality Standards1.5	31

Table 3.1: Water Storage Capacity of KSS	36

Table 3.2: Detail of DMAs with Domestic/Commercial connections	51

Table 3.3: Distribution Pipes to be Replaced	58

Table 3.4: Distribution Pipes for Unserved Areas	58

Table 3.5: Pumping Requirement for KSS	62

Table 3.6: Climatic Parameters for the project	63

Table 3.7: Design Phase for Distribution System	63

Table 3.8: Total Supply to each water works in Phase-1	64

Table 3.9:Major Machinery and Vehicles	66

Table 3.10: Construction Material and Source of Raw Material	72

Table 3.11: Cost of the Project	73

Table 4.1: Comparison of Alternatives	78

Table 5.1: Yearly Mean Temperature (2018-2022)	95

Table 5.2: Mean Monthly Rainfall Record	96

Table 5.3: Mean Yearly Relative Humidity (2018-2022)	97

Table 5.4: Monthly Wind Speed Record	98

Table 5.5: Forests located near the project area	103

Table 5.6: Names of Trees/shrubs & Herbs of the Region	105

Table 5.7: List of indigenous Mammals	106

Table 5.8: List of Reptiles observed in Project Area	106

Table 5.9: List of Amphibians observed in Project Area	107

Table 5.10: Birds Found in Study Area	107

Table 5.11: Administrative subdivision of Rawalpindi	108

Table 5.12: Demography of Rawalpindi (2017 Census)	109

Table 5.13: Detail of Water Quality Sampling Location	123

Table 5.14: Results of Water Quality Analysis	123

Table 5.15: Detail of Air Quality Sampling Locations	127

Table 5.16: Ambient Air Quality Results	127

Table 5.17: Detail of location for Noise Levels Monitoring	129

Table 5.18: Results of Noise level Monitoring	129

Table 5.19: Ambient Air Quality Results at Sangjani Treatment Plant	133

Table 5.20: Results of Noise level Monitoring	133

Table 5.21: Results of Water Quality Analysis	134

Table 6.1: Likelihood Scale	118

Table 6.2: Consequence Scale	119

Table 6.3: Risk Score	119

Table 6.4: Screening of possible Impacts during Design/Pre- Construction phase for Water Supply Component	121

Table 6.5: Screening of Possible Impacts during Construction Phase for Water Supply Component	126

Table 6.6: Noise Produced by Construction Equipment	133

Table 6.7: Estimated Wastewater Generated by Workers in Construction Camps	136

Table 6.8: Screening of Potential Impacts during Operation Phase for Water Supply Component	150

Table 6.9: Screening of possible Impacts during Design/Pre- Construction phase for ‘Solar PV Component’	153

Table 6.10: Screening of Possible Impacts during Construction Phase for ‘Solar PV Component’	157

Table 6.11: Screening of Possible Impacts during Operation Phase for ‘Solar PV Component’	171

Table 7.1 Environmental Management Plan for Water Supply Component	182

Table 7.2: Pre-Construction’ Environmental Monitoring Plan for Baseline Development	200

Table 7.3: Construction Phase Monitoring Requirements	201

Table 7.4: Operation Phase Environmental Monitoring Plan	202

Table 7.5: Capacity Development and Training Programme	205

Table 7.6: Annual Cost Estimates for ‘Pre-Construction Phase’ Environmental Monitoring	206

Table 7.7: Annual Cost Estimates for ‘Construction Phase’ Environmental Monitoring	206

Table 7.8: Annual Cost Estimates for ‘Operation Phase’ Environmental Monitoring	207

Table 7.9: Estimated Costs for EMP Implementation	207

Table 7.10: Cost of Capacity Development and Training Programme for Project Contractor(s)	209

Table 7.11: Environmental Management Plan for Solar PV Installation	209

Table 7.12: Cost Estimates for ‘Pre-Construction Phase’ – Solar	233

Table 7.13: Cost Estimates for ‘Construction Phase’ Environmental Monitoring - Solar	233

Table 7.14: Estimated Costs for EMP Implementation - Solar	234

Table 7.15: Cost of Capacity Development and Training Programme for Project Contractor(s) - Solar	234

Table 8.1: Consultation with Project Affected Peoples	240

Table 8.2: Response of Stakeholder during FGDs	247

Table 8.3: Finding of Consultation with Government Stakeholders	256

Table 8.4: Details of consultations conducted for solar installations	261

Table 8.5: Details of public hearing conducted by Pak EPA	265

## EXECUTIVE SUMMARY

### Project Overview

The proposed sub-project "Augmentation of Surface Water Supply for Khayaban-e-Sir Syed from Khanpur Dam and Creation of DMAs and Solarization component" is a part of the Developing Resilient Environments and Advancing Municipal Services (DREAMS) project which Asian Development Bank (ADB) is financing in the amount of USD 180 million.

The DREAMS project will support inclusive, resilient, and sustainable urban infrastructure and services in two rapidly growing cities in Punjab province---Rawalpindi and Bahawalpur. ---by (i) expanding and modernizing urban water supply systems in Rawalpindi and solid waste collection, treatment, and disposal in Bahawalpur; and (ii) strengthening institutional capacity in service delivery to improve the quality, coverage, efficiency, and reliability of these services, as well as the financial sustainability of the utilities. The project will benefit up to 1.5 million urban dwellers in Punjab, the most populous province of the country.

This sub-project in Rawalpindi deals with the enhancement and sustainable supply of water to the people of Khayaban-e-Sir Syed (KSS), by increasing water supply from surface water source and reducing the load on groundwater along with a solar photovoltaic (PV) component aimed at producing renewable energy on a sustainable basis for use at the project site as well as for exporting the surplus energy to the grid.

The project scope covers the laying of Conductance Main from I-10 Service Road, Islamabad to KSS, creation of District Metering Area (DMA) to manage water flow in different area, replacement of old leaking pipelines and installation of domestic water meters to measure the water use for billing. The proposed project will start from Sector I-10 of Islamabad and will connect different water works of KSS, Rawalpindi with Conductance Main to maintain the pressures till the tail. The project location plan is shown in the below figure. The city of Rawalpindi mainly depends on rainfall for both surface and groundwater resources (as the perched aquifers deplete rapidly due to lesser rainfall). Water supply being the foremost requirement, is currently the most affected municipal service in the city. Citizens are facing a limited supply of water due to depleting water resources and an aging water supply network.

The solar PV component of this project consists of development of a grid connected system of 8.4 MW capacity, which will be installed over 35 acres of land at the site of the Sangjani water treatment plant and the power generated will be used for meeting the project needs any the excess power will be put onto the grid. The site for installation of the solar PV panels and infrastructure is shown as Figure ES.2 below.

KSS is in the WEST-I Zone of Rawalpindi with a 1.48 sq. km area. KSS comprises of three Union Councils (UCs), named as UC-10 (Khayaban-e-Sir Syed (N)), UC-11 (Khayaban-e-Sir Syed (S)), and UC-12 (Dhok Najjo). The KSS (study area) is bounded by three sides, from the north, the IJP road is passing, while on the eastern and western sides, Nallah Lai and Nikki Nullah split the project boundary from the rest of the area. The project location is provided as Figure ES.1 below.

Following are the main components of the proposed activity:

Laying of new Conductance Main from Conductance Main of Tomar Reservoir;

Creation of District Metered Area (DMAs);

Replacement of outlived distribution pipelines;

Installation of pumps at Water Works and Sangjani Water Treatment Plant;

Installation of water meters at household level.

Figure ES.1: Location of the Project

Figure ES.2: Proposed solar PV installation at Sangjani WTP site

### Project Need

Calculations have been carried out to ascertain the current and future water demand of water and the sustainable sources of water supply for KSS. Based on these calculations, new conductance main of High-density polyethylene (HDPE) material with 710mm diameter will be laid down to fulfil the water demand of KSS up to next 30 years (year 2050). Moreover, harvesting of surface water will reduce the pressure on already depleted groundwater resources.

### Project Benefits

The main benefits of this project are as follows:

Shifting onto surface water use to secure ground water resource. Surface water reservoir has sufficient water to cater to the requirements;

Installation of DMA to control and regulate the flow of water to selected areas;

Efficient use of already available OHRs and GSTs;

Control on use of water use by installing water meters;

Replacement of old distribution lines to control leakages and contamination of drinking water;

Power generation from renewable energy using solar PV panels and required infrastructure to be installed at the Sanjrani water treatment plant site to meet the project’s energy needs and for the excess power to be exported onto the grid.

### Policy, Legal and Administrative Framework

The project needs to comply with all the applicable environmental policies, laws, guidelines, acts and legislations of Government of Pakistan and the provincial government as well as international environmental guidelines.

### Environmental Safeguards Category of the Project

According to ADB’s Safeguard Policy Statement (SPS) 2009, a Rapid Environmental Assessment (REA) Checklist was prepared for the proposed project. Based on the REA findings, the project would have some potential adverse environmental impacts, but of lesser degree and/or significance than those of category A projects. These impacts are site-specific, irreversible and mitigation measures can be designed more readily than for category A projects. Hence, the project is categorized as ‘B’ for Environment as per ADB SPS 2009 and an Initial Environmental Examination (IEE) has been conducted.

This is the updated version of the original draft IEE report which was disclosed on the ADB web page in June 2023 (web page link: https://www.adb.org/projects/documents/pak-55236-001-iee-0). This updated version contains details related to the solarization component of the project.

Further as per legal requirement under Section 12 of Punjab Environmental Protection Act, 1997 (Amendment 2012), this project requires an environmental approval from the Punjab EPA Punjab. As a part of the Conductance Main is passing through the federal territory of Islamabad, this project also requires the environmental approval in the form of the No Objection Certificate (NOC) from Pak EPA under Section 12 of Pakistan Environmental Protection Act, 1997. As per Schedule-II (clause F) of Pakistan Environmental Protection Agency (Review of IEE and EIA) Regulations 2000, the proposed project falls in category of projects requiring an Environmental Impact Assessment (EIA) study to be prepared on the basis of the project cost.

### Project proponent

The Program Management Unit (PMU), Punjab Intermediate Cities Improvement Investment Program (PICIIP) is implementation unit/ setup and Local Government and Community Development Department (LG&CD), Government of Punjab (GoPb) is the Executing Agency (EA) of the project.

### Project Alternatives

Four options have been considered for the proposed project. These options are discussed on the basis of technical, environmental, social and economic parameters. Following are the four alternative options:

Alternative I: 		‘No Project’ Option

Alternative – II: 		Capacity Enhancement of Existing Conductance Main

Alternative III: 		Laying of new Conductance Main

Alternative – IV: 	Construction of New OHRs and GSTs

Option III was selected as final option for proposed project because it involves minimum disturbance to environment and it is also cost effective.

The different potential options relating to the proposed solar PV development were assessed. The ‘No project’ scenario is not a viable option since Pakistan is facing an energy deficit and needs to urgently switch from the cost intensive and environmentally unfriendly diesel-based power generation and move towards developing renewable based cheap and reliable energy on a long-term basis.

For the purpose of this proposed solar PV installation, bifacial monocrystalline modules will be utilized in the solar plant. For end of life/failure or at decommissioning, provision in tender agreements with suppliers/vendors will ensure proper and safe collection, recycling and/or disposal, as needed.

### Description of Environment

Considering the potential impacts of the Proposed Project, existing baseline environmental conditions of the proposed project’s Corridor of Impact (CoI) has to be used as a benchmark for comparison of the physical, ecological and socio-economic conditions before and after construction phases of the project. This baseline will also provide the datum for assessing the impacts and suggesting the mitigation measures, which will be implemented effectively at various phases of the proposed project activities.

The data has been collected from the primary and secondary sources. For primary data acquisition, the environment and social team conducted the field visit during the month of December, 2022. The secondary data was collected from published sources/reports and relevant departments, which were also verified through visual observations during detailed surveys. Data has been collected for following elements of environment:

Physical Environment

Biological Environment

Socioeconomic environment

### Environmental Testing & monitoring

Environmental parameters such as air quality, noise level, and drinking water quality were analyzed to assess the baseline conditions of project area. Detailed results of the monitoring are discussed in chapter 5. The test results show that noise levels are exceeding the applicable Punjab / National Environmental Quality Standards (PEQS/NEQS) for day and night at one location (proposed location for Water Tank No. 4). The local standard (Punjab Environmental Quality Standards) is more stringent than WHO for noise levels in commercial and residential zone. The monitoring results of ambient air quality show that concentration of PM10 and PM2.5 is exceeding the prescribed limit values for ambient air as compared with WHO guidelines, while they are within the permissible limits of PEQS/NEQS. The standards for WHO for PM10 and PM2.5 for ambient air are more stringent than PEQS/NEQS. Other monitoring parameters for water, air and noise are within the prescribed limits of NEQS, PEQS and WHO. These monitoring tests were performed by experienced staff from an EPA certified environmental laboratory.

### Sensitive Receptors

Sensitive receptors have been identified which fall under Corridor of Impact (CoI) of 100 meters along the alignment of proposed project. These receptors include places of worship, educational institutes, health facilities and area of spiritual value. Mapping of these receptors has been carried out to assess the impacts and proposed mitigation accordingly.

### Potential Environmental Impacts and Mitigation Measures

Beneficial as well as the potentially adverse environmental and social impacts of the project are evaluated for design/pre-construction, construction and operation phases of the proposed project on the physical, ecological and socio-economic domains of the environment. The appropriate mitigation and remedial measures are proposed accordingly. The impact matrices have been developed for each phase of the project. As given in each matrix, more environmental impacts are envisaged during the construction phase of the project, which will be localized and short term in nature. There will be negligible adverse impacts during operational phase of the proposed project.

### Project Impact Evaluation Matrix

Project impact evaluation matrix have been developed to evaluate the potential impacts of the proposed project during planning, construction and operation phases of the project. Qualitative description of each aspect and the affected environment, in both Right of Way (RoW) and the project’s corridor of impact (CoI) has been prepared and the matrix for design/pre-construction, construction and operational phases of the proposed project for the water supply component are provided in Tables ES.1 to ES.3 while for the solar PV component are provided in Table ES.4 to ES.6 below.

Table ES.1: Screening of possible Impacts during Design/Pre- Construction phase for ‘’Water Supply Component’’

| S/No. | Potential Impacts | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Improper designing of distribution networks including transmission mains (Replacement & New Distribution Lines) | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 2 | Lack of integration of IEE/EMP requirements into Construction bid documents | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 3 | Relocation of existing utilities | 3 / Likely | 2 / Moderate | Medium | Low |
| 4 | Identification of Locations for Labor Camps and ancillary facilities | 3 / Likely | 2 / Moderate | 6 / Medium | Medium |
| 5 | Traffic Issues/ Traffic Management Plan | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 6 | Land acquisition and resettlement impacts | 2 / Unlikely | 2 / Moderate | Low | Low |
| 7 | Topographic Impacts due to Excavations | 2 / Unlikely | 2 / Moderate | Low | Low |
| 8 | Seismic Impacts | 2 / Unlikely | 2 / Moderate | Low | Low |
| 9 | Impact of Water Usage | 2 / Unlikely | 2 / Moderate | Low | Low |

Table ES.2: Screening of Possible Impacts during Construction Phase for ‘’Water Supply Component’’

| Sr. No. | Potential Impacts | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Impact due to Excavation for laying of Conductance Main, replacement of pipelines and Installation of Control Valves | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 2 | Site Accessibility | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 3 | Traffic Issues | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 4 | Resource Conservation | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 5 | Occupational Health & Safety | 3/ Likely | 3/ Moderate | 9/ Medium | Low |
| 6 | Construction Camps/ Camp Site | 5/ Likely | 2/ Moderate | 10/ Medium | Low |
| 7 | Wastewater Generation | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 8 | Solid Waste Generation | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 9 | Disposal of Spoil/ Demolition Waste | 4/ Likely | 2/ Moderate | 8/ Medium | Low |
| 10 | Social and Cultural Conflicts | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 11 | Community Health & Safety | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 12 | Communicable diseases incl. COVID-19 | 2/ Likely | 3/ Moderate | 6/ Medium | Low |
| 13 | Flora | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 14 | Fauna | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 15 | Degradation of Air Quality | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 16 | Noise/Vibration | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 17 | GHG Emission | 3/ Likely | 2/ Moderate | 6/ Medium | No Residual Impact |
| 18 | Site Restorations | 2/ Likely | 10/ Moderate | 10/ Medium | Low |

Table ES.3: Screening of Potential Impacts during Operation Phase for ‘’Water Supply Component’’

| S/No. | Potential Issues | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Water system leaks and water discharges during flushing | 2/ Unlikely | 3/ Major | 6/ Medium | Low |
| 2 | Improved drinking water availability | Positive impacts expected |  |  | Positive residual impact |
| 3 | Improvements in Public Health | Positive impacts expected |  |  | Long term positive residual impact |

Table ES.4: Screening of possible Impacts during Design/Pre- Construction phase for ‘Solar PV Component’

| S/No. | Potential Impacts | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Relocation of existing utilities | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 2 | Traffic Issues/ Traffic Management Plan | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 3 | Lack of integration of IEE/EMP requirements into Construction bid documents | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 4 | Seismic impacts | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 5 | Site and Route selection | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 6 | Land requirements | 2 / Unlikely | 1 / Minor | 2 / Low | Low |

Table ES.5: Screening of Possible Impacts during Construction Phase for ‘Solar PV Component’

| Sr. No. | Potential Impacts | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Traffic Issues | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 2 | Air Quality | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 3 | Noise and Vibration | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 4 | Occupational Health & Safety | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 5 | Community Health and Safety | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 6 | Waste generation | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 7 | Vegetation impacts | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 8 | Fauna impacts | 3/ Likely | 3/ Moderate | 9/ Medium | Low |
| 9 | Biodiversity | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 10 | Interruption to existing facilities | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 11 | Physical landscape | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 12 | Topography impacts | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 13 | Groundwater quality | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 14 | Soil quality | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 15 | Local Economic development | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 16 | Physical Cultural Resources | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 17 | Other Risks | 3/ Likely | 2/ Moderate | 6/ Medium | Low |

Table ES.6: Screening of Potential Impacts during Operation Phase for ‘Solar PV Component’

| S/No. | Potential Issues | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Workforce Organization and Orientation | 2/ Unlikely | 3/ Major | 6/ Medium | Low |
| 2 | Water usage for panel cleaning leading to water shortage | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 3 | Landscape impact | 2/ Unlikely | 3/ Moderate | 6/ Medium | Low |
| 4 | Air quality, noise and vibration | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 5 | Waste generation | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 6 | Ecological impacts | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 7 | Occupational health and safety | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 8 | Green energy-based climate friendly form of power generation | Positive impacts expected |  |  | Positive residual impact |

Critical Risk Level

Significant Risk Level

Medium Risk Level

Low Risk Level

Positive Impacts

### Environmental Management Plan

The IEE has evaluated potential impacts that are likely to arise during proposed project in detail, both negative and positive impacts at each stage of the project and in order to minimize the adverse impacts, mitigation measures has been recommended in the Environmental Management Plan (EMP).

### Institutional Arrangements

Roles and responsibilities of different stakeholders, including PMU, Federal and Punjab EPA, Construction Supervision Consultant (CSC), Contractor and RWASA to implement and supervise the recommended mitigation measures under EMP and environmental monitoring plan has been defined in the IEE report.

### Environment Budget

An estimated cost for implementation of mitigation measures prescribed in the Environmental Management Plan (EMP) has been calculated and given in Chapter 7 of the IEE report. The cost of EMP is PKR 15,960,000 for the water supply component and PKR 1,897,500 for the solar component. The cost includes the hiring of staff, arrangements for water sprinkling, provision of PPEs to workers, tree plantation, the arrangement of first aid kits at the site and waste management etc.

### Public Consultation and Information Disclosure

The Public Consultations (PC), with project Affected People (AP) were carried out along with other stakeholders, including general public, NGOs and relevant government departments. The data of Focus Group Discussions (FGDs) and individual discussions was recorded. The disclosure of relevant information was made according to requirements of ADB SPS 2009, PMU and Punjab EPA. A total of 9 FGDs were conducted for the proposed project, including the consultative session with relevant government departments. Further, public consultations were also carried out with 109 participants, including 82 (75.2%) men and 27 (24.8%) women. The overall response of PC was positive and people showed their consent in favor of the project with a few suggestions which have been incorporated in the EMP.

Additional consultations were conducted during the public hearing, conducted for this sub-project on 13th of June’24 in Rawalpindi and attended by sixty-eight (68) individuals from both the public and private sectors, consisting of local community members, students, academic faculty, owners of local businesses as well as other stakeholders from the project area. They expressed queries and concerns related to the source of the water supply and its geographical coverage of supply as well as other issues such as expected vibration levels during construction works and potential impacts from high dust and noise levels during the works. Responses were provided to all their comments and queries and these are provided along with photographic evidence in Annexure XV of this IEE study.

After completion/revision and approval from the ADB, Punjab EPA and Pak EPA, the IEE study will be disclosed to all the stakeholders as part of public consultation process. The summary of the IEE report will be made available to the stakeholders at ADB website and official website of PMU PICIIP LG&CD, GoPb. The executive summary of the IEE report will also be translated into local language (Urdu) and will be disclosed at various places that are publicly accessible.

In regards to the solar PV installation for the project, a consultation session was conducted with city stakeholders on Environmental Assessment, under the chairmanship of Deputy Director (Development) Rawalpindi, dated 5th August 2023 at 2:00 pm in the Committee Room of Deputy Commissioner, Rawalpindi, for the project “Solarization for Sustainable & Resilient Water Supply System in RAWALPINDI “in which the consultant presented their observation as part of preparing the environmental assessment report.

The Project Support Officer, City Implementation Unit (CIU) Rawalpindi gave a brief introduction of the Project and priority projects of Rawalpindi for the water sector under DREAMS – I then Engineering Design and Construction Management (EDCM) Consultant’s Environment expert presented the Environmental Assessment on the project.

### Grievance Redressal Mechanism

The Grievance Redressal Mechanism (GRM) has been developed under ADB SPS 2009 to deal with complaints at any stage of the proposed project. The GRM will help to receive and resolve the concerns/complaints of Displaced/Affected Persons regarding the project’s social and environment performance. The measures have been identified to mitigate any potential environmental and social impacts due to implementation of proposed project and the mechanism for information disclosures has been provided in the GRM.

### Conclusion and Recommendations

The proposed project is of high significance considering the urgent need for improving sustainable water supply system of KSS Rawalpindi. An action plan, for different stakeholders, with roles and responsibilities has been developed to make project sustainable. Based on the findings of the IEE report, the proposed project is unlikely to cause any significant, irreversible or unprecedented environmental impacts. The potential impacts are localized, temporary in nature and can be addressed through proven mitigation measures. Hence, the classification of the project as Category B, as per ADB SPS 2009 is confirmed. No further study or assessment is required at this stage. The requirements of ADB SPS, PMU and Federal & Punjab EPA shall be communicated to the Contractor(s) by PMU to control any adverse impacts on the environment. The contractor will manage the site-specific impacts by developing and implementing the Site-Specific Environmental Management Plan (SSEMP). template of the same has been provided as an annexure of the IEE report.

## INTRODUCTION

### Overview of the project

The proposed sub-project "Augmentation of Surface Water Supply for Khayaban-e-Sir Syed from Khanpur Dam and Creation of DMAs and Solarization component" is a part of the Developing Resilient Environments and Advancing Municipal Services (DREAMS) project which Asian Development Bank (ADB) is financing in the amount of USD 180 million.

The DREAMS project will support inclusive, resilient, and sustainable urban infrastructure and services in two rapidly growing cities in Punjab province---Rawalpindi and Bahawalpur. ---by (i) expanding and modernizing urban water supply systems in Rawalpindi and solid waste collection, treatment, and disposal in Bahawalpur; and (ii) strengthening institutional capacity in service delivery to improve the quality, coverage, efficiency, and reliability of these services, as well as the financial sustainability of the utilities. The project will benefit up to 1.5 million urban dwellers in Punjab, the most populous province of the country.

This sub-project in Rawalpindi deals with the enhancement and sustainable supply of water to the people of Khyaban-e-Sir Syed (KSS), by increasing water supply from surface water source and reducing the load on groundwater along with a solar photovoltaic (PV) component aimed at producing renewable energy on a sustainable basis for use at the project site as well as for exporting the surplus energy to the grid.

The project scope covers the laying of Conductance Main from I-10 Service Road, Islamabad to Khyaban-e-Sir Syed, Rawalpindi, creation of District Metering Area (DMA) to manage water flow in different area, replacement of old leaking pipelines and installation of domestic water meters to measure the water use for billing. The proposed project will start from Sector I-10 of Islamabad and will connect different water works of KSS, Rawalpindi with Conductance Main to keep the pressures till the tail.

The solar PV component of this project consists of development of a grid connected system of 8.4 MW capacity, which will be installed over 35 acres of land at the site of the Sangjani water treatment plant and the power generated will be used for meeting the project needs any the excess power will be put onto the grid. The site for installation of the solar PV panels and infrastructure is shown as Figure 1.3 below.

The city of Rawalpindi mainly depends on rainfall for both surface and groundwater resources. Water supply being the foremost utility for the citizen is currently the most affected municipal service in the city. Citizens are facing a limited supply of water due to depleting water resources and an aging water supply network. Water availability is a major issue and it will further be aggravated in the coming years.

The Rawalpindi Water and Sanitation Agency (RWASA) is responsible to provide water supply and sewerage services to 1.6 million people living in 66 Union Councils (UCs) of Rawalpindi city. RWASA has divided this jurisdictional area into six water supply zones, namely EAST-1, EAST-2, WEST-1, WEST-2, PP-12 and PP-13. As learned from the RWASA, the Non-Revenue Water (NRW) constitutes almost 30% of the city’s water supply, mainly because of the outlived and rusty water supply infrastructure and due to the absence of net water metering.

KSS is located in the WEST-I Zone of Rawalpindi with a 1.48 sq. km area. There are a total of three Union Councils (UCs) in this area, named as UC-10 (Khayaban-e-Sir Syed (N)), UC-11 (Khayaban-e-Sir Syed (S)) and UC-12 (Dhok Najjo). The study area of KSS is bounded by three sides, from the north, the IJP road is passing, while on the east and west sides, Nallah Lai and Nikki Nullah split the project boundary from the rest of the area of Rawalpindi. Due to the unique area boundary, KSS is considered as the best option for creation of District Metered Areas (DMAs). This project is taken as a pilot project for creation of DMAs. If successful, the same model will be implemented for rest of the areas of Rawalpindi in coming years. The location of proposed project is shown as Figure 1.1 below.

Figure 1.1: Location of the project

The proposed project "has been selected under the proposed DREAMS, formerly known as Punjab Intermediate Cities Improvement Investment Program Phase-II (PICIIP-II), to be financed by the Asian Development Bank (ADB).

This project (DREAMS/ PICIIP-2) represents the second phase of the PICIIP-1, which will continue the momentum of the first phase and ensure development continuity by providing similar urban infrastructure investments and capacity support to additional seven cities in Punjab Province. After detailed deliberations, the top priority projects, to be executed for Rawalpindi city were finalized and named as DREAMS-1.

This is the updated version of the original draft IEE report which was disclosed on the ADB web page in June 2023 (web page link: https://www.adb.org/projects/documents/pak-55236-001-iee-0). This updated version contains details related to the solarization component of the project.

The project original scope along with the additional scope to incorporate the solar PV solarization is provided in the Table 1.1 below.

- Table 1.1: Project Original and revised scope

| Project Original Scope | Project Additional Scope |
| --- | --- |
| Laying of New Conductance Main from Conductance Main of Tomar Reservoir. Creation of District Metered Area (DMAs). Replacement of outlived distribution pipelines. Installation of Pumps at Water Works and Sangjani Water Treatment Plant. Installation of water meters at household level. | A grid connected solar photovoltaic (PV) system of 8.4 MW capacity will be installed over 35 acres of land at the site of the Sangjani water treatment plant and the power generated will be used for meeting the project needs any the excess power will be put onto the grid. |

In addition to the above components, details of sub-projects under DREAMS-1 project are provided below and the proposed solar PV installation to be conducted is shown in Figure 1.3 below.

Figure 1.2: Details of sub-projects under DREAMS-1 Project

Figure 1.3: Proposed solar PV installation at Sangjani WTP site

### Existing Water Supply system of KSS

Currently, there are two main sources of water for residents of KSS. One is from the Khanpur dam and the other is from the existing tube wells, installed by RWASA. The Khanpur dam supplies water to Rawalpindi and Islamabad for domestic consumption and irrigation water to the agricultural lands and industrial areas surrounding the twin cities. The tube wells are discharging a total of 7,200 m3/d of water. A total of 37 tube wells are installed in KSS and out of these, only 31 are functional and 06 are not functional. All the tube wells are of submersible type. Due to the limited capacity of the tube wells, each tube well has a very limited-service area, spread over 3 to 6 streets only. On average, the daily running time for each tube well is 12 -14 hours to meet the water requirements.

### Project Need

According to census reports of 2017, the population of KSS was 64,727 persons and estimated current population is 72,303 persons for year 2022. The future population for 2050 is projected as 114,727 persons. Present average water demand at 182 lpcd i.e., 40 gpcd for 2022 is 12,629 m3/d (2.89 MGD) while future average water demand is calculated as 25,146 m3/d (4.59 MGD). Details of population and water demand are given in the Table 1.2 below.

Table 1.2: Detail of water demand in KSS

| Year | Population | Unit Demand (LPCD) | Av. Water Demand (m3/day) | Av. Water Demand (MGD) | Max Water Demand (m3/day) | Max Water Demand (MGD) |
| --- | --- | --- | --- | --- | --- | --- |
| 2022 | 72303 | 182 | 13159 | 2.89 | 19739 | 4.34 |
| 2025 | 76849 |  | 13987 | 3.07 | 20980 | 4.61 |
| 2030 | 84424 |  | 15365 | 3.38 | 23048 | 5.07 |
| 2040 | 99575 |  | 18123 | 3.98 | 27184 | 5.97 |
| 2050 | 114727 |  | 20880 | 4.59 | 31320 | 6.88 |

It is important to mention here that the groundwater of Rawalpindi is depleting continuously due to over pumping/extraction. Therefore, boring of new tube wells and repairing of abounded tube wells to meet the existing and increasing future demand of water cannot be considered and augmentation of surface water source to meet current and future demand of water is under consideration to make sustainable use of water resources.

The water deficit is calculated for the year 2022 and project design life i.e., year 2050. The increase in population is increasing the demand of water and this supply and demand gap is increasing continuously. The water deficit is calculated for the years 2022, 2025, 2030, 2040 and 2050 and is shown in the Table 1.3 below.

Table 1.3: Water deficit for KSS

| Year | Av. Water Demand (m3/day) | Av. Water Demand (MGD) | Production from Tube Wells (m3/day) | Supply from Khanpur Source (m3/day) | Water Deficit (m3/day) | Water Deficit (MGD) |
| --- | --- | --- | --- | --- | --- | --- |
| 2022 | 13159 | 2.89 | 6546 | 4546 | 2067 | 0.45 |
| 2025 | 13987 | 3.07 |  |  | 2895 | 0.63 |
| 2030 | 15365 | 3.38 |  |  | 4273 | 0.94 |
| 2040 | 18123 | 3.98 |  |  | 7031 | 1.54 |
| 2050 | 20880 | 4.59 |  |  | 9788 | 2.15 |

The intention of RWASA is to reduce the burden on already depleted ground water resource by improving source of surface water supply for KSS. Since the groundwater is depleting continuously, therefore, boring of new tube wells to meet the additional water requirements cannot be considered. However, augmentation of the surface water source to meet future water requirements can be considered as sustainable option of water supply.

As mentioned earlier, most of the existing distribution system is outlived and rusty. There are leakages in these pipelines and need to be replaced. It is suggested that the existing AC/GI water distribution pipelines to be replaced with HDPE pipes with a PN-8 pressure rating. Moreover, some of the pipes which are of smaller diameters also need to be replaced with pipes of larger diameters so that desired quantity of water can be supplied with desired pressures. The figure shows the detail of outlived distribution pipelines to be replaced under proposed project.

The project also involves installation of Water Meters to measure the use of household water for billing. This will bring reduction in water use by reducing wastage.

Figure 1.4: Detail of outlived distribution pipelines to be replaced

### Project Benefits

Execution of the proposed project would be beneficial for the citizens of Khayaban-e-Sir Syed, Rawalpindi. Following benefits of the project are anticipated:

- Enhanced water availability and consistency due to dependence on surface sources;

Improved water quality due to replacement of old leaking distribution system;

Decreased electricity consumption due to reduction in dependence on tubewells;

Improved billing on actual consumption basis due to metering;

Water conservation due to domestic and commercial metering;

Improved pressure in the system;

Enhanced management by the responsible agency.

Power generation from renewable energy using solar PV panels and required infrastructure to be installed at the Sanjrani WTP site to meet the project’s energy needs and for the excess power to be exported onto the grid.

### Environmental Safeguards Category of the Project

According to ADB’s SPS 2009, an Rapid Environmental Assessment (REA) Checklist was prepared for the proposed project. Based on the REA findings, the proposed project would have some potential adverse environmental impacts, but of lesser degree and/or significance than those of category A projects. These impacts are site-specific, irreversible, and mitigation measures can be designed more readily than for category A projects. Hence, the project is categorizing as ‘B’ for Environment as per ADB SPS 2009 and an Initial Environmental Examination (IEE) has been prepared. The REA report has been attached as Annexure-I.

Further as per legal requirement under Section 12 of Punjab Environmental Protection Act, 1997 (Amendment 2012) this project require Environmental Approval from Punjab EPA. As a part of Conductance Main is passing through the Sector I-10 of Islamabad, this project also requires Environmental Approval from Pak EPA under Section 12 of Pakistan Environmental Protection Act, 1997. As per clause F, Schedule-II of Pakistan Environmental Protection Agency (Review of IEE and EIA) Regulations 2022 / 2000 of the acts ibid, the proposed project falls in category of projects requiring Environmental Impact Assessment (EIA) on the basis of project cost.

### Scope and Objectives of the IEE

The scope of IEE will include environmental assessment of project activities including design, construction and operation phases. Following are the objectives of the IEE study:

Assess the existing environmental conditions of project area, including the identification of environmental sensitive receptors and develop a baseline of its prevalent environmental and socioeconomic conditions;

Identify and investigate all impacts of the proposed project during pre-construction/design, construction, operation phases, on the physical, biological and socioeconomic environment of the project area;

To proposed mitigation measures that would help execution agency (RWASA) in conducting the proposed project activities in an environmentally sustainable manner;

To uncover the planning and operational phase impacts up to microenvironment levels in which project is proposed to be sited; and

To develop an Environmental Management Plan (EMP) that would assist execution agency (RWASA) in the effective implementation of the recommendations of the IEE.

This report has been prepared based on the engineering designs, pre-feasibility of the project and estimates of water availability, supply and future demand in Rawalpindi.

### Methodology of IEE Study

The various steps undertaken in the preparation of the IEE are summarized below:

#### Understanding of the Proposed Operation

This involves collecting information from the ADB, PMU PICICP, CIU and RWASA, design team of Osmani & Company Limited (OCL) and technical team of Engineering Design and Construction Management (EDCM) on the proposed project activities and understanding the activities to identify potential impacts of implementing these designs.

#### Review of Legislation and Guidelines

National legislation, international agreements, environmental guidelines both of Punjab Environment Protection Authority (Punjab EPA) and ADB and best industry practices has been reviewed to set environmental standards that RWASA, being the implementation agency (IA), will adhere to all requirements during implementation of the project.

#### Secondary Data Collection

Available published and unpublished information pertaining to the background environment has been obtained and reviewed. All data sources have been carefully reviewed to collect the following information.

Physical environment – topography, geology, seismology, geomorphology, soils, surface and groundwater resources and climate;

Biological environment – habitat types, flora and fauna (particularly rare or endangered species), critical habitats, vegetation and communities within the area;

Physical cultural resources – sites, structures, groups of structures, and natural features and landscapes that have archaeological, paleontological, historical, architectural, religious, aesthetic or other cultural significance; and

Socio-economic environment – settlements, socio-economic conditions, infrastructure and land use.

#### Field Data Collection (Baseline Survey)

Field visits were undertaken consisting of preliminary scoping through survey and assessment activities to establish the potential impacts and categorization of activities and the completion of the REA Checklist was conducted. The key environmental sensitive receptors and stakeholders within the project area were identified.

Baseline surveys were conducted to by the multidisciplinary team of environmentalist, sociologists and data enumerators to identify and establish environmental and social impacts of the proposed project.

#### Public Consultation

Public consultations were carried out with all key stakeholders, particularly local communities residing in the project area, local businesses and government and local government bodies in line with ADB’s SPS 2009 guidelines.

In regards to the solar PV installation for the project, a consultation session was conducted with city stakeholders on Environmental Assessment, under the chairmanship of Deputy Director (Development) Rawalpindi, dated 5th August 2023 at 2:00 pm in the Committee Room of Deputy Commissioner, Rawalpindi, for the project “Solarization for Sustainable & Resilient Water Supply System in RAWALPINDI “in which the consultant presented their observation as part of preparing the environmental assessment report.

The Project Support Officer, City Implementation Unit (CIU) Rawalpindi gave a brief introduction of the Project and priority projects of Rawalpindi for the water sector under DREAMS – I then Engineering Design and Construction Management (EDCM) Consultant’s Environment expert presented the Environmental Assessment on the project.

#### Impact Identification and Assessment

Potential impacts arising from each phase of the proposed project has been identified and assessed by using a risk assessment matrix and field survey data, secondary data, expert opinion and examining previous similar projects in Pakistan. These include effects on physical, biological and socio-economic environment.

#### Recommendations for Mitigation Measures

Mitigation measures to minimize, eliminate or compensate the potential environmental impacts has been recommended. The mitigation measures have been recommended on the basis of past experiences, best industry practices, legislative requirements and professional judgment.

#### Development of Environmental Management Plan (EMP)

An Environmental Management Plan (EMP) has been developed for effective implementation of the recommended mitigation measures. The EMP has included controls to minimize the identified impacts and monitoring program to monitor effect of mitigation measures implemented and residual impacts, if any, during implementation. The EMP has identified roles and responsibilities of all concerned parties during the implementation of the project.

### Project Proponent

The LG&CD, GoPb is the executing agency through Program Management Unit (PMU) of PICIIP. RWASA will be the implementation agency of this project. Contact details of the Executing Agency (EA) are provided below.

Table 1.4: Executing Agency Contact Details

| Description | Executing Agency |
| --- | --- |
| Name of Project and Sub-component | Punjab Urban Development Projects, Loan No. 6029, Engineering Design and Construction Management (PHASE 1), Augmentation of Surface Water Supply for Khayaban-e-Sir Syed from Khanpur Dam and Creation of DMAs. |
| Name of EA | Program Management Unit (PMU), Punjab Intermediate Cities Improvement Investment Program (PICIIP), Local Government and Community Development Department Government of Punjab |
| Address | Plot 41, Block B1, Gulberg III, MM Alam Road, Lahore, Pakistan |
| Telephone | 042-99210013-4 / 042-99268484 |

### Structure of the Report

The IEE report contains eleven chapters as follows:

Introduction - “Introduction” describes introduction, location of project, objective and purpose of the IEE report

Policy, Legal Framework and Administrative Framework - “Environment Legislative Requirements and Framework” elucidates the current legal framework including national and funding agency’s guidelines which is applicable to the proposed project in the context of environment and sustainable development;

Description of the Project - Project Description furnishes an overall description of the project, including its background, features and key components, timeframe and cost;

Analysis of Project Alternatives – discussed the available option and selection of best option based on specific criteria;

Description of Environment - comprises a detailed narrative of the existing (baseline) conditions of the project area, with respect to its physical, biological and socio-economic environment;

Potential Environmental Impacts and Mitigation Measures - elaborates the likely impacts of the project on the physical, biological and socio-economic environment during the construction and operation stages and lays down the proposed measures to mitigate the adverse impacts of the project;

Environmental Management Plan and Institutional Requirements - provides the mechanism to be adopted for the implementation of measures and monitoring the environment during all stages;

Public Consultation, and Information Disclosure - provides summary of consultative sessions with the local community as well as with other stakeholders including local government officials for their opinions and suggestions on the project;

Grievance Redressal Mechanism – Mechanism for complaint management has been described in this chapter.

Conclusion and Recommendations – gives the conclusion of the impact assessment study and recommendations for the construction and operational stages;

References.

### IEE Team

This IEE study has been conducted by the team members provided in the table below.

Table 1.5: IEE Team

| Members | Status in Project |
| --- | --- |
| PMU Team |  |
| Mr. Waqas Afzal | Deputy Director (Social & Environmental Safeguard), Review of the report |
| Syed Osama Faheem Rizvi | Assistant Director M&E, PMU |
| Ms. Shahnaz Kousar Khan | Assistant Director (Monitoring & Evaluation), PRF |
| EDCM Team |  |
| Mr. Kevin Holroyd | Team Leader –EDCM |
| Mr. Farooq Khatak | Deputy Team Leader –EDCM |
| Mr. Muhammad Mujahid | Project In charge -Environment Component |
| Dr. Nadeem | Ecological Expert |
| Ms. Rabia Rafiq | Environmentalist (IEE / EIA specialist) |
| Mr. Babar Ali | Environmentalist (IEE / EIA specialist) |
| Mr. Sami Ullah | Social Expert (Env. Team) |
| Mis. Komal Aroosh | Jr. Social Expert (Female Enumerator) |
| Ms. Shagufta Tahir | Female Enumerator |
| Mr. Asad Yaqoob | GIS Mapping Expert |

### Further Additions & Updating of IEE Study

This version of the report will be further updated once the detailed design is completed and location of exact alignment of works are finalized in the coming months. These revisions shall be incorporated into any subsequent updated versions of this IEE report. The updated IEE study will be submitted for ADB review and clearance and posted at the website of ADB and PMU. The IEE/EMP will be disclosed locally at PMU PICIIP website at least two weeks prior to the next consultation to allow the public time to review the information and form opinions.

## POLICY, LEGAL AND ADMINISTRATIVE FRAMEWORK

### General

This chapter provides an overview of the policy framework and legislation that applies to control the environmental consequences of the proposed project. The project needs to comply with all the applicable environmental policies, laws, guidelines, acts and legislations of Government of Pakistan and the provincial government as well as international environmental guidelines.

### National Policy and Legal Framework

The Pakistan National Conservation Strategy (NCS) that was approved by the federal cabinet in March 1992 is the principal policy document on environmental issues in the country (EUAD/IUCN, 1992). The NCS outlines the country's primary approach towards encouraging sustainable development, conserving natural resources, and improving efficiency in the use and management of resources. The NCS has 68 specific programs in 14 core areas in which policy intervention is considered crucial for the preservation of Pakistan's natural and physical environment. The core areas that are relevant in the context of the proposed project development are ground water depletion, provision of sustainable potable water to citizens while conserving biodiversity.

Prior to the adoption of the 18th Constitutional Amendment, the Pakistan Environmental Protection Act (PEPA) 1997 was the governing law for environmental conservation in the country. Under PEPA 1997, the Pakistan Environmental Protection Council (PEPC) and Pak EPA were primarily responsible for administering PEPA 1997. Post the adoption of the 18th Constitutional Amendment in 2011, the subject of environment was devolved, and the provinces have been empowered for environmental protection and conservation.

### Regulations for Environmental Assessment, Pakistan EPA

Under Section 12 (and subsequent amendment) of the PEPA (1997), a project falling under any category specified in Schedule I of the IEE/EIA Regulations (SRO 339 (I0/2000), requires the proponent of the project to file an IEE with the concerned provincial EPA. Projects falling under any category specified in Schedule II require the proponent to file an EIA with the provincial agency, which is responsible for its review and accordance of approval or request any additional information deemed necessary.

### Regulatory Clearances, Punjab EPA and Pak EPA

In accordance with provincial and Federal regulatory requirements, EIA reports, satisfying the requirements of the Punjab Environmental Protection Act, 1997 (Amendment 2012) and Pakistan Environmental Protection Act, 1997 are to be submitted to Punjab Environmental Protection Agency (Punjab EPA) and Federal Environmental Protection Agency (Pak EPA) for review and approval, and subsequent issuance of Environmental Approval (EA) before the commencement of construction.

### Guidelines for Environmental Assessment, Pakistan EPA

The Pak-EPA has published a set of environmental guidelines for conducting environmental assessments and the environmental management of different types of development projects. The guidelines that are relevant to the proposed project are listed below:

Guidelines for the Preparation and Review of Environmental Reports, Pakistan, EPA1997;

Guidelines for Public Consultations; Pakistan EPA May 1997.

### Punjab Environmental Quality Standards (NEQS) 2000 & 2010

The Punjab Environmental Quality Standards (PEQS), 2000 & 2010, specify the following standards:

Maximum allowable concentration of pollutants (32 parameters) in municipal and liquid industrial effluents discharged to inland waters, sewage treatment facilities, and the sea (three separate sets of numbers);

Maximum allowable concentration of pollutants (16 parameters) in gaseous emissions from industrial sources;

Maximum allowable concentration of pollutants (two parameters) in gaseous emissions from vehicle exhaust and noise emission from vehicles;

Maximum allowable noise levels from vehicles;

Maximum allowable concentration of parameters in drinking water.

### Other Environment Related Legislations

The national and international laws and regulations are provided in the Table 2.1 below.

Table 2.1: Main Strategies/Policies Related to Environment and Relevance to the Project

| Policy/Strategy | Brief Coverage | Relevance to Project |
| --- | --- | --- |
| National Conservation Strategy, 1992 | Pakistan National Conservation Strategy (NCS), which was approved by the federal cabinet in March 1992, is the principal policy document on environmental issues in the Country. The NCS outlines the Country’s primary approach towards encouraging sustainable development, conserving natural resources and improving efficiency in the use and management of resources. The NCS has 68 specific programs in 14 core areas in which policy intervention is considered crucial for the preservation of Pakistan’s natural and physical environment. | The core areas that are relevant in the context of the proposed project are pollution prevention during construction and conserving biodiversity and forestry. |
| National Environmental Policy, 2005 | In March 2005, Government of Pakistan (GoP) launched its National Environmental Policy, which provides a framework for addressing the environmental issues. Section 5 of the policy commits for integration of environment into development planning as instrument for achieving the objectives of National Environmental Policy. It also provides broad guidelines to the Federal Government, Provincial Governments, Federally Administered Territories and Local Governments to address their environmental concerns and to ensure effective management of their environmental resources. | Clause (b) of sub-section 5.1 states that Environmental Protection Act, 1997, will be diligently enforced for all developmental projects. |
| National Forest Policy, 2001 | The goal of this policy is to foster the sustainable development of Renewable Natural Resources (RNR) in Pakistan, through maintenance and rehabilitation of these essential resources and enhancement of sustainable livelihoods of rural masses, particularly women, children and other deserving groups. The various components of the policy include: - Reducing poverty, powerlessness and unemployment; - Population planning in critical ecosystems; - Reducing the impact of socio-economic factors; - Providing substitutes to firewood in the mountain-woods; - Reducing political interferences in Forestry and Wildlife Departments; - Renovating and invigorating the institutions of RNR; - Supporting Local Governments in the sustainable development of their RNR; - Policies for fragile ecosystems; - Riverside forests; - Irrigated plantations; - Preservation of sensitive and unique forests; - Wildlife conservation; - Rangelands and desert ecosystems; and - Planting trees and fodder on farmlands. | The proposed project does not pass through a Reserve Forest or other notified areas. However, other relevant components like wildlife conservation, planting trees will be applicable. |
| Pakistan Labour Policy, 2010 | The main objective of the Labour Policy, 2010 is the social and economic well-being of the labour of Pakistan. The Labour Policy, 2010 has following 4 parts: - Legal Framework; - Advocacy: rights of workers and employers; - Skill development and employment; and - Manpower export. | The labour will be employed for construction and operation of the proposed project. The provision of policy will apply to all the labour employed. |
| National Climate Change Policy, 2012 | The National Climate Change Policy provides a framework for addressing the issues that Pakistan faces or will face in future due to the changing climate. In view of Pakistan’s high vulnerability to the adverse impacts of climate change, in particular extreme events, adaptation effort is the focus of this policy document. The vulnerabilities of various sectors to climate change have been highlighted and appropriate adaptation measures spelled out. The policy cover measures to address issues in various sectors such as water, agriculture, forestry, coastal areas, biodiversity and other vulnerable ecosystems. Notwithstanding the fact that Pakistan’s contribution to global Greenhouse Gas (GHG) emissions is very small, its role as a responsible member of the global community in combating climate change has been highlighted by giving due importance to mitigation efforts in sectors such as energy, forestry, agriculture and livestock. Furthermore, appropriate measures relating to disaster preparedness, capacity building, institutional strengthening; technology transfer; introduction of the climate change issue in higher education curricula; ensuring environmental compliance through Initial Environmental Examinations (IEE) and Environmental Impact Assessments (EIA) in the development process; addressing the issue of deforestation and illegal trade in timber; promoting Clean Development Mechanisms (CDM); and raising Pakistan’s stance regarding climate change at various international forums, have also been incorporated as important components of the policy. The policy thus provides a comprehensive framework for the development of Action Plans for national efforts on adaptation and mitigation. | This policy document is a ‘living’ document and will be reviewed and updated regularly to address emerging concepts and issues in the ever-evolving science of climate change. This policy will accelerate due to the emissions from the construction machinery. |
| National Water Policy 2018 | This first ever water policy was unanimously approved in April 2018 by Council of Common Interest (CCI). The Policy aims at efficient management and conservation of existing water resources, optimal development of potential water resources, steps to minimize time and cost overruns in completion of water sector projects, equitable water distribution in various areas and canal commands, measures to reverse rapidly declining groundwater levels in low-recharge areas, increased groundwater exploitation in high-recharge areas, effective drainage interventions to maximize crop production, improved flood control and protective measures, steps to ensure acceptable and safe quality of water, minimization of salt build-up and other environmental hazards in irrigated areas, institutional reforms to make the managing organizations more dynamic and responsive. The policy covered all water related issues, including water uses, allocation of priorities, integrated planning for development, use of water resources, environmental integrity of the basin, impact of climate change, trans boundary water sharing, irrigated and rain fed agriculture, drinking water and sanitation, hydropower and industry, groundwater, water rights & sustainable water infrastructure, water related hazards, quality management, awareness, conservation, legal and capacity building of infrastructure. | The proposed project will comply with these standards. |
| National Drinking Water Policy, 2009 | The National Drinking Water Policy provides a framework for addressing the key issues and challenges facing Pakistan in the provision of safe drinking water to the people. Drinking water is the constitutional responsibility of the provincial governments and the specific provision function has been devolved to specially created agencies in cities and Town and Tehsil Municipal Administrations under the Local Government Ordinance 2001. | This policy will be applicable as the proposed Project is about the provision of drinking water to residents of Rawalpindi city. |

Table 2.2: Main Legislation/Acts Related to Environment and Their Relevance to the Project

| Act | Brief Coverage | Relevance to Project |
| --- | --- | --- |
| Punjab Environmental Protection 1997 (Amended) Act, 2017 | The Punjab Environmental Protection Act, 1997 (Amended, 2017) is comprehensive legislation and provides the legislative framework for protection, conservation, rehabilitation and improvement of the environment. The ‘environment’ has been defined in the Act as: (a) air, water and land; (b) all layers of the atmosphere; (c) all organic and inorganic matter and living organisms; (d) the ecosystem and ecological relationships; (e) buildings, structures, roads, facilities and works; (f) all social and economic conditions affecting community life; and (g) the interrelationships between any of the factors specified in sub-clauses ‘a’ to ‘f’. The notable points of the law are: - No proponent of a project shall commence construction or operation unless he has filed an EIA / IEE with the Provincial Agency designated by the Provincial EPAs an EIA / IEE, and has obtained an approval; - Establishment and formation of the Punjab Environmental Protection Council (PEPC); - Prohibition of certain discharges or emissions; - Punjab Environmental Quality Standards (PEQS) for wastewater, air emissions and noise; and - Provincial Government can issue notices and enforce them to protect the environment. In the amendment of 2012, legislatives powers related to environment and ecology are given to provincial governments from the Federal government. The provinces are required to enact their own legislation for environmental protection. Other amendments include increasing the penalties for violations. For the proposed Project, Environmental Protection Department (EPD)/Environmental Protection Agency (EPA), Government of Punjab (GoPb) is the concerned authority. The capability of regulatory institutions for environmental management is ultimately responsible for the success of environmental assessments and that development projects are environmentally sound and sustainable. | The provision of the act is applicable to proposed project for conducting an IEE/EIA according to section 12 and to obtain environmental approval from the Punjab EPA. The section 11 of the act is applicable in term of compliance with Punjab Environmental Quality Standards (PEQS). Similarly, section 13 of the act prohibits the import of hazardous waste. The provisions of section 16 are also applicable to comply with the discharge or emission of any effluent, waste, air pollutant or noise or disposal of waste or handling of hazardous substance. Under section 17, penalties will apply if anyone fails to comply with the provisions of section 11, 12, 13 and 16. |
| Pakistan Environmental Protection Agency, (Review of IEE and EIA) Regulations, 2000 | These regulations set out: - Key policy and procedural requirements for filing an EIA / IEE; - The purpose of environmental assessment; - The goals of sustainable development; - The requirement that environmental assessment be integrated with feasibility studies; - The jurisdiction of the Federal and Provincial EPA’s and Planning &Development (P&D) Departments; - The responsibilities of proponents; - Duties of responsible authorities; - Provides schedules of proposals that the project requires either IEE or an EIA; - The environmental screening process of the projects under schedule I, II and III; and - The procedure for the environmental approval for filing the case with the concerned EPA for the granting of the NOC. | The proposed project falls under the Category ‘G’ of “Water Supply and Treatment” of the Schedule I. This category requires an IEE study to be conducted to initiate the process of environmental approval by the Punjab EPA. |
| Punjab Environmental Quality Standards (PEQS), 2016 | PEQS promulgated recently in 2016. Specified standards under PEQS are for: - Drinking Water; - Ambient Air; - Noise; - Industrial Gaseous Emissions; - Municipal and Liquid Industrial Effluents; - Motor vehicle exhaust and noise; and - Treatment of Liquid and Bio-Medical Waste. | All projects to be implemented in Punjab must conform to PEQS during all the phases i.e. construction and operation. |
| Guidelines for the Preparation and Review of Environmental Reports, 1997 | These guidelines describe the format and content of IEE/EIA reports to be submitted to PEPA for obtaining NOC/approval. The guidelines present: - The environmental assessment report format; - Assessing impacts; - Mitigation and impact management and preparing an environmental management plan; - Reporting; - Review and decision making; - Monitoring and auditing; and - Project Management. | The guidelines are applicable for the preparation of the IEE Report. |
| Guidelines for Environmental Assessment | Pak-EPA has published a set of environmental guidelines for conducting environmental assessments and the environmental management of different types of development Projects. The guidelines that are relevant to the proposed Project are listed below. - Guidelines for the Preparation and Review of Environmental Reports, Pakistan Environmental Protection Agency, 1997; - Guidelines for Public Consultation, Pakistan Environmental Protection Agency, May, 1997; and - Sectoral Guidelines: Pakistan Environmental Assessment Procedures, Pakistan Environmental Protection Agency, October 1997. | The guidelines are applicable for the preparation of the IEE Report. |
| Punjab Plantation and Maintenance of Trees Act, 1974 | The Punjab Plantation and Maintenance of Trees Act, (1974) regulates tree plantations and enforces measures for their protection. | The requirements of this act are applicable in terms of planting new trees and their maintenance by the occupier of the existing land who would have the physical possession. |
| Pakistan Antiquities Act 1975 & Punjab Antiquities Amendment Act 2012 | The Punjab Antiquities Amendment Act, 2012 is adopted from the Pakistan Antiquities Act of 1975 with a few minor changes. The Antiquities Act, 1975 (amended in 1990) states the following: - “Ancient” is any object that is at least 75 years old; - All accidental discoveries of artefacts must be reported to the Federal Department of Archaeology; - The Government is the owner of all buried antiquities discovered on any site, whether protected or otherwise; - All new construction within a distance of 200 feet from protected antiquities is forbidden; - No changes or repairs can be made to a protected monument, even if it is owned privately, without approval of the responsible authorities; and - The cultural heritage laws of Pakistan are uniformly applicable to all categories of sites regardless of their state of preservation and classification as monuments of national or world heritage. | The law will be applicable to the project mainly due to its two provisions: According to the law, any construction activity within 61 m or 200 ft. of protected antiquities, are prohibited. The provisions of this act would also be applicable, if any accidental archaeological discoveries may occur during the excavation works for the construction of proposed Project. |
| Pakistan Penal Code, 1860 | The Code deals with the offences where public or private property or human lives are affected due to intentional or accidental misconduct of an individual or organization. The Code also addresses control of noise, noxious emissions and disposal of effluents. | The provisions of the Penal Code, 1860 are applicable to the project in terms of penalties for effecting human lives and public property. It also addresses the control of noise, air emissions and effluent disposal. |
| The Protection against Harassment of Women at the Workplace Act, 2010 | The Protection Against Harassment of Women at the Workplace Act (2010) refers to sexual harassment at the workplace. | This Act will be applicable to the project if women are employed for the construction of the proposed Project. |
| Labour Laws as part of Constitution of Pakistan 1973, | The Constitution of Pakistan contains a range of provisions with regards to labour rights, in particular: - Article 11 of the Constitution prohibits all forms of slavery, forced labour and child labour; - Article 17 provides a fundamental right to exercise the freedom of association and the right to form unions; - Article 25 lays down the right to equality before the law and prohibition of discrimination on the grounds of sex alone; and - Article 37(e) makes provision for securing just and human conditions of work, ensuring that children and women are not employed in vocations unsuited to their age or sex, and for maternity benefits for women in employment. Labour law is controlled at both provincial and national levels with compulsory employment agreements containing the terms set out by the labour laws. The labour laws are a comprehensive set of laws in Pakistan dealing with the following aspects: - Contract of Employment; - Termination of Contract; - Working Time and Rest Time; - Working hours; - Paid Leave; - Maternity Leave and Maternity Protection; - Other Leave Entitlements; - Minimum Age and Protection of Young Workers; - Equality - Pay Issues; - Workers' Representation in the Enterprise; - Trade Union and Employers Association Regulation; and - Other Laws. | The labour laws will be relevant as it would deal with employment of labour for the construction of propose project. Following are the major labour laws which are applicable to the project: Bonded Labour System (Abolition) Act, 1992 Employment of Child Act, 1991 Minimum Wages Ordinance, 1961 Industrial Relations Act, 2010 West Pakistan Minimum Wages for Unskilled Workers’ Ordinance, 1969 |
| Employment of Children Act, 1991 | Article 11(3) of the Constitution of Pakistan prohibits employment of children below the age of 14 years in any construction, or any other hazardous employment. In accordance with this Article, the Employment of Child Act 1991 prohibits child labour (a child is under 14 years old). | The relevance of this act to the project will be to prohibit child employment for construction of the proposed project. |
| Pakistan Climate Change Act, 2017 | This Act aims to meet obligations under international conventions relating to climate change and to provide for adoption of comprehensive adaptation and mitigation policies, plans, programmes, projects and other measures required to address the effects of climate change and for matters connected herewith and ancillary thereto. | This Act will accelerate due to the emissions from the construction machinery. |
| National Clean Air Act, 2000 | The Act aims to control vehicular emissions, pollution from industry, and indoor air pollution in rural and urban areas. | This Act will trigger if vehicles and machinery used for construction activities emanate air pollutants above the permissible limit. |
| Seismic Building Code of Pakistan 2007 | This code stipulates the minimum requirements for seismic safety of building and structures and the provisions of the Building Code of Pakistan (Seismic Provisions-2007) shall apply for engineering design of buildings, like structures and related components. Construction of buildings shall be considered as violation of professional engineering work specified under clause (XXV) of section 2 of the Act. | This Code is applicable to the proposed project as it includes the formation of structures. |
| The Parks and Horticulture Authority Act, 2012 | This act entails regulation, development, and maintenance of public parks, green belts and green areas in the Punjab; regulation of billboards, sky signs and outdoor advertisements; to promote open and unrestricted views of the Punjab. | This act will be applicable if any green belt will be involved. |
| Punjab Municipal Water Act, 2014 | The basic aim of the Act is to recognize, regulate and manage present and future municipal water supply and sanitation services and to establish rights of access to basic water supply and basic sanitation, and to ensure conservation of water resources in the Province. This Act is in draft stage. | This Act will elicit if there is misappropriation of water supply during construction activities. |
| Punjab Environmental Protection (Motor Vehicles) Rules, 2013 | Subject to the provisions of this act, and the rules and regulations, no person shall operate a motor vehicle from which air pollutants and noise are being emitted in an amount, concentration or level which is in excess of the Punjab Environmental Quality Standards, or where applicable the standards established under clause (g) of subsection (1) of section 6 of the act. | This act will be elicited during construction and operational phase due to use of motor vehicles that produces air pollutants and noise. |
| The Punjab Occupational Safety and Health Act, 2019 | This Act entails provision of occupational safety and health of the workers at workplace and to protect them against risks arising out of the occupational hazards; to promote safe and healthy working environment catering to the physiological and psychological needs of the employees at workplace. | The Act will trigger during construction and operational phase to ensure health and safety of workers at workplace associated with the project activities. |
| Electricity Act, 1910 | The Act provides a legal basis for distribution of Power. It enables a licensee to conduct operations for supply of electricity and binds the license to payment of compensation in respect of any damages caused during the construction, Operation and Maintenance (O&M) of Power distribution facilities. | This act will be applicable if any damages occur during construction of the power facilities. |
| Cutting of Trees (Prohibition) Act, 1975 | The Act was enforced in 1975 to place restrictions on cutting of trees in order to restrain unchecked trend of tree felling without replacement plantations. | This act will be applicable to the subject project where the cutting of tree will be involved. |
| National Disaster Management Act, 2010 | National Disaster Management Act, 2010 was passed by Parliament of Pakistan in 2010. The Act applies to whole Pakistan. The Act was passed in backdrop of 2010 Floods in Pakistan and strengthens Disaster Management system. | This act is applicable to the proposed project. The proposed project will require special consideration to disasters and risk management strategies as per the Act. |
| Hazardous Substance Rule, 2003 | The rule describes the procedure of handling, transportation and disposal of hazardous substances and hazardous waste. Inter alia, general safety precautions for handling hazardous substances as well as safety precautions for workers, and notification requirements in the event of an accident are described in these rules. Requirements for project waste management plans are also defined. These include a requirement for updating the plan every three years, the need to provide for management of hazardous waste in a manner that will prevent adverse environmental impacts and to ensure that hazardous and non-hazardous waste are not mixed. | This rule is applicable to the proposed project due to involvement of hazardous waste handling, use and disposal during the construction and operational stages. |

### ADB’s Safeguard Policy Statement (SPS), 2009

The ADB’s SPS 2009 requires that environmental considerations be incorporated into ADB funded projects to ensure that the project will have minimal environmental impacts and be environmentally sound. Health & safety of the local population should also be addressed as well as the project workers as stated in SPS. A Grievance Redress Mechanism (GRM) to receive application and facilitate resolution of affected peoples’ concerns, complaints, and grievances about the project’s environmental performance is also established.

All loans and investments are subject to categorization to determine environmental assessment requirements. Categorization is to be undertaken using REA checklists, consisting of questions relating to (i) the sensitivity and vulnerability of environmental resources in project area, and (ii) the potential for the project to cause significant adverse environmental impacts. Projects are classified into one of the following environmental categories:

Category A: A proposed project is classified as category A if it is likely to have significant adverse environmental impacts that are irreversible, diverse or unprecedented. These impacts may affect an area larger than the sites or facilities subject to physical works. An environmental impact assessment (EIA) is required.

Category B: A proposed project is classified as category B if its potential adverse environmental impacts are less adverse than those of category A projects. These impacts are site-specific, few if any of them are irreversible, and in most cases mitigation measures can be designed more readily than for category A projects. An initial environmental examination (IEE) is required. The project falls in Category B.

- ADB SPS 2009 also guide that the borrower/client will assess the significance of project impacts and risks on biodiversity and natural resources as an integral part of the environmental assessment process

Category C: A proposed project is classified as category C if it is likely to have minimal or no adverse environmental impacts. No environmental assessment is required although environmental implications need to be reviewed.

Category FI: A proposed project is classified as category FI if it involves investment of ADB funds to or through a financial intermediary (FI).

### ADB’s Access to Information Policy (AIP) 2018

ADB’s new Access to Information Policy (AIP), reflects the ADB’s ongoing commitment to transparency, accountability, and participation by stakeholders. The policy contains principles and exceptions to information sharing with external stakeholders, led by a new overarching principle of “clear, timely, and appropriate disclosure.”

### ADB’s Accountability Mechanism Policy 2012

The objectives of the Accountability Mechanism are providing an independent and effective forum for people adversely affected by ADB-assisted projects to voice their concerns and seek solutions to their problems, and to request compliance review of the alleged noncompliance by ADB with its operational policies and procedures that may have caused, or is likely to cause, them direct and material harm. The Accountability Mechanism is a “last resort” mechanism.

### ADB Gender and Development Policy

ADB’s Policy on Gender and Development will adopt mainstreaming as a key strategy in promoting gender equity. Gender considerations will be mainstreamed into all ADB activities, including macroeconomic and sector work, and lending and technical assistance (TA) operations. The key elements of ADB’s policy will include gender sensitivity, gender analysis, gender planning, mainstreaming, and agenda setting.

### Climate Change Risk Management Framework

The climate risk management approach of the ADB aims to reduce risks resulting from climate change to investment projects in Asia and the Pacific. ADB’s framework identifies climate change risks to project performance in the early stages of project development and incorporates adaptation measures in the design of projects at risk. ADB climate risk management framework comprises the following steps:

context-sensitive climate risk screening at the concept development stage to identify projects that may be at medium or high risk.

climate change risk and vulnerability assessment during the preparation of projects at risk

a technical and economic evaluation of adaptation options

identification of adaptation options in project design; and

monitoring and reporting of the level of risk and climate-proofing measures.

### Implications of ADB’s safeguard policies on proposed project

The objectives of ADB’s safeguards are to:

Avoid adverse impacts of projects on the environment and affected people, where possible;

Minimize, mitigate, and/or compensate for adverse project impacts on the environment and affected people when avoidance is not possible; and

Help borrowers/clients to strengthen their safeguard systems.

ADB’s SPS sets out the policy objectives, scope and triggers, and principles for three key safeguard areas:

environmental safeguards,

involuntary resettlement safeguards, and

Indigenous Peoples safeguards.

The objective of the environmental safeguards is to ensure the environmental soundness and sustainability of projects and to support the integration of environmental considerations into the project decision-making process. ADB’s policy principles are summarized in the Table 2.3 below.

Table 2.3: ADB Policy Principles

| No. | Policy principle | Summary |
| --- | --- | --- |
| 1 | Screening and categorization | Screening process initiated early to determine the appropriate extent and type of environmental assessment. |
| 2 | Environmental assessment | Conduct an environmental assessment to identify potential impacts and risks in the context of the project’s area of influence. |
| 3 | Alternatives | Examine alternatives to the project’s location, design, technology, and components and their potential environmental and social impacts, including no project alternative. |
| 4 | Impact mitigation | Avoid, and where avoidance is not possible, minimize, mitigate, and/or offset adverse impacts and enhance positive impacts. Prepare an environmental management plan (EMP). |
| 5 | Public consultations | Carry out meaningful consultation with affected people and facilitate their informed participation. Involve stakeholders early in the project preparation process and ensure that their views and concerns are made known to and understood by decision makers and taken into account. Continue consultations with stakeholders throughout project implementation. Establish a grievance redress mechanism. |
| 6 | Disclosure of environmental assessment | Disclose a draft environmental assessment in a timely manner, in an accessible place and in a form and language(s) understandable to stakeholders. Disclose the final environmental assessment to stakeholders. |
| 7 | Environmental management plan | Implement the EMP and monitor its effectiveness. Document monitoring results and disclose monitoring reports. |
| 8 | Biodiversity | Do not implement project activities in areas of critical habitats. |
| 9 | Pollution prevention | Apply pollution prevention and control technologies and practices consistent with international good practices. Adopt cleaner production processes and good energy efficiency practices. Avoid pollution, or, when avoidance is not possible, minimize or control the intensity or load of pollutant emissions and discharges. Avoid the use of hazardous materials subject to international bans or phase outs. |
| 10 | Occupational health and safety/Community safety. | Provide workers with safe and healthy working conditions and prevent accidents, injuries, and disease. Establish preventive and emergency preparedness and response measures to avoid, and where avoidance is not possible, to minimize, adverse impacts and risks to the health and safety of local communities. |
| 11 | Physical cultural resources | Conserve physical cultural resources and avoid destroying or damaging them. Provide for the use of “chance find” procedures. |

### IFC Environmental, Health and Safety Guidelines for Water and Sanitation

The IFC EHS Guidelines for water and sanitation include information relevant to the operation and maintenance of (i) potable water treatment and distribution systems, and (ii) collection of sewage in centralized systems (such as piped sewer collection networks) or decentralized systems (such as septic tanks subsequently serviced by pump trucks) and treatment of collected sewage at centralized facilities.

Environmental issues associated with water and sanitation projects may principally occur during the construction and operational phases, depending on project-specific characteristics and components.

Guidelines are related to following impacts associated with Drinking water supply and treatment are as follows:

Water Withdrawal

Water Treatment

Solid waste

Wastewater

Hazardous chemicals

Air emissions

Ecological impact

Water Distribution

Water system leaks and loss of pressure

Water discharges

### Comparison of International and Local Environmental Legislations

The ADB SPS, 2009 requires application of pollution prevention and control technologies and practices consistent with international good practice, as reflected in internationally recognized standards. The SPS states that when host country regulations differ from these standards, the EA will achieve whichever is more stringent.

In order to select the most stringent standards applicable, a mix of local (NEQS) and international (IFC) regulations have been selected. The IFC Environmental, Health, and Safety (EHS) Guidelines, General EHS Guidelines and Environmental standards are also applicable. It shall be ensured that all necessary noise mitigation measures are implemented to minimize the noise levels in the project area.

The Table 2-4 presents IFC workplace noise standards that are applicable to the construction workers. It should also be noted that IFC EHS guidelines advise that where existing ambient noise levels already exceed thresholds, the project should not result in an increase of more than 3 dB over existing ambient noise levels at the nearest receptor location off-site.

A comparison of applicable local and international guidelines for ambient air quality has been provided in Table below. In the case of most pollutants, the Pak NEQS standards for ambient air quality are more stringent in comparison to USEPA and WHO/IFC standards. The applicable and most stringent parameters for each respective pollutant are highlighted in green.

Similar to the standards for air quality, the comparison of noise standards provided clearly shows that the Pakistan NEQS standards for noise are more stringent in comparison to the IFC standards. The only exception is the daytime noise level standard for Industrial areas where the IFC standard is more stringent (70 dB (A)) in comparison to NEQS (75 dB (A)) and so for this particular parameter, the IFC standard will be used. Apart from this one exception, the NEQS standards have been used for water quality of development project.

Comparison of International and Local Water Quality Standards has been provided. Standard for Bacterial contamination are same for both PEQS/NEQS and IFC/WHO standard while physical parameters are different. PEQS/NEQS for odor, turbidity, hardness and pH are more stringent while IFC/WHO standards are stringent in metallic contaminations i.e. Arsenic, Barium, Boron, Cadmium and Zinc.

As far as regulations regarding other environmental parameters are concerned such as acceptable effluent disposal parameters, as per the requirements of ADB SPS-2009, the stringent standards among national and international guidelines will be followed for this project.

Table 2.4: IFC Work Environment Noise limits1.2

| Type of Work, Workplace | IFC General EHS Guidelines |
| --- | --- |
| Heavy Industry (no demand for oral communication) | 85 Equivalent level Leq,8hrs |
| Light industry (decreasing demand for oral communication) | 50-65 Equivalent level Leq,8hrs |

Table 2.5: Comparison of International and local Air Quality Standards*1.3

| Pollutants | WHO/IFC |  | Pak. NEQS / PEQS |  |
| --- | --- | --- | --- | --- |
|  | Avg. Time | Standard | Avg. Time | Standard |
| SO2 | 24 hr. | 40 µg/m3 | 24 hrs. | 120 up/m3 |
| CO | 8 hrs. | 4 mg/m3 | 8 hrs. | 5 mg/m3 |
| NO2 | 1 yr. 24 hrs. | 10 µg/m3 25 µg/m3 | Annual Mean 24 hrs. | 40 up/m3 80 up/m3 |
| O3 | 8 hrs. | 100 µg/m3 | 1 hr. | 130 µg/m3 |
| TSP | - | - | Annual Mean 24 hrs. | 360 up/m3 500 up/m3 |
| PM10 | 1 yr. 24 hr. | 15 µg/m3 45 µg/m3 | Annual Mean 24 hrs. | 120 up/m3 150 up/m3 |
| PM2.5 | 1 yr. 24 hr. | 5 µg/m3 15 up/m3 | Annual Average 24 hrs. 1 hr. | 15 up/m3 35 up/m3 15 up/m3 |

*: The standards highlighted in green for each respective pollutant are the most stringent based on a comparison between local and international regulations and thus shall be applicable for the proposed project.

* In instances where the air shed is significantly degraded and the pollutant levels are already exceeding the ambient pollutant concentrations provided in the table above, it shall be ensured that the project activities cause as small an increase in pollution levels as feasible, and amounts to a fraction of the applicable short term and annual average air quality guidelines or standards as established in the project specific environmental assessment.

Table 2.6: Comparison of International and Local Noise Standards1.4

| Category of Area/Zone | Limit in dB(A) Leq |  |  |  |
| --- | --- | --- | --- | --- |
|  | NEQS/PEQS |  | WHO |  |
|  | Day Time 06:00 – 22:00 | Night Time 22:00-06:00 | Day Time 07:00 – 22:00 | Night Time 22:00-07:00 |
| Residential area (A) | 55 | 45 | 55 | 45 |
| Commercial area (B) | 65 | 55 | 70 | 70 |
| Industrial area (C) | 75 | 65 | 70 | 70 |
| Silence zone (D) | 50 | 45 | 55 | 45 |

*: The standards highlighted in green for each respective Area/Zone are the most stringent based on a comparison between local and international regulations and thus shall be applicable for the proposed project.

* In instances where baseline noise levels are already exceeding the standards above, it will need to be ensured that the project activities do not cause an increment of more than 3 dB (A) from the baseline noise levels.

Table 2.7: Comparison of International and Local Water Quality Standards1.5

| Parameter | Unit | NEQS/PEQS |  | WHO/IFC |  |
| --- | --- | --- | --- | --- | --- |
| Bacterial |  |  |  |  |  |
| E-Coli |  | numbers/ml | Must not be detectable in any 100 ml sample | Must not be detectable in any 100 ml sample |  |
| Total Coliform |  | numbers/ml | Must not be detectable in any 100 ml sample | Must not be detectable in any 100 ml sample |  |
| Physical |  |  |  |  |  |
| Color |  | TCU | ≤ 15 TCU | ≤ 15 TCU |  |
| Taste |  | No objectionable/Acceptable | No objectionable/Acceptable | No objectionable/Acceptable |  |
| Odor |  | No objectionable/Acceptable | No objectionable/Acceptable | No objectionable/Acceptable |  |
| Turbidity |  | NTU | < 5 NTU | < 5 NTU |  |
| Total Hardness |  | mg/l | < 500 mg/l | -- |  |
| TDS |  | mg/l | < 1000 | < 1000 |  |
| pH |  |  | 6.5-8.5 | 6.5-8.5 |  |
| Chemical |  |  |  |  |  |
| Aluminum |  | mg/l | ≤0.02 | 0.2 |  |
| Antimony |  | mg/l | ≤0.005 (P) | <0.02 |  |
| Arsenic |  | mg/l | ≤0.05 (P) | 0.01 |  |
| Barium |  | mg/l | 0.7 | 0.7 |  |
| Boron |  | mg/l | 0.3 | 0.3 |  |
| Cadmium |  | mg/l | 0.01 | 0.003 |  |
| Chloride |  | mg/l | <250 | 250 |  |
| Chromium |  | mg/l | ≤0.05 | 0.05 |  |
| Copper |  | mg/l | 2 | 2 |  |
| Cyanide |  | mg/l | ≤0.05 | 0.07 |  |
| Fluoride |  | mg/l | <1.5 | 1.5 |  |
| Lead |  | mg/l | ≤0.05 | 0.01 |  |
| Manganese |  | mg/l | ≤0.5 | 0.5 |  |
| Mercury |  | mg/l | ≤0.001 | 0.001 |  |
| Nickel |  | mg/l | ≤0.02 | 0.02 |  |
| Nitrate |  | mg/l | ≤50 | 50 |  |
| Nitrite |  | mg/l | ≤3 | 3 |  |
| Selenium |  | mg/l | 0.01 )P) | 0.01 |  |
| Residual Chlorine |  | mg/l | 0.2-0.5 at consumer end | - |  |
| Zinc |  | mg/l | 5.0 | 3 |  |

*: The standards highlighted in green for each respective Area/Zone are the most stringent based on a comparison between local and international regulations and thus shall be applicable for the proposed project.

## PROJECT DESCRIPTION

### Project Introduction

The Water and Sanitation Agency (WASA), Rawalpindi, is responsible for providing water supply and sewerage services to the 2 million people living in the 66 Union Councils/Urban Core of Rawalpindi. The WASA has divided its jurisdictional area into six water supply zones, EAST-1, EAST-2, WEST-1, WEST-2, PP-12 and PP-13.

According to the previous master plan of Rawalpindi (1996) and the information provided by the WASA, the per capita water consumption is estimated to be 182 liters (40 gpcd). The domestic water demand is 491407.462 m³/d (108.00164 MGD), almost 259127.13m³/d (57 MGD) less than the current water supply.

The RWASA's estimate of the current water production from all sources (i.e. groundwater and surface water) is 231,851 m3/d (51 MGD), which is over-estimated. The total groundwater production from the tube wells is roughly 159,113 m3/d (35 MGD) with the actual discharge of 127,291 to 136,383m3/d (28-30 MGD). Therefore, the actual volume of water that is being supplied at source, excluding the Non-Revenue Water (NRW) and other losses range from 204,574 to 213,666 m³/d (45-47 MGD).

Some of the water supply lines are outlived. Due to this, problem like pressure drop, NRW and contaminations are increasing. Majority of the lines which needs replacement are of AC and GI material.

The project activities are located in the Rawalpindi city area, including the suburban and the peri-urban areas of the RDA and the Potohar Town. The study area comprises the RWASA jurisdiction, including 3 Union Councils (UCs). The Master Plan (2016-2040), carried out the demand and the projections for the water supply for the three types of the areas, which are:

Urban areas

Peri-urban areas

Rural areas

A map, showing the overall Urban Core area of Rawalpindi is given as below.

Figure 3.1: Urban Core Area of Rawalpindi

### Existing Water Supply System of KSS

Currently, there are two main sources of water for residents of KSS. One is from the Khanpur dam and the other is from the existing tube wells, installed by RWASA. The Khanpur Dam supplies water to Rawalpindi and Islamabad for domestic consumption and irrigation water to the agricultural lands and industrial areas surrounding the twin cities. The tube wells are discharging, a total of 7,200 m3/d water. Total 37 tube wells are installed in KSS and out of these, only 31 are functional and 06 are not functional. All the tube wells are of submersible type. Due to the limited capacity of the tube wells, each tube well has a very limited-service area, spread over 3 to 6 streets only. On average, the daily running time for each tube well is 12 -14 hours, to meet the water requirements.

Water storage includes the ground storage tanks (GSTs) and the overhead reservoirs (OHRs). In KSS, currently there are 3 GSTs, each of 50,000-gallon capacity and 5 OHRs, out of which 4 OHRs have a capacity of 50,000 gallons and one has a storage capacity of 25,000 gallons. All 5 OHRs are all in good working condition, but one of the GSTs, which is located in Shah Najaf Park UC-12, is currently not operational due to the unavailability of transformer.

In addition, one GST with a storage capacity of 50,000 gallons is under construction in Jinnah Park, Sector-3. Also, one GST and an OHR, each of 50,000-gallon capacity, are being tendered by Rawalpindi WASA and will be constructed in Allama Iqbal Park, in UC-10. Figure shows the existing water resources in KSS. Table below gives the detail of storage structures of KSS.

Table 3.1: Water Storage Capacity of KSS

| Sr. No. | UC | Location | Description | Capacity (Cubic Meters) | Capacity (Imperial Gallons) |
| --- | --- | --- | --- | --- | --- |
| 1 | UC-10 | Sector-2 Ground | Existing GST | 227.3 | 50,000 |
| 2 |  |  | Existing OHR | 227.3 | 50,000 |
| 3 |  | Allama Iqbal Park | GST (Under Construction) | 227.3 | 50,000 |
| 4 |  |  | OHR (Under Construction) | 227.3 | 50,000 |
| 5 | UC-11 | Mohammadi Chowk | Existing GST | 227.3 | 50,000 |
| 6 |  |  | Existing OHR | 227.3 | 50,000 |
| 7 |  | Mohra Sharif Park | Existing OHR | 227.3 | 50,000 |
| 8 |  | Jinnah Park, Sector 3 | GST (Tendered) | 227.3 | 50,000 |
| 9 | UC-12 | Shah Najaf Park | Existing GST | 227.3 | 50,000 |
| 10 |  |  | Existing OHR | 227.3 | 50,000 |
| 11 |  | Ladies Park | Existing OHR | 113.7 | 25,000 |
| Total Storage Capacity |  |  |  | 2386.7 | 525,000 |

With a total length of 39.19 km, the distribution network in KSS ranges in size from 3 inches (75mm) to 8 inches (200mm). The pipe materials are AC, PVC, HDPE, and G. I. The pipes have been in place for 15 to 33 years.

Due to a lack of Operation and Maintenance funding, water supply systems have been deteriorated. The infrastructure for supplying water is outdated, especially AC and G.I. pipes, with leaking pipes and inadequate pressure for consumers.

Corrosion over time has decreased the carrying capacity of MS pipelines. Due to the poor quality of the materials used, shoddy workmanship, corrosion, and aging of the installations, a lot of water is wasted through the distribution system's leaky pipes, joints, valves, and fittings causing the pressure to drop, the supply to decrease, and the quality of the water to worsen. Detail of existing water supply system in KSS is shown in  Error! Reference source not found..

Figure 3.2: Existing Water Resources of KSS

Figure 3.3: Existing Water Supply System in KSS

### Components of Project

The proposed project consists of the following sub-components;

- Formation of DMA for KSS

Water Metering

Design of Bulk Supply Main and Laying of New Conductance Main (Bulk Supply Main)

Installation of Conductance Main / Bulk Water Supply Lines

Design of Distribution Network and Replacement of Outlived Pipelines

Pumping Requirements at Water works and Sangjani Water Treatment Plant (WTP)

The detailed alignment of Existing Conductance Main and New Conductance Main (Bulk Supply Line) along with water works is shown below in Figure 3.4 below.

### Creation of DMAs for KSS

District metering is a technique used to improve water distribution system (WDS) management. It divides a WDS into smaller portions called District Metered Areas (DMAs). This technique reduces the NRW by timely detection of leakage points. Valves are applied to control the flow of water into each DMA. It helps to measures the quantity of water flow into system and make sure the 24/7, uninterrupted supply of water into system. A typical view of DMA is shown in Figure 3.5 below.

Figure 3.4: Alignment of Existing Conductance Main and Proposed Conductance Main

Figure 3.5: A typical Thematic view of DMA

### Selection of Study Area for DMAs

The main purpose of selecting KSS as a district metered zone is that, this area is planned and bounded by Nallah Lai and Nikki on two sides and AJP road on the third side, making this area independent zone. As mentioned earlier, there are 3 UCs in KSS and these are taken as 3 sub-zones (Zone-A, B, and C) because their boundaries are distinctive and the laid pipe network is also bifurcated.

### Design Philosophy of DMAs

Based on the population served and the water source's availability, the KSS is split into three sub-DMAs (Zone-A, B, and C). Each DMA is intended to serve at least 6,000–8,000 households. There are existing tube Wells in the system, along with their associated distribution network. All zones will have clearly defined boundaries and isolation valves to keep them separate.

Water will be supplied to each DMA, using control valves to distribute water to the current distribution system. The distribution network will only be fed through OHR/GST during normal operation, while during peak hours, the demand will be satisfied by direct pumping, from either the tube wells or the GST. As a result, residents of the project area will have continuous access to water. Bulk vales will be installed to control and monitor the flow of water.

The supply to each DMA shall be recorded and monitored along with the water supplied to end consumers using domestic and commercial water meters. It will help to identify the leakages in the system as well as the illegal house connections or theft.

Figure 3.6: Proposed DMAs, / Zones in Khayaban-e-Sir Syed

### DMA Boundaries from Water GEMS Model

In Water GEMS, pressure zone manager is used to identify no. of DMAs and their boundaries. The red colour shows the pipe network of DMA-1, blue for DMA-2 and Green for pipes of DMA-3.

### Isolation Plan of DMAs

Isolation of network is done by using sluice/gate valves at desired location in the network. In KSS, isolation plan for each DMA has been created by installing sluice valves at all boundary nodes (inlets and outlets) of the network.  Isolation maps of DMA 1, 2 and 3 are shown in Figure 3.8 and Figure 3.10.

Figure 3.7: Water supply boundaries of DMAs derived from Hydraulic Modelling

Figure 3.8: Isolation Map of DMA-1

Figure 3.9 Isolation Map of DMA-2

Figure 3.10 Isolation Map of DMA-3

### Water Metering

The proper metering in the KSS is proposed to get the record of the water supplied and obtained from the source. It will result in identifying wastages in the system and water conservation as the customer will have to pay for the quantity of water consumed as per actual.

### Water Meters Required for KSS

Two types of water meters are required in proposed project.

Flow meters for Bulk Supply Mains

Domestic Water Meters

### Flow Meters for Bulk Supply Main

Flow meters are to be installed at water entry point of all bulk supply mains and sub-main. This will give the total volume of water that is being supplied to the system. It shows the location plan of all flow meters that are to be installed at various locations of bulk supply mains.

### Domestic Water Meters

Keeping in view the spatial plan findings, population of 2025 was taken to find the number of house connections. Total number of domestic connections were calculated, assuming 7 people per house, while commercial connections were assumed as 5% of residential connections. Population and domestic/ commercial connections in each DMA are given in the table below.

Table 3.2: Detail of DMAs with Domestic/Commercial connections

| UC No. | Proposed DMA | Population (2025) | No. of Domestic Connection | No. of Commercial Connection |
| --- | --- | --- | --- | --- |
| UC-10 | DMA-1 | 22746 | 3249 | 162 |
| UC-11 | DMA-2 | 24201 | 3457 | 173 |
| UC-12 | DMA-3 | 29902 | 4272 | 214 |
| Total |  | 76849 | 10978 | 549 |

Location map of water meters to be installed in the distribution network of KSS at feeding mains i.e., at main lines which are coming from storages and supplying water to the network is shown in Figure 6.12.

Figure 3.11: Locations of Flow Meters on Bulk Supply Mains

Figure 3.12 Location of water meters at Feeding Mains

### Design of Bulk Supply Mains

Keeping in view the present and future water supply demand to KSS, following design has been considered for KSS.

A mild steel (MS) supply main of 300 mm (ID) is currently supplying water to KSS. It takes water from a 600 mm (ID) Supply Main, which is bifurcated from a 1000 mm (ID) main line, coming from Tomar reservoir. The Tomar reservoir is fed by a 1400 mm main line from Sangjani treatment plant based on Khanpur dam.

As this supply line is not enough to fulfil even the current water demands, a new supply main of 620 mm (ID) or 710 mm (OD) HDPE with pressure rating of PN-10 has been proposed to meet the maximum water demand of KSS up to the year 2050. This new line shall be laid parallel to the existing main line on the other side of the road (Service Road West I-10) in the green strip in Sector I-10 area. This line will run parallel to IJP Road and will cross the IJP Road to enter into Khyaban-e-Sir Syed (KSS) at CDA Stop.

In this option, Existing Supply Main is set to supply water to UC-12 of KSS and shall be discontinued at connection with new proposed line at Main KSS Road and new proposed line will be supplying water to the water works of UC-10 and UC-11 through sub-mains. The new line is designed on maximum day demand of whole KSS. This new line can also be used to supply water to UC-12 as well in future if existing line is to be made non-functional due to any reason such as corrosion, aging or diversion. A layout of proposed bulk supply main is shown in Figure 3.13.

Figure 3.13 Layout of Proposed Bulk Supply Line

### Hydraulic Model of Bulk Supply Mains

The proposed bulk supply line shall be supplying water on working head. The available head at bifurcation point (start point of proposed line) is 22.3 m of water. Bentley Water GEMS software is used to create a hydraulic model of the line. HDPE pipe with pressure rating PN-10 is used for all pipes. The maximum pipe diameter comes out to be 710 mm (OD) and minimum diameter used is 280 mm (OD). Hydraulic model layout of the proposed bulk supply line is shown below in Figure 6.14.

### Excavation

Trenches shall be excavated for the laying of pipe at adequate depth as per the design. The estimated depth for Conductance Main is 8-10 feet and for replaceable distribution lines is 2.5-3 feet. It will involve soft soil cutting in the green area parallel to I-10 Service Road West and may involve cutting of pavement structure in Khayaban-e-Sir Syed, if required.

### Bedding and Laying/ Jointing

Sand bedding shall be provided in layers for laying of pipe. Butt fission jointing of pipe shall be carried out for HDPE pipes.

### Testing

Testing of pipe shall be carried out as per ASTM standard F-2164/ AWWA M5.

### Restoration of damaged Utility Services

Any utilities, if encountered during the laying of bulk supply line shall be restored.

### Provision of Thrust Blocks

Thrust blocks shall be provided at bends to ensure pipe stability in case of change in velocity and pressure at bends.

### Backfilling

Backfilling of the trenches shall be done as per the designed backfilling detail.

### Restoration of Road/ Street

Restoration of pavements (rigid/flexible) at all roads/streets shall be done.

### Pipe Jacking

In order to cross 710 mm dia (OD) Bulk Supply Line from IJP Road (Islamabad), pipe jacking shall be done. Length of jacking is 100m.

Figure 3.14 Layout of Hydraulic Model from Water GEMS

### DESIGN OF DISTRIBUTION NETWORK

The water supply distribution system is designed to meet the peak demands of an area. The main source of water for KSS is surface water source from Khanpur dam and groundwater from tube wells. The minimum pressure for the water supply distribution network of KSS is 20 psi and the maximum pressure is 70 psi.

### Replacement of Outlived Pipes

Old leaking pipes causes an insufficient pressure at the consumer's end. Due to a lack of funding for operation and maintenance, WSS systems have deteriorated.

Corrosion over time, especially in unlined MS pipes cause pipes to disintegrate. Due to poor construction, faulty materials, corrosion, and aging installations the water distribution pipelines of KSS needs replacement to control the wastages and mixing of wastewater into water supply system.

Outlived distribution network of pipe material AC and GI shall be replaced with HDPE pipe. Approximately 4 km length of PVC pipes are also suggested to be replaced with HDPE in order to maintain pressures in the system. The details of distribution network pertaining to replacement and additional pipes for unserved areas are shown in the table below.

Table 3.3: Distribution Pipes to be Replaced

| Sr. No. | Existing Type | Existing Dia (ID) (mm) | Length (m) | New Dia (OD) (mm) | Replacement Material Type | Pressure |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | AC | 75 | 2379 | 90 | HDPE | PN-8 |
| 2 | AC | 100 | 166 | 110 | HDPE | PN-8 |
| 3 | AC | 150 | 1074 | 180 | HDPE | PN-8 |
| 4 | AC | 200 | 1045 | 225 | HDPE | PN-8 |
| 5 | GI | 50 | 1145 | 90 | HDPE | PN-8 |
| 6 | PVC | 75 | 2063 | 90 | HDPE | PN-8 |
| 7 | PVC | 100 | 1149 | 110 | HDPE | PN-8 |
| 8 | PVC | 150 | 813 | 180 | HDPE | PN-8 |

Table 3.4: Distribution Pipes for Unserved Areas

| Sr. No. | New Dia (OD) (mm) | Length (m) | Pipe Material | Pressure |
| --- | --- | --- | --- | --- |
| 1 | 90 | 850 | HDPE | PN-8 |

Moreover, some of the pipes which are of smaller diameters are also need to be replaced with pipes of higher diameters so that desired quantity of water can be supplied with desired pressures. Figure 3.15 shows the layout plan that highlights the detail of outlived pipes that need to be replaced.

### Distribution Lines for Unserved Areas

In KSS, some of the areas/streets are not getting any supply from existing network. As the total supply includes the share of those unserved areas as well. So, the supply to those areas is considered and new distribution lines are proposed. Figure 3.16 shows the layout of new distribution lines to be provided in unserved areas of KSS.

#### Excavation

Trenches shall be excavated for the laying of pipe of adequate depth as per the design. It will involve soft soil/pavement cutting as per the requirement.

#### Bedding and Laying/ Jointing

Sand bedding shall be provided in layers for laying of pipe. Butt fission jointing of pipe shall be carried out for HDPE pipes.

#### Testing

Testing of pipe shall be carried out as per ASTM standard F-2164/ AWWA M5.

#### Restoration of damaged Utility Services

Identification of existing utilities will be made by developing test pits. Any utilities if encountered during the laying of distribution network shall be restored to its original by repairing and replacing the disturbed utilities.

#### Backfilling

Backfilling of the trenches shall be done as per the design which will be developed at the time of project execution. The same design will be shared with PMU.

### Restoration of Road/ Street

Restoration of pavements (rigid/flexible) at all roads/streets shall be done.

Figure 3.15 Outlived Distribution Pipes to be Replaced

Figure 3.16 Layout of Distribution Lines for Unserved Areas

### Pumping Requirement

Pumps and pumping machinery are used to lift water from a source or to boost water from low point to high areas. Pumps being used are mostly centrifugal and turbine pumps.

### Pumping at Sangjani Treatment Plant

Sangjani Water Treatment Plant (WTP) treats the water from Khanpur dam, which is the source of surface water for Khayaban-e-Sir Syed. The treated water is transported to Tomar reservoir from where it gravitates to KSS. The share of Rawalpindi WASA is supplied through 2 working and 1 standby pumps having capacity of 0.8 MG/hr. Based on the site visit and discussion with the stakeholders, the current status of pumps is as follows.

1 x pump: Working Condition

2 x pumps: Not in good working condition

Based on assessment of pumping requirement, RDLO 500-835 A SB P F/or equivalent having Flow of 3,637 m3/h, with head of 100 m, Motor 1,400 kW 6.6 kV @ 6 pole has been considered for replacement, which would be finalized at the at the detailed design stage.

### Pumping Requirement for Water Supply Distribution in KSS

In KSS, depending upon the water requirement, each water works is allocated with pump of required capacity and pumping head.

There are seven water works in KSS, out of which four water works consists of, both a GST and an OHR; two water works have OHR only; and one water works has a GST only from which water is directly pumped to the distribution network.

gives the characteristics of water supply pumps of all water works.

Table 3.5: Pumping Requirement for KSS

| Sr. No. | UC | WW Location | Description | Capacity (Gallons) | Pumps Req. in Phase-1 (Including 1 Stand-by) | Pumps Req. in 2040 | Pumping Head (m) | Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UC-10 | Sector-2 Ground | GST + OHR | 50000 Each | 2 x 1 Cusec | 1 x 2 Cusec | 45 | Turbine |
| 2 |  | Allama Iqbal Park | GST+OHR (Tendered) | 50000 Each | 2 x 1 Cusec | 1 x 2 Cusec | 45 | Turbine |
| 3 | UC-11 | Mohammadi Chowk | GST+OHR | 50000 Each | 2 x 1 Cusec | 1 x 1.5 Cusec | 45 | Turbine |
| 4 |  | Mohra Sharif | OHR | 50000 | 2 x 1 Cusec | 1 x 1.5 Cusec | 45 | Centrifugal |
| 5 |  | Jinnah Park, Sector 3 | GST (Under Construction) | 50000 | 2 x 1 Cusec | 1 x 1.5 Cusec | 45 | Turbine |
| 6 | UC-12 | Shah Najaf Park | GST+OHR | GST: 50000; OHR: 25000 | 2 x 1 Cusec | 1 x 2 Cusec | 45 | Turbine |
| 7 |  | Ladies Park | OHR | 50000 | 2 x 1 Cusec | 1 x 2 Cusec | 45 | Centrifugal |

### Climatic Consideration

Local climatic conditions, i.e., the temperature, wind velocity, thunderstorm levels, relative humidity, etc., control the selection of materials to be used. The Table 3.6 below shows the climatic parameters that have been considered in designing of the project.

Table 3.6: Climatic Parameters for the project

| Maximum Temperature | 48.3oC |
| --- | --- |
| Minimum Temperature | -3.9oC |
| Annual Mean temperature (2022) | 43oC (maximum) to 30oC (minimum) |
| Maximum Rainfall | 340 mm/month |
| Annual Relative Humidity | 39.6% |

### Schedule of Project Implementation

#### Design Phases

The design of distribution network is performed using Bentley Water GEMS software. Four phases of design are proposed as given the Table 3.7 below. The first phase of the project is covered under the scope of current ADB funding. Hence, detail of the first phase are given below.

Table 3.7: Design Phase for Distribution System

| Sr. No. | Phase | Duration (Year) |
| --- | --- | --- |
| 1 | Phase-1 | 2022-25 |
| 2 | Phase-2 | 2025-30 |
| 3 | Phase-3 | 2030-40 |
| 4 | Phase-4 | 2040-50 |

#### Phase-1 (2022-25):

It is the first phase of planning, in this phase a new proposed supply main of 710mm (OD) dia HDPE pipe is to be bifurcated from 1000mm supply main coming from Tomar reservoir. The new main line is then linked with all water works of UC-10 and UC-11. The existing supply main of 300mm is to be supplied to the water works of UC-12. In this phase all the AC and GI pipes of distribution network of KSS are to be replaced with HDPE pipes. A length of 554 m of existing PVC pipe of 75mm dia will also be replaced with 100mm dia and 1325 m length with 150mm dia. Moreover, some additional pipes are to be laid down in unserved areas. All the working Tube Wells (31 No's) shall remain functional in the 1st phase. Total supply to each water works is given in the Table 3.8 below.

Table 3.8: Total Supply to each water works in Phase-1

| Sr. No. | UC | Location | Description | Capacity of WW (Gallons) | Population 2025 (Persons) | Av Demand (MGD) | Supply from TWs (MGD) | To be Supplied from Khanpur (MGD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UC-10 | Sector-2 Ground | GST + OHR | 50000 Each | 11373 | 0.45 | 0.46 | 0.22 |
| 2 |  | Allama Iqbal Park | GST+OHR (Tendered) | 50000 Each | 11373 | 0.45 |  | 0.22 |
| 3 | UC-11 | Mohammadi Chowk | GST+OHR | 50000 Each | 8067 | 0.32 | 0.33 | 0.21 |
| 4 |  | Mohra Sharif | OHR | 50000 Gallon | 8067 | 0.32 |  | 0.21 |
| 5 |  | Jinnah Park, Sector 3 | GST (Under Construction) | 50000 Gallon | 8067 | 0.32 |  | 0.21 |
| 6 | UC-12 | Shah Najaf Park | GST+OHR | GST: 50000; OHR: 25000 | 14951 | 0.60 | 0.65 | 0.27 |
| 7 |  | Ladies Park | OHR | 50000 Gallon | 14951 | 0.60 |  | 0.27 |

### Construction phase activities

The activities to be conducted during construction phase of the project are provided below:

Mobilization of Contractor

Development of Construction and Labor Camps

Excavations for lying of Bulk Water Supply Lines

Excavation for replacement of AC /GI Pipelines

Installation of domestic meters

Excavations for replacement of control Valves and installation of DMAs

Installation of Pumps and axillary machinery at GSTs and OHRs for water flow

Installation of Pumps at Sangjani WTP

One of the first activities to be completed by the Contractor shall be the establishment of the construction and labor camp. The Contractor will also establish construction yards and sites (including storage facilities), offices and a workshop.

The project will be divided into construction work packages and these packages will be awarded to the selected project Contractors or single contractor whichever is feasible.

- The construction activity (for first phase) will be completed in, approximately twenty-four months. The selected Contractors shall have the option to select suitable site(s) located near the project sites to establish his labor camps. If private land is selected, the contractor shall enter into contract with the private owner. During the first phase of the project, an estimated 50-60 persons, consisting of both semi-skilled and skilled human resource will be required.

The location of storage materials and camps will be critical. Since the project contractor(s) will be responsible for identifying the suitable locations for storage and labor camps from the private sector, thus there will need to be clear guidelines for this process, which will need to be closely monitored by the implementing agency. The contractor will identify the location and get approval from PMU for material storage and camp sites. PMU will carry out detailed survey to identify social and environmental impacts of the storage facility and camp site on local community and environment.

As far as possible, the project design team shall be assigned the task to identify the suitable location(s) for storage of materials since inappropriate storage of materials may result disruption of the traffic movement because the area of I-10 Islamabad and KSS is densely populated.

The scope of works for installation of the solar PV system will be as follows:

Site establishment, setting up of construction site offices, storage areas etc;

Importing of materials and equipment;

Clearing of any trees and vegetation;

Establishment of any access ways and drainage system across the site;

Earthworks to create level construction platform;

Installation of foundations using pre-cast concrete blocks, earth screw anchors or piles (whichever is most suitable);

Installation of solar PV modules;

Excavation of cable trenches;

Installation of electrical equipment/connection of internal electrical connections;

External connection for power evacuation (BPC);

Testing and commissioning;

Landscaping works and demobilization of construction site, waste disposal etc.

#### Labor camp site

Accommodation

Kitchen

Dining area

Sanitation facilities

Septic tank

Liquid and solid waste disposal facilities

Generator(s), for operation when the power supply from the grid station will not available

#### Construction camp site

Uncovered material storage

Covered material storage

Parking for vehicles and plant

Batching plant (if required)

Generator(s)

Site offices

#### Workshop site

Workshop

Storage area

### Construction Machinery Requirement

For storing materials, stocking equipment and parking machinery and vehicles, the Contractor(s) shall require open and accessible sites close to the labor camps. The Contractor(s), at his own expense, but keeping in view his contractual obligations to honor the applicable national and international guidelines regarding level of pollution, shall make the arrangements.

The  outlines the approximate number of major machinery and vehicles that are envisaged to be required for the project construction works.

Table 3.9:Major Machinery and Vehicles

| Sr. No. | Machinery / Equipment | Quantity required* |
| --- | --- | --- |
|  | Excavators | 2 |
|  | Dumpers | 2 |
|  | Loaders | 2 |
|  | Power Generators | 2 |
|  | Tractor Trolley | 2 |
|  | Compactor / Roller | 1 |
|  | Crane | 1 |
|  | Concrete Pump | 1 |
|  | Vibro Hammer | 1 |
|  | Watering Tanks (moveable) | 1 |
|  | Cars/Pickups | 4 |

* Number of machineries is indicative and can be changed subject to working schedule.

### Solar PV System Installation

The work under this section will consists of supply, installation, testing & commissioning of the grid connected 8.4 MW solar PV power system over 35 acres of land at Sangjani Water Treatment Plant. The Contractor shall furnish all labor, materials, services and all skilled supervision necessary for the construction, erection, installation and connection of all equipment.

The solar PV power system equipment shall comply with following international standards wherever applicable.

IEC 60364‐7‐712: Electrical Installations of Buildings: Requirements for Solar PV power supply systems.

IEC 61727 or similar: Utility Interface Standard for PV power plants >10kW.

IEC 62103, 62109 and 62040 (UL 1741): Safety of Static Inverters‐ Mechanical and Electrical safety aspects.

IEC 62116: Testing procedure of Islanding Prevention Methods for Utility‐Interactive PV Inverters.

PV Modules: IEC 61730‐Safety qualification testing, IEC 61701‐Operation in corrosive atmosphere.

IEC 61215: Crystalline Silicon PV Modules qualification.

String/array junction boxes/: IP65, Protection Class II, IEC 60439‐1, 3.

The Inverter shall be rated for IP54.

The AC distribution boards shall be rated IP54.

The different components to be installed will be as follows:

#### Solar Modules

Power output: 90% power output at end of 10th year and 80% power output at the end of 25th year.

Solar photovoltaic panels will be used by the solar power plant for generating electrical power by converting solar radiation into direct current. This phenomenon takes place due to the photovoltaic effect exhibited by the semiconductors.

The solar PV modules shall be fixed firmly on top of the pole / structure using mounting structure, with suitable tilt and inclination to receive maximum sunlight.

The PV module shall be of half-cut mono‐crystalline technology.

The PV module vendor and its model shall be same for whole project.

Efficiency of individual PV modules shall be at least 19% or higher, under Standard Testing Conditions (STC).

A suitable number of solar PV modules shall be connected in a series string. A suitable number of series strings shall be connected in parallel to formulate a series‐parallel array.

The PV strings and array shall be designed to match the inverter input specifications.

Maximum DC output voltage of the array shall not exceed than as specified for inverter.

The module shall be provided with junction box with provision of by‐pass diodes.

Plug‐in connectors shall be external MC4 type or equivalent.

The front surface of the module shall consist of impact resistant and toughened glass.

The module frame shall be made of corrosion resistant material.

DC negative conductor shall be bonded to the ground.

#### Inverter

The inverter specifications are as follows:

Output: 97% or higher.

Product service: 20 years.

The KW ratings of inverters shall be chosen as per the PV system wattage, however vendor of all the inverters shall be same for whole project.

The sine wave output of the inverter shall be suitable for connecting to 400V, 3 phase AC LT voltage grid.

The peak inverter output efficiency shall exceed 97% and higher.

The nominal AC frequency tracking range shall be ‐ 6Hz to +5Hz.

Power factor:

Lagging 0.8 or higher

Leading 0.8 or higher

The inverter shall use transformer/transformer less topology.

The inverter shall have string fuse/ failure detection, ground fault detection, grid islanding protection, suitable DC/AC fuses/circuit breakers and voltage surge protection.

The inverter shall have internal protection against any sustained faults in DC inputs by providing DC reverse-polarity protection / reverse current protection and in mains AC grid circuits/lightning etc.

Inverter shall provide display of PV array DC voltage, current and power, AC output voltage and current (All 3 phases), AC power, Power Factor and AC energy (All 3 phases and cumulative) and output frequency etc.

The Inverter should have provisioning for remote monitoring.

Operating temperature range between 0 - 60°C.

Protection rating:

Electronics- IP66

Connection points/area - IP54

#### Charge Controller

Product service: up to 20 years.

#### Mounting structure for Solar PV array/tree

Structural & foundation: 20 years.

The proposed Solar PV system shall be designed to maintain the overall aesthetics.

The mounting structure for module array shall allow ventilation for cooling and also ease of cleaning and maintenance of panels.

Proper location and spacing of the PV modules arrays should be selected for minimum shadow effect.

Material for solar and module array support structure shall be fabricated using corrosion resistant material.

All the support structure welded joints shall be adequately treated to resist corrosion.

The support structure shall be free from corrosion when installed.

The structure of solar module array shall be designed so that PV modules can be placed and fixed with ease.

Screw fasteners shall use existing mounting holes provided by module manufacturer. No additional holes shall be drilled on module frames. Module fasteners/clamps shall be adequately treated to resist corrosion.

The support structure shall withstand the weight of solar PV modules installed on each solar modules on each PV array at buildings and other installed equipment.

The structure shall be designed to withstand operating environmental conditions for a period of minimum 20 years.

The structure should have provision for connecting the earthing cables.

#### Balance of System (BoS)

Product service: 20 years.

#### Equipment for solar PV system cleaning

The following equipment for cleaning of the solar PV panels will be installed:

1 HP electric water motor pump.

Flexible water pipe with stand (it shall have access to every solar panel)

Mid & low-pressure nozzles to splash water.

2 nos. of each min. 6 feet long retractable brushes & viper.

Brush fiber shall be made of polystyrene, soft enough to protect the photovoltaic cells, yet enough to remove debris.

High pressure cleaning equipment should not be used to clean solar panels as this may cause water to get under the glass.

#### Power Evacuation

The power generated will be connected to the national grid through the nearest substation to evacuate any excess power that is being generated and is not required to be exported onto the grid.

#### Site Drainage

To prevent any damage to the solar equipment from flash flooding and soil erosion, an effective watershed management and drainage system inside and outside of the site boundary will be implemented during the construction and operation phases. Run-off from precipitation will be collected and drained through a specified points or outlets at the project site.

#### Security Measures (Fencing and Lighting)

The solar equipment will be secured with chain link fencing up to 2.5 m high to prevent access; to ensure the safety of the adjacent local communities and to provide security for the solar photovoltaic panels. In addition to this, two security personnel will be deputed on rotation basis (all day and night hours, alternately) to prevent the entry of unauthorized persons to the site.

Figure 3.17: Conceptual design of Solar PV system installation at Sangjani WTP

### Construction Activities for Solar PV installation

#### Solar Power Plant Design, Installation and Supply Contractor

A Contractor will be selected to develop the solar power equipment on a Design, Installation and Supply basis through a competitive bidding process. The Design, Installation and Supply contractor will undertake the detailed design, carry out civil works, supply and install the solar power equipment, construct the power evacuation facilities to evacuate power at the grid connection point and set up the operation and control systems required for solar power operation. The Design, Installation and Supply contractor will also continue to be engaged during operation through warranties to rectify any issues arising from equipment defects or workmanship issues.

#### Temporary Facilities

Construction site offices, stores for fuel, oil and chemicals, storage/laydown areas for materials, plant and equipment, sanitation and welfare facilities including washrooms, cooking, and eating areas and worker camps will need to be temporarily set up within the allocated land, on completion of construction these will be removed from site.

#### Construction Methods

Construction of the solar power facility will be semi-mechanized. This means a combination of both machine and labor will be utilized. The use of machines will mainly be for earthworks and excavation and the installation of foundations. There is the possibility that soil compaction and/or piling will be required due to ground condition at the site, although it is to be avoided if possible.

#### Construction Materials

Construction materials will be required for the solar power equipment, including gravel for any access ways that might be required, concrete for foundations, hot dip galvanized steel or aluminium for solar PV module tables and concrete, reinforcement bars and bricks etc. No borrow areas or quarries will be opened to provide construction materials. Solar PV modules and electrical equipment meeting the technical specification will be procured from licensed manufacturers. No CFCs, PCBs or asbestos containing materials will be used in construction.

#### Manpower requirement

At most, 60 construction workers are expected to reside at the solar power facility during the construction period. Skilled workers will need to come from outside the local community due to the need for them to have suitable qualifications and experience to work on construction site, but unskilled manual labor may be sourced from the local communities.

#### Power Requirement

The project site has grid connection available from the Rawal lake water filtration plant which will be available for use, although the Contractor may choose to use temporary diesel generator set during construction as backup power.

#### Water Requirement

There will be water required during construction for drinking and dust suppression. During the construction, it is estimated about 60 laborers will be stationed at the project site requiring about 10 cubic of water. For human consumption (drinking, bathing, cooking, toilet etc.) alone, the estimated water requirement is about 10 m3 per day.

### Operation phase of Solar installation

#### Solar Power Plant Owner and Operator

SCADA system will be installed for remotely controlling and monitoring the solar power equipment. The Rawalpindi WASA staff will be trained by the Design, Installation and Supply contractors to operate, maintain, and carry out repairs of the solar power facility. However, some of the operational activities, such as, cleaning of solar photovoltaic panels and maintenance and service of key components such as connector boxes, switchgear, and transformers may be assigned to third party contractors.

#### Manpower Requirement

Once the construction activities are completed, only a few people will be required for operation and maintenance of the plant. Only 3 employees for this facility are expected to be engaged.

#### Power Requirement

The project site has grid connection available from the Sangjani water treatment plant which will be available for use in case of any emergencies.

#### Water Requirement

The main consumption of water is for cleaning of the solar PV modules with minimal requirement for domestic usage. Cleaning of all modules will be done only once per month. It will be possible to clean all the panels by using requiring 200 m3 of water on the basis of 24m3 of water per MW of installed solar PV.

### Decommissioning

The lifespan of the solar plant is 25 years (solar photovoltaic panels typically last 25-30 years) after which decommissioning will be initiated by Rawalpindi WASA. Decommissioning will involve removal of the solar plant infrastructure including all equipment, structures, foundations, access paths and fences; disposing of or recycling its components; and restoring the land to its original state. Rawalpindi WASA is anticipated to appoint a third-party contractor to undertake decommissioning work.

Decommissioning activities will be like those involved during construction, the main difference being that there will be many solar PV panels and transformers containing oil to dispose of in an environmentally safe and sound manner. The Rawalpindi WASA focal staff will seek guidance from the concerned EPA staff on the disposal of hazardous waste. Recyclable waste can be sold to licensed waste collectors who should reuse/recycle or dispose of the waste according to type to suitably licensed and engineered waste management facilities. If at the time, Rawalpindi WASA is to pursue an alternative scenario such as repowering using new solar PV panels, this will be subject to national environment laws and regulations in force at the time.

### Construction Materials Requirement

The common source of the material required for civil works are described in Table 3.10 below.

Table 3.10: Construction Material and Source of Raw Material

| Sr.# | Raw Material | Estimated Quantity | Source |
| --- | --- | --- | --- |
| 1 | Earth Material | 50,126 Cft | Available locally, borrowed from the lands acquired for the project. |
| 2 | Aggregate | 263,023 Cft | Available at many sources within the vicinity of the site. No aggregates shall be sourced from river or stream beds. |
| 3 | Riprap material | 12,100 Cft | Will be taken from local suppliers |
| 4 | Sand | 66,516 Cft | Sand is available in local market. |
| 5 | Water | 5000 lit/day | Water will be taken from existing network of water supply. |
| 6 | Cement | 24,899 Cft. | Ordinary Portland Cement is suitable, which is available at various factories in Pakistan mainly from Kalar Kahar and local market. |
| 8 | Energy | Given in relevant section | Electricity supplies are available at the site through IESCO. |

### Operation Phase Details for Proposed project

#### Scope of Activities

The activities to be conducted during the operational phase of proposed project are provided below.

24/7 Operation at OHRs and GSTs to provide uninterrupted supply of water.

Regular inspection of Sluice valves and air valves

Condition monitoring of Conductance Main and Distribution lines

Provision of spare parts and consumables

Repair/refurbishment arrangements.

Revenue collection for water supply from residents of UCs falling under jurisdiction of RWASA.

### Operation Equipment and Machinery

Equipment used during operation of the proposed sub-project will be Pumps and auxiliary machinery to maintain pressure in pipelines and lifting water from GSTs to OHRs, leakage inspection tools, water meters to be installed, machinery to be used to discard the existing tube wells.

### Manpower Requirement

It is expected that existing organizational capacity of RWASA may be sufficient to handle the components of proposed project. However, additional staff may be required for inspection and maintenance of DMAs, Domestic Meters, Control Valves etc. it is estimated that around 10 to 15 additional staff will be required during operation phase.

### Cost of Project

The estimated cost of the proposed project has been calculated based of the scope of work. The Table below provides the breakup of the project cost.

Table 3.11: Cost of the Project

| Sr. No. | Description | Qty. | Unit | Rough Cost Estimate |
| --- | --- | --- | --- | --- |
| 1) | Proposed Conductance main (OD) |  |  |  |
| i) | 710 mm HDPE PN-10 | 3100 | m | 167,662,217 |
| ii) | 500 mm HDPE PN-10 | 250 | m | 7,002,424 |
| iii) | 450 mm HDPE PN-10 | 500 | m | 11,546,282 |
| iv) | 355 mm HDPE PN-10 | 450 | m | 6,871,114 |
| v) | 315 mm HDPE PN-10 | 1200 | m | 14,956,625 |
| vi) | 280 mm HDPE PN-10 | 450 | m | 4,231,406 |
| vii) | 200 mm MS SCH - 40 | 400 | m | 6,731,415 |
| 2) | Replacement of Water Supply Distribution Pipelines (OD) |  |  |  |
| i) | 90mm HDPE PN-8 | 5600 | m | 14,107,466 |
| ii) | 110mm HDPE PN-8 | 200 | m | 587,041 |
| iii) | 180mm HDPE PN-8 | 3100 | m | 14,807,527 |
| iv) | 225mm HDPE PN-8 | 1100 | m | 5,253,596 |
| v) | Provision for unforeseen laying (10% of the Item. 2) | 1 | Job | 3,475,263 |
| iv) | House connection | 11000 | Nos | 36,045,900 |
| v) | Commercial Connection | 550 | Nos | 1,802,295 |
| 3) | Additional Pipeline for Unserved Area (OD) |  |  |  |
| i) | 90mm HDPE PN-8 | 850 | m | 1,539,870 |
| 4) | Installation of pipe sleeves for Road Crossings | 130 | m | 1,808,198 |
| 5) | Installation of Water Meter |  |  |  |
| i) | Domestic 20mm dia | 11000 | Nos | 249,287,500 |
| ii) | Commercial 25mm dia | 550 | Nos | 12,464,375 |
| 6) | Installation of Flow Meter |  |  |  |
| i) | Bulk Flow meter i/c chamber (OD) |  |  |  |
| a) | 1016 mm dia | 1 | Nos | 1,441,843 |
| b) | 710 mm dia | 1 | Nos | 1,441,843 |
| c) | 600mm dia | 1 | Nos | 1,441,843 |
| d) | 315mm dia | 3 | Nos | 4,325,529 |
| e) | 300mm dia | 3 | Nos | 4,325,529 |
| f) | 280 mm dia | 3 | Nos | 4,325,529 |
| g) | 200 mm dia | 7 | Nos | 10,092,901 |
| ii) | at TWs 110 mm dia | 31 | Nos | 44,697,133 |
| 7) | Installation Charges of Bulk/Domestic Meters | 1 | Job | 53,719,715 |
| 8) | SCADA Solution Package for Metering | 1 | Job | 12,000,000 |
| 9 | Installation of isolation (sluice) valves i/c Chambers |  |  |  |
| i) | 3" i/d (75 mm) | 1 | Nos | 158,143 |
| ii) | 4" i/d (100 mm) | 6 | Nos | 959,253 |
| iii) | 8" i/d (200 mm) | 14 | Nos | 2,638,035 |
| iv) | 10" i/d (250 mm) | 3 | Nos | 620,590 |
| v) | 12" i/d (300 mm) | 2 | Nos | 709,809 |
| vi) | 18" i/d (450 mm) | 1 | Nos | 1,086,525 |
| vii) | 24" i/d (600mm) | 1 | Nos | 436,276 |
| viii) | 28” i/d (700mm) | 1 | Nos | 485,677 |
| ix) | 36" i/d (900mm) | 1 | Nos | 584,479 |
| x) | 40" i/d (1000mm) | 1 | Nos | 633,880 |
| 10 | Installation of Air Release Valves i/c Chambers | 6 | Nos | 930,456 |
| 11 | Fire Hydrant dia 180 mm | 6 | Nos | 1,181,933 |
| 12 | Pipe Jacking for bulk supply line |  |  |  |
| i) | 900mm dia MS Sleeve | 190 | m | 61,794,566 |
| 13 | Connection with Existing 1000 mm dia Line | 1 | Job | 25,344 |
| 14 | Installation of pumps at Sangjani WTP | 2 | Nos | 308,729,205 |
|  | Civil works | 1 | Job | 5,000,000 |
| 15 | Installation of pumps at GST/OHR in KSS |  |  |  |
| i) | Centrifugal pumps - 1 cusec | 4 | Nos | 18,855,229 |
| ii) | Turbine pumps - 1 cusec | 10 | Nos | 59,269,955 |
| 16 | Thrust Block 2m x 1m x 2m | 200 | Nos | 9,445,680 |
| Total Cost (PKR) |  |  |  | 1,171,534,414 |
| 5% for Restoration of utilities, Env protection works, Traffic diversions and unforeseen |  |  |  | 58,576,721 |
| Grand Total Cost (PKR) |  |  |  | 1,230,111,135 |
| Total Cost (Million PKR) |  |  |  | 1,230 |
| Total Cost (Million USD) |  |  |  | 4.43 |

### Electricity Required

Keeping in views the nature of project, estimation of electrical requirement is difficult at this stage specially for construction phase. However, the contractor/s will use their own generators for electricity supply during construction phase.

### Water Required for Construction and Operation

Approximately 50-60 workers will be involved in different phases of construction of each component of proposed project. The major component which required larger number of workforce is Laying of New Conductance Main and Replacement of AC/GI Pipelines. It is estimated that per worker use of water will be 100 liters per day. An average of 5000 liters/day water will be required during construction phase while during regular operation phase about 1500 liters/day water will be required.

### Generation of Solid Waste

The major source of solid waste is construction waste during construction phase. Approximately 552,664 Cft. construction waste will be produced by excavation of soil for lying of new pipelines and replacing of old distribution pipelines. Almost 80% of this material will be reused in backfilling the excavations while remaining will be used for backfilling of low laying areas by contracting with private housing societies.

Other source of solid waste will be from Worker Camps. It is estimated that 150 workers will be involved during the construction phase. As per Design criteria of Public Health Engineering for water supply, sewerage and storm water drain, the per capital and per day generation of solid waste is 0.6kg. It is estimated that 15 workers will be residing in the camps and estimated quantity of solid waste will be 90kg. Negligible quantity of solid waste will be generated during regular operation of proposed project as it does not involve any processing or manufacturing activities. Based on the number of employees, it is estimated that 5 to 6 kg solid waste will be produced during operation phase.

### Generation of Wastewater and its disposal mechanism

The estimated quantity of wastewater from worker camps will be 6000 liters per day. Major source of wastewater from worker's camps will be toilets and kitchens. The wastewater will be treated at septic tank and the treated wastewater will be discharged into nearby drain by submerged pipes to avoid spillage of wastewater. The installation of septic tanks at camp sites will be responsibility of the contractor. PMU will ensure that treatment process is followed and no wastewater is discharged untreated. Wastewater generation during regular operation will be about 1000 liters per day.

## ANALYSIS OF ALTERNATIVES

### Overview

Project alternatives are studied as a part of this IEE process. Alternative’s analysis has been conducted in detail to foresee environmental, economic and social impact of each alternative. This section also provides an overview of various commercially available technological options and siting of proposed project to make the project environmentally sound.

Project alternatives has been studied keeping in view number of parameters including no project option, single bulk supply option, technological / selection of material, construction of new storage facilities.

The designing of proposed project component is based on the increasing water demand of KSS with respect to increasing population. The project is designed to cater the water demands of up to 2050.

This process of analysis of the different alternatives for Augmentation of Water Supply and Creation of DMAs for KSS ensures that a well-informed decision is taken regarding the selection of the most optimal option, amongst the possible options that are brought into consideration.

The discussion and analysis of alternatives considers all practicable strategies that will promote the elimination of negative environmental impacts, identified. This section is critical in consideration of the ideal development with minimal environmental disturbance.

The report identifies potential environmental impacts of each alternative, develops comparative analysis matrix with respect to components of augmentation of water supply project and environmental impacts to suggest the most feasible alternative that can be taken up for development.

### Design and Siting of Alternatives

The alternative for following project components have been discussed:

Alternative I: 		No Project Option

Alternative – II: 		Capacity Enhancement of Existing Conductance Main

Alternative III: 		Lying of new Conductance Main

Alternative – IV: 	Construction of New OHRs and GSTs

#### Alternative-I 'No Project Option'

The “no project option” means no action alternative is required to ensure the consideration of the original environment without any development. This is necessary for the decision-makers in consideration of all possibilities. The development will have a minimal effect on the physical environment. In terms of the social environment, the “no-action” alternative would result in shortage of clean water to residents of KSS, eliminate job opportunities, create insecurities of future water demand, cause provision of contaminated water supply due to leaked distribution lines, uncontrolled and unmanaged water usage, losses of collection revenue to RWASA.

The old distributions pipelines were laid down back in early 1990's and some of them are even 33-year-old. These pipelines are leaking at certain points and result in contamination of water supply to residents. The old lines are rusted and lower the water quality as well.

The existing Conductance Main, from Tomar reservoir is insufficient to cater the increasing water demand of KSS. If new Conductance Main is excluded from the option, then KSS will face extreme water shortage by 2030.

Due to leakage from the replaceable Distribution Pipelines, leakage water cause damage to roads and other permanent structures like houses and shops. The no project conditions will result in further worsening the present environmental conditions and increased disturbance to residents of area. Socio-economic conditions will also be deteriorated due to expenditure on repair work of structures and medical expenses to cure diseases caused by drinking of contaminated water. This moisture on walls and basements will also cause algal and fungal growth which will result in different diseases.

#### Decision

The “No project option” reveals the degraded water supply and seepage on walls and basements will cause diseases to residents, extreme water shortage in coming years and loss of revenue to RWASA, therefore is not recommended.

#### Alternative-II ‘Capacity Enhancement of Existing Conductance Main’

#### Strengths and Opportunities

The strength to this option is that existing conductance Main will be replaced with line of bigger diameter within existing ROW. No additional land will be required and the same crossing under IJP road will be used. No major disturbance will be caused to any physical, biological and social part of the environment.

#### Weaknesses and Threats

During the time of replacement of existing Conductance Main, the residents of KSS will not receive water supply until completion of project and this will cause serious social impact on residents. More cost will be involved in removal of existing conductance main and laying of new conductance main.

#### Decision

This option is not recommended because people of KSS will suffer with no water supply for long period of time. This option is not cost effective as well, so this option is rejected.

#### Alternative-III ‘Lying of New Conductance Main'

#### Strengthen the Opportunities

If one additional Conductance Main of 620 mm (ID) or 710 mm (OD) HDPE is laid down parallel to the existing Conductance Main, will enhance the water capacity of KSS without any halt of water supply to people of KSS. It will also ensure the continuous supply to KSS even in case of repair and maintenance of one Conductance Main. ROW is also available to lay new Conductance Main and no land acquisition involved. New line will fulfill the demand of water for next 30 years even in case of failure of Existing Conductance Main due to any reason.

#### Weaknesses and Threats

It will investment of major cost involves which require large amount of funds. Further, new excavation will cause temporary halt in free movement of people of project area specially along the Corridor of Impact (CoI).

#### Decision

This alternative option is selected because it will continue the supply of water to the residents even during construction of new line for water capacity enhancement of KSS. Further, funds are available under Dream-I project of Local Government and Community Development Department Government of Punjab. Diversion routes will be allocated for movement of traffic under traffic management plan.

#### Alternative-IV ‘Construction of New OHRs and GSTs'

#### Strengthen the Opportunities

The strength to this option is that increase in water supply needs additional storage to make sure 24/7 water supply to residents of KSS. For this construction, existing Parks of PHA are proposed. The area of KSS is densely populated with no open land available. If private land acquisition will be involved it will involve the involuntary resettlement and dislocation. Therefore, as alternate option parks of PHA are proposed.

#### Weaknesses and Threats

The parks are used by residents for walk and jogging. These parks are recreational place for large number of people as large population use them. During public consultation the respondents replied that patients with diseases related to heart and diabetes use these parks for their recommended exercise and walks to maintain their health. Further, there is only one park dedicatedly for women titled as Ladies park which is proposed for construction of OHR and GST. Furthermore, these parks are the only green places of KSS with ornamental plants, grass land and trees.

#### Decision

Based on the social and environmental impacts, the option for construction of New OHR and GSTs in Public Parks of PHA is rejected. It will cause social disturbance to people. Already available water storage facilities will be utilized with additional utilization on household storage tanks.

#### Comparison Analysis of Alternatives

The comparison between three alternatives based on environmental, social and economic impacts is provided in the table below.

Table 4.1: Comparison of Alternatives

| Parameters | New Conductance Main | Construction of New OHRs & GSTs in Parks | Rehabilitation of Existing Conductance Main |
| --- | --- | --- | --- |
| Cost | For new Conductance Main large amount of funds required which are available under Dream-1 project | Huge amount of funds is required for this construction. However, RWASA is already constructing new OHR and GSTs. Therefore, this will be dead investment to build additional OHRs and GSTs | Rehabilitation of existing Main involve additional cost for removal of existing Conductance Main. |
| Environmental Aspects | No disturbance to any protected area or national reserve. No cutting of trees and disturbance to local fauna and flora. Only small green strip will of Sector I-10 will be disturbed. | Tree cutting in parks, deterioration of green lawns and ornamental plants of parks. | No major environmental Impact envisaged as existing Right of Way (ROW) will be utilized. |
| Social Aspects | No involvement of land acquisition so no involuntary resettlement or dislocation involved. Temporary halt in movement of people will occur. | Social issues will be created by disturbing existing parks which are used by people of KSS for jogging and walks. Construction activities may involve serious encounters between public and workers | During rehabilitation/ replacement of existing conductance, water supply to locals will be shut down for many days which is not acceptable. |

#### Decision

Thus, option for lying of new Conductance Main was selected with minimum disturbance to local environment, therefore is recommended. It will fulfil the water needs of KSS for next 30 years (till 2050).

### Options for Water Meters

A key element of the sustainability of water utilities is accurate measurement of customer water consumption. There are various types of water meters, Mechanical and Static types are widely used throughout the world. The specifications and use of these water meters varies. The Static Water Meters are selected to be installed in KSS under the proposed project based of their specifications.  The specifications of both the types are given here;

#### Mechanical Water Meters (Option-1)

The specification of Mechanical Water Meters is given below;

#### Technology

Volumetric Rotary Piston/Notating.

Multi Jet.

#### General Features:

Possibility of both Visual and Automated Meter Reading outputs.

Tamper proof.

Provided with non-return valve.

Un- affected by grit and particulates.

Provided with wire and lead seal.

Not affected by Magnetic field.

#### Certifications:

ISO: 4064 (International Organization Standardization) compliant

OIML: R49 (International organization for legal Metrology) and

MID/2004/22/EEC/AWWA/JWWA

#### Detailed Metrological Specifications:

Accuracy:				Class II

Material:				Brass/Nonferrous Metal

Protection Class:			I P-68

Ambient Temperature:			+50 to 55 0C

Liquid Temperature:           		upto 50 0C

Maximum Working Pressure: 		16 bar

Flow rate: `Q3/Q1			160

Installation:				Any Direction and ii. Horizontal (MultiJet)

Useful Life:				15 years

#### Static Water Meters (Option-2)

The specification of Static Water Meters is given below

#### Technology:

Ultrasonic Technology

Electromagnetic

Fluidic Oscillation

#### General Features:

No moving parts

Both Visual (LCD) with or without Automated Meter Reading outputs

As per class 2 of OIML/ISO specifications (current)

Tamper Proof

Un-affected by grit and particulates

No reverse flow measurement

#### Certification:

ISI 4064 complaint

OIML: R49 (International organization for metrology) /MID /2004 /22/ EEC/ AWWA /JWWA

Detailed Metrological Specifications:

Accuracy:				Class II

Material:				Brass/Nonferrous Metal

Protection Class:			I P-68

Ambient Temperature:			+50 to 55 0C

Liquid Temperature:           		upto 50 0C

Maximum Working Pressure: 		16 bar

Flow rate: `Q3/Q1			160

Installation:				Horizontal or vertical position

Battery Life:				10 years

### Selected Option:

Based on the best technological option, Static Water Meters have been selected for installation in KSS.

### Siting the Contractor’s Facilities at Appropriate Site, Be it Public Land or Private Land

#### Strengths and Opportunities

The option will create an operationally adjustable opportunity wherein the best site will be chosen by the contractor. Of course, all sites will be approved by the Engineer.

The contractor will not be bound to remain restricted to the sites proposed by the client Engineer and can plan his operations according to his operational convenience and availability of facilities and services. Where enough state land is not available, and private land is available adjacently, the contractor can make necessary adjustments without changing the site.

#### Weaknesses and Threats

To the extent that the contractor moves out of state lands, establishment of camps, yards and facilities close to markets or Environmental Sensitive Receptors can cause social problems. Adverse effects may be caused on movement of local population. Possibilities of spread of Dengue, COVID-19 and other infectious diseases will get greater chances of spread from construction camps.

#### Decision

Settling the contractor on state land is the best option. But no state land available in KSS. Therefore, sitting of camps at private land is preferable but, client holds no responsibility for contractor’s cost and commitments.

### Labor

#### All Local Labor

- Strengths and Opportunities

All labor will be local and their employment will fulfill a major social requirement of the contract. Local economy will benefit and poverty alleviation will take place. The project will become socially acceptable. A big advantage of local labor will be that most of them will come from and go to their homes daily and there will be very little pressure on labor camps.

Weaknesses and Threats

Three types of labor will be required; unskilled, semiskilled and skilled. All types of labor cannot be available locally. So binding contractor to employ all labor locally can put limitation on contractor.

Decision

It is not possible to rely entirely on local labor for all trades. So this option is not acceptable. However, all unskilled labor openings must be given to locals.

#### All Labor from Outside

- Strengths and Opportunities

Labor from outside will be pre-selected and economically sound. More efficient more dependable and well-trained man power will be available. The labor will not be able to exert undue pressure on the contractor or the project. The outside labor will set model for untrained labor which may improve. Spending by outside labor will bring additional benefit, to local economy.

Weaknesses and Threats

Bringing outside labor will take away economic benefits of employment from the local communities and they will not develop a sense of ownership for the project. The project objective of poverty alleviation, social uplift and capacity building will be defeated. Serious social problem can also be created.

Decision

In case the labor is brought from outside, the social losses will be much higher than the economic gains. However, the ergonomic limitations may necessitate bringing in a small percentage of outside manpower especially at skilled level. But as far as possible the local human resource will have to be preferred.

#### An Admixture of Local and Outside Labor as per Skill Required

Strengths and Opportunities

All (100%) unskilled labor can be hired locally. Semiskilled labor can be local as per their skill-wise availability, may be up to 50% of the total. Most of the senior technicians and skilled workers will have to be brought from outside, if not available locally. This flexibility will be possible only under the option of admixture of labor.

- Weaknesses and Threats

Because of the presence of the outsiders there are likely to be social losses. Income of local communities will reduce.

Decision

From the project implementation point of view, this option presents the best combination of local and outside labor. This option also presents an opportunity of transfer of skills from outside technicians to local workers.

### Technology Alternatives

#### Material for Replaceable Distribution Lines

A comparison of materials was carried out for replacement of AC and G pipelines. Initially it was proposed to replace the lines with same materials. Detail analysis of available option was carried out and the following conclusion was drawn;

Asbestos Cement has been banned due to its negative impacts on environment and health of workers.

Galvanized Iron pipes will be rusted after few years and there will be leakage from them causing contamination of water supply to users.

HDPE has long life and is corrosion resistant. It is easy to use and easily available in local market. Further, it will be cost effective at longer run.

#### Decision

It is concluded that for improvement of water supply in KSS social and environment friendly technologies are required to avoid any potential negative social and environmental impacts. Hence, the best material option for New Conductance Main and replacement of outlived pipelines will be HDPE with long working life and safe for drinking water supply. Further, to avoid cut of IJP road, technological option of jacking the pipe under IJP road will be used.

### Solar PV Installation Component

#### With’ and ‘Without’ Project Scenario

The ‘No project’ scenario is not a viable option since Pakistan is facing an energy deficit and needs to urgently switch from the cost intensive and environmentally unfriendly diesel-based power generation and move towards developing renewable based cheap and reliable energy on a long-term basis.

Also, the benefits from the addition of this solar installation project are primarily in terms of offsetting energy production primarily from fossil fuel-based generation (resulting in fuel savings), but also include significant benefits from more efficient operation of diesel generators, less diesel generator maintenance and depreciation and delayed purchase of new generator capacity in later years.

The Table 8.1 below presents a comparison of the ‘with project’ and ‘without project’ scenario.

Table 8.1 – ‘With’ and ‘Without’ Project Scenario

| No. | Parameter | With Project Scenario | Without Project Scenario |
| --- | --- | --- | --- |
| 1 | Electricity | Major benefits are as follows: - Improve the electricity supply and stability of the national grid; - Diversify power generation mix; - Increase percentage of clean energy; - Supply less expensive than current oil-based generation; - Improve electricity supply and stability of the national grid. | Negative effect, increase in greenhouse gas (GHG) emissions due to continued reliance on inefficient oil-based energy generation. |
| Environment |  |  |  |
| 2 | Effect on protected, sensitive or forest areas | No effect, avoids national protected areas, environmentally sensitive and forest areas. | No effect. |
| 3 | Effect on endangered species | No endangered species identified in project area. | No effect. |
| 4 | Tree Cutting, vegetation removal. | No major effect anticipated. | No effect. |
| 5 | Air emissions | Minor effect during construction; increase in air emissions because of construction activities / trenching, dust generation. Major effect during operation: Decrease in GHG emissions. | Negative effect, Increase in GHG emissions due to continued reliance on fossil-fuel based energy generation. |
| 6 | Water Supply | Minor effect during construction and operation; Increase in groundwater withdrawal compared to without project scenario. Shall comply with project EMP as prescribed in clearances, permits and licenses obtained from relevant authorities. Major effect during operation; improved water accessibility for different purposes due to relevant power available for pumping etc. as required. | No effect. |
| Social |  |  |  |
| 7 | Disturbances to people/ communities. | Minor effect during construction phase; temporary and localized impact due to power outage, dust and noise due to movement of machinery and construction vehicles; shall comply with project EMP (community health and safety plan, traffic control and road management plan, noise and dust control plan) Influx of migrant workers; shall comply with project EMP (emphasis on local hiring). | No issue. |
| 8 | Effect of business. | Construction activities may employ local populace generating economic and livelihood generation opportunities | No issue. |
| 9 | Status of living. | Major effect, improved access to electricity, reduced domestic load for persons such as women involved in cooking activities or in accessing water supplies. | No change. |
| Economic |  |  |  |
| 10 | Economic Development | Greater rate of economic development expected. | Slow development. |

## Description of Environment

### General

This section provides the description of the baseline conditions along the proposed Project. Considering the potential impacts of the Proposed Project, existing baseline environmental conditions of the proposed project’s Corridor of Impact (CoI) has to be used as a benchmark for comparison of the physical, ecological and socio-economic conditions before and after construction phases of the Project. This baseline will also provide the datum for assessing the impacts and suggesting the mitigation measures, which will be implemented effectively at various phases of the proposed project activities.

The description of the environment is site-specific and includes the information of physical environment including topographic condition, geological setting, water quality (surface water and groundwater), ambient air quality, climate of the area, land use, description of ecological habitats, located in the overall environmental study area and socio-economic conditions of the area.

With due regard to baseline environmental conditions, the impact of project interventions is addressed and mitigation measures are proposed in the foregoing sections. The baseline information also assists in identifying specific issues to be monitored during project implementation as well as during the operational phase.

### Site Visit for baseline visit

The data presented in the forthcoming sections has been collected from the primary and secondary sources. For primary data acquisition, the Environment and Social team conducted the field visit during the months of December 2022.

The major objective of the field visits was to collect the baseline data on physical, eco-biological and social aspects along with identification, assessment, and categorization of the significant environmental and social impacts of the proposed Project. The required information/ data of a baseline study was classified into two main categories i.e. Primary data and Secondary data. The primary data was collected directly from the concerned communities and affected households while secondary data was the information gathered through secondary sources such as PMU, EDCM, census report, research publications, topographic sheets, other published data related to the project such as wildlife, livestock, fishery, forestry, agriculture, horticulture, climate etc.

- Corridor of Impact: The proposed areas where the project activities will be carried out is highly populate areas of Islamabad and Rawalpindi. The Project Corridor of Impact (COI) based on nature and scope of the project is referred to all those areas in the region within 100m from the project line and which may be affected directly or indirectly by the project activities. The COI is applicable for physical environment including the environmental sensitive receptors like educational institutes, health facilities and areas of religious importance. This COI is not applicable for biological and socioeconomic environment.

Right of Way:  The area of actual working space is called as Right of Way (ROW). The required ROW for proposed project is estimated to be 8 feet.

Figure 9.1 shows the location of project marked with COI.

This Chapter gives an overview of the environmental and social baseline information of proposed project area, with special reference to impacts on the CoI. The information has been compiled from primary and secondary sources of data. Major environmental components are:

Physical Environment

Biological Environment

Socioeconomic Environment

### Physical Environment

The following section provides an overview of the information on physical environment of the data, collected from primary as well as secondary sources. The major parameters covered include Topography, Geology, Seismicity, Soil, Climate, Water Resources, ambient Air Quality & Noise, Solid Waste and Land-use along the proposed project. Figure 5.1 shows location map of Rawalpindi-Islamabad showing geographical locations of the area.

Figure 5.1 Location of proposed project marked with COI

Figure 5.2 Geographical Map of Rawalpindi-Islamabad

### Topography

The terrain in the metropolitan area of Islamabad-Rawalpindi consists of plains and mountains whose total relief exceeds 1,175 m. Three general physiographic zones trend generally east-northeast. The northern part of the metropolitan area lies in the mountainous terrain of the Margala Hills, a part of the lower and outer Himalayas, which also includes the Hazara and Kala Chitta Ranges. The Margala Hills, which reach 1,600-m altitude near Islamabad.

South of the Margala Hills is a southward-sloping piedmont bench underlain primarily by folded sandstones and shales of the Rawalpindi Group (Miocene). Although the relief of the piedmont area is generally low and dominated by extensive plains of windblown silt, the piedmont area also includes many ridges and valleys that have been buried by alluvial deposits from the hills.

In the southernmost part of the area, the Soan River valley extends generally along the axis of the Soan syncline at an altitude of about 425 m.

The site for Conductance Main is mostly plain with gentle slope towards KSS from Sector I-10 area which will provide gravity flow to water. The other component of  the project, replacement of distribution pipelines, creation of DMAs, installation of pumps and auxiliary machinery etc. are also present in flat area of KSS.

### Land use of the Proposed Project

There are different types of land use i.e. open land near Sector I-10, built-up, graveyards, green belt, road/track, stream/ Nullah, residential and commercial structures, and parks etc. majority of the project area is densely populated. Figure 5.3 shows the land use around the project site.

Figure 5.3 Land Use around the project site

| Area for lying of new Conductance Main | Shabbir Road, Khyaban-e-Sir Syed |
| --- | --- |
| School at KSS | Park at KSS |
| Graveyard in KSS | Sector 3, KSS |
| Area under Islamabad |  |
| Service Road Sector I-10 | Sector I-10 |
| IJP Road |  |

### Soils

The same is true for various faunal species, which are important for adding the nutrients in the soil by the breakdown of organic matter and making it suitable for the agriculture. The project area does not have any valuable minerals. The soil of Rawalpindi is different in character and generally inclined to be dry.

However, it is rich in potential plant nutrients. The clay in the region has five distinct strata. Coarse pebbles together with sand and clay make up the lowest layer which is of alluvium nature deposited by older system of Soan Basin over the pebble bed. The alluvium deposits same river system comprises the subsequent layers. The air borne top layer of silt or clay is called loess and the top most one is gravely conglomerate

### Geologic Setting

The area in and around Rawalpindi and Islamabad has a complex geological history of mountain formation, alluvial-loessic deposition and erosion cycles. In the west of the Potohar Uplands, the main depositions of loess are from the Pleistocene period. Streams and ravines cut the loess plain, affected by gully erosion and steep slopes. This type of land is unsuitable for cultivation. The areas are composed of sandstone and limestone. There is an extensive area with exposed bedrock and fragmentary, thin soil formations.

The dominant factor controlling the geology of the Islamabad-Rawalpindi area is the convergence of the Pakistan-India and Eurasian tectonic plates and the collision between the plates that began about 20 million years ago. This process produced complex structure and stratigraphy in the Islamabad-Rawalpindi area.

The sedimentary rocks of the Islamabad area record a long period of gentle geologic fluctuations and slow deposition while the Pakistan-India tectonic plate drifted northward across the Indian Ocean, followed by much more vigorous tectonic processes and rapid deposition in the shorter period since the Pakistan-India and Eurasian plates converged. Consequently, the 150-million-year (m.y.) period from deposition of the Samana Suk Formation (Middle Jurassic) to the beginning of deposition of the Murree Formation (lower Miocene) is represented by only about 675 m of primarily marine sedimentary rocks, whereas the last 20 m.y. are represented by more than 7,572 m of continental sedimentary rock

The gravel and loess are especially important to the environmental geology because they form most of the building foundations and because gravel is the primary ground-water aquifer. West of Rawalpindi, plains of thick, easily eroded loess is extensively dissected into shallow badland valleys. East of Rawalpindi, the folded ridges of Rawalpindi Group rocks rise above the alluvial cover to form prominent hills. Urban development is concentrated in the piedmont bench area, which is little dissected in its northern part, where Islamabad is located, but is more deeply dissected toward the south near the Soan River, where Rawalpindi is located.

Important minerals like limestone, marble, and fireclay are found in the area. Limestone is found abundantly in Margalla Hills and marble in the western section of Margalla range. Limestone is common in the lower hills in Rawalpindi Tehsil, and deposits can also be found in the Kankar Plains. Clay soils in the District exhibit five strata, from bottom to top: (i) coarse pebbles with sand or clay; (ii) an alluvial stratum deposited by an older river system in the Soan Basin, (iii) alluvial deposits of the present river system (iv) an airborne top layer of silt or clay (loess), and (v) conglomerate and loose gravel deposits.

### Water Resources

Water resources can be categorized as groundwater resource and surface water Resource. Following is the detail of both resources.

### Groundwater

A supplemental network of municipal and private wells as deep as 200 meters (m) produces ground water primarily from quaternary alluvial gravels. The altitude of the water table decreases from about 600ft at the foot of the Margala Hills to less than 450ft near the Soan River, so that the saturated zone generally lies 2–20 m below the natural ground surface. The strata of the project area are water bearing and alluvial deposits, giving groundwater potential throughout the district.

The water table is seasonal at some places and dug wells generally run dry during summer season at some parts of Rawalpindi district. The local population is generally reliant on water supply from RWASA. Some people have installed their own bore holes and use water from them. The chemical quality of ground water in the district varies in different areas and at different depths. The results of analysis of water samples taken from two locations of KSS are shown as Annexure-II of the IEE Report.

### Surface Water

The proposed project area is divided into two main portions, one is area for Laying of New conductance main from 620mm to 900mm Distribution Main of Tomar Reservoir, starting from Service Road West of Sector I-10 of Islamabad and passing through IJP Road. The other major components of project fall in Khyaban-e-Sir Syed (KSS) including sub-division of Bulk Water Supply line to connect it with all water works of KSS.

The Soan and Kurang Rivers are the main streams draining the area. Their primary tributaries are the Ling River, draining northwestward into the Soan; Gumreh Kas, draining westward into the Kurang from the area between the Kurang and Soan; and Lei Nala, draining southward into the Soan from the mountain front and urban areas. The Kurang and Soan Rivers are dammed at Rawal and Sambli Lakes, respectively, to supply water for the urban area.

Nullah Lai has three major tributaries (Saidpur Kas, Tenawali Kas, and Bedarawali Kas) in the administrative jurisdiction of Islamabad. In the jurisdiction of Rawalpindi below Katarian Bridge, other three major tributaries (Nikki Lai, Pir Wadhai Kas, and Dhoke Ratta Nullah) fall into the Nullah Lai. In addition, there are six other drainage channels joining the Nullah after the Dhoke Ratta and finally the Nullah falls down in the Soan River.

Nullah Lai basin drains a total area of 235 km2 south of Margalla Hills, with 70% of the watershed falling within the territory of Islamabad and the remaining 30% within the downstream Rawalpindi municipal and cantonment limits. Stretching from the Margalla hills in the federal capital Islamabad at north-western edge until the Soan River at south-eastern edge in the District Rawalpindi the maximum length of the Lai Nullah does not exceed 45 km.

### Seismicity

On the basis of SAARC Seismic Microzonation, Pakistan has been divided into four zones. Seismic Zoning Map of Pakistan showing proposed project site area is presented as Figure 5.4, indicating different zones. Being spread over I-10 and IJP road of Islamabad and KSS of Rawalpindi, the project site falls in Seismic Zone 2B according to the Seismic Zoning Map of Pakistan. Zone 2B (moderate limit of high damage) represents peak ground acceleration (PGA) from 0.16 to 0.24g.

Figure 5.4 Seismic Map of Pakistan

### Climate

The Project area is classified as humid subtropical climate. The humid subtropical climates are characterized by typically mild, dry and short winters. Summers in Project area are long with higher temperatures starting from mid-April and reaching peak temperatures in June/July. The temperatures often exceed 40 °C in summers. The summers are typically followed by monsoons where heavy rains are experienced. The project area receives rains in all the seasons but monsoon rain is pronounced and constitutes a definite rainy season between the month of July and September. Average high temperatures decrease during the monsoon season, but humidity increases. This results in hot and humid conditions, similar to summers in other humid subtropical climates.

The following sections present the climate details of the meteorological gauging station situated near the proposed project. Airport, Islamabad meteorological station was identified near to the proposed project.

The monthly data for Temperature, Rainfall, and Relative Humidity from year (2018-2022) for the above-mentioned Weather Stations for the Project Area are presented in subsequent section.

### Temperature

The proposed project area will be subjected to long periods of high temperature in summers and low temperature in winter. The mean monthly temperature (max & min) of Rawalpindi for the 2018 to 2022 years are 38, 37, 36, 38 & 28C respectively. The mean yearly temperature data is presented below.

The temperature rises rapidly until June and the temperature drops with advent of monsoon in July. The temperature rises rapidly until June and the temperature drops with advent of monsoon in July. The months of December and January experience a severe drop in temperature and, the temperature falls below the freezing point in areas above 1800 masl. Summer is marked with monsoon rains, which occur usually in July/August. Yearly average temperature data is presented below.

Table 5.1: Yearly Mean Temperature (2018-2022)

| Sr. No. | Year | Mean Max. Temperature (0C) | Mean Min. Temperature (0C) | Mean Temperature (0C) |
| --- | --- | --- | --- | --- |
| 1 | 2022 | 43 | 38 | 28 |
| 2 | 2021 | 41 | 31 | 38 |
| 3 | 2020 | 41 | 30 | 36 |
| 4 | 2019 | 42 | 32 | 37 |
| 5 | 2018 | 30 | 30 | 38 |

*Source: Worldweatheronline.com

Figure 5.5 Mean Yearly Temperature

According to the Metrological Gauging Station, the coldest month is January in which the mean minimum temperature is 3.2°C and the mean maximum temperature is 17.9°C. June is the hottest month with the mean minimum temperature of 23.7°C and the mean maximum temperature as 38.4°C.

### Rainfall

Mean monthly rainfall data and the number of rainy days recorded at climatic stations in the vicinity of the project area are given below. The average annual rainfall of the project area is 1248 mm.

The maximum rainfall occurs during the months of July and August, which is about 50% of the annual rainfall. Winter rains generally occur during the months of January to March, whereas, November is normally the months with least precipitation.

Table 5.2: Mean Monthly Rainfall Record

| Sr. No. | Months | Mean Monthly Rainfall (mm) |
| --- | --- | --- |
|  |  | Airport |
| 1 | Jan | 56.2 |
| 2 | Feb | 84.1 |
| 3 | Mar | 92.4 |
| 4 | Apr | 63.2 |
| 5 | May | 35.8 |
| 6 | Jun | 75.3 |
| 7 | Jul | 307.8 |
| 8 | Aug | 340.3 |
| 9 | Sep | 110.7 |
| 10 | Oct | 31.7 |
| 11 | Nov | 14.4 |
| 12 | Dec | 36.3 |
| Annual |  | 1248.5 |

Source: Pakistan Meteorological Department

Graphical representations of month-wise precipitation of meteorological gauging station are provided below. The maximum rainfall occurs during the monsoon season in the months of July and August.

Figure 5.6 Mean Monthly Rainfall

### Relative Humidity

The relative humidity data is given in Error! Reference source not found.. The data reveals that at 00:00 hours, the relative humidity levels are generally higher while lower relative humidity levels are recorded at 12:00 hours. Relative humidity levels are mostly high during the month of August, whereas, these are extremely lower during the highest temperature i.e., May and June.

Table 5.3: Mean Yearly Relative Humidity (2018-2022)

| Sr. No. | Yearly | Humidity |
| --- | --- | --- |
| 1 | 2022 | 41.5 |
| 2 | 2021 | 38.0 |
| 3 | 2020 | 41.0 |
| 4 | 2019 | 43.0 |
| 5 | 2018 | 34.5 |

*Source: Worldweatheronline.com

Graphical representations of month-wise Relative Humidity of Airport meteorological gauging stations are provided below in Figure 5.7.

Figure 5.7 Mean Yearly Humidity (2018-2022)

### Wind Speed

The mean monthly wind speed at Airport, Islamabad met stations around proposed project area is given below. The data reveals that at 00 UTC, the wind speeds are generally lower while higher wind speeds are recorded at 12 UTC. During summers wind speeds are generally higher than wind speeds in winters.

Table 5.4: Monthly Wind Speed Record

| Sr. No. | Months | Wind Speed (knots) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 00 UTC | 03 UTC | 12 UTC |
| 1 | January | 0.7 | 0.6 | 6.4 |
| 2 | February | 1.2 | 1.1 | 8.8 |
| 3 | March | 1.6 | 1.4 | 9.5 |
| 4 | April | 2.4 | 2.2 | 10.0 |
| 5 | May | 2.9 | 2.6 | 9.9 |
| 6 | June | 2.8 | 3.5 | 9.4 |
| 7 | July | 3.6 | 3.5 | 8.3 |
| 8 | August | 3.1 | 2.4 | 6.7 |
| 9 | September | 1.5 | 1.5 | 5.9 |
| 10 | October | 0.8 | 0.7 | 4.9 |
| 11 | November | 0.6 | 0.4 | 4.5 |
| 12 | December | 0.4 | 0.4 | 5.0 |

Source: Pakistan Meteorological Department

Graphical representations of month-wise wind speed are provided below in Figure 5.8.

Figure 5.8 Mean Monthly Wind Speed

The maximum wind speed recorded at Airport weather station is 10.0 knots in the month of April and the minimum wind speed is 4.5 knots in the month of November at 12 UTC.

### Wind Rose

The wind rose for Rawalpindi where majority of the project components are present shown in Figure 5.9. The figure shows that predominant wind direction is from south west and west directions.

Figure 5.9 Wind Rose of Rawalpindi

### Floods

Disaster management in Pakistan basically revolves around flood disasters, with a primary focus on rescue and relief. The existing institutional structures responsible for disaster management are geared to reactive short-term responses. Experience has shown that abnormal heavy rains in catchment areas result in flash floods in Lai Nullah especially in 2001, 2010, 2011, 2012, 2014 and 2015.

Though the flood water recedes within a period of 3-8 hours, it causes considerable damage in the city area. Local drains from Chah Sultan and Cantonment area also fall in it, which further aggravates the situation, especially when these drains cannot find their way into Nullah Lai, which is already in flood, causing inundation of surrounding low lying areas (back water effect). Flash urban floods due to Lai Nullah are being planned every year for medium and high risk. The potential sites/vulnerable areas have been pointed out :

Dhoke Naju

Zain-ul-Haq Colony

Both sides of Bridge at Ratta.

Bhoosa Godown

Mohanpura Neae Jinnah Road Bridge.

Javaid Colony,

Tipu Road,

Chamanzer Colony,

Nadeem Colony.

Dhoke Chiragh Din

Mohallah Raja Sultan

New phagwari

Mohallah Raja Sultan

Gawalmandi.

Dhoke Ellahi Buksh.

Mainly heavy rainfall during monsoon season is the main triggering factor for urban flash flooding along with contributing factors of throwing of garbage in the main stream of La Nullah as well as in tributaries of nearby vulnerable areas. The areas are marked on the map shown in Figure 5.10 below.

The management and/or administration of the Lai Nullah area, there are a number of other agencies that have a role in the this area, namely: the Capital Development Authority (CDA), responsible for master planning of Islamabad; the Small Dams Organization (SDO) is responsible for maintenance of Rawal Dam, of the main sources for water supply in the twin cities; the Rawalpindi Development Authority (RDA) deals with surface drainage or flood mitigation projects; the Tehsil Municipal Administration (TMA) is responsible for the execution and management of development plans, the enforcement of all municipal laws and rules, and the prevention of encroachments. Finally, after the 2001 floods, it was decided that the joint flood management problem of Islamabad and Rawalpindi should be tackled by the Flood Forecasting Commission (FFC) through medium and long term planning and implementation of measures. FFC was assigned the task to act as a coordinating agency between the Federal Capital and Rawalpindi Administrations.

Extreme flood years were 1981, 1988, 1997 and 2001, the latter having been the largest among the recorded events (considered as a national disaster). The intensity and amount of rainfall caused the water level of Lai Nullah and its tributaries to rise remarkably and flooding Rawalpindi (where the damages were several times larger than in Islamabad).

Housing; Agriculture and Livestock; and Transport and Communications sectors suffered the most significant damage, at USD 5.6 billion, USD 3.7 billion, and USD 3.3 billion, respectively. Sindh is the worst affected province with close to 70 percent of total damages and losses, followed by Balochistan, Khyber Pakhtunkhwa, and Punjab.

The Ministry of Planning, Development and Special Initiatives led the Post-Disaster Needs Assessment (PDNA), which was conducted jointly with the ADB, the European Union (EU), the United Nations agencies with technical facilitation by the United Nations Development Programme (UNDP) and the World Bank. The PDNA, in addition to estimating damages, economic losses and recovery and reconstruction needs, also assesses broader macro-economic and human impacts and recommends principles along which to develop a comprehensive recovery and reconstruction framework.

Figure 5.10 Flood prone areas of Rawalpindi

### Ecological Environment

In order to identify ecological resources, ecological baseline survey was carried out by EDCM team. Detailed surveys were conducted for project scoping during December 2022. The area for Conductance Main fall under Sector I-10 is green patch with grass and shrubs. However, the existing RO will be utilized for lying of Conductance Main. Further, no tree cutting in involved in this proposed project. There will be disturbance to green patches of public parks during joining of Bulk Water Supply lines with existing OHRs and GSTs which are mostly present in existing parks of PHA. Approval from PHA will be taken before execution of the project. Majority of the other components of project are located within KSS, which is densely populated area of Rawalpindi district. No worth mentioned ecological feature has been noticed during site visit and review of literature.

All the available literature was thoroughly reviewed to have a better understanding of the project area and its surroundings including habitat, flora and fauna. The route alignment was thoroughly examined based on the primary and secondary (Flora of Pakistan-MI-Sheikh, Birds of Pakistan-ZB-Mirza-Mammals of Pakistan-TJ Roberts, Manual of silviculture for Pakistan [1965] Champion, Sir H.G.; Pakistan Forest Inst., Peshawar eng; Seth, S.K. Khattak, G.M.) data. The stakeholder Forest, Wildlife, EPA, Fisheries and Agricultural departments were also consulted. This survey broadly covers ecosystem sensitivities (If any), vegetation, other flora and fauna.

### Ecosystem

The climate of the project area has a typical version of humid subtropical and dry climate, with five seasons. The study area majorly lies in sub-tropical scrub zone. The dominant trees species of this natural zone are, Kau (Olea ferruginea), Phulai (Acacia modesta) and sanatha (Dodonaea viscosa).

In order to improve the landscape and living environment of the Rawalpindi traits like color or blossom, shade, form or shape, scent and free from disease and nuisance are considered during the selection of trees and shrubs for given location. So, the study area in general is under significant biotic pressure and under retrogression due to various factors such as unsustainable practices, Natural hazards causing the habitat loss, soil erosion and lack of proper functioning of ecosystems. Areas have been accessible to humans for a long time resulting in low diversity and wildlife abundance.

### Protected areas

No protected area of biological habitat is present within or near the proposed routes of project. There is no Game Reserve, Reserve Forests and wildlife sanctuaries reported in the project alignment. Further the nearest forests are enlisted under Table 5.5 below. However, no impact is anticipated due to the proposed interventions as none of the national forest or protected area is present within 10km radius of proposed project components. The area of forests is marked on the map, shown as Figure 5.11.

Table 5.5: Forests located near the project area

| Sr. No. | Name | Area (Acre) |
| --- | --- | --- |
| 1 | Takhar Pari Forest | 2,210 |
| 2 | Khairi Murat Reserved Forest | 12,680 |
| 3 | Dhungi Reserved Forest | 4,746 |
| 4 | Adiala Reserved Forest | 1,330 |

Figure 5.11 Location of Forests near the project site

### Flora

As climate of study area is subtropical, the vegetation of the area falls under subtropical broad leaved evergreen scrub type as per phyto-geographical classification of the area. The tract, in which the project site exists, was once covered with native vegetation consisting, of trees and thick cover of bushy vegetation, with the onslaught of civilization, this vegetation was cleared for agricultural purposes.

As the project area is part of the Potohar plateau, the vegetation is characteristic of that tract. Kau (Olea cuspidata) is the climax species. However, in the project and the study area, Phulahi (Acacia modesta) and Kikar (Acacia nilotica) dominate. Shisham (Dalbergia sissoo), Mesquit (Prosopis juliflora) and Paper Mulberry (Broussonetia papyrifera) have also grown as secondary species with Phulahi. With the onslaught of civilization and urbanization of the tract, the original vegetation in and around the tract was removed and replaced by ornamental plants and lawns, which is the dominant land use in the tract. The detail of vegetative species of the region are given in Table 5.6 below:

Table 5.6: Names of Trees/shrubs & Herbs of the Region

| Sr. No. | Common Name | Scientific Name |
| --- | --- | --- |
| 1 | Banyan/Bhor | Ficus benghalensis |
| 2 | Phulai | Acacia modesta |
| 3 | Wild olive | Olea ferruginea |
| 4 | Sanatha | Dodonaea viscosa |
| 5 | Dhak | Butea frondosa |
| 6 | Anar | Punica granatum |
| 7 | Pear | Pyrus pasha |
| 8 | Sukh Chain | Pongamia pinnata |
| 9 | Amaltas | Cassia fistula |
| 10 | Bottle Brush | Callistemon spp |
| 11 | Jasmine | Jasminum humile |
| 12 | Holmskioldia | Holmskiodia sanguine |
| 13 | Beri | Zizyphus mauritiana |
| 14 | Royle's Spike Thorn | Gymnosporia royleana |
| 15 | Baikarh | Adhatoda vasica |
| 16 | Zebrawood | Pistacia integerrima |
| 17 | Poplar | Populus sp. |
| 18 | Devil tree | Alstonia scholaris |
| 19 | Bougainvillea | Bougainvillea spectabilis |
| 20 | Sukh chain | Pongamia pinnata |
| 21 | Dharek | Melia azedarach |
| 22 | Eucalyptus | Eucalyptus camaldulensis |

Source: TAS for LAI Expressway Project, 2022

### Fauna

The project area, on account of nature of vegetation and topography, once rich in vegetation and wildlife has now reduced its potential due to over hunting, loss of proper habitat, conversion of forest land. The wild mammals were not spotted commonly. However, domestic mammals were found to be common around the project area.  Fauna of the proposed area consists of mammals, reptiles, amphibians and birds. The details are given below.

#### Mammals

The study area was once considered as good habitat to many wild species but due expansion of population and commercialization the habitats of many species are decaled as degraded.

However, there is no wild mammalian specie observed in the Project area except some domesticated animals including cats, dogs, etc. However, the following Error! Reference source not found. shows the list of mammalian species present in the region, as the project area is to be considered highly degraded in terms of wild fauna. Jackals and foxes are seen occasionally in Islamabad and Sangjani area of proposed project site. Wild boars and Hares are found on all the low hills and in most of the ravines.

Table 5.7: List of indigenous Mammals

| Sr. No. | Mammals | Scientific Name | IUCN Status |
| --- | --- | --- | --- |
| 1 | Jackal | Canis aureus | LC |
| 2 | Fox | Vulpes vulpes | LC |
| 3 | Jungle Cat | Felis chaus | LC |
| 4 | Palm Squirrel | Funambulus palmarum | LC |
| 5 | Mongoose | Herpestes auropunctatus | LC |
| 6 | Indian mole rat | Rattus rattus | LC |
| 7 | Field mouse | Funambulus pennant | LC |
| 8 | Porcupine | Hystrix indica | LC |
| 9 | Indian Hare | Lepus nigricollis | LC |
| 10 | Cape hare | Lepus capensis | LC |
| 11 | Masked palm civet | Paguma larvata | LC |
| 12 | Wild boar | Sus scrofa | LC |

Source: TAS for LAI Expressway Project, 2022

LC = Least Concern

#### Reptiles

Reptiles reported in the project area and its vicinity are given in Error! Reference source not found.Other varieties of snakes reported in the project area are Rat Eaters, Sang Choor and a snake locally known as Phissi.

Table 5.8: List of Reptiles observed in Project Area

| Sr. No. | Local/ English Name | Scientific Name | IUCN Status |
| --- | --- | --- | --- |
| 1 | Fringed toed Lizard | Acanthodactylus cantoris | LC |
| 2 | Spiny tailed Lizard | Uromastyx hardwickii | VU |
| 3 | Common Krait | Bungarus caeruleus | LC |
| 4 | King Cobra | Ophiophagus Hannah | VU |
| 5 | Viper | Vipera xanthina | LC |

Source: TAS for LAI Expressway Project, 2022

LC = Least Concern

VU = Vulnerable

#### Amphibians

Amphibians found in the project area are given below.

Table 5.9: List of Amphibians observed in Project Area

| Sr. No. | Local/ English Name | Scientific Name | IUCN Status |
| --- | --- | --- | --- |
| 1 | Common frog | Rana tigrina | LC |
| 2 | Common toad | Bufo bufo | LC |
| 3 | Marble frog | Uperodon systoma | LC |

Source: TAS for LAI Expressway Project, 2022

LC = Least Concern

#### Birds

Many bird species have been reported in and around the Study Area. These include passage migrants, vagrant, resident, breeding and irregular visitors. The migratory birds descend from higher altitudes during the winter months. Avifauna of the tract consists of small and medium sized birds of different colors, flying from one tree to the other or from crop to crop. Most common birds are House Sparrow, House Crow and Mynah. Birds like Cuckoo, Bulbul, Hoopoe, Parrots, Blue Birds, and Little Egrets etc were frequently sighted. The common birds observed and reported in the study area are given in Table 5.10 below:

Table 5.10: Birds Found in Study Area

| Sr. No. | Common Name | Scientific Name | IUCN Status |
| --- | --- | --- | --- |
| 1 | Rock Pigeon | Columba livia | LC |
| 2 | Myna | Acrido therestritis | LC |
| 3 | Grey Partridge | Perdix perdix | LC |
| 4 | House Sparrow | Passer domesticus | LC |
| 5 | Red-billed chough | Pyrrhocorax pyrrhocorax | LC |
| 6 | Magpie | Pica pica | LC |
| 7 | Yellow-billed Chough | Pyrrhocorax graculus | LC |
| 8 | Grey shrikes | Lanius excubitor | LC |
| 9 | Eastern Spotted doves | Spilopelia chinensis | LC |

Source: TAS for LAI Expressway Project, 2022

LC = Least Concern

#### Aquatic Life of the Project region

The Nullah Lai and Niki Nullah are the main water bodies located near project site. Due to discharge of wastewater from residential and commercial areas, these water bodies have no aquatic life of worth mentioning.

### Socio-economic Environment

This section includes a summary of the prevailing socio-economic conditions in the project area and the population that will be potentially affected by the Project. To ascertain the socio-economic condition of the project area, primary and secondary data was collected including social and physical infrastructure in the project area.

To assess the socioeconomic conditions of the project area, consultations are being carried out with participants including male and female. Households (HH) has been studied during field survey based on individual interviews as well as focus group discussions/ public consultations. Consultation participant include project affected people and community residing along the route of Conductance Main and Replacement of AC/GI Pipelines. Further the consultations were carried out at previously selected sites for construction of OHRs and GSTs for storage of additional water. In addition, the secondary data, including Economic Survey of Pakistan (2018-19), Bureau of Statistics (2017-18) and District Population Census report.

Detailed surveys were conducted for project scoping during the month of December 2022. For the purpose of the environmental and social assessment and sensitive receptor data collection, a Corridor of Impact of 100 meters was taken. Most of the field data collection was carried out within this corridor though where relevant data was also collected from a wider area along the proposed project site. The reason for selecting this corridor is to cover those areas that have a potential to be affected by the project activities.

The names of the major settlements falling within the close vicinity of project area are Sector 1, 2 and 3 of Khyaban-e-Sir Syed. These sectors are comprised on three union councils, UC 10, UC11, UC-12. The area for proposed Conductance Main start from Service Road West of Sector I-10 of Islamabad and runs through the IJP road before entering into KSS. Khayaban-e-Sir Syed (KSS) is located in the WEST-I Zone of Rawalpindi with a 1.48 sq. km area.

#### Administrative Setup

The proposed project components are located in two territories, Rawalpindi and Islamabad. Start of New Conductance Main is from green area of Sector I-10 of Islamabad and it will run parallel to IJO road and enter into Rawalpindi near CDA Stop. Rest of the project components are located in district Rawalpindi.

Rawalpindi district is governed by the Rawalpindi District Council, while several regions sub-divided into one Municipal Corporation, two Cantonment Boards and Seven Tehsils:

Rawalpindi Municipal Corporation

Rawalpindi Cantonment Board

Chaklala Cantonment Board

The Table 5.11 below shows the administrative subdivision of Rawalpindi district.

Table 5.11: Administrative subdivision of Rawalpindi

| Sr. No. | Tehsil | Headquarters | Area (km2) | Population (2017) |
| --- | --- | --- | --- | --- |
| 1 | Taxila | Taxila | NA | 677,951 |
| 2 | Rawalpindi | Rawalpindi | NA | 3,258,547 |
| 3 | Gujar Khan | Gujar Khan | 1,466 | 678,503 |
| 4 | Kallar Syedan | Kallar Syedan | 421 | 217,273 |
| 5 | Kahuta | Kahuta | NA | 220,576 |
| 6 | Potohar | Potohar | NA | NA |

#### Demography and Population

Khayaban-e-Sir Syed (KSS) is located in the WEST-I Zone of Rawalpindi with a 1.48 sq. km area. There is a total of three Union Councils (UCs) in this area named UC-10 (Khayaban-e-Sir Syed (N)), UC-11 (Khayaban-e-Sir Syed (S)), and UC-12 (Dhok Najjo). The current population of KSS is 69,388 persons while the future population for 2050 is projected as 138,165 persons. Sector I-10 of Islamabad is located at the boundary with Rawalpindi, divided by IJP Road. It is a lightly built area, located on the southwestern edge of the city.

According to Census 2017, the male population of Rawalpindi was 2,736,180 and female population was 2,665,089. The ratio of people living in rural and urban area was:2,396,672 (44.36%) lived in rural areas and 3,005,708 (55.64%) lived in urban areas.

I-10 borders Rawalpindi to the south and neighbor’s I-9 and I-11, while sectors H-9, H-10, and H-11 are located adjacent. Like most other I-sectors, I-10 is primarily a part of the industrial zone. It consists of open land with dense vegetation. It is a densely populated area. Figure 5.12 shows the socio-economic feature of the project area. Demographic features of Rawalpindi are shown in Figure 5.13. According to Census report 2017, the demographic features of Rawalpindi including total area, population, population density and annual population change is given below.

Table 5.12: Demography of Rawalpindi (2017 Census)

| Districts | Area (km²) | Population (2017) | Density (people/km²) | Annual Population Change 1998 to 2017 |
| --- | --- | --- | --- | --- |
| Rawalpindi | 5,285 | 5,402,380 | 1022 | 2.5% |

Source: District wise population Census 2017 by Pakistan Bureau of Statistics: Government of Pakistan

Figure 5.12: Photographs depicting Socio-economic Conditions of the project area

Figure 5.13 Demographic features of Rawalpindi

#### Religion

Majority of the population of project area is Muslims. Cultural festivals are mostly linked with traditional religious events. According to the 2017 census Muslims were the overwhelming majority religion with 98.01%, while Christians were 1.90% of the population.

#### Archaeological and Cultural Site

No archaeological and cultural site was observed in close proximity of proposed components of Augmentation of Water supply and Creation of DMAs project. However, if any archaeological antiquity discovered Archaeological Chance Find procedure shall be adopted. Archaeological Chance Find procedure has been attached as Annexure-IV.

#### Ethnicity in the Project Area

During Social survey of the proposed project site it was found that the entire population is Muslims and they did not consider themselves to be called any other type of population such as indigenous peoples as the ADB’s SPS 200. None of these castes may be considered as indigenous people (IP) based on ADB SPS definition.

#### Language and Dialects

The major language of the area is Hindko, Punjab, Urdu and Pashto. People also speak other languages like Kashmiri, Saraiki and Brahvi. The Figure 5.13 shows detail of mother tongue of people of study area.

#### Dwellings

Housing conditions of the respondents have been analyzed according to the type of houses in which they were residing. The house or building constructed with concrete or burnt bricks fall in pacca category whereas house or building constructed with burnt bricks with mud comes under semi-pacca category while house constructed with mud bricks or temporary wooden logs etc. are categorized as kacha house. All the respondents in project area are living in category of Pacca Houses.

#### Economics of Rawalpindi / Islamabad

The economy of majority of the people of study area is based on their private business and government jobs. KSS is among the posh area of Rawalpindi with high rage in per capita income. Some family members are settled outside the country and send remittances to family members in Pakistan. Majority of the people of Rawalpindi and Islamabad are directly or indirectly depends on mining, Government/private jobs in educational/government institutions for their income.

#### Education Facilities in Project area

The cities of Islamabad and falls under category of large cities and they are among the hub of educational facilities. Large number of Government and private Schools, Colleges and Universities are present in the study area. The Error! Reference source not found. shows the presence of large number of Schools and other educational institutes.

#### Social Amenities in the project area

During the field survey, the access/ availability of the social amenities/ basic infrastructure was asked from the surveyed households as well as physically observed at site. It was noted that facilities such as Electricity, Sui Gas, Water Supply, Telephone, Sewerage Drainage, school are available in Rawalpindi and Islamabad.

#### Major Source of Drinking Water

Currently, there are two main source of water supply in people of project area. The major source is water supply from RWASA and CDA. While the other source of water is ground water. Some households have installed their own electrical pumps on bore holes.

#### Health Facilities in project area

Health facilities in the form of Government and Private health care institutes is present in and around the project area. Error! Reference source not found. (Sensitive Receptors) shows the presence of health care facilities. Being the major cities, Rawalpindi and Islamabad has primary, secondary and tertiary health care facilities. PIMS, Shifa International, CMH, Rawal Foundation, Ripha International Hospital, Islamabad International Hospital and Research Centre etc.

### Environmental Sensitive Receptors of the Proposed Project

Figure 4.13: Land Use of the Project Area.

Sensitive receptors are people/places more susceptible to the adverse effects of exposure to the pollutants and social disturbance, due to the developmental projects. Thus, sensitive receptors are necessary to be identified, to evaluate the potential impacts of the proposed project on public health and the environment and adopt necessary mitigation measures to minimize the impact.

The sensitive receptors identified for the proposed project within or near the CoI are religious, cultural places, educational institutes, health care facilities etc. They are prone to sensitivity during construction phase, due to emission of air pollutants, noise and vibration, traffic jams/access, temporary edifice of construction camps, and mobilization issues. The map of sensitive receptors of the study area is shown in Figure 5.14. Also, the Figure 5.15 shows the detail of environmental sensitive receptors along the COI of proposed project area.

Figure 5.14: Environmental Sensitive Receptors

Figure 5.15: Detail of Environmental Sensitive Receptors

| Sr. No. | Pictorial View | Coordinates |  | Description |
| --- | --- | --- | --- | --- |
| 1 |  | 33.639506° N 73.033523° E |  | Mosque and Imam Bargah |
| 2 |  | 33.639662° N 73.033478° E |  | Medical Clinic |
| 3 |  | 33.637842° N 73.034778° E |  | Mosque |
| 4 |  | 33.639803° N 73.03334° E |  | Medical Clinic |
| 5 |  | 33.640191° N 73.032611° E |  | Mosque |
| 6 |  | 33.639655° N 73.031878° E |  | Sabzi Mandi |
| 7 |  | 33.629229° N 73.047936° E |  | Park |
| 8 |  | 33.628729° N 73.047883° E |  | School |
| 9 |  | 33.64021° N 73.048305° E |  | Imam Bargah |
| 10 |  | 33.640023° N 73.048063° E |  | High School for Girls |
| 11 |  | 33.638661° N 73.048698° E |  | School |
| 12 |  | 33.638769° N 73.04904° E |  | Mosque |
| 13 |  | 33.638633° N 73.049137° E |  | School |
| 14 |  | 33.6375° N 73.045844° E |  | School |
| 15 |  | 33.630873° N 73.050167° E |  | Medical Clinic |
| 16 |  | 33.630177° N 73.052296° E |  | Historical Monument |
| 17 |  | 33.630216° N 73.050183° E |  | School |
| 18 |  | 33.633093° N 73.049102° E |  | School |
| 19 |  | 33.632687° N 73.049287° E |  | School |
| 20 |  | 33.630117° N 73.049952° E |  | Mosque |
| 21 |  | 33.634418° N 73.048256° E |  | School |
| 22 |  | 33.631393° N 73.049854° E |  | School |
| 23 |  | 33.630118° N 73.049952° E |  | School |
| 24 |  | 33.63006° N 73.049967° E |  | Utilities |
| 25 |  | 33.629666° N 73.050711° E |  | School |
| 26 |  | 33.629545° N 73.0501° E |  | Mosque and School |
| 27 |  | 33.633966° N 73.048741° E |  | Mosque |
| 28 |  | 33.6352° N 73.047953° E |  | Graveyard |
| 29 |  | 33.63716° N 73.046442° E |  | Mosque |
| 30 |  | 33.637686° N 73.046202° E |  | School |
| 31 |  | 33.637739° N 73.04617° E |  | School |
| 32 |  | 33.638358° N 73.045881° E |  | School |
| 33 |  | 33.639147° N 73.045121° E |  | Mosque |
| 34 |  | 33.638423° N 73.045858° E |  | School |
| 35 |  | 33.639557° N 73.044772° E |  | School |
| 36 |  | 33.639557° N 73.044772° E |  | School |
| 37 |  | 33.640228° N 73.044196° E |  | School |
| 38 |  | 33.640256° N 73.044044° E |  | Mosque |
| 39 |  | 33.640598° N 73.043707° E |  | CNG Filling Station |
| 40 |  |  | 33.640434° N 73.043887° E | CNG Filling Station |

### Environmental Monitoring for Water Supply Works

Environmental monitoring was carried out for water, air, and noise levels at two locations within the proposed project site. The two locations were analysed to create environmental baseline of air, water, and noise. Further, the locations were selected because as per the initial planning of the project, maximum construction activities were proposed at these two locations. Furthermore, the terrain of the area is similar, and nature of activities/business is also the same for whole KSS. Therefore, these two sites were selected for baseline environmental monitoring.

The locations were selected for environmental monitoring based on the number of environmental receptors (like educational institutes, health care facilities, places of worship and spiritual importance) and the level of construction activity involved. The monitoring was carried out based on the initial planning of project that construction of OHR and GSTs will be carried to enhance the storage capacity of KSS. Environmental monitoring was carried out in December 2022 by EPA certified laboratory. Location of the sampling points are marked on the layout map. The map of project site marked with monitoring locations is shown as Figure 5.16. The results of environmental monitoring is attached as Annexure-II of the IEE report. The monitoring is carried out with calibrated equipment. Calibration certificates of the equipment are attached as Annexure III.

Figure 5.16 Locations of Environmental Monitoring Points for Water Supply Works

#### Water Quality

Water sampling was carried out at two locations (i) proposed underground tank-2 (W-1) (ii) proposed underground tank-4 (W-2) on 1st and 2nd December 2022. Ground water samples were taken for analysis of physical, chemical and biological parameters defined in NEQS/PEQS and WHO guidelines. It is pertinent to mention here that the sampling locations were selected based on the initial designing of project in which construction of Overhead Reservoirs (OHRs) and Ground Storage Tanks (GSTs) were part of the design. Keeping in view the sensitivity of environmental receptors, these two sampling sites were identified. The samples were taken from tap water of houses near the parks. The analysis results of water samples are given below. The lab reports are attached as Annexure-II of the IEE report.

#### Locations and Reason for selection of Monitoring point

The location and rationale of the water sampling is given below in Table 5.13.

Table 5.13: Detail of Water Quality Sampling Location

| Sample | Coordinated | Location | Sample Taken | Rationale of Selection |
| --- | --- | --- | --- | --- |
| W.1 | Lat. 33.630233 Long.73.051757 | Near proposed underground tank-2 | Drinking water (Tap Water) | The sampling location was selected to check if there is mixing of wastewater occurs due to leaked pipelines. The second reason for selection of these points is that these points were near the proposed construction site. |
| W.2 | Lat. 33.638687 Long.73.04889 | Near proposed underground tank-4 | Drinking Water (Tap water) | The sampling location was selected to check if there is mixing of wastewater occurs due to leaked pipelines. The second reason for selection of these points is that these points were near the proposed construction site. |

The sampling locations were identified to check the water quality near proposed OHRs and GSTs because these were supposed to be area of heavy construction. Further, the water sampling location were marked to check the level of contamination due to leaked pipelines of KSS. Sampling location and coordinates are marked on the pictures given in Figure 11.1.

#### Results of Analysis

It is evident from the laboratory results that all the analyzed parameters are within compliance of limiting values set by WHO, PEQS, and NEQS for drinking water. The results show that the water is fit for drinking purpose as no biological contamination is found in water sample and the physical and chemical parameters are also fit for drinking purpose. The monitoring results of water samples is given below in Table 5.14. The complete results of the drinking water quality samples are presented as Annexure-II.

Table 5.14: Results of Water Quality Analysis

| Underground Tank-2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sr. No. | Parameters | PEQS/ NEQS |  | WHO |  | Unit | Result | Method |
| 01 | pH | 6.5-8.5 |  | -- |  | -- | 6.9 | SMWW 4500*B |
| 02 | TDS | <1000 |  | 1000 |  | Mg/l | 758 | SMWW 2540C |
| 03 | Chloride | <250 |  | 250 |  | Mg/l | 54 | SMWW 4500Cl-B |
| 04 | Fluoride | ≤1.5 |  | 1.5 |  | Mg/l | 0.3 | U.S. EPA 9214 |
| 05 | Taste | Nonobjectionable/Acceptable |  | -- |  |  | Acceptable | Organoleptic |
| 06 | Odor | Nonobjectionable/Acceptable |  | -- |  |  | Acceptable | Organoleptic |
| 07 | Colour | ≤15 |  | -- |  | TCU | ND | SMWW 2120C |
| 08 | Nitrate | ≤50 |  | 50 |  | Mg/l | 25 | SMWW 4500NO3-B |
| 09 | Nitrite | ≤3 |  | -- |  | Mg/l | ND | SMWW 4500NO2-B |
| 10 | Lead | ≤0.05 |  | 0.01 |  | Mg/l | ND | U.S. EPA-200.7 |
| 11 | Total Hardness as CaCO3 | <500 |  | -- |  | Mg/l | 468 | SMWW 2340C |
| 12 | Turbidity | <5 |  | -- |  | NTU | ND | SMWW 2130B |
| 13 | Zinc | 5 |  | 3 |  | Mg/l | 0.1 | U.S. EPA-200.7 |
| 14 | Aluminum | ≤0.2 |  | 0.2 |  | Mg/l | 0.1 | U.S. EPA-200.7 |
| 15 | Cadmium | 0.01 |  | 0.003 |  | Mg/l | ND | U.S. EPA-200.7 |
| 16 | Selenium | 0.01 |  |  |  | Mg/l | ND | U. S. EPA-200.7 |
| 17 | Copper | 2 |  | 2 |  | Mg/l | ND | U.S. EPA-200.7 |
| 18 | Boron | 0.3 |  | 0.3 |  | Mg/l | 0.1 | U.S. EPA-200.7 |
| 19 | Barium | 0.7 |  | 0.7 |  | Mg/l | 0.2 | U.S. EPA-200.7 |
| 20 | Antimony | ≤0.005 |  | ≤0.02 |  | Mg/l | ND | U.S. EPA-200.7 |
| 21 | Arsenic | ≤0.05 |  | 0.01 |  | Mg/l | ND | U.S. EPA-200.7 |
| 22 | Cyanide | ≤0.05 |  | 0.07 |  | Mg/l | ND | SMWW 4500 CN-F |
| 23 | Mercury | ≤0.001 |  | 0.001 |  | Mg/l | ND | U.S. EPA-200.7 |
| 24 | Nickel | ≤0.02 |  | 0.02 |  | Mg/l | ND | U.S. EPA-200.7 |
| 25 | Residual Chlorine | 0.2-0.5 |  | -- |  | Mg/l | ND | SMWW 4500-Cl B |
| 26 | Total Coliform | ----- |  | 0/100ml |  | ------ | ND | SMWW 9221 B |
| 27 | Fecal Coliform | 0/100ml |  | 0/100ml |  | ml | ND | SMWW 9221 F |
| 28 | E. Coli | 0/100ml |  | 0/100ml |  | ml | ND | SMWW 9221 F |
| 29 | Manganese | < 0.5 |  |  |  | Mg/l | 0.1 | U. S. EPA-200.7 |
| 30 | Phenolic Compounds as (Phenols) | NGVS |  |  |  | --- | ND | SMWW 5530 C |
| Underground Tank-4 |  |  |  |  |  |  |  |  |
| Sr. No. | Parameters |  | PEQS/ NEQS |  | WHO | Unit | Result | Method |
| 01 | pH |  | 6.5-8.5 |  | -- | - | 6.9 | SMWW 4500*B |
| 02 | TDS |  | <1000 |  | 1000 | Mg/l | 614 | SMWW 2540C |
| 03 | Chloride |  | <250 |  | 250 | Mg/l | 35 | SMWW 4500Cl-B |
| 04 | Fluoride |  | ≤1.5 |  | 1.5 | Mg/l | 0.3 | U.S. EPA 9214 |
| 05 | Taste |  | Nonobjectionable/Acceptable |  | -- | Unobjec. | Acceptable | Organoleptic |
| 06 | Odor |  | Nonobjectionable/Acceptable |  | -- | Unobjec. | Acceptable | Organoleptic |
| 07 | Color |  | ≤15 |  | -- | TCU | ND | SMWW 2120 C |
| 08 | Nitrate |  | ≤50 |  | 50 | Mg/l | 15.7 | SMWW 4500NO3-B |
| 09 | Nitrite |  | ≤3 |  | -- | Mg/l | ND | SMWW 4500NO2-B |
| 10 | Lead |  | ≤0.05 |  | 0.01 | Mg/l | ND | U.S. EPA-200.7 |
| 11 | Total Hardness as CaCO3 |  | <500 |  | -- | Mg/l | 404 | SMWW 2340C |
| 12 | Turbidity |  | <5 |  | -- | NTU | 0.1 | SMWW 2130B |
| 13 | Zinc |  | 5 |  | 3 | Mg/l | 0.1 | U.S. EPA-200.7 |
| 14 | Aluminum |  | ≤0.2 |  | 0.2 | Mg/l | 0.1 | U.S. EPA-200.7 |
| 15 | Cadmium |  | 0.01 |  | 0.003 | Mg/l | ND | U.S. EPA-200.7 |
| 16 | Selenium |  | 0.01 |  |  | Mg/l | ND | U. S. EPA-200.7 |
| 17 | Copper |  | 2 |  | 2 | Mg/l | ND | U.S. EPA-200.7 |
| 18 | Boron |  | 0.3 |  | 0.3 | Mg/l | 0.1 | U.S. EPA-200.7 |
| 19 | Barium |  | 0.7 |  | 0.7 | Mg/l | 0.2 | U.S. EPA-200.7 |
| 20 | Antimony |  | ≤0.005 |  | ≤0.02 | Mg/l | ND | U.S. EPA-200.7 |
| 21 | Arsenic |  | ≤0.05 |  | 0.01 | Mg/l | ND | U.S. EPA-200.7 |
| 22 | Cyanide |  | ≤0.05 |  | 0.07 | Mg/l | ND | SMWW 4500 CN-F |
| 23 | Mercury |  | ≤0.001 |  | 0.001 | Mg/l | ND | U.S. EPA-200.7 |
| 24 | Nickel |  | ≤0.02 |  | 0.02 | Mg/l | ND | U.S. EPA-200.7 |
| 25 | Residual Chlorine |  | 0.2-0.5 |  | -- | Mg/l | ND | SMWW 4500-Cl B |
| 26 | Total Coliform |  | ---- |  | 0/100ml | Number/100ml | ND | SMWW 9221 B |
| 27 | Fecal Coliform |  | 0/100ml |  | 0/100ml | Number/100ml | ND | SMWW 9221 F |
| 28 | E. Coli |  | 0/100ml |  | 0/100ml | Number/100ml | ND | SMWW 9221 F |
| 29 | Manganese |  | < 0.5 |  |  | Mg/l | ND | U. S. EPA-200.7 |
| 30 | Phenolic Compounds as (Phenols) |  | NGVS |  |  | --- | ND | SMWW 5530 C |

#### Ambient Air Quality

No major anthropogenic source of air pollution was found near and around the proposed project site, other than local traffic and household activities like cooking and cleaning. The monitoring parameters selected for analysis of ambient air are based on the parameters defined in NEQS, PEQS and WHO for air quality. Following parameters were analyzed to check air quality of proposed project area.

Respirable particulate matter: (PM10, PM2.5)

Sulfur dioxide (SO2)

Oxides of Nitrogen (NOX): Mainly Nitrogen dioxide (NO2) and Nitric oxide (NO)

Carbon Monoxide (CO)

Ozone (O3)

Suspended Particulate Matter (SPM)

#### Reason for selection of Monitoring point

The sampling locations were identified to check the ambient air quality near the proposed construction sites of OHRs and GSTs. These sites were selected based on the number of sensitive receptors (like educational institutes, health care facilities, places of worship and spiritual importance) and level of construction. The construction of GST would have involved emission of fugitive dust and exhaust from working of heavy machinery. The ambient air quality analysis was carried out at proposed Underground Tank-2 (A-1) & proposed Underground Tank-4 (A-2). The rationale for selection and detail of sampling locations is given below in Table 5.15.

Table 5.15: Detail of Air Quality Sampling Locations

| Sample ID | Coordinates | Location | Rational for selection |
| --- | --- | --- | --- |
| A-1 | Lat.33.63908 Long. 73.04876 | Near proposed Underground Tank-2 | The sampling location was identified based on the number of sensitive receptors (like educational institutes, health care facilities, places of worship and spiritual importance) and level of construction activity. |
| A-2 | Lat. 33.630185 Long. 73.051599 | Near proposed Underground Tank-4 | The sampling location was identified based on the number of sensitive receptors (like educational institutes, health care facilities, places of worship and spiritual importance) and level of construction activity. |

Sampling location and coordinates are marked on the pictures given in Figure 5.16.

#### Results of Analysis

Ambient Air Quality sampling was carried out for 24 hours (Except CO) at two locations; (i) Near proposed Underground Tank-2 (A-1) (ii) Near proposed Underground Tank-2 (A-2) on 1st and 2nd December, 2022. The ambient air quality was monitored by using the Environment S.A, Ambient Air Quality Monitoring Station. For better and reliable results, calibrated equipment was used, and the calibration certificate is attached as Annexure-II of the IEE report. The instrument has built-in mechanism for recording and calculated the reading. The Summery of air quality analysis is given below in Table 5.16. The complete Laboratory reports are attached as Annexure-II of the IEE report.

All parameters analyzed are within NEQS/PEQS limits for ambient air. However, values of PM10, NO2 & PM2.5 exceeded the limiting values set by WHO at both points. The major source of particulate matter is traffic and commercial activities in the area.

Table 5.16: Ambient Air Quality Results

| Near Underground Tank-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sr. No | Parameters | PEQS/NEQS | WHO | Units | Results | Test Method |  |
| 1 | PM10 | 24 hours = 150 | 45 | µg/m3 | 133 | Particular Sensor |  |
| 2 | PM2.5 | 24 hours = 35 | 15 | µg/m3 | 30.2 | Particular Sensor |  |
| 3 | CO | 10 (8 hrs.) | 4 | mg/m3 | 1.5 | Non Dispersive Infrared Absorption (NDIR) |  |
| 4 | NO | 24 hours-40 | 1 hr. 200 | µg/m3 | 15.3 | Chemiluminescence Detection |  |
| 5 | NO2 | 24 hours-80 | 25 | µg/m3 | 28.5 | Chemiluminescence Detection |  |
| 6 | SO2 | 24 hours-120 | 24hrs40 | µg/m3 | 13 | UV Flourescence (UVF) |  |
| 7 | O3 | 1 hour -130 | 100 (8 hrs) | µg/m3 | 19 | Non Dispersive UV Absorption |  |
| 8 | SPM | 24 hours-500 | -- | µg/m3 | 417 | Particular Sensor |  |
| Near Underground Tank-4 |  |  |  |  |  |  |  |
| Sr. No. | Parameters | PEQS/NEQS | WHO | Units | Results |  | Test Method |
| 1 | PM10 | 24 hours = 150 | 45 | µg/m3 | 133 |  | Particular Sensor |
| 2 | PM2.5 | 24 hours = 35 | 15 | µg/m3 | 34.5 |  | Particular Sensor |
| 3 | CO | 10 (8 hrs.) | 4 | mg/m3 | 1.5 |  | Non Dispersive Infrared Absorption (NDIR) |
| 4 | NO | 24 hours-40 | 1 hr. 200 | µg/m3 | 13.7 |  | Chemiluminescence Detection |
| 5 | NO2 | 24 hours-80 | 25 | µg/m3 | 26.5 |  | Chemiluminescence Detection |
| 6 | SO2 | 24 hours-120 | 24hrs 40 | µg/m3 | 11.5 |  | UV Fluorescence (UVF) |
| 7 | O3 | 1 hour -130 | 100 (8 hrs) | µg/m3 | 18.8 |  | Non Dispersive UV Absorption |
| 8 | SPM | 24 hours-500 | -- | µg/m3 | 458 |  | Particular Sensor |

#### Noise Level

This section defines the baseline ambient noise levels in the CoI in a manner that can be used for the assessment of the noise impact of the proposed project. Noise levels were measured at two selected locations on 1st and 2nd December 2022. The sampling locations for noise levels was selected based on the presence of environmental receptors and the level of proposed construction activity. The detailed monitoring report is attached as Annexure-II

#### Reason for selection of Monitoring point

The sampling locations were identified to check the noise levels near the treatment plant (source) and the other sample was taken from Underground Tank-2 & Underground Tank-4. KSS is densely populated area with large number of receptors and people to be affected by project activity. Baseline has been established to check the level of impact during construction phase of the proposed project. Sampling location and coordinates are marked on the pictures given in Figure 5.16. The monitoring was carried out for 24 hrs. Leq values of noise levels are calculated and added in the results.

The location of noise levels sampling with rationale for selection of site is given below in Table 5.17 below.

Table 5.17: Detail of location for Noise Levels Monitoring

| Sample ID | Coordinates | Location | Rational for selection |
| --- | --- | --- | --- |
| N-1 | Lat 33.63908 Long. 73.04876 | Near proposed Underground Tank-2 | The sampling location was identified based on the number of sensitive receptors (like educational institutes, health care facilities, places of worship and spiritual importance) and level of construction activity. |
| N-2 | Lat. 33.630185 Long.73.051599 | Near proposed Underground Tank-4 | The sampling location was identified based on the number of sensitive receptors (like educational institutes, health care facilities, places of worship and spiritual importance) and level of construction activity. |

#### Results and Analysis

The results of noise levels monitoring are presented below in Table 5.18 below.

- It is evident from below table that noise levels at both monitored points exceeds the limiting values, set by PEQS/NEQS & WHO of noise for day and night. The major reason of this increased noise levels is the traffic and other commercial activities.

Table 5.18: Results of Noise level Monitoring

| Near Underground Tank-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sr. No. | Results (db(A) |  | PEQS/NEQS Limits |  | WHO Limits |  | Remarks |
|  | Day | Night | Day Time | Nighttime | Day | Night |  |
| 1 | 59 | 51 | 55 | 45 | 55 | 45 | Within limits |
| Near Underground Tank-4 |  |  |  |  |  |  |  |
| Sr. No | Results (db(A) |  | PEQS/NEQS Limits |  | WHO limits |  | Remarks |
|  | Day | Night | Day Time | Night Time | Day | Night |  |
| 1 | 67 | 61 | 55 | 45 | 55 | 45 | Exceeding Prescribed limits |

The photographic evidence of baseline environmental monitoring of drinking water, ambient air and noise levels is presented below as Figure 5.17.

Figure 5.17: Environmental Monitoring Photo Log

| Water Sampling location near Underground Tank-2 & Tank-4 |  |
| --- | --- |
| Ambient Air monitoring location near Underground Tank-2 & Tank-4 |  |
| Noise monitoring location near Underground Tank-2 & Tank-4 |  |

### Environmental Monitoring for Solar installations

Monitoring using the EPA certified laboratory, WELCOS, was conducted for air quality, noise and water quality near the Sangjani water treatment plant site with the results provided as Annexure XVI. The map showing the monitoring locations for the different environmental parameters is provided as Figure 5.18 below. Ambient air quality and noise were monitored at one location while tap water sample was obtained to assess the ground water quality.

Figure 5.18: Monitoring point for ambient Air Quality and Noise for Solar installation at Sangjani Treatment Plant

#### Ambient Air Quality

All parameters analyzed are within NEQS/PEQS limits for ambient air. Only exceedance in value of PM10 was observed while all other ambient quality parameters were observed to be within the applicable limits as shown in Table 5.19 below.

Table 5.19: Ambient Air Quality Results at Sangjani Treatment Plant

| Sr. No | Parameters | PEQS/NEQS | WHO | Units | Results | Test Method |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | PM10 | 24 hours = 150 | 45 | µg/m3 | 57.9 | Particular Sensor |
| 2 | PM2.5 | 24 hours = 35 | 15 | µg/m3 | 13.36 | Particular Sensor |
| 3 | CO | 10 (8 hrs.) | 4 | mg/m3 | 0.19 | Non Dispersive Infrared Absorption (NDIR) |
| 4 | NO | 24 hours-40 | 1 hr-200 | µg/m3 | 6.11 | Chemiluminescence Detection |
| 5 | NO2 | 24 hours-80 | 25 | µg/m3 | 7.23 | Chemiluminescence Detection |
| 6 | SO2 | 24 hours-120 | 24hrs-40 | µg/m3 | 5.27 | UV Flourescence (UVF) |

#### Noise Level

The results of noise levels monitoring are presented below in Table 5.20 below. It is evident from below table that noise levels at the monitored location exceeds the limiting values, set by PEQS/NEQS & WHO of noise for day and night. The major reason of this increased noise levels is the traffic and other commercial activities.

Table 5.20: Results of Noise level Monitoring

| Near Underground Tank-2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sr. No. | Results (db(A) |  | PEQS/NEQS Limits |  | WHO Limits |  | Remarks |
|  | Day | Night | Day Time | Night Time | Day | Night |  |
| 1 | 59.64 | 51.23 | 55 | 45 | 55 | 45 | Within limits |

#### Water Quality

It is evident from the laboratory results that all the analysed parameters are within compliance of limiting values set by WHO, PEQS and NEQS for drinking water. The results show that the water is fit for drinking purpose as no biological contamination is found in water sample and the physical and chemical parameters are also fit for drinking purpose. The monitoring results of water samples is given below in Table 5.21. The complete results of the drinking water quality samples are presented as Annexure XVI.

Table 5.21: Results of Water Quality Analysis

| Sr. No. | Parameters | PEQS/ NEQS | WHO | Unit | Result | Method |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | pH | 6.5-8.5 | -- | -- | 7.6 | SMWW 4500*B |
| 02 | TDS | <1000 | 1000 | Mg/l | 369 | SMWW 2540C |
| 03 | Chloride | <250 | 250 | Mg/l | 112 | SMWW 4500Cl-B |
| 04 | Fluoride | ≤1.5 | 1.5 | Mg/l | 0.86 | U.S. EPA 9214 |
| 05 | Taste | Nonobjectionable/Acceptable | -- |  | Acceptable | Organoleptic |
| 06 | Odor | Nonobjectionable/Acceptable | -- |  | Acceptable | Organoleptic |
| 07 | Colour | ≤15 | -- | TCU | 8 | SMWW 2120C |
| 08 | Nitrate | ≤50 | 50 | Mg/l | 1.2 | SMWW 4500NO3-B |
| 09 | Nitrite | ≤3 | -- | Mg/l | 0.2 | SMWW 4500NO2-B |
| 10 | Lead | ≤0.05 | 0.01 | Mg/l | 0.007 | U.S. EPA-200.7 |
| 11 | Total Hardness as CaCO3 | <500 | -- | Mg/l | 258 | SMWW 2340C |
| 12 | Turbidity | <5 | -- | NTU | 5 | SMWW 2130B |
| 13 | Zinc | 5 | 3 | Mg/l | 1.03 | U.S. EPA-200.7 |
| 14 | Aluminum | ≤0.2 | 0.2 | Mg/l | ND | U.S. EPA-200.7 |
| 15 | Cadmium | 0.01 | 0.003 | Mg/l | ND | U.S. EPA-200.7 |
| 16 | Selenium | 0.01 |  | Mg/l | ND | U. S. EPA-200.7 |
| 17 | Copper | 2 | 2 | Mg/l | ND | U.S. EPA-200.7 |
| 18 | Boron | 0.3 | 0.3 | Mg/l | 0.1 | U.S. EPA-200.7 |
| 19 | Barium | 0.7 | 0.7 | Mg/l | 0.18 | U.S. EPA-200.7 |
| 20 | Antimony | ≤0.005 | ≤0.02 | Mg/l | ND | U.S. EPA-200.7 |
| 21 | Arsenic | ≤0.05 | 0.01 | Mg/l | ND | U.S. EPA-200.7 |
| 22 | Cyanide | ≤0.05 | 0.07 | Mg/l | ND | SMWW 4500 CN-F |
| 23 | Mercury | ≤0.001 | 0.001 | Mg/l | ND | U.S. EPA-200.7 |
| 24 | Nickel | ≤0.02 | 0.02 | Mg/l | 0 | U.S. EPA-200.7 |
| 25 | Chlorine | 0.2-0.5 | -- | Mg/l | 0.19 | SMWW 4500-Cl B |
| 26 | Total Coliform | ----- | 0/100ml | ------ | 0 | SMWW 9221 B |
| 27 | E. Coli | 0/100ml | 0/100ml | ml | 0 | SMWW 9221 F |
| 28 | Manganese | < 0.5 |  | Mg/l | ND | U. S. EPA-200.7 |

Note: ND – Not Detected.

## POTENTIAL ENVIRONMENTAL IMPACTS AND MITIGATION MEASURES

### General

This chapter identifies the beneficial as well as the potentially significant adverse environmental and social impacts during design/pre-construction, construction and operation phases of the proposed project on the physical, ecological and socio-economic domains of the environment. The appropriate mitigation and remedial measures are proposed in this chapter. A project impact evaluation matrix has been developed to evaluate the potential impacts of the proposed project. Potential impacts arising from design, construction and operational phases of water supply scheme have been identified and assessed on the basis of field data, secondary data, expert opinion and examining previous similar projects in Pakistan. These include effects on physical, biological and socio-economic environments. Impacts associated with design, construction, operation phases of project components water transmission mains, distribution networks, storage tanks have been detailed in the section. A brief qualitative description of each aspect and the affected environment in both RoW and the project’s COI is presented below.

### Methodology for Impact Screening

The methodology for assessing the risk level associated with each potential impact is presented below.

Risk is assessed as the likelihood that the activity will have an effect on the environment as well as the consequence of the effect occurring. It is often described like this:

Risk = Likelihood × Consequence

Table 6.1: Likelihood Scale

| Likelihood | Definition | Scale |
| --- | --- | --- |
| Certain | Will certainly occur during the activity at a frequency greater than every week if preventative measures are not applied | 5 |
| Likely | Will occur more than once or twice during the activity but less than weekly if preventive measures are not applied | 3 |
| Unlikely | May occur once or twice during the activity if preventive measures are not applied | 2 |
| Rare | Unlikely to occur during the project | 1 |

Table 6.2: Consequence Scale

| Consequence | Definition |  | Score |
| --- | --- | --- | --- |
| Catastrophic | The action will cause unprecedented damage or impacts on the environment or surrounding communities. |  | 5 |
| Major | The action will cause major adverse damage on the environment or surrounding communities |  | 3 |
| Moderate | No or minimal adverse environmental or social impacts |  | 2 |
| Minor | No or minimal adverse environmental or social impacts |  | 1 |

Table 6.3: Risk Score

| Likelihood | Consequence |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Catastrophic | Major | Moderate | Minor |
|  | Certain | 25 | 15 | 10 | 5 |
|  | Likely | 15 | 9 | 6 | 3 |
|  | Unlikely | 10 | 6 | 4 | 2 |
|  | Rare | 5 | 3 | 2 | 1 |

Total Risk:	Significant: 15-25

Medium: 6-10

Low 1-5

Any ‘Medium’ to ‘Significant’ risk requires an environmental management measure to manage the potential environmental risk. Judgment will be required concerning the application of an environmental management measure to mitigate low risk situations.

### Potential Positive Impacts

The positive impacts due to the proposed project are mentioned below:

The project will provide sustainable and clean water source in serving areas.

Access to clean water will improve the general health status of the area and reduce exposure to waterborne diseases, ultimately, health budget of the project area will be decreased.

The project will result in conservation of groundwater resource of project area by augmenting more surface water from WTP.

People will save lot of cost which they need to spend on purchase of water containers, especially during summer season.

The project will result in improved access to water for nearby communities and other major settlements.

The sale of food and other services to immigrant workers during the construction phase of the project will lead to some direct incomes as a form of project impact.

The communities may be presented in form of temporary employment, when engaged to assist surveyors. Also, experts involved in feasibility studies and other pre-project consultations may lead to some level of purchases of items from petty traders and farmers during their presence in the communities. Some form of temporary employment would be generated for community members during the construction phase by the project directly. However, completion of the project enhancing the employment possibilities of inhabitants.

### Potential Adverse Impacts

Apart from positive impacts, there are some potential adverse environmental and social impacts on the local environment. The proposed project is divided into three stages:

Pre-construction / Planning and Design Stage,

Construction Stage and

Operation and Maintenance (O&M) Stage.

The pre-construction stage includes all stages before the construction stage (i.e. site investigation work i.e. topographical, seismic studies etc.). Construction stage includes all stages from mobilization of Contractor to the completion of the project and the operation stage starts after the construction stage, which includes the inspection and repair works.

Adverse impacts envisaged at these three stages of the proposed water supply project along with their proposed remedial or mitigation measures are given in the following section.

### Design/Pre-Construction Phase

#### Impact Screening Matrix

The ‘activity wise’ screening of potential impacts during the design/pre-construction phase is provide in Table 6.4 below.

Table 6.4: Screening of possible Impacts during Design/Pre- Construction phase for Water Supply Component

| S/No. | Potential Impacts | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Short term, Long term) |
| --- | --- | --- | --- | --- | --- |
| 1 | Improper designing of distribution networks including transmission mains (Replacement & New Distribution Lines) | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 2 | Lack of integration of IEE/EMP requirements into Construction bid documents | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 3 | Relocation of existing utilities | 3 / Likely | 2 / Moderate | Medium | Low / |
| 4 | Traffic Issues/ Traffic Management Plan | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 5 | Land acquisition and resettlement impacts | 2 / Unlikely | 2 / Moderate | Low | Low |
| 6 | Topographic Impacts due to Excavations | 2 / Unlikely | 3 / Moderate | Moderate | Low |
| 7 | Seismic Impacts | 2 / Unlikely | 2 / Moderate | Low | Low |
| 8 | Water Usage/ Extraction from Current Sources | 2 / Unlikely | 2 / Moderate | Low | Low |

Critical Risk Level

Significant Risk Level

Medium Risk Level

Low Risk Level

### Adverse Impacts during Planning and Design Phase

Potential adverse impacts during the planning and design stage of the project are given below.

#### Improper Designing of Distribution Networks and Conductance Main (Replacement and New Distribution Lines)

- Impacts

If location of water distribution networks is not carefully selected considering topography, geology and catchment of watershed results in degradation of channel embankments, reduce water flow or increased chances of structure settlement due to weak geological conditions/weak bearing capacity of soil.

The water shall be conveyed to distribution network is also primarily designed on gravity. If not designed properly while maintaining minimum head, the system will require high energy for pumping water to the desired storage tanks.

Mitigation Measures

- The following mitigation measures will be implemented:

Factors such as site capacity, accessibility, stability, environmental sensitivity, land use, socio-economic receptors and climate hazards will be studied and site will be selected accordingly.

Contractor will prepare and obtain approval of SSEMP through PMU/CSC with consent of ADB environment team, before establishing any camp site.

Water supply network will be designed based on gravity flow to facilitate uninterrupted water supply through gravity system.

Residual Impact

If the proposed mitigation measures are implemented, no significant adverse residual environmental effects are likely to occur. However, in case improper designing of conductance main, the residual impact will be low adverse in nature.

#### Lack of Integration of IEE/EMP Requirements into Construction Bid Documents

- Impacts

The bidding documents must reflect the requirement to select a qualified and experienced Contractor from the perspective of ensuring implementation of required environmental safeguards provisions during project development.

Mitigation Measures

The environmental staff of PMU will be assigned the task to check that design and bid documents are responsive to key environmental, social and safety considerations, and that the proposed method of work reflects the boundaries defined in the EMP. The bid documents must include the EMP and its implementation cost must be reflected in the BOQ.

IEE/EMP implementation and monitoring requirements must be part of bidding documents and necessary contractual binding must be agreed by project contractors before award of contract.

Project contractors shall have qualified and experienced environmental staff to plan, arrange, implement, monitor and report IEE/EMP requirements.

Residual Impact

Upon integration of EMP requirements into construction contract agreement, and upon implementation of EMP in true letter and spirit, the residual environmental effects are expected to be low in nature.

#### Relocation of Utilities

Utility services such as sewerage lines and telephone lines lying along the project alignment may need to be relocated temporarily or permanently to prevent damage to them.

- Mitigation Measures

Detailed filed surveys will be conducted to assess any sub-surface utilities that might be present under the proposed project alignment within the right of way of the project, shall be relocated prior to the commencement of the work.

Based on the utilities that are identified during the detailed surveys, a relocation plan of those utilities will be developed by the respective line agencies in close coordination.

In case any utilities can be allowed to remain in place and avoidance of their damage is possible, the contractor will need to be aware of the location of these services so that disruptions are not caused. In such scenario, the responsibility for any repair of damaged services will lie with the contractor.

Residual Impact

Provided the proposed mitigative measures are implemented, the magnitude and extent of the environmental effects will be low. Therefore, no significant adverse residual environmental effects are likely to occur.

#### Traffic Issues

- Impacts

In particular, the lying of water pipelines could result in heavy congestion due to the movement of construction heavy machinery and equipment with significant delays for traffic apart from posing a significant risk for vehicles.

Mitigation Measures

A comprehensive traffic management plan shall be developed by the contractor and approved by the Executing Agency or the Project Management Unit that will contain the traffic diversion onto alternate routes and management of traffic flows to minimize the congestion and the possibility of accidents. This plan shall be dependent to the EMP and will be implemented by the contractor.

Residual Impacts

After implementing the traffic management plan, the residual impacts will be low.

#### Land Acquisition and Resettlement Impacts

- Impacts

The proposed project involves the distribution of the water supply network. This water distribution network or pipes will be laid within the existing network of water pipes, so no land acquisition is involved in this proposed project.

Mitigation Measures

No mitigation measures are required.

Residual Impacts

No residual impacts so it will remain low.

#### Topographic Impacts due to Excavation

- Potential Impacts

The project area has plain topography but no land acquisition is involved for the proposed project. However, excavation of land is involved for the proposed project. A minor physical change in the topography due to excavation of land is expected in the project area. This adverse impact is low in nature.

Mitigation Measures

Mitigation measures will involve adoption of best engineering design measures keeping in view of the aesthetics of the project area and provision of green areas for the landscape in design. The proposed design has tried to minimize these impacts.

Residual Impact

No residual effects from this component of the project are anticipated as no significant topographical change is anticipated.

#### Seismic Impacts

- Potential Impact

In Building Code of Pakistan (Seismic Provisions, 2007), the whole region of Pakistan is classified into five main Seismic Hazard zones i.e., i) Zone-1, ii) Zone-2A, iii) Zone-2B, iv) Zone-3, v) Zone-4 based on the peak horizontal ground acceleration and its ultimate tendency to damages during seismic events. The proposed project area falls in the seismic zone classified as “Zone-2B” with g-value 0.16 to 0.24 and is considered as ‘moderate’ hazard seismicity zone.

No change (+/-) in the impact of seismology risk of the area is expected during the project construction phase as none of the project activities is expected to be of such a powerful extent to influence the tectonic risk. This adverse impact is medium in nature.

Mitigation Measures

The proposed structure should be designed and constructed keeping in consideration high intensity earthquakes. For seismic hazard analysis, updated structural and seismic evaluations should be conducted by the design engineer/consultant. Moreover, geo-technical investigations must be conducted prior to construction phase.

Residual Impacts

Upon implementation of mitigation measures suggested above, no significant residual impact is anticipated.

#### Water Usage/ Extraction from Current Sources

- Potential Impact

Excessive extraction of water from reservoir will cause impact on the other uses of water. However, the impact of water usage or its extraction from water source is low/ negligible in nature as the source has capacity to manage water supply for the current project.

Mitigation Measures

Water must be extracted as planned and over extraction must be avoided; and

Regular measurement/ monitoring must be done to avoid the over extraction of water from the source.

RWASA has conducted studies for available water resources for Rawalpindi, including the water available from Khanpur Dam. Already existing 300 mm MS pipe is extracting water for KSS from a 600 mm (ID) supply main which is bifurcated from a 1000 mm (ID) main line coming from Tomar reservoir. The Tomar reservoir is fed by a 1400 mm main line from Sangjani treatment plant based on Khanpur dam.

Khanpur Dam has a 51 MGD (231,851 m3/d) design capacity. The actual water supply from this dam to Rawalpindi city is only 27,277 m3/d (6 MGD). The dam has the capacity to provide more water to Rawalpindi for drinking purposes.

Residual Impacts

Upon implementation of mitigation measures suggested above, no significant residual impact is anticipated.

### Adverse Impacts during Construction Phase

#### Impact Screening Matrix

The screening of potential impacts during the construction phase is provided in Table 6.5 below.

Table 6.5: Screening of Possible Impacts during Construction Phase for Water Supply Component

| Sr. No. | Potential Impacts | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Short term, Long term) |
| --- | --- | --- | --- | --- | --- |
| 1 | Soil erosion and cutting of roads and pavements due to excavations for laying of pipelines | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 2 | Site Accessibility | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 3 | Traffic Issues | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 4 | Resource Conservation | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 5 | Occupational Health & Safety | 3/ Likely | 3/ Moderate | 9/ Medium | Low |
| 6 | Construction Camps/ Camp Site | 5/ Likely | 2/ Moderate | 10/ Medium | Low |
| 7 | Wastewater Generation | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 8 | Solid Waste Generation | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 9 | Disposal of Spoil/ Demolition Waste | 4/ Likely | 2/ Moderate | 8/ Medium | Low |
| 10 | Social and Cultural Conflicts | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 11 | Community Health & Safety | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 12 | Communicable diseases incl. COVID-19 | 2/ Likely | 3/ Moderate | 6/ Medium | Low |
| 13 | Flora | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 14 | Fauna | 3/ Likely | 2/ Moderate | 6/ Medium | Low/ |
| 15 | Degradation of Air Quality | 3/ Likely | 2/ Moderate | 6/ Medium | Low/ |
| 16 | Natural and Man Made Hazards | 3/ Likely | 2/ Moderate | 6/ Medium | Low/ |
| 16 | Noise/Vibration | 3/ Likely | 2/ Moderate | 6/ Medium | Low/ |
| 17 | GHG Emission | 3/ Likely | 2/ Moderate | 6/ Medium | No Residual Impact |
| 18 | Site Restorations | 2/ Likely | 10/ Moderate | 10/ Medium | Low/ |

Critical Risk Level

Significant Risk Level

Medium Risk Level

Low Risk Level

#### Impacts due to Excavation for Conductance Mains, Water Distribution Networks and Installation of Control Valves

- Impacts

Construction activity of water distribution network and transmission main will be conducted along the roads within the existing Right of Way (ROW) of the water lines.

The excavation will be done for laying of Conductance Main from Sector I-10 of Islamabad area will take place on the green strip of Service Road of I-10. Open trench of around 1.5-meter depth will be excavated for laying of Conductance Main (Bulk Supply Line) and the trench of around 2.5 feet will be excavated for replacement of distribution lines along the existing ROW.

Cutting of roads and pavements will be involved in excavation activity.

The excavated soil, placed along the trench may get disturbed due to wind, rain water and the movement of workers, vehicles and pedestrians, and spill onto road way – disturbing road users, creating dust, road safety issues, etc., and also into nearby open drains.

There is also a possibility of silt runoff during rainy season causing soil erosion. During the rain, the eroded soil mix with stagnant water to transform into slush, which can affect movement of vehicles and machinery and construction work as well as limit the movements of local people.

The invasive nature of excavation will result to impacts to the sensitive receptors of sub project locations such as movement of community to schools, hospitals and mosques.

Mitigation measures

Mitigation measures adopted for construction of water supply networks and Conductance Main are provided below:

These anticipated impacts are temporary and for short duration. Physical impacts will be reduced by the method of working and scheduling of work, whereby the project components will be (i) constructed by small teams working at a time; (ii) any excavation done near sensitive area like school, religious places and house will be protected as per standard construction practices.

IJP road will not be disturbed and the impact will be mitigated by construction methodology (strategy). The Conductance Main will be jacked under the IJP road and no road cutting will be done.

No additional land is required. All the construction work will be done on the existing ROW of roads.

Prior to starting of work, the contractor shall prepare a method statement for water supply pipeline works. This shall be simple and explain the contractor’s work process that is actually conducted on site, with safety and safeguard concerns.

Almost 80% of this material will be reused in backfilling the excavations while remaining will be used for backfilling of low laying areas by contracting with private housing societies.

Good engineering practices will help to control or minimize the soil erosion both at the construction sites and in peripheral areas. All the disturbed areas need to be protected against soil erosion by stripping and stockpiling of all the available topsoil for later re-vegetation.

Special slope protection measures will be adopted in the sensitive areas and along the shoulders of roads and excavations shall be kept confined to the specified foundation spots as per the approved engineering drawings.

Barrication shall be carried out with steel sheets to control soil erosion due to wind, movement of pedestrians and vehicles.

Method Statement is very important, particularly for water supply pipeline works along the roads.

Method Statement can be prepared for each stretch (say 1 km) /specific site based on the project area.

Method Statement shall be in a Table format with appended site layout map and cover the following:

- Work description

No. of workers (skilled & unskilled)

Details of Plant, equipment & machinery, vehicles

Work duration (total and activity-wise for example for pipe laying, from excavation to road resurfacing/testing) (helmet, gloves, boots, etc.) details for each type of work

Risks/hazards associated with the work (for example, Trench excavation will have risks such as trench collapse, persons/vehicles falling into trench, structural risk to nearby buildings, damage to buildings, infrastructure etc.)

Detail the sequence of work process (step-by-step) including specific details of each work

Contractor’s supervision & management arrangements for the work

Emergency: Designate (i) responsible person on site, and (ii) first aider

Typical site layout plan including pipe trenching, placement of material, excavated earth, barricading etc.

The following shall be included in the site layout plan:

Provide barricading/security personnel at the site to prevent entry/trespassing of pedestrian/vehicles into the work zone.

Location of temporary stockpiles and provision of bunds.

Separation of stockpiles areas with worker's/vehicle movement paths to avoid disturbing the stockpiled soil.

Wetting of soil to arrest dust generation by sprinkling water

Waste/surplus soil utilization and disposal plan – indicate expected duration of temporary stockpiling along the trench at each site and identify final surplus soil utilization/disposal site in consultation with CSC/PMU.

PMU LG&CD will ensure the identification of disposal sites for spoil.

Stock piling of excavated material at places that are congested will be avoided as these piles can create traffic issues and public nuisance.

The excavated material shall be stored at least 3 feet away from the edges of excavation to avoid the chances of slope failure and pouring of excavated material into trenches.

Residual Impact

After implementation of the proposed mitigation measures, the residual impacts would be low in nature.

#### Site Accessibility during Construction

- Potential Impact

The proposed project can be approached through various roads. The construction material, equipment and machinery will be arrived at proposed workshop area and construction camps sites. From the camp, the material, equipment and machinery will be transferred to work site by multiple routes.

Mitigation Measures

The accessibility of road for local population will be ensured by implementation of Traffic Management Plan. The framework for Traffic Management Plan is given as Annexure-V of this IEE report.

Selection of access roads/tracks should be based upon certain criteria to ensure high stability, no or minimum tree cutting and crop damage, minimum disturbance to nearby settlements and other social amenities;

The approach roads should be selected to avoid any soil degradation and erosion impacts;

Special care should be taken for known sensitive areas to ensure that all interested parties are aware of the type and location of working. Measures should be taken where necessary to protect such areas physically; and

Mitigations for the loss of trees and crops, soil erosion, soil contamination, air and noise pollution, waste generation, and loss of agricultural land provided in below sections will be adopted.

Residual Impacts

Provided the proposed mitigative measures are implemented, the environmental impacts will have low magnitude and will be reversible. Therefore, no significant adverse residual effects are expected.

#### Traffic Issues

- Potential Impact

The construction phase of the proposed project, the laying of the water supply pipelines, could result in heavy congestion due to the movement of construction heavy machinery and equipment’s with significant delays for the traffic apart from posing a significant accident risk for vehicles.

The efficient management of traffic once the construction activity commences will be critical in order to minimize the risk of possible road accidents and construction related hazards.

Mitigation Measures

To minimize traffic problems in the proposed project area, following measures will be considered:

- Movement of vehicles carrying construction materials and equipment/machinery will be restricted during the daytime to reduce traffic load and inconvenience to the local population;

- Construction vehicles, machinery and equipment will be parked at designated areas (at construction camps site) to avoid un-necessary congestions on roads;

- The speed of the vehicles will be controlled (at 20 to 25 km/hr) to reduce the probability of severe accidents, soil erosion, debris flows due to vibrations and dust emission;

- Damages of roads due to construction vehicles will be instantly repaired and/or compensated after the completion of work;

- Proper sign boards will be provided for smooth flow of traffic;

- Period of construction and area / location of construction site shall be informed to public in general and specifically to local residents; and

- Any closure of the roads (especially main roads) and deviations / diversions proposed should be informed to the riders through standard signs and displays.

- Traffic Management Plan will be implemented to avoid traffic accidents, jams/public inconvenience. Guideline for Traffic Management Plan has been prepared and given as Annexure-V.

Residual Impacts

Upon implementation of the proposed mitigation measures, no significant adverse residual effects are anticipated. Further, traffic management plan has been developed and after its implementation, the residual impact will be low.

#### Degradation of Ambient Air Quality

- Potential Impacts

Air quality will be affected by fugitive dust emissions from construction machinery; dust from the unpaved surface and construction vehicles. Emissions may be carried over longer distances depending upon the wind speed, direction, temperature of surrounding air and atmospheric stability.

Potential impacts on the air quality during the construction stage will be due to the fugitive dust and the exhaust gases generated in and around the construction site. Dust is a major component of air pollution, generated mainly from the following construction activities:

Site clearance and use of heavy vehicles and machinery/equipment etc. at construction site;

Procurement and transport of construction materials such as sand, cement, etc. to the construction site;

Other Gaseous emissions during construction result from operating of construction vehicles, plant and equipment;

The sensitive receptors identified within or near the project length are educational institutions and religious and cultural places etc. which may be affected during construction phase, due to emission of air pollutants. The construction activities will also result in increased air pollution in the area. The overall impact on the quality of air during the construction phase will, however, be temporary and limited to the project’s implementation phase only. This impact is high adverse in nature.

Mitigation Measures

The major source of air emission is fugitive dust emitted from construction machinery and movement of vehicles. The impacts construction phase of the proposed project could be effectively mitigated by the implementation of simple procedures by the Contractor including but not limited to the following:

- All vehicles, machinery, equipment and generators used during construction activities should be kept in good working condition and be properly tuned and maintained in order to minimize the exhaust emissions;

- Open burning of solid waste from the Contractor’s camps and at construction site should be strictly banned;

- Preventive measures against dust should be adopted for on-site mixing and unloading operations;

- Construction and spoil materials will be transported trucks covered with tarpaulins and all vehicles (e.g., trucks, equipment, and other vehicles that support construction works) will comply with the PEQS (as amended) for carbon emissions and noise;

- Contractors shall comply with submitted work schedule, keeping emission operations away from sensitive receptors and residential areas;

- Regular water sprinkling of the site should be carried out to suppress excessive dust emission(s);

- Emissions from power generators and construction machinery are important point sources at the construction sites. Proper maintenance and repair is needed to minimize the hazardous emissions;

- Construction equipment is generally left idling while the operators are on break or waiting for the completion of another task. Emissions from idling equipment tend to be high. Existing idling control technologies, which automatically shut the engine off after a pre-set time can reduce emissions, without intervention of the operators;

- Along the project alignment with water pipelines and trunk sewer being laid, water will be sprinkled at high frequency especially during summer season at all construction sites to suppress dust emissions;

- Construction workers should be provided with masks for protection against the inhalation of dust;

- Vehicles carrying construction material or mucking material will be covered with tarpaulin sheets to avoid the dust emissions;

- Stockpile material will also be covered with tarpaulin sheets to avoid the dust emission;

- Vehicles used for construction should be tuned properly and regularly to control emission of exhaust gases;

- Regular monitoring of air quality in accordance with PEQS, 2016 and WHO Guidelines; and

- Dust Management plan has been developed and will be implemented by contractor to manage dust emission. Dust management plan is attached as Annexure-IX of this report.

Residual Impact

It is anticipated that air emissions will not exceed the ambient air quality standards after implementation of the suggested mitigation measures.

#### Noise level

- Potential Impact

The laying of water pipelines and installation of water pumps will result in different construction equipment and machinery being used which will generate high noise levels at the project sites and in the project areas.

The noise and vibration will be produced due to the operation of construction machinery and equipment. Sources of noise and vibration during construction are heavy machinery such as bulldozers, excavators, concrete mixing plant, pneumatic drills and other equipment’s. Noise and vibration are perceived as one of the most undesirable consequences of construction activity. The above machinery is expected to generate noise levels that would be severe in the project area.

The cumulative effects from several machines can be significant and may cause significant nuisances. However, these increased noise levels will prevail only for a short duration during the construction stage. The noise produced by construction equipment is presented in Table 6.6 below as reference.

Table 6.6: Noise Produced by Construction Equipment

| Sr. No. | Equipment | Noise dB(A) at 15ft from Source |
| --- | --- | --- |
| 1 | Wheeled loading | 90 |
| 2 | Grader | 90 |
| 3 | Bulldozer | 86 |
| 4 | Sprayer | 87 |
| 5 | Power generator | 98 |
| 6 | Impact drill | 87 |
| 7 | Truck | 92 |
| 8 | Concrete mixer | 91 |
| 9 | Concrete pump | 85 |
| 10 | Pneumatic hammer and rock crusher | 98 |
| 11 | Breaker | 84 |
| 12 | Pneumatic spanner | 95 |

Source: Quagliata, A., Ahearn, M., Boeker, E., Roof, C., Meister, L., & Singleton, H. (2018). Transit Noise and Vibration Impact Assessment Manual (No. FTA Report No. 0123).

The likely impacts due to noise are:

Psychological effects of distraction of attention, irritation and short temperedness in the exposed persons due to persistently higher noise levels; and

Noisy settings and higher background levels can cause temporary threshold shift and the consequent habit of speaking loud, which may cause damage to vocal cords in the persons exposed.

The sensitive receptors identified within or near the RoW are educational institutions and religious and cultural places etc.  which may affect during construction phase, due to noise and vibration.

Though the construction method has not been determined yet, however it is believed that the adopted method of construction shall produce less noise and vibration if the suggested mitigation measures are adopted. This impact is high adverse in nature.

Mitigation Measures

There are a variety of ways by which construction equipment and worksite noise can be controlled. The following is a list of ways to control noise level at the worksite of the proposed project:

Quieter Equipment: A cost-effective way to reduce noise at a construction worksite is to buy quiet equipment. In addition, equipment in use should be the most suitable for the job. Avoid using equipment that is over-powered and, conversely, avoid using under powered equipment. Whenever possible the quietest equipment alternative should be used. In general, electronic powered equipment is quieter than diesel powered equipment and hydraulically powered equipment is quieter than pneumatic power.

Modifying Existing Old Equipment: The most common way to reduce the noise levels of common construction equipment is through worksite modifications. Some common worksite modifications consist of retro-fitting existing equipment with damping materials and mufflers.

Barrier Protection: An effective way of reducing noise is to locate noisy equipment behind purpose-built barriers. The barriers can be constructed on the work site from common construction building material (plywood, block, stacks or spoils) or the barriers can be constructed from commercial panels which are lined with sound absorbing material to achieve the maximum shielding effect possible. To be effective, the length of the barrier should be greater than its height. The noise source should not be visible and barrier should be located as close as possible to either the noise source or the receiver.

Work Activity Scheduling: Work activity scheduling are administrative means to control noise exposure. Planning how noise sources are sited and organized on a work site can reduce noise hazards. Jobs can be rotated so that exposure time is limited. Transferring workers from a high exposure task to a lower exposure task could make the employee’s daily noise exposure acceptable.

Maintenance: Increased attention to maintenance of tools and equipment will reduce worksite noise levels. Maintaining plant and equipment in good order not only increases its life, but makes it safer to use and quieter. Loose and worn parts should be fixed as soon as possible. Ideally, the worksite should have a system in place for checking and servicing the various machines and power tools.

Use of low noise machinery, or machinery with noise shielding and absorption;

Contractors shall comply with submitted work schedule, keeping noisy operations away from sensitive receptors, residential areas; implement regular maintenance and repairs; and employ strict implementation of operation procedures;

The Contractor shall comply with PESQ, 2016.

In addition to the above mentioned mitigative measures, Noise and Vibration Management Plan has been developed and appended as Annexure-X.

Residual Impact

Provided the proposed mitigative measures are implemented as suggested, the environmental effects due to noise will have a low magnitude. Therefore, no significant adverse residual impacts on the acoustic environment are likely to occur. Therefore, the residual impacts due to noise and vibration will be low.

#### Vibration Induced Impacts

- Potential Impact

The duration and amplitude of vibration generated by construction equipment varies widely depending on the type of equipment and the purpose for which it is being used. In assessing vibration from construction equipment, it is useful to categorize the equipment by the nature of the vibration generated. Various equipment categories according to type of vibration and/or activities in each category are listed below:

Equipment or activities typical of continuous vibration include:

Excavation equipment;

Static compaction equipment;

Tracked vehicles;

Road traffic;

Vibratory pile drivers;

Pile-extraction equipment; and

Vibratory compaction equipment.

Energy from construction equipment is transmitted into the ground and transformed into vibration, which weakens with distance. The magnitude and attenuation of ground vibration is dependent on the following:

The efficiency of the energy transfers mechanism of the equipment (i.e. impulsive, reciprocating, rolling or rotating equipment);

The frequency content;

The impact medium stiffness;

The type of wave (surface or body); and

The ground type and topography.

Any vibration would result in nuisance effects, and will be localized and temporary and will unlikely result in structural damage to buildings or walls of the adjacent private properties. However, there is likelihood of human discomfort and damage to infrastructures (crack formation) due to the vibration caused by construction machinery. This impact is high adverse in nature.

Mitigation Measures

All vibration producing machinery will be identified by the contractor and vibration producing machinery will be operated at low frequency to reduce the vibration impacts.

Vibration monitoring will be carried out in accordance with German Standard DIN 4150 (heritage structures) and BS 7385: Part 2 – 1993 (other structures).

vibration monitoring will be undertaken at the nearest buildings to the construction sites during times of vibration intensive works

Where exceedances of the criteria are recorded, corrective actions would be implemented where soils at risk of vibration induced settlement are identified.

A regular programme of vibration monitoring shall be implemented as a minimum in accordance with of this document.

The Contractor will submit the proposed method, the frequency and the location of monitoring site to the Planning Authority for agreement prior to commencing works as part of vibration management plan.

#### Wastewater Generation

- Potential Impact

One (01) construction camp is proposed to be established during construction period for the proposed project.

Wastewater will be generated at the construction camp and from construction activities. If the generated wastewater is not properly treated or disposed of, this may contaminate the surface water sources such as nullahs, drains, water channels etc. apart from soil contamination. Water from dewatering activities (during rainy season) has the potential to contain suspended solids and oil and grease and if disposed of untreated may affect the soil quality. The Error! Reference source not found. below shows anticipated composition and estimate of the wastewater to be generated by construction workers during the course of the construction phase of the project assuming that on average the water demand per person is 50 liters per day and that 80% of the water demand will become wastewater.

Table 6.7: Estimated Wastewater Generated by Workers in Construction Camps

| Sr. No. | No. of Workers* | Estimated Total Water Demand** (liters/day) | Estimated Wastewater Generated (liters/day)*** |
| --- | --- | --- | --- |
| 1 | 150 | 7,500 | 6,000 |

*= “Tentative Work Force Requirements Including Client and Contractor Staff”

** = (150) x (40) = 7,500 liters/day

*** = (7,500) x (80%) =6,000 liters/day

This 6,000 liters/day of wastewater is from the construction camp. This impact can be categorized as medium adverse in nature.

Mitigation Measures

To dispose the liquid waste generated from the construction activities, the following steps will be taken by the contractor:

Domestic and chemical effluents from the construction camp will be disposed by the development of on-site sanitation systems i.e. septic tanks.

Proper monitoring to check the compliance of PEQS will be carried out; and

Sewage from construction camps will be disposed of after proper pre-treatment and processes such as soakage pit.

The contractor(s) will be responsible to submit details of site-specific wastewater management plan along with details of wastewater collection, transportation and its disposal.

Residual Impacts

Provided the proposed mitigative measures are implemented, no significant adverse residual environmental impacts are likely to occur and the residual impact is low.

#### Solid Waste Generation

- Potential Impact

Considering the labourers (about 150 in numbers) residing in the construction camp and the locally available labour, an average solid waste generation rate of 0.6 kg/capita/day is adopted for the estimation of solid waste generation. Based on this assumption, a total of about 90 kg of solid waste will be generated from construction camp on daily basis. The major components of the labour camp waste will be garbage, putrescible waste, rubbish and small portion of ashes and residues. Other type of wastes may include inorganic construction wastes including hazardous waste.

The estimated quantity of excavation material for ROW clearance cannot be provided at design stage. The contractor will provide the estimated quantity of excavation material during the construction phase. These wastes will be generated due to the construction activities and the materials used for construction. This waste would require proper disposal to minimize land and water contamination. This impact can be categorized as medium adverse in nature.

Mitigation Measures

All the solid waste from the camps will be properly collected at source by placing containers and disposed of through proper solid waste management system. The contractor will coordinate with local representatives and administration of the concerned solid waste management department for the disposal of solid waste;

The contractor must develop a plan of action with the help of concerned solid waste management department for transporting the waste to the disposal site;

Toxic waste will be handled, stored, transported and disposed separately;

The waste will be properly sealed in containers with proper labels indicating the nature of the waste; and

Solid waste will be segregated at source so that it can be re-used or recycled.

Waste Management Plan will be developed to implement an efficient and responsive solid waste management system during construction phase. Recyclable wastes e.g. steel bars will be sold to waste vendors;

Reusable material will be used as a filling material during ground levelling;

Solid waste generated during construction will be safely disposed in demarcated waste disposal sites and the contractor will provide a proper waste management plan; and

The site will be restored back to its original conditions after construction completion.

Residual Impact

Upon implementation of the proposed mitigation measures, no significant adverse residual effects are anticipated and the residual impacts will be low.

#### Disposal of Spoil and Demolition Waste

- Potential Impact

During construction of the project, especially during the excavation and dismantling of the existing distribution pipeline, spoil/ demolition waste will be generated which need to be addressed. However, this impact is categorized as medium adverse in nature. Approximately 552,664 Cft. Spoil will be produced. Almost 80% of this material will be reused in backfilling. As the project involve replacement of AC and GI pipelines with HDPE pipeline, the redundant AC pipelines will be removed and must be handled with care to avoid release of asbestos fibers into environment.

Mitigation Measures

Following mitigation measures shall be followed:

Waste to be reused in the project or by any other interested parties.

80% of the excavated material will be re used at the project site in backfilling of the excavation / trenches for Conductance Mains and distribution network. The remaining material will be disposed off at designated site of TMA Rawalpindi.

It is important to mention here that contract with rapidly developing housing societies shall be made to use leftover soil for earth filling / backfilling of low lying areas. This will reduce the burden on landfills.

All material that is produced shall be transported in tarpaulin covered trucks for disposal at the designated site. At the disposal site, the demolition waste will be disposed off in accordance with the best practices to ensure no residual impact.

Disposal trucks will be properly maintained to minimize oil leakages during the construction.

ADB's Good Practice Guidance for the Management and Control of Asbestos, Protecting Workplaces and Communities from Asbestos Exposure Risk shall be followed during the replacement of old AC distribution pipelines of KSS. The project contract/Manager shall implement the following guidelines.

Employers/project managers shall engage suitably qualified, experienced and licensed (if required) asbestos contractors for asbestos removal work.

Employers/project managers shall ensure the asbestos removal work does not result in unacceptable risk to other workers and communities.

Employers/project managers shall ensure that the area of asbestos removal is appropriately validated following asbestos removal work by a suitably qualified, experienced and licensed (if required) competent person prior to allowing access to the area.

Asbestos specialists shall be hired by the contractor to remove AC pipes.

Demolition waste will not be disposed off at any other area other than the designated area.

The Asbestos Cement Pipelines falls under Class II non—friable. For handling of this waste the workers must be properly trained with annual refreshers.

Framework for Asbestos Management Plan is given as Annexure-XI

Asbestos containing material (ACM) will not be used as a new material in rehabilitation works. Existing asbestos-cement pipes will be removed and disposed of following the applicable IFC (2007) guidelines for asbestos-containing materials (ACM) for its removal and disposal.

The Contractor must develop a feasible occupational health asbestos action plan, as per ACM Health & Safety Management Plan.

Continuous supervision by Competent supervisor must be carried out

Engineering Controls including demarcation, wet methods, drop cloths, clean-up and disposal, etc. must be adopted for transportation of redundant AC pipelines.

Try as much as possible to remove large lengths of pipeline to avoid cutting and damage to pipeline to control release of asbestos fibers.

Respiratory Protection in the form of full face mask and face shield along with protective clothing including gloves, apron etc. must be provided to workers who will be involved in handling of AC pipelines.

Disposal of the AC pipelines must be ensured at designated landfill site. The pipelines must be wrapped in double layer of at least 6mil thick plastic sheet.

Residual Impact

After implementation of the proposed mitigation measures, the residual impacts would be low in nature.

#### Resource Conservation

- Potential Impact

During the construction, overburden on local resources is possible if construction facilities such as workers camp and construction camp built near rural areas and can create problems for local communities. There can be a conflict for resources between workers and local community.

Mitigation:

Use potable water bowsers for construction works and mineral water bottles/ ground water for drinking purpose;

Plan for the provision/purchase of adequate insulation to reduce heat loss through construction plants;

Reduction of wastage of water through training of workers involved in water use;

Reuse of construction waste materials may be adopted wherever possible;

Aggregates will not be sourced from river and stream beds. Construction material will be sourced from Margalla and Sargodha quarry areas;

Diesel and fuels with low sulphur content should be used to operate construction machinery and equipment;

Efficient and well -maintained equipment and machinery should be used;

The equipment and machinery should be turned off when not in use;

Use of solar panels at construction camps may be considered and plan for use of solar panels at operational phase may also be considered; and

A good camp design and an efficient worksite management plan can help the contractor to reduce the water demand, wastewater and solid waste volumes to the lowest levels.

Resource Conservation Plan has been developed for the proposed project and attached as Annexure-VI.

Residual Impact

Provided the proposed mitigative measures are implemented as suggested, the environmental impacts will have low magnitude on resource conservation. Therefore, low residual impacts due to the project are anticipated.

#### Flora

- Potential Impact

No vegetation exists within the existing right of the water pipe lines and the proposed project deals only the laying of water pipe lines within the existing right of way along the existing pipelines. Therefore, no damage to the existing flora is envisaged due to the laying of proposed water supply lines. However, the installation of water pumps may damage the grass near the existing overhead water reservoirs.

This impact is negligible and temporary in nature.

Mitigation Measures

Cutting and / or disturbance of trees shall be avoided, as far as possible;

A tree plantation program shall be implemented with the recommendations and technical support of Punjab, Forest Department for the enhancement ecological condition of the area; This program is requirement of local EPA as well as local government.

Final and actual plantation and trees plantation cost will be finalized after assessment by Punjab, Forest Department.  All the activities will be executed as per agreement/MOU between proponent and Punjab, Forest department while strictly considering law of land.

Open fires should be banned in the area to avoid hazards of fire in the area;

Damage to the natural vegetation may be minimized by establishing camp sites, workshops and concrete mixing plants on waste/barren land rather than on forested or green land. However, if such type of land is not available, it shall be ensured that minimum clearing of the vegetation is carried out and minimum damage is caused to trees and undergrowth;

Construction vehicles, machinery and equipment will remain confined within their designated areas of movement;

The Contractor’s staff and labor shall be strictly directed not to damage any vegetation such as trees or bushes; and

Contractor shall provide gas cylinders at the camps for cooking purposes and cutting of trees/bushes for fuel shall be strictly prohibited.

Residual Impacts

Given the type of vegetation involved, and the presence of type of land adjacent to the Project Site, the residual effects on vegetation are considered not significant.

#### Fauna

- Potential Impact

During construction phase the existing population of birds, mammals and reptiles of the construction areas will be affected due to disturbance arising from construction activities involving excavation, movement of machinery and vehicular traffic, movement of labor, camping, etc. The existing fauna will leave the directly affected areas due to construction activities and human intervention. Some animals particularly reptiles may get killed during the earthwork’s operations. Moreover, the movements of the mammals and reptiles will be restricted during the construction phase.

Birds as well, will tend to move away from the construction areas and find shelter and food elsewhere due to the activities mentioned above for fear of being hunted / trapped.

Noise generated from machinery will even scare the wildlife residing in habitats located at some distance from the construction areas.

Food and refuse at the Contractor’s camps may attract animals that might result into encounter with the workers. This impact is indirect, site-specific, temporary, reversible, possible and medium significant.

Mitigation Measures

Care shall be taken during construction activities for avoiding purposely or chance killing of animals;

If any wild species and habitat is found during construction, it must be dealt carefully and local wildlife department officials should be informed;

Hunting, poaching and harassing of wild animals shall be strictly prohibited, and Contractor shall be required to instruct and supervise its labor force accordingly and clear orders should be given in this regard;

The Contractor must be held responsible for instructing his work force accordingly and for enforcing this restriction. In addition, this shall have to be controlled by the Wildlife Department.

Special measures shall be adopted to minimize impacts on the wild birds, such as avoiding noise generating activities during the critical periods of breeding;

Noise generating activities shall not be carried out during the night by the work force;

Wastes of the camps shall be properly disposed of to prevent it being eaten by animals, as it may be hazardous to them; and

Noise produced due to construction activities may be kept to acceptable level.

Residual Impacts

The residual, after implementation of suggested mitigation measures, will be low.

#### Social and Cultural Conflicts

- Potential Impact

During the construction phase of the proposed project, conflicts may arise between labor force and local community. Use of local resources and products by the construction workers can generate stress on the local resources. Furthermore, difference in cultural values may also cause discomfort to local residents. This impact is negative, local, low, short-term and probable.

Mitigation Measures

Local labor especially from nearby communities should be given preference for the construction works;

Careful planning and training of work force to minimize disturbance to the local people;

Public notification through print or electronic media during the entire construction phase to avoid any inconvenience in accessibility to the locals; and

Adequate training especially for the transitive workforce of the station (involved both in the construction process and in the commissioning) to regard the customs of the area so that the locals do not feel insecure.

Residual Impacts

Implementation of mitigation measures will reduce the risk of conflicts in society and will results into short term and low residual impacts.

#### Emission of Green House Gases (GHGs)

- Potential Impact

The main sources of greenhouse gases (CO2, CH4, NOx etc.) during the construction activities of the proposed project will include both mobile and stationary sources. The mobile source will be the construction and transportation vehicles while the stationary source will be the backup generator for electricity. Emission of greenhouse gases cause global warming and other climatic changes on regional and global scale. This impact can be categorized as low adverse in nature.

Mitigation Measures

Regular motioning of the vehicles for engine efficiency;

Avoid idling of construction vehicles;

Alternative energy resources shall be considered where possible; and

The Contractor shall ensure the compliance with NEQS/PEQS.

Residual Impacts

After implementation of suggested mitigation measures the residual impact will be low.

#### Natural and Man-Made Hazards

- Potential Impact

The construction of the proposed project may encounter hazards such as earthquakes, and fires may occur. Lack of Emergency Response Plan (ERP) or an inefficient response plan may lead to an accident or critical injury. This impact is medium adverse in nature.

Natural disasters (earthquakes & floods) and man-made disasters like excavation collapse, structural damage due to deep excavations and fire accidents may result in injuries, financial losses and may even lead to deaths.

Release of natural gas, due to damage to gas pipelines may also occur during excavations operations.

Incidence of collapse of any nearby structure due to deep excavation can also take place during the construction activities which needs to be avoided.

Mitigation Measures

- Mitigation measures include the following:

An ERP for earthquakes and manmade disasters should be developed by the contractor, in coordination with SC and the proponent (WASA), should be implemented in close consultation with the RESCUE 1122 Services and other concerned departments;

Training of the contractor and WASA staff and employees regarding the emergency procedures and plans should be regularly conducted;

In case of excavation near to the existing structure is required, then the appropriate controls measures must be taken before start of excavation;

Bracing / Wall strengthening/support for deep excavation need to be placed to avoid collapse;

Emergency numbers should be clearly posted at all disposal stations;

Minor incidents and near misses should be reported, and preventive measures should be formulated accordingly by the WASA Management.

A climate change resilience plan or climate change adaptation measure will be adopted to avoid any climate change impacts of the project.

Emergency Response Plan has been developed to manage all potential and actual emergency situation during construction phase. The same is attached as Annexure-VIII of the report.

Residual Impact

Successful implementation of mitigation measures suggested here and in Emergency Response Plan (attached as Annexure) will keep the residual impact to low.

#### Occupational Health and Safety

- Potential Impact

Occupational Health and Safety (OHS) related impacts will arise during construction stage activities including clearing of earth, levelling, compaction, foundations, finishing, testing & commissioning. In nutshell, occupational health and safety issues associated with the construction of proposed project will primarily include physical hazards; chemical hazards; and noise.

Noise: Construction and maintenance personnel may be potentially exposed to high levels of noise from heavy equipment operation and from working in proximity to vehicular traffic. As most of these noise sources can be prevented, by using noise barriers, new machinery and by using personal hearing protection by exposed personnel and implementation of work rotation programs to reduce cumulative exposure.

In addition, disasters such as earthquakes and fires may occur.

Lack of Emergency Response Plan (ERP) or an inefficient response plan may lead to an accident or critical injury. Considering these consequences, this impact can be categorized as direct, moderate, site-specific, medium term, temporary, medium probability and irreversible.

Mitigation Measures

Following mitigation measures shall be adopted by the contractor to control accidents due to deep excavations:

Proper barricading shall be applied to all excavation deeper than 6feet.

Warning taps shall be applied to trenches which are around 3 feet deep.

Isolation of area must be done during excavation activities to control accidents.

Damping down of area to control fugitive dust.

Where necessary, apply green sheet to control fugitive dust, especially during high wind season.

Material shall be stored at least 3 feet away from the edges of excavations.

Designated entry and exit points at all deep excavations.

If any confined space encounters (excavation deeper than 10 feet or any main hole etc.), procedure for confined space entry shall be followed.

For further details, framework for Occupational Health and Safety Management plan has been developed to handle any health and safety issue of workers and community. Mitigation Measures to prevent and control physical hazards include:

#### Moving Equipment and Traffic Safety

Establishment of work zones to separate workers on foot from traffic and equipment by:

Routing of traffic to alternative roads when possible;

Where worker exposure to traffic cannot be completely eliminated, use of protective barriers to shield workers from traffic vehicles, or installation of channeling devices (e.g. traffic cones and barrels) to delineate the work zone;

Regulation of traffic flow by warning lights, avoiding the use of flaggers if possible;

Reduction of maximum vehicle speeds in work zones; and

Training of workers in safety issues related to their activities, such as the hazards of working on foot around equipment and vehicles; and safe practices for work at night and in other low-visibility conditions, including use of high-visibility safety apparel and proper illumination for the work space (while controlling glare so as not to blind workers and passing motorists).

Provide appropriate PPE in conjunction with training, use, and maintenance of the PPE.

Furthermore, the noise reduction options that should be considered which include:

Selecting equipment with lower sound power levels;

Installing suitable mufflers on engine exhausts and compressor components;

Installing vibration isolation for mechanical equipment;

Providing noise protection PPEs (ear plugs/ear muffs) to the construction workers;

Re-locating noise sources to fewer sensitive areas to take advantage of distance and shielding;

Developing a mechanism to record and respond to complaints; and

Regular monitoring of noise levels at active sites or near noise producing equipment/machinery and compare it to the available occupational noise standards.

#### Monitoring of OHS Activities:

During the construction phase of proposed project, occupational health and safety monitoring programs of the contractor (s) should verify the effectiveness of prevention and control strategies. The selected indicators should be representative of the most significant occupational, health, and safety hazards at the construction site and camps and the implementation of prevention and control strategies. The occupational health and safety monitoring program should include:

Regular inspection and testing of all safety features and hazard control measures focusing on engineering and personal protective features, work procedures, places of work, installations, equipment, and tools used;

Surveillance of the working environment: The contractors should document compliance using an appropriate combination of portable and stationary sampling and monitoring instruments. Monitoring and analyses should be conducted according to internationally recognized methods and standards. Monitoring methodology, locations, frequencies, and parameters should be established individually for each project following a review of the hazards;

Continuous and efficient surveillance of worker’s health during the entire construction phase by the nominated officials of contractors; and

Training: Training activities for employees (construction contractor & supervision consultant staff) and visitors should be adequately monitored and documented (curriculum, duration, and participants). Emergency exercises, including fire drills, should be documented adequately.  Service providers and contractors should be contractually required to submit to the employer adequate training documentation before start of their assignment.

Contractor(s) shall prepare a comprehensive OHS Plan as part of Site Specific Environmental Management Plan (SSEMP) including work method statements for all hot works to avoid incidents during construction stage. A guideline OHS Plan is provided in Annexure-VII.

Residual Impacts

Upon implementation of the proposed mitigation measures suggested above, and also upon implementation of the Occupation Health and Safety Plan (Attached as Annexures), low residual impacts are anticipated.

#### Construction Camps/Camp Sites

- Potential Impact

Approximately one (01) construction camp is proposed to be established during construction period for the subject project. Due to the proposed camp site, loss of vegetation and dissatisfaction of rehabilitation measures during and after completion of construction phase may occur. This may include waste, soil pollution, groundwater pollution, dust, etc. However, the impact will be medium adverse in nature. For these impacts, mitigation measures have been developed to minimize the likelihood, extent or duration of their occurrence and any associated adverse effects.

Mitigation Measures

Environmental

Implement landscaping plan for all facilities in areas where high landscape value and visual vulnerability to the proposed activities warrants site-specific landscape restoration measures.

Operate equipment in a manner sympathetic to the ambient noise environment. Do not leave equipment idling unnecessary.

Provide adequate warnings of impeding works to all potential receptors within a 1 km corridor surrounding the RoW via public notices and local news.

Social

State land will be a first preference for worker camp location, followed by land where there is a willing lessee.

Employment policies which aim to maximize job opportunities for local people will help to minimize tensions caused by different socio-cultural values.

Training will be provided to all staff on camp management rules and overall discipline and cultural awareness. This will include, in appropriate languages    abriefing on camp rules

A community relations orientation to increase awareness about the local area, cultural sensitivities and the project Code of Conduct

Awareness-raising on health considerations, including sexually transmitted diseases (STDs) and Covid 19.

The construction contractor is required to develop a Construction Camp Management Plan to address:

    - Discipline;

    - Community liaison;

    - Ethnic tensions and;

    - Communicable diseases.

A Code of Conduct and Camp Rules will be required within the Construction Camp Management Plan, which provides policies and a disciplinary framework with respect to worker behavior.

Camp Location

The construction contractor will be required to assess the environmental/social sensitivity of any additional or alternative sites prior to their approval for adoption.

Some additional mitigation measures should include:

The contractor(s) should provide plan to the Proponent for removal & rehabilitation of site upon completion;

Photographical and botanical inventory of vegetation before clearing the site; and

Compensatory plantation to be scheduled when construction works near end.

Residual Impacts

The residual impacts are envisaged to be low after implementation of suggested preventive and mitigatory measures.

#### Community Health and Safety

- Potential Impact

Community health and safety issues during the construction of proposed project may include dust, noise, and vibration from construction vehicle transit, and communicable disease associated with the influx of temporary construction labor. Significant community health and safety issues associated with the projects may also include:

Pedestrian safety;

Traffic safety; and

Emergency preparedness.

Further, the laying of water supply pipelines, in particular, will require the digging of trenches for laying these lines will pose a risk of community members falling into these trenches. The construction areas located near the, settlements, may cause accident for the people moving near to these areas. Considering these consequences, this impact can be categorized as direct, moderate, site-specific, medium term, temporary, medium probability and irreversible.

Mitigation Measures

The World Bank/IFC EHS Guidelines, 2007, defines community health and safety in terms that guarantee as-built infrastructure conforms to acceptable standards (structural safety, flood and fire risk), water supply sources are of suitable potable quality, emergency response planning is in place for built environments, traffic safety provisions are enforced alongside transport hazard assessment and mitigation, and disease prevention measures are taken. The Contractor will prepare the site specific community health and safety plan in compliance with relevant sections of the IFC General Environmental Health and Safety Guidelines (WB/IFC 2007), Pakistan Labor Laws, Punjab Occupational Safety and Health Act (2019);

The Contractor will clearly barricade work areas to prevent access by the public, while ensuring passage by providing safe pathways for pedestrians around construction zones;

The Contractor will exclude parking, waiting vehicles and vendors from areas adjacent to the work by means of clearly marked barricades and posted signage;

The Contractor will remove excavated earth, spoil, rubble, cut vegetation and refuse whether generated by the project or discarded by third parties from areas within the construction zone, where it has potential to interfere with the public or generate dust;

The Contractor will provide temporary lighting to facilitate construction during night time;

The Contractor will remove hazardous conditions on construction sites that cannot be controlled effectively with site access restrictions and will barricade any excavations and materials placed near the public place (if applicable);

Hard Barricades will be provided at the excavation deeper than 1.5 meter as per “NZS 3845:1999 Road safety barrier systems”;

The Contractor will promptly reinstate any services and reinstall any physical facilities that are cut, disconnected or damaged during construction, and maintain or provide temporary services that are interrupted by construction. The Supervisory Consultant will inspect and certify the adequacy of all reinstated services and facilities;

Installation and maintenance of speed control and traffic calming devices at pedestrian crossing areas especially near the settlements.

An Emergency Preparedness and Response Plan (EPRP) in coordination with the local emergency responders to provide timely first aid response in the event of accidents and hazardous materials response in the event of spills;

Instruct foremen to strictly enforce the keeping out of non-working persons, particularly children, off work sites;

Timely public notification on planned construction works;

Close consultation with local communities to identify optimal solutions for diversions to maintain community integrity and social links;

Any environmental condition that is disagreeable to the public and causes an avoidable nuisance can be addressed with additional provisions over and above those described above, as determined necessary by the supervisory consultant.

These requirements will be incorporated into the bidding specification and contract documents, and will be binding on the contractor, at risk of penalty for noncompliance, as charges to be recovered from contractor for unsafe act or condition.

Residual Impacts

Upon implementation of the proposed mitigation measures suggested above, and also upon implementation of the proposed Health and Safety Plan, no residual impacts are anticipated.

#### Communicable Diseases

- Potential Impact

The laborers in the Contractor Camp, truck drivers and like personnel who interact with each other have the potential for the spread of HIV/AIDS if the incidence exists. Majority of the people living in the surrounding of the Project, and potential Labor are not aware of the source, mode of communication or consequences of HIV/AIDS. Although their religious and cultural value system, to a large extent excludes the outbreak or rapid communication of HIV/AIDS, yet its occurrence in such a situation cannot be precluded. It is necessary that awareness and preventive campaigns are run from time to time in the Labor camps and the field offices of the Project to prevent the communicable diseases like Cholera, Typhoid and Tuberculosis.

There is a chance of spreading of an epidemic of Coronavirus disease (COVID-19) due to close interaction of the labor force during construction not only among the workers but also in the area. This impact is high adverse in nature.

Mitigation Measures

The Contractor shall:

Arrange to run an active campaign, in the labour camp, to make people aware of the cause, mode of transmission and consequences of HIV/AIDS;

Latest / Updated SOPs by WHO, national and provincial Government related to the construction industry to control spreading of COVID-19, should be implemented by the contractor and should be strictly monitored;

Strengthen the existing local health & medical services for the benefit of labour as well as the surrounding villages;

Ensure cleanliness and hygienic conditions at the labour camp by ensuring proper drainage and suitable disposal of solid waste. Inoculation against Cholera will be arranged at intervals recommended by the Health department;

Locating a labour camp at least away from the villages (local settlement), and

Keep all the camps, offices, material depots, machinery yards and work sites open for the inspection of health and safety measures and related documents.

Residual Impacts

Successful implementation of suggested mitigation measures in above section in implementation of WHO guidelines for prevention of COVID-19 will reduces the chances of spread of communicable diseases among workers and nearby community. Therefore, the impact will be low.

### Impacts during Site Restorations

- Potential Impacts

After completion of construction activity, the project facilities will be restored as close to its original condition as possible. One of the important tools is the photographic record of project facilities e.g., campsite(s) prior to set-up will be taken and will be compared after site restoration.

Unattended construction waste and excavated material along the RoW of transmission and water supply mains will be source of bad aesthetics within city. Before closure of a typical construction day, the area needs to be cleared from all type of waste and construction material.

Following measures/ actions will be adopted to restore the site:

Demobilization of all equipment and machinery;

Disposal of any waste material remaining at the time of completion of the operation;

Backfilling of all excavation followed by compactions;

Dismantling and removal of fence or barriers surrounding the campsite area;

General restoration of the site area including landscaping and restoration of drainage where required.

PMU through Contractor will ensure that restoration of construction works at water transmission and supply mains will be carried out by contractors.

PMU will ensure periodic monitoring of such restorations.

Contractors will develop site restoration protocols and will submit to PMU for review and approval.

Construction site restoration protocols will be part of bidding documents and constructions contracts.

Construction contractor will add restorations costs into BOQs.

Residual Impacts

The proposed site will be restored near to its original conditions and the residual impacts will be low.

### Anticipated Impacts during Operation Phase

The potential impacts from operation of the water supply system in KSS are provided below.

Table 6.8: Screening of Potential Impacts during Operation Phase for Water Supply Component

| S/No. | Potential Issues | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Short term, Long term) |
| --- | --- | --- | --- | --- | --- |
| 1 | Water system leaks and water discharges during flushing | 2/ Unlikely | 3/ Major | 6/ Medium | Low/ Short term |
| 2 | Improved drinking water availability | Positive impacts expected |  |  | Long term positive residual impact |
| 3 | Improvements in Public Health | Positive impacts expected |  |  | Long term positive residual impact |

Critical Risk Level

Significant Risk Level

Medium Risk Level

Low Risk Level

Positive Impacts

### Water System Leaks and Water Discharges during Flushing

- Potential Impacts

Water system leaks can reduce the pressure of the water system compromising its integrity and ability to protect water quality (by allowing contaminated water to leak into the system) and increasing the demands on the source water supply, the quantity of chemicals, and the amount of power used for pumping and treatment. Leaks in the distribution system can result from improper installation or maintenance, inadequate corrosion protection, settlement, stress from traffic and vibrations, frost loads, overloading, and other factors.

Water supply lines may be periodically flushed to remove accumulated sediments or other impurities that have accumulated in the pipe. Flushing is performed by isolating sections of the distribution system and opening flushing valves or, more commonly, fire hydrants to cause a large volume of flow to pass through the isolated pipeline and suspend the settled sediment. The major environmental aspect of water pipe flushing is the discharge of flushed water, which may be high in suspended solids, residual chlorine, and other contaminants that can harm surface water bodies. Recommended measures to prevent, minimize, and control impacts from flushing of mains include:

Mitigation Measures

Ensure construction meets applicable standards and industry practices;

Conduct regular inspection and maintenance;

Implement a leak detection and repair program (including records of past leaks and unaccounted-for water to identify potential problem areas);

Consider replacing mains with a history of leaks of with a greater potential for leaks because of their location, pressure stresses, and other risk factors.

Discharge the flush water into a municipal sewerage system with adequate capacity;

Discharge the flush water into a separate storm sewer system;

Minimize erosion during flushing, for example by avoiding discharge areas that are susceptible to erosion and spreading the flow to reduce flow velocities.

Residual Impact

The implementation of suggested mitigation measures will reduce impacts on the environment and the residual impacts will be low.

### Improved Drinking Water Availability

- Potential Impacts

The continued supply of treated quality water by proposed project will be an indispensable facility in Khyaban-e-Sir Syed (KSS), Rawalpindi. The proposed water supply system will facilitate the domestic as well as commercial water requirements of people living in KSS, Rawalpindi. With the replacement of outlived/rusted pipeline water shortages and leakage/wastage issues shall also be resolved. Moreover, the proposed project will provide improved and sustainable potable water availability to citizens of for next fifty years. The project will reduce abstraction of ground water from tube wells and water bores omitting chances of ground water depletion and reduce operation cost for RWASA.

Furthermore, project implementation will also create job opportunities during operation, thereby improving the socioeconomic condition of the local people and help in improving their quality of life. Thus, the ‘no project’ option is not a viable option.

Mitigation measures

No mitigation measures required.

### Improvements in Public Health

- Potential Impacts

The clean potable water will reduce water borne disease, improve public health and ultimately reduced pressure on health care system.

- Mitigation measures

- No measures required.

### Solar PV Installation

### Impacts and mitigation measures during pre-Construction phase

The potential adverse environmental impacts associated with the project are avoided or minimized by: (i) careful site selection and route alignment; (ii) integrating key measures that will permanently become part of the infrastructure and will be included in the project detailed engineering design; (iii) implementation of environmental mitigation measures for identified impacts; and (iv) ensuring project environment management readiness. These are provided below. Also, the screening matrix for the pre-construction phase is provided in the Table 6.9 below.

Table 6.9: Screening of possible Impacts during Design/Pre- Construction phase for ‘Solar PV Component’

| S/No. | Potential Impacts | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Relocation of existing utilities | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 2 | Traffic Issues/ Traffic Management Plan | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 3 | Lack of integration of IEE/EMP requirements into Construction bid documents | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 4 | Seismic impacts | 3 / Likely | 2 / Moderate | 6 / Medium | Low |
| 5 | Transmission line interconnection Route selection | 2 / Unlikely | 1 / Minor | 2 / Low | Low |
| 6 | Land requirements | 2 / Unlikely | 1 / Minor | 2 / Low | Low |

Critical Risk Level

Significant Risk Level

Medium Risk Level

Low Risk Level

Positive Impacts

#### Relocation of Utilities

Potential impacts

Utility services such as sewerage lines and/or telephone, electricity or gas pipelines lying in the project area or across the project site may need to be relocated temporarily or permanently to prevent damage to them.

- Mitigation measures

Detailed filed surveys will be conducted to assess any sub-surface utilities that might be present under or across the project site or in the project area and they shall be relocated prior to the commencement of the work.

Based on the utilities that are identified during the detailed surveys, a relocation plan of those utilities will be developed by the respective line agencies in close coordination.

In case any utilities can be allowed to remain in place and avoidance of their damage is possible, the contractor will need to be aware of the location of these services so that disruptions are not caused. In such scenario, the responsibility for any repair of damaged services will lie with the contractor.

- Residual Impact

Provided the proposed mitigative measures are implemented, the magnitude and extent of the environmental effects will be low. Therefore, no significant adverse residual environmental effects are likely to occur.

#### Traffic Issues

- Potential impacts

In particular, the movement of heavy equipment and machinery for the solar PV installation works could result in heavy congestion due to the movement of construction heavy machinery and equipment with significant delays for traffic apart from posing a significant risk for vehicles.

- Mitigation Measures

A comprehensive traffic management plan shall be developed by the contractor and approved by the Executing Agency or the PMU that will contain the traffic diversion onto alternate routes and management of traffic flows to minimize the congestion and the possibility of accidents. This plan shall be dependent to the EMP and will be implemented by the contractor.

- Residual Impacts

After implementing the traffic management plan, the residual impacts will be low.

#### Lack of Integration of IEE/EMP Requirements into Construction Bid Documents

Potential Impacts

The bidding documents must reflect the requirement to select a qualified and experienced Contractor from the perspective of ensuring implementation of required environmental safeguards provisions during project development.

- Mitigation Measures

Appointment of at least one environment focal person and one social focal person within the PMU to be in charge of EMP coordination, implementation and site inspections including project GRM;

Appointment of at least one environment, health and safety officer within the Contractor staff to be in charge of EMP coordination, implementation, site inspections and information disclosure and consultations;

Appointment of at least one GRM focal person to be in charge of project GRM coordination, handling complaints, dispute resolution, site visits and information disclosure and consultations.

The environmental staff of PMU will be assigned the task to check that design and bid documents are responsive to key environmental, social and safety considerations, and that the proposed method of work reflects the boundaries defined in the EMP. The bid documents must include the EMP and its implementation cost must be reflected in the BOQ.

IEE/EMP implementation and monitoring requirements must be part of bidding documents and necessary contractual binding must be agreed by project contractors before award of contract.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Seismic Impacts

- Potential Impacts

In Building Code of Pakistan (Seismic Provisions, 2007), the whole region of Pakistan is classified into five main Seismic Hazard zones i.e., i) Zone-1, ii) Zone-2A, iii) Zone-2B, iv) Zone-3, v) Zone-4 based on the peak horizontal ground acceleration and its ultimate tendency to damages during seismic events. The proposed project area falls in the seismic zone classified as “Zone-2B” with g-value 0.16 to 0.24 and is considered as ‘moderate’ hazard seismicity zone.

No change (+/-) in the impact of seismology risk of the area is expected during the project construction phase as none of the project activities is expected to be of such a powerful extent to influence the tectonic risk. This adverse impact is medium in nature.

- Mitigation Measures

The proposed structures shall be designed and constructed keeping in consideration high intensity earthquakes. For seismic hazard analysis, updated structural and seismic evaluations should be conducted by the design engineer/consultant. Moreover, geo-technical investigations must be conducted prior to construction phase.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Transmission Line Interconnection Route Selection

Potential impacts

The power generated from the solar PV installation will need to be evacuated through interconnection to the nearest substation. The route of the transmission line for the interconnection must be selected to ensure the minimal distance and scope of work is conducted while avoiding and/or minimizing any potential impacts on identified receptors, particularly any areas of importance such as protected areas, habitats and species of conservation value, physical cultural resources (PCRs) etc.

The EPCM consultants have already conducted a feasibility study and identified the most feasible interconnection route which will avoid and/or minimize any potential impacts on identified receptors.

Mitigation measures

No measures required.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Land requirements

Potential impacts

The proposed solar installation will be developed on publicly owned land. Thus, no land acquisition and/or resettlement will be applicable for this project development and all civil works will be conducted within the acquired land area.

Mitigation measures

No measures required.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Landscape Impact

Potential impacts

The solar plant will create a visual impact and may result in induced access to the area. The site is not surrounded by habitation or permanent structures. Steps to reduce visual impacts e.g. maintenance of hedges and field margins around the fenced perimeter will be incorporated after completion of civil works and during the start of operation. This project will showcase green development of the power sector in Pakistan as well as preserve the local biodiversity. Solar panel glare issues are not anticipated for solar PV plants.

- Mitigation measures

- No measures required.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

### Impacts and mitigation measures during Construction Phase

The screening matrix for the construction phase is provided in the Table 6.10 below.

Table 6.10: Screening of Possible Impacts during Construction Phase for ‘Solar PV Component’

| Sr. No. | Potential Impacts | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Traffic Issues | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 2 | Air Quality | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 3 | Noise and Vibration | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 4 | Occupational Health & Safety | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 5 | Community Health and Safety | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 6 | Waste generation | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 7 | Vegetation impacts | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 8 | Fauna impacts | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 9 | Biodiversity | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 10 | Interruption to existing facilities | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 11 | Physical landscape | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 12 | Topography impacts | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 13 | Groundwater quality | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 14 | Soil quality | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 15 | Local Economic development | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 16 | Physical Cultural Resources | 2 / Unlikely | 1 / Minor | 2 / Low | Low |
| 17 | Other Risks | 3/ Likely | 2/ Moderate | 6/ Medium | Low |

Critical Risk Level

Significant Risk Level

Medium Risk Level

Low Risk Level

Positive Impacts

#### Traffic Issues

Potential Impact

The construction phase of the proposed solar installation, particularly the transportation to site of the solar PV panels and associated electronical equipment and infrastructure could result in congestion due to the movement of construction heavy machinery and equipment with significant delays for the traffic apart from posing a significant accident risk for vehicles.

The efficient management of traffic once the construction activity commences will be critical in order to minimize the risk of possible road accidents and construction related hazards.

Mitigation Measures

In order to minimize traffic problems in the proposed project area, following measures will be considered:

- Movement of vehicles carrying construction materials and equipment/machinery will be restricted during the daytime to reduce traffic load and inconvenience to the local population;

- Construction vehicles, machinery and equipment will be parked at designated areas (at construction camps site) to avoid un-necessary congestions on roads;

- The speed of the vehicles will be controlled (at 20 to 25 km/hr) to reduce the probability of severe accidents, soil erosion, debris flows due to vibrations and dust emission;

- Damages of roads due to construction vehicles will be instantly repaired and/or compensated after the completion of work;

- Proper sign boards will be provided for smooth flow of traffic;

- Period of construction and area / location of construction site shall be informed to public in general and specifically to local residents; and

- Any closure of the roads (especially main roads) and deviations / diversions proposed should be informed to the riders through standard signs and displays.

- Traffic Management Plan will be implemented to avoid traffic accidents, jams/public inconvenience. Guideline for Traffic Management Plan has been prepared and given as Annexure-V.

- Residual Impacts

Upon implementation of the proposed mitigation measures, no significant adverse residual effects are anticipated. Further, traffic management plan has been developed and after its implementation, the residual impact will be low.

#### Air Quality

Potential impacts

Potential air quality impacts during construction include dust emissions associated with earthworks and gaseous emissions from construction activities and heavy vehicles used to support the construction works. Some concerning activities are wind-borne dust emissions from cleared areas, stockpiles, and unpaved site roads; truck-derived dust emissions from traffic on haulage routes and dumping; and dozer- and excavator-derived dust emissions from clearing operations and earthworks.

Moderate temporary air quality impacts during peak construction phase of the project are anticipated because of construction activities scheduled in the dry season and due to fugitive dust generation associated with construction works and transport (of materials, equipment and machinery), loading, unloading and haulage of materials and corresponding increase in vehicular emissions. Use of construction machinery and equipment and movement of transport vehicles may also affect levels of nitrogen oxides (NOx) and sulfur oxides (SOx), affecting air quality. Building access roads to and within the project site (asphalt road pavement) will produce fumes containing small quantities of toxic and hazardous chemicals such as volatile organic compounds (VOC) and poly-aromatic hydrocarbons (PAH).

Mitigation measures

The measures to be implemented will be as follows:

Particulate dust generation will be minimized by restricting land clearing to the areas flagged for immediate use, progressively rehabilitating exposed soils as soon as practicable, avoiding clearing and earthworks activities during dry, windy conditions and applying water to all cleared, road and stockpile areas whenever weather conditions have the potential to mobilize fugitive dust.

Open burning (e.g. of vegetation, waste, hydrocarbons) will be avoided.

A detailed management and monitoring plan for air quality is provided in the EMP for the Project, which will monitor emissions generated during construction.

General air quality management and mitigation measures are as follows:

  - Conduct awareness trainings on dust emissions for project employees and workforce;

  - Restrict speed limit on unsealed roads to minimize dust generation;

  - Conduct topsoil stripping when soil is moist to the extent practical;

  - Monitor for air quality impacts near any sensitive receptors identified near construction areas and along the access roads;

  - Use low-sulphur fuels where practical;

  - Source low-emission diesel equipment (e.g. Tier 2 or better), where practical.

The Contractor(s) will conduct air quality monitoring at the solar site, once before start of the civil works to establish the baseline, quarterly during the civil works and once after completion of the civil works to monitor and mitigate exceedances (if any) with respect to the applicable ambient air quality standards.

Air monitoring will be conducted utilizing handheld air monitoring devices at select locations identified on site map.

Open and exposed land areas at solar plant will be sprayed with water to suppress dust levels, particularly during the dry season.

Use of chemical dust suppressants will be prohibited.

Aside from this, works sites will be temporarily barricaded to control dust levels.

Storage areas/warehouse for materials required for construction and civil works will be provided onsite (construction camps at the solar plant) to reduce construction vehicle trips of transporting materials and minimize stockpiling.

Vehicles transporting materials that generate dust will be covered with tarps. Construction vehicles and machinery will be maintained to a high standard to minimize vehicular emissions and noise.

Log of monitoring/incidences of non-compliance and rectification will be recorded and maintained.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Noise and vibration

Potential impacts

Noise impacts will be temporary and localized at all construction sites as construction machinery and vehicles generate noise as they operate. Other noise sources include loading, unloading and haulage of equipment and materials. Significant noise impacts will be experienced by construction site workers/operators; construction machinery may produce noise levels up to 90 A-weighted decibels (dBA).

For the solar plant, only construction workers will be this close to the machinery for extended periods of time. Measurement of noise levels at the plant site will be conducted once prior to start of the civil works to establish the baseline, quarterly during the civil works and once after completion of the civil works (the latter will be conducted at the solar plant only/upon commissioning of the substation) to monitor and mitigate exceedances (if any), with respect to the ambient noise standards. The potential impacts due to noise and vibration will be mitigated by the following measures as set out in the EMP.

Potential vibration and noise impacts include those associated with vehicle movements, heavy machinery and construction activities. Since there are no nearby residents around the project, potential noise and vibration impacts are primarily issues for workers onsite. Sources of noise and vibration associated with project construction include:

Light vehicle movements associated with personnel arrival/departure and onsite activities;

Earthworks in the project area using heavy machinery (clearing, grubbing and cutting);

Earthworks associated with upgrade of the project internal roads;

Concrete foundation works;

Building and facility construction;

Heavy vehicle movements associated with delivery of equipment, building materials and infrastructure.

Mitigation measures

The measures to be implemented will be as follows:

Noise and vibration associated with vehicles will be minimized by imposing a speed limit on site roads, by scheduling the majority of vehicle movements to daylight hours, minimizing air/exhaust braking where this would cause disturbance, and establishing soil berms or vegetation along roads to reduce the impact of traffic noise.

Equipment with lower noise levels will be selected where practical and the operation of noisy equipment will be limited to daylight hours where practical.

Schedule project supply deliveries to daylight hours.

Maintain road surface to reduce rumble.

Conduct periodic auditing of noise levels to ensure minimal potential impacts.

Noise monitoring will be conducted utilizing handheld noise monitors at select locations identified on site map.

Log of monitoring/incidences of non-compliance and rectification will be recorded and maintained.

Drivers will be required to observe low speed wherever necessary and no blowing of horns.

Construction activities utilizing heavy machinery work will be restricted between 8 AM – 6 PM.

Advance warning to communities will be provided with respect to the timing of noisy activities.

Construction vehicles and machinery will be maintained to a high standard to minimize emissions and noise.

All construction workers / operators will use appropriate PPE, including ear defenders, when operating machinery.

Implement CEMP sub-plans:

- The Noise and Dust Control Plan

- Traffic and Road Management Plan

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Occupational Health and Safety

Potential impacts

During the construction phase, more workers will be employed to support project construction, to be housed on site in the construction camps. The key factors that will pose risks to worker health and safety are movement of heavy machinery and high volumes of traffic on site as well as any potential injuries resulting from electrical and civil works while developing the project infrastructure.

Mitigation measures

The measures to be implemented will be as follows:

The project will demarcate exclusion and safety zones around any high-risk areas.

The project will strictly require all workers wear personal protective equipment appropriate to their duties with signage at designated sites.

Clinical assistance will be available to project personnel and medical emergency evacuation procedures will be developed.

Fatigue will be monitored and managed during travel and during on site operations.

Standard Operating Procedures (SOPs) for the safe handling, storage and transportation of associated facilities and materials.

The project will strictly implement the management and monitoring measures as per EMP for water quality, waste management, air, health and safety, noise and vibration and transport measures.

Implement CEMP sub-plan on Occupational Health and Safety (OHS) Plan.

Provide sanitary facilities and wash areas, safe drinking water and garbage bins.

Provide health assessments (health and fitness to workers once every two months).

Periodic training will be provided to workers in all aspects of the ERP and OHS

Coordinate with nearest hospital for arrangements in case of accidents.

Provide first aid treatment for construction sites and camp.

Assess safety risks and safety protocols (such as for electrical works, working at heights, etc.), and implement.

Workers will be equipped with PPE, including proper safety clothes and protection gear/equipment to avoid accidents.

Provide communication devices to designated site officers / engineers.

Electrical safety risks will be assessed and safety protocols will be developed and implemented such as for electrical works, working at heights, etc.

All works at height will be prohibited during night time, periods of fog and strong wind.

All workers climbing towers will have a Safety Certificate.

All towers, steel structures and equipment will be properly earthed and equipped with lightening protection.

When testing electrical equipment, all unrelated works in the flagged zone – marked as danger zone- will be stopped and unrelated workers will leave the zone.

Records of health assessments/incidents/accidents/near-miss/fatalities will be maintained.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Community Health and Safety

Potential impacts

Site preparation and civil works, movement of vehicles bringing equipment, materials, supplies and machinery may interfere with road crossings, flow of traffic and may cause an increase in traffic congestion. These activities may pose safety risks to the communities residing in close proximity to the work sites and general public. In addition, construction camps and an influx of workers may cause social conflict or even lead to the spread of any communicable diseases. Contractors will obtain permissions from relevant authorities before commencement of activities. A code of conduct will be developed for all relevant stakeholder and workers to keep the work organized and safe.

The Project may also generate (fugitive) dust mainly from upgrading any access roads, clearing any vegetation/forestland for infrastructure and transportation of materials and equipment. Noise from these activities may also disturb nearby residents at night as well as impact on daily economic/business activities. Also, during the construction phase, the project may lead to a number of key risks for community health, food security and safety, including:

Safety impacts will be primarily associated with accidents involving company vehicles along the project access road. During construction, in particular, there will be a significant increase in project traffic.

Potential psycho-social and health impacts among project personnel associated with air quality, noise, and vibration effects.

Mitigation measures

Security fencing and patrols to prevent safety impacts from any construction activities undertaken during construction.

Potential noise impacts will be reduced to acceptable levels through scheduled daytime construction, while dust generation will be managed through regular water spraying on major dust sources associated with the project.

The project will strictly implement the management and monitoring measures as per EMP for water quality, waste management, air, health and safety, noise and vibration, and transportation; and sub-plans, including Emergency Preparedness and Response Plan, Community Health and Safety Plan, Stakeholder Engagement Plan, Traffic and Transport Management Plan and Waste Management Plan.

Provide healthcare services for project employees with an appropriately staffed site clinic

Effectively control the risks associated with accidents involving multiple vehicles, interactions between different types of vehicles with consideration for the volume and speed of traffic, and vehicle and person interactions.

Obeying speed limits and apply additional caution by considering environmental conditions such as time of day, visibility, precipitation.

Implement an effective site waste management strategy to prevent the incidence of vector-borne diseases.

Develop a management strategy for worker accommodation in line with international/national standards to minimize spread of infectious disease and food-related illnesses.

Adhere to strict schedule for completion of civil works and avoid prolonged construction and disturbance.

Implement CEMP sub-plans.

Community Health and Safety Plan.

Traffic and Road Management Plan.

Provide perimeter fencing to mitigate trespassing (fencing will be constructed with adequate ground clearance for passing of wild animals/other species at the solar plant site).

Provide sufficient lights, clear warning signs and danger signals.

Assign security personnel to prevent accidents, trespassing and pilferage.

Warning signs and cones will be installed in and around the transmission tower site and along roads, with clearly marked danger zones.

Safety flags and flag persons will be used, as needed.

Record of incidents/accidents/near-miss/fatalities associated with the project will be maintained.

Records of issues raised will be maintained in accordance with GRMs.

Outreach to local communities to disseminate knowledge about safety and ERPs at or near solar plant will be provided via information disclosure and consultation activities, project information booklet (PIB) / frequently asked questions (FAQ) flyers.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Waste Generation

Potential impacts

Impacts due to waste generation during construction stage have been assessed for all infrastructure types under the solar plant and discussed as follows:

Impacts on resource use and impacts associated with disposal will arise from waste generated during site preparation and civil works. This includes generation of inert wastes e.g. spoil, biodegradable wastes e.g. cleared vegetation, construction debris, packaging waste, metal scrap, domestic waste.

Impacts of disposal and wind-blown litter will be seen at the construction areas, whereas the impacts of resource use are national and global.

Mitigation measures

Establish a covered onsite sorting and recycling area away from any drainage sources.

Transport of recyclables, scrap, discarded equipment to depots, dedicated storage yards for resale or auction to authorized dealers. The broken or damaged solar panels will be sold to a solar panel specializing panel recycling.

For other type of wastes, licensed companies will be hired to collect, transport and dispose of wastes at licensed dump facilities.

Biodegradable waste such as cleared vegetation may be provided to local communities for use.

Provide multiple waste containers at construction camps.

Waste burning will be prohibited.

No final waste disposal on site.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Vegetation impacts

#### Potential impacts

The most significant impact to terrestrial habitats and flora associated with the project is expected to occur during the construction phase when habitats within the project footprint will be cleared. Since the project site is located far away from any areas of note, thus habitat clearance will not result in the loss of natural or modified habitats and no critical habitats have been found.

Mitigation measures

Construction camps to be set up at least 400 m from any water source or drainage channels;

Access to adjacent properties will be maintained, as necessary;

Vegetation clearances will be strictly restricted to work sites;

Use of herbicides/pesticides will be prohibited for vegetation clearing to prevent soil contamination;

Excess spoil will be backfilled onsite or spread out in a manner that causes no disturbance to local drainage pattern;

Implement the following CEMP sub-plans:

- Strictly implement the Spoils Disposal plan

- Drainage and Storm-water Management Plan

- Site Rehabilitation and Clean-up Plan after completion of civil works to preconstruction conditions, including vegetation planting to stabilize soil where it does not compromise the intended function.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Fauna Impacts

Potential impacts

No major loss to any fauna in the project area is expected. Any potential impact will largely be localized and restricted to the limited vegetation to be removed from the project site. As preparatory works begin, the on-site species will move into other parts of their range. Other priority birds, reptiles and small mammals have a wide habitat tolerance and are likely to move into similar habitat elsewhere.

Mitigation measures

The project will design the solar plant layout to avoid most sensitive habitats, including maintaining the buffer zone for instance to keep habitats as far as possible.

The project will avoid direct and indirect impacts to any areas of biodiversity sensitivity.

Habitat clearance will be undertaken in a progressive and sensitive manner to enable priority fauna to move away from the area of works and to avoid isolating fauna in fragmented areas of habitat.

Adverse impact to habitats, flora and fauna arising from fugitive dust will be minimized through the regular application of water to reduce dust levels in areas of biodiversity sensitivity (particularly during the dry season), the stabilization of landforms and the implementation of erosion prevention measures.

Raise awareness with workforce and local villagers about importance of protecting forest resources and conservation important species and their habitats.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Biodiversity

Potential impacts

Vegetated areas occurring within the project footprint will also be cleared. Potential impacts will include:

Release of suspended sediment from construction activities

Accidental spills of diesel fuel

Mitigation measures

The potential impacts on ecological resources will be mitigated by the following measures as set out in the EMP:

Minimize vegetation clearing confined to the footprint of the solar plant area.

No use of chemicals (pesticides/herbicides).

Maintain hedges and field margins along solar plant perimeter.

After completion of civil works, implement Site Rehabilitation and Clean Up Plan, including landscaping along the fenced perimeter of the solar plant, re-seeding most or all of the site with native species of plants.

Provision for adequate ground clearance under fencing for allowing passage of animals/wild species to prevent habitat fragmentation.

Restoring temporarily disturbed areas to pre-construction conditions.

Trees below 3 m will not be cut or felled, minimum lopping and pruning will be undertaken as necessary.

Any identified bat roosting sites will not be disturbed.

Restoring temporary land use (used for material storage) to pre-construction conditions.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Interruption to Existing Utilities

Potential impacts

In order to reduce impacts on existing utilities (such as power outages), the solar plant construction works will be scheduled in a phased manner. All activities will be supervised by PMU Environment and Social Officer.

Use of Existing Access Roads. Delivery of equipment, materials and machinery to work sites at the solar plant may lead to increase in traffic and road congestion. It may cause interference with road crossings during stringing of conductors. Movement of transport vehicles will also result in moderate air quality impacts (dust and increase in vehicular emissions) as well as an increase in noise (due to operation of transport vehicles) during peak construction period (these are discussed under Impact on Air Quality, Noise and Vibration).

- Mitigation measures

The potential impacts due to increase in traffic and road congestion will be mitigated through the following measures as set out in the EMP:

Implement Traffic and Road Management Plan.

Follow planned transportation routes and delivery schedule.

Any traffic detours to have danger and clearly visible warning signs as well as flag persons.

Compliance with local speed limits vehicle load carrying capacity and other road regulations.

Any damage to roads to be borne by Contractor(s).

Record of incidents/accidents/near-miss/fatalities/road damage will be maintained.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Physical Landscape

Potential impacts

The solar installation may involve some vegetation clearance and soil compaction. Due to the topographical location of the project, the changes to physical landscape in the project development area will not be visible or have an impact beyond the project’s immediate vicinity.

The solar project has been designed to minimize adverse physical impacts to the surrounding landscape by minimizing the project footprint and blending major project components with the surroundings to the extent practicable.

Mitigation measures

No measures required.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Topography

Potential impacts

Land leveling and grading works may alter drainage patterns at the solar plant site that may result in surface erosion, generation of spoil and localized flooding. Impacts will also result from installation of equipment, installation of cable routes (above ground and/or underground) and commissioning of new equipment. These impacts will be temporary and localized at the construction sites within the solar project boundaries.

Mitigation measures

Clear demarcation of work sites, no encroachment outside the demarcated zone;

Access to adjacent properties and agricultural land will be maintained, as necessary;

Construction camp/ workers accommodation set up at least 400 m from natural streams or any other water source at site.

Use of herbicides/pesticides will be prohibited for vegetation clearing to prevent soil contamination;

Implement engineering and biological measures to prevent surface erosion such as provision of silt traps or sowing soil-binding grass, as needed;

Restore loose soil from foundations through ramming, if required;

Implement the following CEMP sub-plans;

Spoils Disposal plan;

Drainage and Storm-water Management Plan

Also, site rehabilitation and clean-up plan will be implemented after completion of civil works to restore works site to pre-construction conditions, including landscaping along the fenced perimeter, maintain hedges and field margins to reduce visual and dust impact, re-seeding most or all of the site with native plant species (soil binding grass) to stabilize the soil and restore habitat as far as possible.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Groundwater Quality

Potential impacts

There are some potential effluences for groundwater contamination, i.e. contamination of soil and groundwater with oil, grease and hydrocarbon pollution associated with vehicles and heavy machinery and with pathogen and nutrient pollution associated with human activity, accommodation, sewage treatment and putrescible waste.

Mitigation measures

All chemicals and hydrocarbons will be handled and stored appropriately according to the EMP and associated standard operation procedures, with adequate bunding, hardstand preparation including lining, concreting or compaction, as well as inspection, auditing and contingency measures.

Septic tank will be installed to keep wastewater from toilet and kitchen to prevent pollution to groundwater.

Groundwater quality is to be monitored using groundwater monitoring bores installed up and down gradient of potential contamination zones. Important quality parameters to be monitored as per applicable guidelines.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Soil Quality

Potential impacts

Vegetation will be cleared in preparation for the construction of project components. Measures are therefore required to minimize soil erosion (by rain and wind) and prevent sediment release. Soil compaction within active construction areas is likely due to heavy traffic passage and construction activities. Soil contamination from accidental leaks/spills of hydrocarbons, oil, grease, chemicals and sewage/wastewater effluent is also possible and will require management.

Mitigation measures

The project has been designed to minimize long-term negative impacts to soil in the project site by recovering topsoil stripped from clearing and construction activities and subsequently for progressive and final rehabilitation.

Land clearing is to be limited to the immediate project site required for development to the extent practical.

Activities involving chemicals or hydrocarbons (including storage of broken and redundant solar panels) are to be restricted to designated areas that have been prepared with appropriate bunding, a concrete or compacted earth base, and fully contained by drainage controls.

An appropriate sanitation system is to be provided to collect all waste and sewage for proper storage and disposal;

Strict controls on the use of sanitation facilities will be implemented through education and enforcement.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Local Economic Development

Potential impacts

Despite the generally positive impacts the Project will have on economic development and employment, there is a risk of inequitable employment, training and procurement opportunities or inadequate communication leading to community dissatisfaction.

Mitigation measures

The project will implement the preferential local recruitment and procurement policies in place. Implementation of social management and monitoring as per the EMP and sub-plans: Stakeholder Engagement Plan and Grievance Mechanism.

Provide a grievance mechanism for the Project.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Physical Cultural Resources (PCRs)

Potential impacts

There are no known PCRs in the project area of the solar plant. However, site preparation and civil works during the construction stage have the potential to disturb as yet unknown PCRs.

Mitigation measures

A ‘Chance Find’ Procedure will be implemented as necessary. All activities will be supervised by the PMU Environment and Social Officer.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Other Risks

Potential impacts

Forest/vegetation catching fire in the dry season are risks that need to be accounted for in the planning, design and management of the Project.

Mitigation measures

The project will construct a fire path break around the project boundary to prevent forest/vegetation fires from reaching the solar plant and other facilities.

The project will maintain equipment ready for fire-fighting (e.g. water trucks with water cannons) and train personnel to respond to forest fires in the project area.

Implement and provide inductions to all staff on the Emergency Preparedness and Response Procedures and measures described in the EMP.

Ongoing engagement with government authorities and consultation with local communities will form part of a proactive approach to prevent or reduce risk of natural hazards to project activities and communities.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

### Operation Phase

The screening matrix for the construction phase is provided in the Table 6.11 below.

Table 6.11: Screening of Possible Impacts during Operation Phase for ‘Solar PV Component’

| S/No. | Potential Issues | Likelihood (Certain, Likely, Unlikely, Rare) | Consequence (Catastrophic, Major, Moderate, Minor) | Risk Level (Significant, Medium, Low) | Residual Impact (Low, medium, High) |
| --- | --- | --- | --- | --- | --- |
| 1 | Workforce Organization and Orientation | 2/ Unlikely | 3/ Major | 6/ Medium | Low |
| 2 | Water usage for panel cleaning leading to water shortage | 3/ Likely | 2/ Moderate | 6/ Medium | Low |
| 3 | Landscape impact | 2/ Unlikely | 3/ Moderate | 6/ Medium | Low |
| 4 | Air quality, noise and vibration | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 5 | Waste generation | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 6 | Ecological impacts | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 7 | Occupational health and safety | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 8 | Glare from solar panels may blind pilots or air traffic control towers | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 9 | Interference with navigation and communication systems of aircrafts (electromagnetic interference) | 2/ Unlikely | 1/ Minor | 2/ Low | Low |
| 10 | Green energy-based climate friendly form of power generation | Positive impacts expected |  |  | Positive residual impact |

Critical Risk Level

Significant Risk Level

Medium Risk Level

Low Risk Level

Positive Impacts

#### Workforce Organization and Orientation

#### Potential impacts

The plant workforce will possess limited capacities in regard to national and ADB specific safeguard requirements.

Mitigation measures

The Plant operator will conduct briefing and orientation for all project staff, workers and its turnkey contractor/sub-contractors (facility operators):

EMP, GRM, Information disclosure and meaningful consultation, environmental monitoring and reporting requirements.

Responsibilities of the contractors in implementing SOPs and monitoring environmental compliance with environmental performance indicators and EMP.

Responsibility of the Contractors in engaging with affected persons for project GRM.

Record and maintain briefing and orientation events log with duration and list of attendees.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Water usage for panel cleaning

Potential impacts

There could be a potential shortage of water in the project area and/or lowering of water table due to cleaning of solar panels.

Mitigation measures

The following measures will be implemented:

Maintain storm water retention pond for use for PV cleaning.

Periodic cleaning of storm water retention pond to remove debris.

No use of chemicals/detergents for cleaning purposes.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

#### Air Quality, Noise and Vibration

Potential impacts

Operation of the solar plant will not impact the air quality except for some fugitive dust generation due to movement of project vehicles. Noise-generating equipment (such as inverters/transformers) at the substation will be enclosed and periodic maintenance of equipment will be conducted to minimize disturbance.

- Mitigation measures

- No measures required.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

### Waste Generation

- Potential impacts

While it is not expected that solar PV panels of the solar plant will require replacement since their life span is 25+ years; however, some may fail due to rain, humidity or heat. Inverters at the substation will likely be rebuilt as opposed to replaced. Small amounts of domestic solid waste will also be generated at the solar plant site during operation. Lack of proper handling, storage and disposal of domestic waste and/or broken PV panels may result in adverse impacts.

- Mitigation measures

The potential impacts due to waste generation will be mitigated by following the measures as set out in the EMP:

- Utilize licensed vendors/suppliers for collection, transportation and disposal of broken/unused PV panels.

- Store broken/redundant panels in covered bunded areas.

- Implement SOP for Materials Management and Waste Management.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

### Ecological Impacts

- Potential impacts

The large area of PV panels in the solar plant may create a ‘lake effect’ which could attract birds that mistake the solar panels for a water body and the hot panels could kill or seriously injure birds, which attempt to land on them especially the young inexperienced ones.

- Mitigation measures

Visual deterrents and flight diverters will be provided at the solar plant, particularly close to the storm water retention pond.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

### Occupational Health and Safety

- Potential impacts

Contractors will comply with the relevant safety measures required by law and as per international best practices.

- Mitigation measures

The potential impacts to workers will be mitigated through the following measures as set out in the EMP:

- Implement SOP for Occupational Health and Safety.

- Compliance to relevant national electrical safety standards.

- Provide and maintain signage as per Institute of Electrical and Electronics Engineers standards at dangerous places for warning of electrical hazards.

- Provide and maintain health assessment by a competent medical practitioner for all workers.

- Provide periodic training to all workers with access to electrical and hazardous conditions and workers will be certified to work on site.

- Provide and maintain workers appropriate PPE.

- Equipment and tools will be inspected before use to ensure proper and safe operation.

- Appropriate grounding and deactivation of live power equipment during maintenance work or if working in close proximity to the equipment; provision of lightning arrestors as appropriate.

- Record of health assessments, incidents, accidents, near-miss, fatalities will be maintained.

- EMF levels expected to be below the limits set by International Commission on Non-Ionizing Radiation Protection which is 4.17 kV/m for electric field and 833 miliGauss for magnetic field;

- Periodic EMF monitoring using handheld devices to be conducted, as required.

- Residual Impacts

Upon implementation of mitigation measures suggested above, low or no significant residual impact is anticipated.

### Glare from solar panels might blind pilots or air traffic control towers

- Potential impacts

The proposed solar PV installation site is located 23.4 km from the nearest airfield i.e PAF Noor Khan Airbase, which is used by the Airforce as well as for movement of VIPs arriving and departing from the country. In view of this considerable distance and since the proposed site is not located in the flight path of any incoming or outgoing air traffic, thus no impacts are expected from potential glare issues from the proposed solar installation.

- Mitigation measures

No mitigation measures required.

- Residual impacts

No residual impacts are expected to take place.

### Interference with navigation and communication systems of aircrafts (electromagnetic interference)

- Potential impacts

The proposed solar PV installation site is located 23.4 km from the nearest airfield i.e PAF Noor Khan Airbase, which is used by the Airforce as well as for movement of VIPs arriving and departing from the country. In view of this considerable distance and since the proposed site is not located in the flight path of any incoming or outgoing air traffic, no impacts from interference with the navigation and communication systems of aircrafts is expected.

- Mitigation measures

No mitigation measures required.

- Residual impacts

No residual impacts are expected to take place.

### Green energy-based climate friendly form of power generation

- Potential impacts

The proposed solar PV installation will result in green and renewable based energy generation which will be sustainable in the long term.

- Mitigation measures

No mitigation measures required.

- Residual impacts

Positive residual impacts are expected to take place.

### Decommissioning

The solar plant facility lifespan is expected to be >20 years, at which point it might be decommissioned. Typical activities during the decommissioning and site reclamation phase include facility removal, breaking up of concrete pads and foundations, removal of access roads that are not maintained for other uses, re-contouring the surface (if required), and land re-vegetation and replantation of trees. Associated impacts include erosion, noise, dust and vehicle exhaust and the need to properly manage large amounts of debris, solar panels, wire and cabling, electronics, etc.

Solar plant and its power production may have a much longer lifespan that the solar PV plants (order of 45 to 60 years), which may be further extended through maintenance and periodic equipment replacement.

The EMP provides a preliminary assessment of potential mitigation measures/safeguards that may be implemented at a minimum of six months prior to plant closure at that time. This will be followed by a detailed decommissioning and site reclamation plan to be developed at that time. The preliminary assessment will include measures for avoiding and/or minimizing surface erosion, impact to air and water quality, noise and vibration, flow alteration, waste generation, impact to flora and fauna, traffic related issues as well as health and safety of workers and communities.

### Cumulative Impacts

Based on the scoping exercise of the site and based on discussions with the public sector agencies responsible for development in the project area. No other infrastructure works are planned to be carried in the project area while these project works shall be conducted. Thus, no cumulative impacts are expected.

### Indirect and Induced Impacts

The potential impact of development of the works under this sub-project in the project area has been examined, which indicated that the existing and planned infrastructure would be adequate to accommodate any potential population intake as a result of the proposed project development. Impacts on the environment from air emissions, traffic and noise have also been assessed and have found to be acceptable and within the carrying capacities of the environmental media.

Thus, negative indirect and induced impacts from the proposed works are not expected.

## ENVIRONMENTAL MANAGEMENT, MONITORING AND REPORTING PLAN

### Introduction

The IEE has identified potential impacts that are likely to arise during proposed project of water supply system for KSS in detail, both negative and positive impacts at each stage of the project. To minimize the adverse impacts, this IEE study has recommended mitigation measures in the Environmental Management Plan (EMP). The proposed mitigation measures have been based on the understanding of the sensitivity and behavior of environmental receptors in the project area, legislative controls that apply to the project and a review of good industry practices for projects of similar nature. For residual impacts (impacts remaining after applying the recommended mitigation measures) and for impacts in which there can be a level of uncertainty in prediction at the IEE stage, monitoring measures have been recommended to ascertain these impacts during the course of the project activities.

The EMP is developed to eliminate and/or mitigate the impacts envisaged at the design, construction and operation stages.

The detailed EMP provided in this document as Table 7.1 ensures that proposed project has no detrimental effect on the surrounding environment. The Plan shall act as a guideline for incorporating environmental measures to be carried out by the contractors engaged for the proposed project. It shall also be used for other parties concerned for mitigating possible impacts associated with each project and will form part of the contract documents to be considered alongside the specifications. This Plan shall act as the Environmental Management and Monitoring Plan during the construction and operation phase of the project and will allow for prompt implementation of effective corrective measures.

### Environmental Management Plan

The EMP given hereunder in this IEE report ensures the following:

Delivery of the prescribed environmental outcomes during all phases of proposed project;

Formulating a system for compliance with applicable legislative requirements and obligations and commitments;

Ensure that project design process incorporates best practice environmental design and sustainability principles to minimize potential impacts of construction and operation on the environment and community;

Ensure that the construction and operation work procedures minimize potential impacts on the environment and community;

Develop, implement and monitor measures that minimize pollution and optimize resource use.

### Objectives of EMP

The EMP provides a delivery mechanism to address potential impacts of the project activities, to enhance project benefits and to outline standardized good practice to be adopted for all project works. The EMP has been prepared with the objectives of:

Defining the roles and responsibilities of the project proponent for the implementation of EMP and identifying areas where these roles and responsibilities can be shared with other parties involved in the execution and monitoring of the project;

Outlining mitigation measures required for avoiding or minimizing potential negative impacts assessed by environmental study;

Developing a monitoring mechanism and identifying requisite monitoring parameters to confirm effectiveness of the mitigation measures recommended in the study;

Defining the requirements for communication, documentation, training, monitoring, management and implementation of the mitigation measures.

### Environmental Management, Monitoring and Reporting

During the construction phase, the overall responsibility for the implementation and monitoring of the EMP rests with the Project Director (PD), Project Management Unit (PMU), and LG&CD Department. The PD at the PMU, using the Construction Supervision Consultant (CSC), will supervise the implementation of the proposed mitigation measures and monitor the implementation progress in the field.

During the operation phase, the overall responsibility for the implementation and monitoring of the EMP rests with Managing Director (MD) WASA, Rawalpindi.

The specific roles and responsibilities for environmental management and monitoring are provided in Table 7.1 below.

### Inclusion of EMP in Contract documents

In order to make Contractors fully aware and responsible of the implications of the EMP and to ensure compliance, it is recommended that mitigation measures be treated separately in the tender documentation and that payment milestones shall be linked to performance, measured by execution of the prescribed mitigation measures. Such a procedure will help ensure adequate management of project impacts is carried out during the construction and operation phases, where a consistent approach will be expected on behalf of the Contractor so that data and information collected from monitoring programs is comparable with baseline monitoring data.

The Contractor shall be made accountable through contract documents and/or other agreements for fulfilling the environmental safeguard obligations and delivering on the environmental safeguard components of the Project. Contractors shall be prepared to co-operate with the executing agency and supervising consultants and local population for the mitigation of adverse impacts. After the EMP’s inclusion in the contract documents, the Contractor will be bound to implement the EMP and will engage appropriately trained environmental and social management staff to ensure the implementation and effectiveness of the mitigation measures.

The Contractor is required to bid for executing the EMP, including the recommended mitigation measures and monitoring programs, as part of its Bill of Quantities (BOQ).

### Institutional Arrangements

The environmental management plan will require involvement of the following organizations for its implementation during construction and operation phases of the project.

#### Role of PMU/CIU, LG&CD Department, Punjab

The PMU will:

Provide support to ADB missions;

Coordinate activities with all stakeholders, review consultants, proposals, and provide overall guidance during various stages of project preparation;

Manage and ensure safeguard due diligence and disclosure requirements including resettlement and environmental safeguards in accordance with ADB’s Safeguard Policy Statement (2009) and Federal & Punjab Government requirements;

Have competent staff to manage contractors and ensure implementation of recommended mitigation measures;

Manage and ensure effective implementation of the gender action plan;

Preparation of Semi Annual Environmental Monitoring Reports (SAEMRs);

Ensure submission of all IEE requirements as per law by responsible entities; and

Monitoring of activities of the entire project.

#### Role of the ADB

The ADB will:

Ensure project compliance with ADB SPS 2009;

Support the coordination and administration of the project;

Provide guidance to PMU, LG&CD and R-WASA on implementation issues and project design;

Disclose all safeguards documents, and monitor safeguards implementation;

Monitor and report project performance;

Conduct periodic review of the project.

#### Role of Construction Supervision Consultant (CSC)

The CSC will be responsible for the following items:

Incorporates into the project design the environmental protection and mitigation measures identified in the EMP for the design stage;

Hire qualified and experienced  staff to monitor and

Ensure implementation of mitigation measures suggested in the EMP.

Addition of one international consultant to supervise the implementation process.

Assists PMU to ensure that all environmental requirements and mitigation measures from the IEE and EMP are incorporated in the bidding and contracts documents.

Assists in preparation of SAEMRs and review of construction bidding documents;

Prior to construction, review and approved the Contractor Site Specific Environmental Management Plans (SSEMPs) with consent of ADB environment team.

Undertakes environmental management capacity building activities for relevant project focal staff including staff from contractors.

#### Role of Federal and Punjab EPAs

The Federal & Punjab EPAs will have the following responsibilities with regards to this project:

Provides regulatory compliance works for the project.

Reviews and approves environmental assessment report of the proposed project submitted by PMU.

Issues environmental clearance certification for the Project based on their mandate and regulations.

Undertakes monitoring of the project’s environmental performance based on their mandate.

#### Role of Project Contractor

The project contractor will be responsible for following items:

Implementation of, or adherence to, all provisions of the IEE and EMP;

Preparation of SSEMPs as required. SSEMPs will be prepared by Contractor’s Environment Specialist, site in charge, HSE staff and project technical team before their mobilization and it will be submitted to Engineer of construction supervision consultant/PMU for review and approval. The SSEMP template for guidance of contractors is provided as Annexure-XII.

Contractor’s environmental performance will rest with the person holding the highest management position within the contractor’s organization. Reporting to their management, the contractor’s site managers will be responsible for the effective implementation of the EMP.

Preparation of monthly environmental monitoring reports.

The Contractor will be required to have qualified and experienced Environmental Specialists in their team to ensure all mitigation measures are implemented during the different development phases of the project.

#### Role of WASA, Rawalpindi

The R-WASA will be responsible for following items:

Implementation of, or adherence to, all provisions of the IEE and EMP

Preparation of site specific EMPs for operations phase

R-WASA will be responsible to ensure that contractors engaged during operation phase of WTP are executing activities in compliance to IEE/EMP.

R-WASA will be required to have qualified Environmental Specialist designated to ensure all mitigation measures are implemented in true letter and spirit.

R-WASA will design and drive behavior change campaigns to increase public participation and cooperation. Public cooperation will be extended through incentives and penalties to the public.

R-WASA will plan customer feedback surveys in order to ensure sustainable service delivery and to remove gaps in the system.

Table 7.1 Environmental Management Plan for Water Supply Component

| Project Stages | Section | Aspects | Impact | Mitigation Measures Recommended |  | Responsibility |  |  |  | Timing |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Execution |  | Monitoring |  |  |  |
| Design/Pre- Construction Phase | 1.1 | Impacts due to Improper designing of distribution networks including transmission mains (replacement & New Distribution Lines) | If location of water distribution networks is not carefully selected considering topography, geology and catchment of watershed results in degradation of channel embankments, reduce water flow or increased chances of structure settlement due to weak geological conditions/weak bearing capacity of soil. The water shall be conveyed to distribution network is also primarily designed on gravity. If not designed properly while maintaining minimum head, the system will require high energy for pumping water to the desired storage tanks | - Factors such as site capacity, accessibility, stability, environmental sensitivity, land use, socio-economic receptors and HOD hazards have been studied and site has been selected accordingly. - Contractor will prepare and get approval of SSEMP through PMU/CSC with consent of ADB Environment Team, before establishing any camp site. - Water supply network design shall facilitate uninterrupted water supply through gravity system without technical constraints. |  | Design Consultant |  | PMU, LG&CD |  | BC: during detailed designing of the project |  |
|  | 1.2 | Impacts due to Lack of integration of IEE/EMP requirements into Construction bid documents | The bidding documents must reflect the requirement to select a qualified and experienced Contractor from the perspective of ensuring implementation of required environmental safeguards provisions during project development. | The Environmental staff of PMU will be assigned the task to check that design and bid documents are responsive to key environmental, social and safety considerations, and that the proposed method of work reflects the boundaries defined in the EMP. The bid documents must include the EMP and its implementation cost must be reflected in the BOQ. IEE/EMP implementation and monitoring requirements must be part of bidding documents and necessary contractual binding must be agreed by project contractors before award of contract. Project contractors shall have qualified and experienced environmental staff to plan, arrange, implement, monitor and report IEE/EMP requirements. |  | Design Consultant |  | PMU, LG&CD |  | BC: during detailed designing of the project |  |
|  | 1.3 | Relocation of Utilities Utility services such as sewerage lines and telephone lines lying along the project alignment may need to be relocated temporarily or permanently to prevent damage to them. |  |  | Detailed filed surveys will be conducted to assess any sub-surface utilities that might be present under the proposed project alignment within the right of way of the project, shall be relocated prior to the commencement of the work. Based on the utilities that are identified during the detailed surveys, a relocation plan of those utilities will be developed by the respective line agencies in close coordination. In case any utilities can be allowed to remain in place and avoidance of their damage is possible, the contractor will need to be aware of the location of these services so that disruptions are not caused. In such scenario, the responsibility for any repair of damaged services will lie with the contractor. |  | Design Consultant |  | PMU, LG&CD |  | BC: during detailed designing of the project |  |
|  | 1.4 | Development of Traffic Management Plan | Heavy congestion due to the movement of construction heavy machinery and equipment with significant delays for traffic apart from posing a significant risk for vehicles. | A comprehensive traffic management plan shall be developed by the contractor and approved by the Executing Agency or the Project Management Unit that will contain the traffic diversion onto alternate routes and management of traffic flows to minimize the congestion and the possibility of accidents. This plan shall be dependent to the EMP and will be implemented by the contractor. |  | Design Consultant |  | PMU, LG&CD |  | BC: during detailed designing of the project |  |
|  | 1.5 | Topographic Impacts due to Excavation | Excavation of land is involved for the proposed Project. A minor physical change in the topography due to excavation of land is expected in the project area. | Mitigation measures will involve adoption of best engineering design measures keeping in view of the aesthetics of the project area and provision of green areas for the landscape in design. The proposed design has tried to minimize these impacts. |  | Design Consultant |  | PMU, LG&CD |  | BC: during detailed designing of the project |  |
|  | 1.6 | Seismic Impacts | Fatalities Damages to structures etuis expected during the project | The proposed structure should be designed and constructed keeping in consideration high intensity earthquakes. For seismic hazard analysis, updated structural and seismic evaluations should be conducted by the design engineer/consultant. Moreover, geo-technical investigations must be conducted prior to construction phase. |  | Design Consultant |  | PMU, LG&CD |  | BC: during detailed designing of the project |  |
|  | 1.7 | Water Usage/ Extraction from Water Source | impact on the other uses of water. | Water must be extracted as planned and over extraction must be avoided; and Regular measurement/ monitoring must be done to avoid the over extraction of water from the source. |  | Design Consultant |  | PMU, LG&CD |  | BC: during detailed designing of the project |  |
| Construction Phase | 2.1 | Impact due to Excavation for Laying of Conductance Main, Distribution of Water Network and Installation of Control Valves | Open trench of around 1.5-meter depth will be excavated for laying of Conductance Main (Bulk Supply Line) from I-10 to IJP road and then inside KSS. Replacement of distribution lines the trench of around 2.5 feet will be excavated along the existing ROW. Excavated soil will be placed along the trench. The pipeline is to be laid along the existing road. The excavated soil, placed along the trench may get disturbed due to wind, rain water and the movement of workers, vehicles and pedestrians, and spill onto road way – disturbing road users, creating dust, road safety issues, etc., and also into nearby open drains. Construction of the pipelines involves quite simple techniques of civil works, the invasive nature of excavation will result to impacts to the sensitive receptors of sub project locations such as residents, business and community in general. Soil erosion may also occur along the stockpiles material especially during winds and rains which may ultimately turn into sluch. | IJP road will not be disturbed and the impact will be mitigated by construction methodology (strategy). The Conductance Main will be jacked under the IJP road and no road cutting will be done. Prior to starting of work, the contractor shall prepare a method statement for water supply pipeline works. This shall be simple and explain the contractor’s work process that is actually conducted on site, with safety and safeguard concerns. Almost 80% of this material will be reused in backfilling the excavations while remaining will be used for backfilling of low laying areas by contracting with private housing societies. Good engineering practices will help to control or minimize the soil erosion both at the construction sites and in peripheral areas. All the disturbed areas need to be protected against soil erosion by stripping and stockpiling of all the available topsoil for later re-vegetation. Special slope protection measures will be adopted in the sensitive areas and along the shoulders of roads and excavations shall be kept confined to the specified foundation spots as per the approved engineering drawings. Method Statement is very important, particularly for water supply pipeline works along the roads. Method Statement can be prepared for each stretch (say 1 km) /specific site based on the project area. Method Statement shall be in a Table format with appended site layout map and cover the following: ▪Work description ▪No. of workers (skilled & unskilled) ▪Details of Plant, equipment & machinery, vehicles Work duration (total, and activity-wise, for example for pipe laying, from excavation to road resurfacing/testing) PPE (helmet, gloves, boots, etc.) details for each type of work Details of materials at each site (type & quantity) Risks/hazards associated with the work (for example, Trench excavation will have risks such as trench collapse, persons/vehicles falling into trench, structural risk to nearby buildings, damage to buildings, infrastructure etc.) Construction waste/debris generated (details & quantity) Detail the sequence of work process (step-by-step) including specific details of each work Contractor’s supervision & management arrangements for the work Emergency: Designate (i) responsible person on site, and (ii) first aider Typical site layout plan including pipe trenching, placement of material, excavated earth, barricading etc. The following shall be included in the site layout plan: Provide barricading/security personnel at the site to prevent entry/trespassing of pedestrian/vehicles into the work zone. Location of temporary stockpiles and provision of bunds Separation of stockpiles areas with workers/vehicle movement paths to avoid disturbing the stockpiled soil Wetting of soil to arrest dust generation by sprinkling water Waste/surplus soil utilization and disposal plan – indicate expected duration of temporary stockpiling along the trench at each site and identify final surplus soil utilization/disposal site in consultation with CSC/PMU. PMU LG&CD will ensure the identification of disposal sites for unsuitable excavated material. Stock piling of excavated material at places that are congested will be avoided as these piles can create traffic issues and public nuisance. |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.3 | Site Accessibility during Construction | The proposed project can be approached through various roads. The construction material, equipment and machinery will be arrived at proposed workshop area and construction camps sites. From the camp, the material, equipment and machinery will be transferred to work site by multiple routes | Selection of access roads/tracks should be based upon certain criteria to ensure high stability, no or minimum tree cutting and crop damage, minimum disturbance to nearby settlements and other social amenities; The approach roads should be selected to avoid any soil degradation and erosion impacts; Special care should be taken for known sensitive area to ensure that all interested parties are aware of the type and location of working. Measures should be taken where necessary to protect such areas physically; and Mitigations for the loss of trees and crops, soil erosion, soil contamination, air and noise pollution, waste generation, and loss of agricultural land provided in below sections will be adopted. |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.4 | Traffic Issues | The construction phase of the proposed project, the laying of the water supply pipelines, could result in heavy congestion due to the movement of construction heavy machinery and equipment’s with significant delays for the traffic apart from posing a significant accident risk for vehicles. The efficient management of traffic once the construction activity commences will be critical in order to minimize the risk of possible road accidents and construction related hazards | To minimize traffic problems in the proposed project area, following measures will be considered: Movement of vehicles carrying construction materials and equipment/machinery will be restricted during the daytime to reduce traffic load and inconvenience to the local population; Construction vehicles, machinery and equipment will be parked at designated areas (at construction camps site) to avoid un-necessary congestions on roads; The speed of the vehicles will be controlled (at 20 to 25 km/hr) to reduce the probability of severe accidents, soil erosion, debris flows due to vibrations and dust emission; Damages of roads due to construction vehicles will be instantly repaired and/or compensated after the completion of work; Proper sign boards will be provided for smooth flow of traffic; Period of construction and area / location of construction site shall be informed to public in general and specifically to local residents; and Any closure of the roads (especially main roads) and deviations / diversions proposed should be informed to the riders through standard signs and displays. Traffic Management Plan will be implemented to avoid traffic accidents, jams/public inconvenience. Guideline for Traffic Management Plan has been prepared and given as Annexure-V |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.5 | Resource Conservation | During the construction, overburden on local resources is possible if construction facilities such as workers camp and construction camp built near rural areas and can create problems for local communities. There can be a conflict for resources between workers and local community. | Use potable water bowsers for construction works and mineral water bottles/ ground water for drinking purpose; Plan for the provision/purchase of adequate insulation to reduce heat loss through construction plants; Reduction of wastage of water through training of workers involved in water use; Reuse of construction waste materials may be adopted wherever possible; Aggregates will not be sourced from river and stream beds. Construction material will be sourced from Margala and Sargodha quarry areas. Diesel and fuels with low sulphur content should be used to operate construction machinery and equipment; Efficient and well maintained equipment and machinery should be used; The equipment and machinery should be turned off when not in use; Use of solar panels at construction camps may be considered and plan for use of solar panels at operational phase may also be considered; and A good camp design and an efficient worksite management plan can help the contractor to reduce the water demand, wastewater and solid waste volumes to the lowest levels. Resource Conservation Plan has been developed for the proposed project and attached as Annexure-VIII. |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.6 | Construction Camps/ Camp Sites related impacts. | Approximately one (01) construction camp is proposed to be established during construction period for the subject project. Due to the proposed camp site, loss of vegetation and dissatisfaction of rehabilitation measures during and after completion of construction phase may occur. This may include waste, soil pollution, groundwater pollution, dust, etc. However, the impact will be medium adverse in nature. For these impacts, mitigation measures have been developed to minimize the likelihood, extent or duration of their occurrence and any associated adverse effects. | Environmental Reinstate any temporary facilities to pre-existing conditions in ecologically sensitive areas. Implement landscaping plan for all facilities in areas where high landscape value and visual vulnerability to the proposed activities warrants site-specific landscape restoration measures. Operate equipment in a manner sympathetic to the ambient noise environment. Do not leave equipment idling unnecessary. Provide adequate warnings of impeding works to all potential receptors within a 1 km corridor surrounding the RoW via public notices and local news. Social State land will be a first preference for worker camp location, followed by land where there is a willing lessee. Employment policies which aim to maximize job opportunities for local people will help to minimize tensions caused by different socio-cultural values. Training will be provided to all staff on camp management rules and overall discipline and cultural awareness. This will include, in appropriate languages: A briefing on camp rules A community relations orientation to increase awareness about the local area, cultural sensitivities and the project Code of Conduct Awareness-raising on health considerations, including sexually transmitted diseases (STDs). The construction contractor is required to develop a Construction Camp Management Plan to address: Discipline; Community liaison; Ethnic tensions and; Communicable diseases. A Code of Conduct and Camp Rules will be required within the Construction Camp Management Plan, which provides policies and a disciplinary framework with respect to worker behaviour. Camp Location The construction contractor will be required to assess the environmental/social sensitivity of any additional or alternative sites prior to their approval for adoption. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.7 | Wastewater Generation | If the generated wastewater is not properly treated or disposed of, this may contaminate the surface water sources such as nullahs, drains, water channels etc. apart from soil contamination. Water from dewatering activities (during rainy season) has the potential to contain suspended solids and oil and grease and if disposed of untreated may affect the soil quality. | To dispose the liquid waste generated from the construction activities, the following steps will be taken by the contractor: Domestic and chemical effluents from the construction camp will be disposed by the development of on-site sanitation systems i.e. septic tanks. Proper monitoring to check the compliance of PEQS will be carried out; and Sewage from construction camps will be disposed of after proper pre-treatment and processes such as soakage pit. The contractor(s) will be responsible to submit details of site-specific wastewater management plan along with details of wastewater collection, transportation and its disposal. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.8 | Disposal of Spoil & Demolition Waste | During construction of the project, especially during the dismantling of the existing distribution pipeline, spoil/ demolition waste will be generated which need to be addressed. However, this impact is categorized as medium adverse in nature. As the project involve replacement of AC and GI pipelines with HDPE pipeline, the redundant AC pipelines will be removed and must be handled with care to avoid release of asbestos fibers into environment | Waste to be reused in the project or by any other interested parties. All material that is produced shall be transported in tarpaulin covered trucks for disposal at the designated site. At the disposal site, the demolition waste will be disposed off in accordance with the best practices to ensure no residual impact. Disposal trucks will be properly maintained to minimize oil leakages during the construction. Demolition waste will not be disposed off at any other area other than the designated area as chances of asbestos presence in the existing pipelines cannot be ignored. In case asbestos is present in existing pipelines, they need to be demolished ensuring internationally acceptable protocol as part of the best practices to properly handle and dispose of the asbestos to prevent any damage to the health of all individuals exposed to the asbestos. Following precautionary measures must be adopted to handle redundant AC pipelines; The Asbestos Cement Pipelines falls under Class II non—friable. For handling of this waste the workers must be properly trained with annual refreshers. Continuous supervision by Competent supervisor must be carried out Engineering Controls including demarcation, wet methods, drop cloths, clean-up and disposal, etc. must be adopted for transportation of redundant AC pipelines. Try as much as possible to remove large lengths of pipeline to avoid cutting and damage to pipeline to control release of asbestos fibers. Respiratory Protection in the form of full face mask and face shield along with protective clothing including gloves, apron etc. must be provided to workers who will be involved in handling of AC pipelines. Disposal of the AC pipelines must be ensured at designated landfill site. The pipelines must be wrapped in double layer of at least 6mil thick plastic sheet. |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.9 | Solid Waste Generation | Considering the labourers (about 150 in numbers) residing in the construction camp and the locally available labour, an average solid waste generation rate of 0.6 kg/capita/day is adopted for the estimation of solid waste generation. Based on this assumption, a total of about 90 kg of solid waste will be generated from construction camp on daily basis. The major components of the labour camp waste will be garbage, putrescible waste, rubbish and small portion of ashes and residues. Other type of wastes may include inorganic construction wastes including hazardous waste. The estimated quantity of excavation material for ROW clearance cannot be provided at design stage. The contractor will provide the estimated quantity of excavation material during the construction phase. These wastes will be generated due to the construction activities and the materials used for construction. This waste would require proper disposal to minimize land and water contamination. This impact can be categorized as medium adverse in nature | All the solid waste from the camps will be properly collected at source by placing containers and disposed of through proper solid waste management system. The contractor will coordinate with local representatives and administration of the concerned solid waste management department for the disposal of solid waste; The contractor must develop a plan of action with the help of concerned solid waste management department for transporting the waste to the disposal site; Toxic waste will be handled, stored, transported and disposed separately; The waste will be properly sealed in containers with proper labels indicating the nature of the waste; and Solid waste will be segregated at source so that it can be re-used or recycled. Waste Management Plan will be developed to implement an efficient and responsive solid waste management system during construction phase. Recyclable wastes e.g. steel bars will be sold to waste vendors; Reusable material will be used as a filling material during ground levelling; Solid waste generated during construction will be safely disposed in demarcated waste disposal sites and the contractor will provide a proper waste management plan; and The site will be restored back to its original conditions after construction completion. |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.10 | Emission of Green House Gas (GHGs) | The main sources of greenhouse gases (CO2, CH4, NOx etc.) during the construction activities of the proposed project will include both mobile and stationary sources. The mobile source will be the construction and transportation vehicles while the stationary source will be the backup generator for electricity. Emission of greenhouse gases cause global warming and other climatic changes on regional and global scale. This impact is can be categorized as high to low adverse in nature | Regular motioning of the vehicles for engine efficiency; Avoid idling of construction vehicles; Alternative energy resources shall be considered where possible; and The Contractor shall ensure the compliance with NEQS/PEQS, 2016. |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.11 | Potential Emergency | The construction of the proposed project may encounter emergencies. In addition, disasters such as earthquakes and fires may occur. Lack of Emergency Response Plan (ERP) or an inefficient response plan may lead to an accident or critical injury. This impact is medium adverse in nature | Emergency Response Plan has been developed to manage all potential and actual emergency situation during construction phase. The same is attached as Annexure X of the report. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.12 | Natural and Man Made Disaster | The construction of the proposed project may encounter hazards such as earthquakes, and fires may occur. Lack of Emergency Response Plan (ERP) or an inefficient response plan may lead to an accident or critical injury. This impact is medium adverse in nature. Natural disasters (earthquakes & floods) and man-made disasters like excavation collapse, structural damage due to deep excavations and fire accidents may result in injuries, financial losses and may even lead to deaths. Release of natural gas, due to damage to gas pipelines may also occur during excavations operations. Incidence of collapse of any nearby structure due to deep excavation can also take place during the construction activities which needs to be avoided | Mitigation measures include the following: An ERP for earthquakes and manmade disasters should be developed by the contractor, in coordination with SC and the proponent (WASA), should be implemented in close consultation with the RESCUE 1122 Services and other concerned departments; Training of the contractor and WASA staff and employees regarding the emergency procedures and plans should be regularly conducted; In case of excavation near to the existing structure is required, then the appropriate controls measures must be taken before start of excavation; Bracing / Wall strengthening/support for deep excavation need to be placed to avoid collapse; Emergency numbers should be clearly posted at all disposal stations; Minor incidents and near misses should be reported, and preventive measures should be formulated accordingly by the WASA Management. A climate change resilience plan or climate change adaptation measure will be adopted to avoid any climate change impacts of the project. Emergency Response Plan has been developed to manage all potential and actual emergency situation during construction phase. The same is attached as Annexure-VIII of the report. Annexure-VIII of the report. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.13 | Degradation of Ambient Air Quality | Air quality will be affected by fugitive dust emissions from construction machinery; dust from the unpaved surface and construction vehicles. Emissions may be carried over longer distances depending upon the wind speed, direction, temperature of surrounding air and atmospheric stability. Potential impacts on the air quality during the construction stage will be due to the fugitive dust and the exhaust gases generated in and around the construction site. Dust is a major component of air pollution, generated mainly from the following construction activities: Site clearance and use of heavy vehicles and machinery/equipment etc. at construction site; Procurement and transport of construction materials such as sand, cement, etc. to the construction site; Other Gaseous emissions during construction result from operating of construction vehicles, plant and equipment; The sensitive receptors identified within or near the project length are educational institutions and religious and cultural places etc. which may be affected during construction phase, due to emission of air pollutants. The construction activities will also result in increased air pollution in the area. The overall impact on the quality of air during the construction phase will, however, be temporary and limited to the project’s implementation phase only. This impact is high adverse in nature. | All vehicles, machinery, equipment and generators used during construction activities should be kept in good working condition and be properly tuned and maintained in order to minimize the exhaust emissions; Open burning of solid waste from the Contractor’s camps and at construction site should be strictly banned; Preventive measures against dust should be adopted for on-site mixing and unloading operations; Construction and spoil materials will be transported trucks covered with tarpaulins and all vehicles (e.g., trucks, equipment, and other vehicles that support construction works) will comply with the PEQS (as amended) for carbon emissions and noise; Contractors shall comply with submitted work schedule, keeping emission operations away from sensitive receptors and residential areas; Regular water sprinkling of the site should be carried out to suppress excessive dust emission(s); Emissions from power generators and construction machinery are important point sources at the construction sites. Proper maintenance and repair is needed to minimize the hazardous emissions; Construction equipment is generally left idling while the operators are on break or waiting for the completion of another task. Emissions from idling equipment tend to be high. Existing idling control technologies, which automatically shut the engine off after a pre-set time can reduce emissions, without intervention of the operators; Along the project alignment with water pipelines and trunk sewer being laid, water will be sprinkled at high frequency especially during summer season at all construction sites to suppress dust emissions; Construction workers should be provided with masks for protection against the inhalation of dust; Vehicles used for construction should be tuned properly and regularly to control emission of exhaust gases; and Regular monitoring of air quality in accordance with PEQS, 2016. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.14 | Noise | The laying of water pipelines and installation of water pumps will result in different construction equipment and machinery being used which will generate high noise levels at the project sites and in the project areas. The noise and vibration will be produced due to the operation of construction machinery and equipment. Sources of noise and vibration during construction are heavy machinery such as bulldozers, excavators, concrete mixing plant, pneumatic drills and other equipment’s. Noise and vibration are perceived as one of the most undesirable consequences of construction activity. The above machinery is expected to generate noise levels that would be severe in the project area. The cumulative effects from several machines can be significant and may cause significant nuisances. However, these increased noise levels will prevail only for a short duration during the construction stage. | Quieter Equipment: A cost-effective way to reduce noise at a construction worksite is to buy quiet equipment. In addition, equipment in use should be the most suitable for the job. Avoid using equipment that is over-powered and, conversely, avoid using under powered equipment. Whenever possible the quietest equipment alternative should be used. In general, electronic powered equipment is quieter than diesel powered equipment and hydraulically powered equipment is quieter than pneumatic power. Modifying Existing Old Equipment: The most common way to reduce the noise levels of common construction equipment is through worksite modifications. Some common worksite modifications consist of retro-fitting existing equipment with damping materials and mufflers. Barrier Protection: An effective way of reducing noise is to locate noisy equipment behind purpose-built barriers. The barriers can be constructed on the work site from common construction building material (plywood, block, stacks or spoils) or the barriers can be constructed from commercial panels which are lined with sound absorbing material to achieve the maximum shielding effect possible. To be effective, the length of the barrier should be greater than its height. The noise source should not be visible and barrier should be located as close as possible to either the noise source or the receiver. Work Activity Scheduling: Work activity scheduling are administrative means to control noise exposure. Planning how noise sources are sited and organized on a work site can reduce noise hazards. Jobs can be rotated so that exposure time is limited. Transferring workers from a high exposure task to a lower exposure task could make the employee’s daily noise exposure acceptable. Maintenance Increased attention to maintenance of tools and equipment will reduce worksite noise levels. Maintaining plant and equipment in good order not only increases its life, but makes it safer to use and quieter. Loose and worn parts should be fixed as soon as possible. Ideally, the worksite should have a system in place for checking and servicing the various machines and power tools. Use of low noise machinery, or machinery with noise shielding and absorption; Contractors shall comply with submitted work schedule, keeping noisy operations away from sensitive receptors, residential areas; implement regular maintenance and repairs; and employ strict implementation of operation procedures; The Contractor shall comply with PESQ, 2016. |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.14 | Vibrations | The duration and amplitude of vibration generated by construction equipment varies widely depending on the type of equipment and the purpose for which it is being used. In assessing vibration from construction equipment, it is useful to categorize the equipment by the nature of the vibration generated. Various equipment categories according to type of vibration and/or activities in each category are listed below: Equipment or activities typical of continuous vibration include: Excavation equipment; Static compaction equipment; Tracked vehicles; Road traffic; Vibratory pile drivers; Pile-extraction equipment; and Vibratory compaction equipment. Energy from construction equipment is transmitted into the ground and transformed into vibration, which weakens with distance. The magnitude and attenuation of ground vibration is dependent on the following: The efficiency of the energy transfers mechanism of the equipment (i.e. impulsive, reciprocating, rolling or rotating equipment); The frequency content; The impact medium stiffness; The type of wave (surface or body); and The ground type and topography. Any vibration would result in nuisance effects, and will be localized and temporary and will unlikely result in structural damage to buildings or walls of the adjacent private properties. However, there is likelihood of human discomfort and damage to infrastructures (crack formation) due to the vibration caused by construction machinery. This impact is high adverse in nature. | All vibration producing machinery will be identified by the contractor and vibration producing machinery will be operated at low frequency to reduce the vibration impacts. Vibration monitoring will be carried out in accordance with German Standard DIN 4150 (heritage structures) and BS 7385: Part 2 – 1993 (other structures). Vibration monitoring will be undertaken at the nearest buildings to the construction sites during times of vibration intensive works Where exceedances of the criteria are recorded, corrective actions would be implemented where soils at risk of vibration induced settlement are identified. A regular programme of vibration monitoring shall be implemented as a minimum in accordance with of this document. The Contractor will submit the proposed method, the frequency and the location of monitoring site to the Planning Authority for agreement prior to commencing works |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.15 | Flora | No vegetation exists within the existing right of the water pipe lines and the proposed project deals only the laying of water pipe lines within the existing right of way along the existing pipelines. Therefore, no damage to the existing flora is envisaged due to the laying of proposed water supply lines. However, the installation of water pumps may damage the grass near the existing overhead water reservoirs | Cutting and / or disturbance of trees shall be avoided, as far as possible; A tree plantation program shall be implemented with the recommendations and technical support of Punjab, Forest Department for the enhancement ecological condition of the area; Final and actual plantation and trees plantation cost will be finalized after assessment by Punjab, Forest Department. All the activities will be executed as per agreement/MOU between proponent and Punjab, Forest department while strictly considering law of land. Open fires should be banned in the area to avoid hazards of fire in the area; Clearing of vegetation cannot be avoided at the areas specified for project structures, but damage to the natural vegetation may be minimized by establishing camp sites, workshops and concrete mixing plants on waste/barren land rather than on forested or green land. However, if such type of land is not available, it shall be ensured that minimum clearing of the vegetation is carried out and minimum damage is caused to trees and undergrowth; Construction vehicles, machinery and equipment will remain confined within their designated areas of movement; The Contractor’s staff and labor shall be strictly directed not to damage any vegetation such as trees or bushes; and Contractor shall provide gas cylinders at the camps for cooking purposes and cutting of trees/bushes for fuel shall be strictly prohibited. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.16 | Fauna | During construction phase the existing population of birds, mammals and reptiles of the construction areas will be affected due to disturbance arising from construction activities involving excavation, movement of machinery and vehicular traffic, movement of labor, camping, etc. The existing fauna will leave the directly affected areas due to construction activities and human intervention. Some animals particularly reptiles may get killed during the earthwork’s operations. Moreover, the movements of the mammals and reptiles will be restricted during the construction phase. Birds as well, will tend to move away from the construction areas and find shelter and food elsewhere due to the activities mentioned above for fear of being hunted / trapped. Noise generated from machinery will even scare the wildlife residing in habitats located at some distance from the construction areas. Food and refuse at the Contractor’s camps may attract animals that might result into encounters with workers. This impact is indirect, site-specific, temporary, reversible, possible and medium significant. | Care shall be taken during construction activities for avoiding purposely or chance killing of animals; If any wild species and habitat is found during construction, it must be dealt carefully and local wildlife department officials should be informed; Hunting, poaching and harassing of wild animals shall be strictly prohibited, and Contractor shall be required to instruct and supervise its labor force accordingly and clear orders should be given in this regard; The Contractor must be held responsible for instructing his work force accordingly and for enforcing this restriction. In addition, this shall have to be controlled by the Wildlife Department. Special measures shall be adopted to minimize impacts on the wild birds, such as avoiding noise generating activities during the critical periods of breeding; Noise generating activities shall not be carried out during the night by the work force; Wastes of the camps shall be properly disposed of to prevent it being eaten by animals, as it may be hazardous to them; and Noise produced due to construction activities may be kept to acceptable level. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.17 | Community Health and Safety | Community health and safety issues during the construction of proposed project may include dust, noise, and vibration from construction vehicle transit, and communicable disease associated with the influx of temporary construction labor. Significant community health and safety issues associated with the projects may also include: Pedestrian safety; Traffic safety; and Emergency preparedness. Further, the laying of water supply pipelines, in particular, will require the digging of trenches for laying these lines will pose a risk of community members falling into these trenches. The construction areas located near the, settlements, may cause accident for the people moving near to these areas. Considering these consequences, this impact can be categorized as direct, moderate, site-specific, medium term, temporary, medium probability and irreversible | The World Bank/IFC EHS Guidelines, 2007, defines community health and safety in terms that guarantee as-built infrastructure conforms to acceptable standards (structural safety, flood and fire risk), water supply sources are of suitable potable quality, emergency response planning is in place for built environments, traffic safety provisions are enforced alongside transport hazard assessment and mitigation, and disease prevention measures are taken. The Contractor will prepare the site specific community health and safety plan in compliance with relevant sections of the IFC General Environmental Health and Safety Guidelines (WB/IFC 2007), Pakistan Labor Laws, Punjab Occupational Safety and Health Act (2019); The Contractor will clearly barricade work areas to prevent access by the public, while ensuring passage by providing safe pathways for pedestrians around construction zones; The Contractor will exclude parking, waiting vehicles and vendors from areas adjacent to the work by means of clearly marked barricades and posted signage; The Contractor will remove excavated earth, spoil, rubble, cut vegetation and refuse whether generated by the project or discarded by third parties from areas within the construction zone, where it has potential to interfere with the public or generate dust; The Contractor will provide temporary lighting to facilitate construction during night time; The Contractor will remove hazardous conditions on construction sites that cannot be controlled effectively with site access restrictions and will barricade any excavations and materials placed near the public place (if applicable); Hard Barricades will be provided at the excavation deeper than 1.5 meter as per “NZS 3845:1999 Road safety barrier systems”; The Contractor will promptly reinstate any services and reinstall any physical facilities that are cut, disconnected or damaged during construction, and maintain or provide temporary services that are interrupted by construction. The Supervisory Consultant will inspect and certify the adequacy of all reinstated services and facilities; Installation and maintenance of speed control and traffic calming devices at pedestrian crossing areas especially near the settlements; An Emergency Preparedness and Response Plan (EPRP) in coordination with the local emergency responders to provide timely first aid response in the event of accidents and hazardous materials response in the event of spills; Instruct foremen to strictly enforce the keeping out of non-working persons, particularly children, off work sites; Timely public notification on planned construction works; Close consultation with local communities to identify optimal solutions for diversions to maintain community integrity and social links; Any environmental condition that is disagreeable to the public and causes an avoidable nuisance can be addressed with additional provisions over and above those described above, as determined necessary by the supervisory consultant. These requirements will be incorporated into the bidding specification and contract documents, and will be binding on the contractor, at risk of penalty for noncompliance, as charges to be recovered from contractor for unsafe act or condition. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.18 | Occupational Health and Safety | Occupational Health and Safety (OHS) related impacts will arise during construction stage activities including clearing of earth, levelling, compaction, foundations, finishing, testing & commissioning. In nutshell, occupational health and safety issues associated with the construction of proposed project will primarily include physical hazards; chemical hazards; and noise. Noise: Construction and maintenance personnel may be potentially exposed to high levels of noise from heavy equipment operation and from working in proximity to vehicular traffic. As most of these noise sources can be prevented, by using noise barriers, new machinery and by using personal hearing protection by exposed personnel and implementation of work rotation programs to reduce cumulative exposure. In addition, disasters such as earthquakes and fires may occur. Lack of Emergency Response Plan (ERP) or an inefficient response plan may lead to an accident or critical injury. Considering these consequences, this impact can be categorized as direct, moderate, site-specific, medium term, temporary, medium probability and irreversible | Complying with the safety precautions for the construction workers as per International Labour Organization (ILO) Convention No. 62, as far as applicable to the Project Contract; Formulation of layout plan for camp site, indicating safety measures taken by the contractor, e.g. firefighting equipment, safe storage of hazardous material, first aid, security, fencing, and contingency measures in case of accidents; Work safety measures and good workmanship practices are to be followed by the contractor to ensure no health risks for labourers; Provision of adequate sanitation, washing, cooking and dormitory facilities including light up to satisfaction; Proper maintenance of facilities for workers shall be monitored; Provision of protective clothing for labourers handling hazardous materials, e.g. helmet, adequate footwear for bituminous pavement works, protective goggles, gloves, etc. Ensure strict use of wearing these protective clothing during work activities; Contractor will ensure the provision of medicines, first aid kits, ambulance etc. at the camp site; Provision of proper safety and diversion signage, particularly at sensitive/accident-prone spots; and Disasters such as earthquakes, flooding, due to heavy rain and other manmade disasters such as fires may occur, and that must be considered for minimizing their impacts. Therefore, an Emergency Response Plan for earthquakes and manmade disasters should be developed by the construction contractor; and Contractor(s) shall prepare a comprehensive OHS Plan including work method statements for all hot works to avoid incidents during construction stage. A guideline OHS Plan is provided in Annex-VI. The Contractor will prepare the site specific occupational health and safety plan in compliance with relevant sections of the IFC General EHS Guidelines (WB/IFC 2007), Pakistan Labor Laws and Punjab Occupational Safety and Health Act (2019). |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site |  |
|  | 2.19 | Social and Cultural Conflicts | During the construction phase of the proposed project, conflicts may arise between labor force and local community. Use of local resources and products by the construction workers can generate stress on the local resources. Furthermore, difference in cultural values may also cause discomfort to local residents. This impact is negative, local, low, short-term and probable | Local labor especially from nearby communities should be given preference for the construction works; Careful planning and training of work force to minimize disturbance to the local people; Public notification through print or electronic media during the entire construction phase to avoid any inconvenience in accessibility to the locals; and Adequate training especially for the transitive workforce of the station (involved both in the construction process and in the commissioning) to regard the customs of the area so that the locals do not feel insecure. |  | Construction Contractor |  | CSC, PMU |  | During Construction |  |
|  | 2.20 | Communicable diseases incl. COVID-19 | The laborers in the Contractor Camp, truck drivers and like personnel who interact with each other have the potential for the spread of HIV/AIDS if the incidence exists. Majority of the people living in the surrounding of the Project, and potential Labor are not aware of the source, mode of communication or consequences of HIV/AIDS. Although their religious and cultural value system, to a large extent excludes the outbreak or rapid communication of HIV/AIDS, yet its occurrence in such a situation cannot be precluded. It is necessary that awareness and preventive campaigns are run from time to time in the Labor camps and the field offices of the Project to prevent the communicable diseases like Cholera, Typhoid and Tuberculosis. There is a chance of spreading of an epidemic of Coronavirus disease (COVID-19) due to close interaction of the labor force during construction not only among the workers but also in the area. This impact is high adverse in nature. | All workers must perform complete sanitization at the site as per SOPs/guidelines issued by WHO and the national guidelines issued by the Government of Pakistan (GOP)20. All workers must wear a mask as soon as they arrive at site and must keep wearing it at all times while present at the work site/hospital premises. As soon as workers arrive at work site, their body temperature must be checked and in case any worker is assessed to be running a fever or suffering from a flu or cough, he must be informed to leave immediately and self-isolate for a two-week period and not report for work until this two-week mandatory period has been completed. At the work site(s), social distancing measures must be strictly implemented and gathering of workers at any location at the work site(s) must be strictly forbidden. In case of workers not taking this measure seriously, strict penalties must be imposed to ensure implementation. The work tasks must be divided into shifts, as far as possible, to reduce the workforce present at the work site(s) at any one moment and improve the working speed/efficiency. All workers will be strictly advised to wash their hands as frequently as practicable and not to touch their face during work. A supply of safe drinking water will be made available and maintained at the project site(s). COVID awareness sign boards must be installed at the camp clinic and at the work site(s). Contact details of all workers will be kept in a register on site in order to efficiently trace and manage any possible workers that might experience symptoms of COVID-19. Prohibition of entry for local community/any unauthorized persons at work sites. Proper hygiene practices in the toilets and washrooms will be implemented with proper and adequate use of soaps and disinfectant spray. Social distancing must be maintained during the pick-up and dropping off of workers from their residences to and from the work site(s). |  | Construction Contractor |  | CSC, PMU |  | During Construction at Active Construction Site/ Camps |  |
| Operational Phase | 3.1 | Water system leaks and water discharges during flushing | Water system leaks can reduce the pressure of the water system compromising its integrity and ability to protect water quality (by allowing contaminated water to leak into the system) and increasing the demands on the source water supply, the quantity of chemicals, and the amount of power used for pumping and treatment. Leaks in the distribution system can result from improper installation or maintenance, inadequate corrosion protection, settlement, stress from traffic and vibrations, frost loads, overloading, and other factors. Water supply lines may be periodically flushed to remove accumulated sediments or other impurities that have accumulated in the pipe. Flushing is performed by isolating sections of the distribution system and opening flushing valves or, more commonly, fire hydrants to cause a large volume of flow to pass through the isolated pipeline and suspend the settled sediment. The major environmental aspect of water pipe flushing is the discharge of flushed water, which may be high in suspended solids, residual chlorine, and other contaminants that can harm surface water bodies. Recommended measures to prevent, minimize, and control impacts from flushing of mains include | Ensure construction meets applicable standards and industry practices; Conduct regular inspection and maintenance; Implement a leak detection and repair program (including records of past leaks and unaccounted-for water to identify potential problem areas); Consider replacing mains with a history of leaks of with a greater potential for leaks because of their location, pressure stresses, and other risk factors. Discharge the flush water into a municipal sewerage system with adequate capacity; Discharge the flush water into a separate storm sewer system Minimize erosion during flushing, for example by avoiding discharge areas that are susceptible to erosion and spreading the flow to reduce flow velocities. |  | WASA, Rawalpindi |  | Env. Expert/ EPAs |  | During Operations |  |

| CSC | Construction Supervision Consultant |
| --- | --- |
| BC | Before Construction |
| DC | During Construction |
| PMU | Project Management Unit |
| DO | During Operation |

### Monitoring Parameters

A monitoring plan for the pre-construction/design, construction and operation phases of the project, indicating environmental parameters, mitigation measures of the negative environmental impacts, responsible authority and frequency is provided below as Table 7.1, Table 7.2 and Table 7.3 below.

During the procurement/pre-construction period, the monitoring activities will focus on (i) checking the contractor’s bidding documents particularly to ensure that all necessary environmental requirements have been included; and (ii) checking that the contract documents’ references to environmental mitigation measures requirements have been incorporated as part of contractor’s assignment and making sure that any advance works are carried out in good time.

During the construction period, the monitoring activities will focus on ensuring that any required environmental mitigation measures are implemented to address possible impacts.

In general, the construction impacts will be manageable, and no insurmountable impacts are predicted, provided that the EMP is implemented to its full extent as required in the Contract documents. However, experience suggests that some Contractors may not be familiar with this approach or may be reluctant to carry out some measures. For the proposed project, in order that the Contractor is fully aware of the implications of the EMP and to ensure compliance, environmental measures must be costed separately in the tender documentation and listed as BOQ items, and that payment milestones must be linked to environmental performance, Vis a Vis the carrying out of the EMP.

The effective implementation of the EMP will be audited as part of the loan conditions by ADB, and as part of regulatory/NOC compliance by Federal and Punjab EPA. In this regard, the PMU/CSC will guide the design engineers and Contractors on the environmental aspects and necessary EMP documentation. Monitoring during operation phase of water supply system will be carried out by WASA Rawalpindi with support from PMU.

Table 7.2: Pre-Construction’ Environmental Monitoring Plan for Baseline Development

| Parameter to be measured | Objective of Monitoring | Parameters to be Monitored | Measurements | Location | Frequency | Responsibility |
| --- | --- | --- | --- | --- | --- | --- |
| Ambient Air Quality | To establish baseline air quality levels | CO, NO2 & PM10 (particulate matter smaller than 10 microns) concentration at receptor level,PM2.5 | 1-hr and 24-hr concentration levels | At least two random receptor locations (especially one in Federal Area and One in Punjab) in the project area, both upwind and downwind | Once | CSC |
| Ambient Noise | To establish baseline noise levels | Ambient noise level near receptors in project area | A-weighted noise levels 24 hours, readings taken at 15 s intervals over 15 min. every hour, and then averaged | At two random receptor locations in the project area | Once | CSC |
| Wastewater Quality in vicinity of project area | To establish wastewater quality in project area | Wastewater quality in project area | Water samples for comparison against NEQS parameters | At two locations within the project area | Once | CSC |

Table 7.3: Construction Phase Monitoring Requirements

| Project Activity and Potential Impact | Objective of Monitoring | Parameters to be Monitored | Measurements | Location | Frequency | Responsibility |
| --- | --- | --- | --- | --- | --- | --- |
| Noise Disturbance due to noise from construction activity | To determine the effectiveness of noise abatement measures on sound pressure levels | Ambient noise level at different locations in project area | A-weighted noise levels 24 hours, readings taken at 15 s intervals over 15 min. every hour at 15 m from receptors, and then averaged | At two random sensitive receptor locations in project area | Quarterly basis on a typical working day | Contractor’s Environmental officer, CSC |
| Air Quality Dust emissions from construction vehicles and equipment | To determine the effectiveness of dust control program on dust at receptor level | CO,NO2 & PM10 (particulate matter smaller than 10 microns) ,PM2.5 concentration at receptor level | 1-hr and 24-hr concentration levels | At two random sensitive receptor locations in project area | Quarterly basis on a typical working day | Contractor’s Environmental officer, CSC |
|  |  | Visible dust | Visual observation of size of dust clouds, their dispersion and the direction of dispersion | Construction site | Once daily during peak construction period | Contractor’s Environmental officer, CSC |
| Water quality monitoring for drinking water and wastewater | To determine the effectiveness of mitigation measures | As per WHO/NEQS | Water samples for comparison against NEQS parameters | Workers camp and active construction site | Quarterly basis on a typical working day | Contractor’s Environmental officer, CSC |
| Safety precautions by Safety workers | To prevent accidents for workers and general public | Number of near miss events and accidents taking place | Visual inspections | Construction site | Once Daily | Contractor’s Environmental officer, CSC |
| Soil Contamination | To prevent contamination of soil from oil and toxic chemical spills and leakages | Incidents of oil and toxic chemical spills | Visual inspections | At construction site and at vehicle and machinery refuelling & maintenance areas | Once a month | Contractor’s Environmental officer, CSC |
| Solid Waste & Effluent disposal Insufficient procedures for waste collection, storage, transportation and disposal | To check the availability of waste management system and implementation | Inspection of solid and liquid effluent generation, collection, segregation, storage, recycling and disposal will be undertaken at all work sites in project area | Visual inspections | At work sites in project area | Once daily | Contractor’s Environmental officer, CSC |

Table 7.4: Operation Phase Environmental Monitoring Plan

| Parameter to be measured | Objective of Monitoring | Parameters to be Monitored | Measurements | Location | Frequency | Responsibility |
| --- | --- | --- | --- | --- | --- | --- |
| Waste Water Quality | To determine the effectiveness of mitigation measures | As per WHO/NEQS | Water samples for comparison against NEQS parameters | upstream and downstream locations of nullahs | Annual basis on a typical working day | WASA Rawalpindi |
| Water Quality | To assess whether provided portable water is within limits of PEQS. | As per PEQS | Water samples for comparison against NEQS parameters | At storage tanks and few samples will be taken from users taps | Annual basis on a typical working day.Monthly | WASA Rawalpindi |

### Environmental Training

#### Capacity Building and Training

Capacity building and training programs are necessary for the project staff in order to control the negative impacts resulting from the project construction and during its operation phase. They will also require trainings on monitoring and inspecting of such a project for environmental impacts and for implementation of mitigation measures.

The details of capacity building and training program are presented in the Table 7.5.

Table 7.5: Capacity Development and Training Programme

| Provided by | Organized by | Contents | Target Audience | Venue | Duration |
| --- | --- | --- | --- | --- | --- |
| Pre-construction Phase PMC offering specialized services in environmental management and monitoring. | CSC & PMU | Short seminars and courses on: - Environmental Management Plan and Environmental Monitoring Plan. - Group exercise and participatory workshop to measure effectiveness of program. | Contractor staff | WASA Rawalpindi Office | One day long training seminar including group exercise/workshop. |
| Construction Phase PMC offering specialized services in social management and monitoring. | CSC & PMU | Short seminar on Environmental risks associated with construction phase. - Development of Environmental - Performance Indicators - Occupational Health and Safety (OHS) issues. - Group exercise and participatory workshop to measure effectiveness of program. | Contractor staff | WASA Rawalpindi Office | One day long training seminar including group exercise/workshop. |

### Environmental Management Costs for Water Supply Component

The Table 7.6 below provides cost estimates for ‘Pre-Construction phase’ monitoring while Tables 7.7 and 7.8 provides cost estimates for ‘Construction phase’ and ‘Operation phase’ monitoring of key environmental parameters.

The costs associated with implementation of the EMP and the necessary mitigation measures are provided as Table 7.9 below. The Table 7.10 below provides the cost for capacity development and training programme for project contractors for the proposed project.

Table 7.6: Annual Cost Estimates for ‘Pre-Construction Phase’ Environmental Monitoring

| Monitoring Component | Parameters | Quantity | Amount PKR | Details |
| --- | --- | --- | --- | --- |
| Air Quality | CO, NO2, SO2, O3 PM10, PM2.5 | 2 (Once only at 2 locations) | 60,000 | 2 readings @ PKR 30,000 per sample |
| Noise Levels | dB(A) | 2 (Once only at 2 locations) | 60,000 | 2 readings @ PKR 30,000 per reading |
| Groundwater Quality | PEQS /NEQS | 2 (Once only at 2 locations) | 60,000 | 2 readings @ PKR 30,000 per sample |
| Contingencies |  |  | 9,000 | 5% of monitoring cost |
| Total (PKR) |  |  | 189,000 |  |

Table 7.7: Annual Cost Estimates for ‘Construction Phase’ Environmental Monitoring

| Monitoring Component | Parameters | Quantity | Amount PKR | Details |
| --- | --- | --- | --- | --- |
| Wastewater Quality | PEQS / NEQS / WHO | 16 (Quarterly at 2 locations) | 480,000 | 16 readings @ PKR 30,000 per sample |
| Drinking water quality | PEQS / NEQS / WHO | 16 (Quarterly at 2 locations) | 480,000 | 16 readings @ PKR 30,000 per sample |
| Air Quality | CO, NO2, PM10.PM2.5 | 16 (Quarterly basis at 2 Locations) | 480,000 | 16 readings @ PKR 30,000 per sample |
| Noise Levels | dB(A) | 16 (Quarterly basis at 2 locations) | 160,000 | 16 readings @ PKR 10,000 per reading |
| Total |  |  |  | 1,600,000.0 |
| Contingencies |  |  | 80,000 | 5% of monitoring cost |
| Total (PKR) |  |  | 1,680,000* |  |

*Subject to monitoring requirements of EPAs as per conditions of Environmental Approval for construction phase

Table 7.8: Annual Cost Estimates for ‘Operation Phase’ Environmental Monitoring

| Monitoring Component | Parameters | Quantity | Amount PKR | Details |
| --- | --- | --- | --- | --- |
| Water Quality | PEQS / NEQS / WHO | 12 | 360,000 | 12 readings @ PKR 30,000 per sample |
| Wastewater Quality | PEQS / NEQS / WHO | 1 | 30,000 | 1 reading @ PKR 30,000 per sample |  |
| Total |  |  |  | 390,000.0 |  |
| Contingencies |  |  | 19,500.0 | 5% of monitoring cost |
| Total (PKR) |  |  | 409,500** |  |  |

**Subject to monitoring requirements of EPAs as per conditions of Environmental Approval for Operation Phase

Table 7.9: Estimated Costs for EMP Implementation

| Item | Sub-Item | Estimated Total Cost (PKR) |
| --- | --- | --- |
| Contractor Environment Specialist | 1 person for 24 months (@ 100,000 per month) | 2,400,000 |
| Monitoring Activities | Provided separately in Tables 7.7 and 7.8. | - |
| Mitigation Measures | As prescribed under EMP and IEE. | 4,000,000 |
| (i) Water sprinkling | To suppress dust emissions | 800,000 |
| (ii) Solid waste collection & disposal (including hazardous waste) | From construction sites (based on initial estimates) | 1,500,000 |
| (iii) Plantation around project | To plant vegetation all along the route of Conductance Main and at Water works | 1,500,000 |
| (iv) Cost of Hard Barricades, shoring, restoration and noise barriers | Provision of hard barricades in case the trench is 1.5 meter deep | 4,000,000 |
| (v) SSEMP preparation and Implementation | Preparation and Implementation of SSEMP | 1,000,000 |
| Total |  | 15,200,000 |
| Contingencies | 5% of EMP implementation cost | 760,000 |
| Total Estimated Cost (PKR) |  | 15,960,000 |

Table 7.10: Cost of Capacity Development and Training Programme for Project Contractor(s)

| Provided by | Organized by | Contents | No. of training events | Duration | Cost (PKR) |
| --- | --- | --- | --- | --- | --- |
| Pre-construction Phase Monitoring Consultants / Organizations offering specialized services in environmental management and monitoring | CSC & PMU | Short seminars and courses on: Environmental Management Plan and Environmental Monitoring Plan | Two seminars for Contractor management staff and project staff | 1 day | 200,000/Training |
| Construction Phase Monitoring Consultants / Organizations offering specialized services in environmental management and monitoring | CSC & PMU | Short seminars on Environmental risks associated with construction phase. Development of Environmental Performance Indicators Occupational Health and Safety (OHS) issues | Two seminars for Contractor management staff and project staff dealing in environment and social issues | 1 day | 200,000/Training |
| Total |  |  | 400,000 (PKR 0.4 million) |  |  |

Table 7.11: Environmental Management Plan for Solar PV Installation

| Project Stages | Section | Aspects | Impact | Mitigation Measures Recommended | Responsibility |  |  | Timing |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Execution | Monitoring |  |  |
| Design/Pre- Construction Phase | 1.1 | Impacts due to Lack of integration of IEE/EMP requirements into Construction bid documents. | The bidding documents must reflect the requirement to select a qualified and experienced Contractor from the perspective of ensuring implementation of required environmental safeguards provisions during project development. | - Appointment of at least one environment focal person and one social focal person within the PMU to be in charge of EMP coordination, implementation and site inspections including project GRM; - Appointment of at least one environment, health and safety officer within the Contractor staff to be in charge of EMP coordination, implementation, site inspections and information disclosure and consultations; - Appointment of at least one GRM focal person to be in charge of project GRM coordination, handling complaints, dispute resolution, site visits and information disclosure and consultations. - The environmental staff of PMU will be assigned the task to check that design and bid documents are responsive to key environmental, social and safety considerations, and that the proposed method of work reflects the boundaries defined in the EMP. The bid documents must include the EMP and its implementation cost must be reflected in the BOQ. - IEE/EMP implementation and monitoring requirements must be part of bidding documents and necessary contractual binding must be agreed by project contractors before award of contract. | Design Consultant | PMU, LG&CD | BC: during detailed designing of the project. |  |
|  | 1.2 | Relocation of Utilities | - Utility services such as sewerage lines and/or telephone, electricity or gas pipelines lying in the project area or across the project site may need to be relocated temporarily or permanently to prevent damage to them. | - Detailed field surveys will be conducted to assess any sub-surface utilities that might be present under or across the project site or in the project area and they shall be relocated prior to the commencement of the work. - Based on the utilities that are identified during the detailed surveys, a relocation plan of those utilities will be developed by the respective line agencies in close coordination. - In case any utilities can be allowed to remain in place and avoidance of their damage is possible, the contractor will need to be aware of the location of these services so that disruptions are not caused. In such scenario, the responsibility for any repair of damaged services will lie with the contractor. | Design Consultant | PMU, LG&CD | BC: during detailed designing of the project. |  |
|  | 1.3 | Traffic issues | - Heavy congestion due to the movement of construction heavy machinery and equipment with significant delays for traffic apart from posing a significant risk for vehicles. | A comprehensive traffic management plan shall be developed by the contractor and approved by the Executing Agency or the PMU that will contain the traffic diversion onto alternate routes and management of traffic flows to minimize the congestion and the possibility of accidents. This plan shall be dependent to the EMP and will be implemented by the contractor. | Design Consultant | PMU, LG&CD | BC: during detailed designing of the project |  |
|  | 1.4 | Seismic Impacts | Damage to project infrastructure could take place. | The proposed structures shall be designed and constructed keeping in consideration high intensity earthquakes. For seismic hazard analysis, updated structural and seismic evaluations shall be conducted by the design engineer/consultant. Moreover, geo-technical investigations will be conducted prior to construction phase. | Design Consultant | PMU, LG&CD | BC: during detailed designing of the project |  |
| Construction Phase | 2.1 | Traffic Issues | - The construction phase of the proposed solar installation, particularly the transportation to site of the solar PV panels and associated electronical equipment and infrastructure could result in congestion due to the movement of construction heavy machinery and equipment with significant delays for the traffic apart from posing a significant accident risk for vehicles. | - Movement of vehicles carrying construction materials and equipment/machinery will be restricted during the daytime to reduce traffic load and inconvenience to the local population; - Construction vehicles, machinery and equipment will be parked at designated areas (at construction camps site) to avoid un-necessary congestions on roads; - The speed of the vehicles will be controlled (at 20 to 25 km/hr) to reduce the probability of severe accidents, soil erosion, debris flows due to vibrations and dust emission; - Damages of roads due to construction vehicles will be instantly repaired and/or compensated after the completion of work; - Proper sign boards will be provided for smooth flow of traffic; - Period of construction and area / location of construction site shall be informed to public in general and specifically to local residents; and - Any closure of the roads (especially main roads) and deviations / diversions proposed should be informed to the riders through standard signs and displays. - Traffic Management Plan will be implemented to avoid traffic accidents, jams/public inconvenience. | Construction Contractor | CSC, PMU | During Construction at Active Construction Site |  |
|  | 2.2 | Air Quality | - Potential air quality impacts during construction include dust emissions associated with earthworks and gaseous emissions from construction activities and heavy vehicles used to support the construction works. Some concerning activities are wind-borne dust emissions from cleared areas, stockpiles, and unpaved site roads; truck-derived dust emissions from traffic on haulage routes and dumping; and dozer- and excavator-derived dust emissions from clearing operations and earthworks. | - Particulate dust generation will be minimized by restricting land clearing to the areas flagged for immediate use, progressively rehabilitating exposed soils as soon as practicable, avoiding clearing and earthworks activities during dry, windy conditions and applying water to all cleared, road and stockpile areas whenever weather conditions have the potential to mobilize fugitive dust. Open burning (e.g. of vegetation, waste, hydrocarbons) will be avoided. A detailed management and monitoring plan for air quality is provided in the Environmental and Social Management Plan (EMP) for the Project, which will monitor emissions generated during construction. General air quality management and mitigation measures are as follows: Conduct awareness trainings on dust emissions for project employees and workforce; Restrict speed limit on unsealed roads to minimize dust generation; Conduct topsoil stripping when soil is moist to the extent practical; Monitor for air quality impacts near any sensitive receptors identified near construction areas and along the access roads; Use low-sulphur fuels where practical; Source low-emission diesel equipment (e.g. Tier 2 or better), where practical. The Contractor(s) will conduct air quality monitoring at the solar site, once before start of the civil works to establish the baseline, quarterly during the civil works and once after completion of the civil works to monitor and mitigate exceedances (if any) with respect to the applicable ambient air quality standards. Air monitoring will be conducted utilizing handheld air monitoring devices at select locations identified on site map. Open and exposed land areas at solar plant will be sprayed with water to suppress dust levels, particularly during the dry season. Use of chemical dust suppressants will be prohibited. Aside from this, works sites will be temporarily barricaded to control dust levels. Storage areas/warehouse for materials required for construction and civil works will be provided onsite (construction camps at the solar plant) to reduce construction vehicle trips of transporting materials and minimize stockpiling. Vehicles transporting materials that generate dust will be covered with tarps. Construction vehicles and machinery will be maintained to a high standard to minimize vehicular emissions and noise. Log of monitoring/incidences of non-compliance and rectification will be recorded and maintained. | Construction Contractor | CSC, PMU | During Construction at Active Construction Site |  |
|  | 2.3 | Noise and Vibration | Noise impacts will be temporary and localized at all construction sites as construction machinery and vehicles generate noise as they operate. Other noise sources include loading, unloading and haulage of equipment and materials. | Noise and vibration associated with vehicles will be minimized by imposing a speed limit on site roads, by scheduling the majority of vehicle movements to daylight hours, minimizing air/exhaust braking where this would cause disturbance, and establishing soil berms or vegetation along roads to reduce the impact of traffic noise. Equipment with lower noise levels will be selected where practical and the operation of noisy equipment will be limited to daylight hours where practical. Schedule project supply deliveries to daylight hours. Maintain road surface to reduce rumble. Conduct periodic auditing of noise levels to ensure minimal potential impacts. Noise monitoring will be conducted utilizing handheld noise monitors at select locations identified on site map. Log of monitoring/incidences of non-compliance and rectification will be recorded and maintained. Drivers will be required to observe low speed wherever necessary and no blowing of horns. Construction activities utilizing heavy machinery work will be restricted between 8 AM – 6 PM. Advance warning to communities will be provided with respect to the timing of noisy activities. Construction vehicles and machinery will be maintained to a high standard to minimize emissions and noise. All construction workers / operators will use appropriate PPE, including ear defenders, when operating machinery. Implement CEMP sub-plans: - The Noise and Dust Control Plan - Traffic and Road Management Plan | Construction Contractor | CSC, PMU | During Construction at Active Construction Site |  |
|  | 2.4 | Occupational Health and Safety | During the construction phase, more workers will be employed to support project construction, to be housed on site in the construction camps. The key factors that will pose risks to worker health and safety are movement of heavy machinery and high volumes of traffic on site as well as any potential injuries resulting from electrical and civil works while developing the project infrastructure. | The project will demarcate exclusion and safety zones around any high-risk areas. The project will strictly require all workers wear personal protective equipment appropriate to their duties with signage at designated sites. Clinical assistance will be available to project personnel and medical emergency evacuation procedures will be developed. Fatigue will be monitored and managed during travel and during on site operations. Standard Operating Procedures (SOPs) for the safe handling, storage and transportation of associated facilities and materials. The project will strictly implement the management and monitoring measures as per EMP for water quality, waste management, air, health and safety, noise and vibration and transport measures. Implement CEMP sub-plan on Occupational Health and Safety (OHS) Plan. Provide sanitary facilities and wash areas, safe drinking water and garbage bins. Provide health assessments (health and fitness to workers once every two months). Periodic training will be provided to workers in all aspects of the ERP and OHS Coordinate with nearest hospital for arrangements in case of accidents. Provide first aid treatment for construction sites and camp. Assess safety risks and safety protocols (such as for electrical works, working at heights, etc.), and implement. Workers will be equipped with PPE, including proper safety clothes and protection gear/equipment to avoid accidents. Provide communication devices to designated site officers / engineers. Electrical safety risks will be assessed and safety protocols will be developed and implemented such as for electrical works, working at heights, etc. All works at height will be prohibited during night time, periods of fog and strong wind. All workers climbing towers will have a Safety Certificate. All towers, steel structures and equipment will be properly earthed and equipped with lightening protection. When testing electrical equipment, all unrelated works in the flagged zone – marked as danger zone- will be stopped and unrelated workers will leave the zone. Records of health assessments/incidents/accidents/near-miss/fatalities will be maintained. | Construction Contractor | CSC, PMU | During Construction at Active Construction Site |  |
|  | 2.5 | Community Health and Safety | Site preparation and civil works, movement of vehicles bringing equipment, materials, supplies and machinery may interfere with road crossings, flow of traffic and may cause an increase in traffic congestion. These activities may pose safety risks to the communities residing in close proximity to the work sites and general public. | Security fencing and patrols to prevent safety impacts from any construction activities undertaken during construction. Potential noise impacts will be reduced to acceptable levels through scheduled daytime construction, while dust generation will be managed through regular water spraying on major dust sources associated with the project. The project will strictly implement the management and monitoring measures as per EMP for water quality, waste management, air, health and safety, noise and vibration, and transportation; and sub-plans, including Emergency Preparedness and Response Plan, Community Health and Safety Plan, Stakeholder Engagement Plan, Traffic and Transport Management Plan and Waste Management Plan. Provide healthcare services for project employees with an appropriately staffed site clinic Effectively control the risks associated with accidents involving multiple vehicles, interactions between different types of vehicles with consideration for the volume and speed of traffic, and vehicle and person interactions. Obeying speed limits and apply additional caution by considering environmental conditions such as time of day, visibility, precipitation. Implement an effective site waste management strategy to prevent the incidence of vector-borne diseases. Develop a management strategy for worker accommodation in line with international/national standards to minimize spread of infectious disease and food-related illnesses. Adhere to strict schedule for completion of civil works and avoid prolonged construction and disturbance. Implement CEMP sub-plans. Community Health and Safety Plan. Traffic and Road Management Plan. Provide perimeter fencing to mitigate trespassing (fencing will be constructed with adequate ground clearance for passing of wild animals/other species at the solar plant site). Provide sufficient lights, clear warning signs and danger signals. Assign security personnel to prevent accidents, trespassing and pilferage. Warning signs and cones will be installed in and around the transmission tower site and along roads, with clearly marked danger zones. Safety flags and flag persons will be used, as needed. Record of incidents/accidents/near-miss/fatalities associated with the project will be maintained. Records of issues raised will be maintained in accordance with GRMs. Outreach to local communities to disseminate knowledge about safety and ERPs at or near solar plant will be provided via information disclosure and consultation activities, project information booklet (PIB) / frequently asked questions (FAQ) flyers. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.6 | Waste Generation | Impacts on resource use and impacts associated with disposal will arise from waste generated during site preparation and civil works. This includes generation of inert wastes e.g. spoil, biodegradable wastes e.g. cleared vegetation, construction debris, packaging waste, metal scrap, domestic waste. | Establish a covered onsite sorting and recycling area away from any drainage sources. Transport of recyclables, scrap, discarded equipment to depots, dedicated storage yards for resale or auction to authorized dealers. The broken or damaged solar panels will be sold to a solar panel specializing panel recycling. For other type of wastes, licensed companies will be hired to collect, transport and dispose of wastes at licensed dump facilities. Biodegradable waste such as cleared vegetation may be provided to local communities for use. Provide multiple waste containers at construction camps. Waste burning will be prohibited. No final waste disposal on site. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.7 | Vegetation impacts | The most significant impact to terrestrial habitats and flora associated with the project is expected to occur during the construction phase when habitats within the project footprint will be cleared. | Construction camps to be set up at least 400 m from any water source or drainage channels; Access to adjacent properties will be maintained, as necessary; Vegetation clearances will be strictly restricted to work sites; Use of herbicides/pesticides will be prohibited for vegetation clearing to prevent soil contamination; Excess spoil will be backfilled onsite or spread out in a manner that causes no disturbance to local drainage pattern; Implement the following CEMP sub-plans: Strictly implement the Spoils Disposal plan Drainage and Storm-water Management Plan Site Rehabilitation and Clean-up Plan after completion of civil works to preconstruction conditions, including vegetation planting to stabilize soil where it does not compromise the intended function. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.8 | Fauna impacts | No major loss to any fauna in the project area is expected. Any potential impact will largely be localized and restricted to the limited vegetation to be removed from the project site. | The project will design the solar plant layout to avoid most sensitive habitats, including maintaining the buffer zone for instance to keep habitats as far as possible. The project will avoid direct and indirect impacts to any areas of biodiversity sensitivity. Habitat clearance will be undertaken in a progressive and sensitive manner to enable priority fauna to move away from the area of works and to avoid isolating fauna in fragmented areas of habitat. Adverse impact to habitats, flora and fauna arising from fugitive dust will be minimized through the regular application of water to reduce dust levels in areas of biodiversity sensitivity (particularly during the dry season), the stabilization of landforms and the implementation of erosion prevention measures. Raise awareness with workforce and local villagers about importance of protecting forest resources and conservation important species and their habitats. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.9 | Biodiversity | Vegetated areas occurring within the project footprint will also be cleared. | Minimize vegetation clearing confined to the footprint of the solar plant area. No use of chemicals (pesticides/herbicides). Maintain hedges and field margins along solar plant perimeter. After completion of civil works, implement Site Rehabilitation and Clean Up Plan, including landscaping along the fenced perimeter of the solar plant, re-seeding most or all of the site with native species of plants. Provision for adequate ground clearance under fencing for allowing passage of animals/wild species to prevent habitat fragmentation. Restoring temporarily disturbed areas to pre-construction conditions. Trees below 3 m will not be cut or felled, minimum lopping and pruning will be undertaken as necessary. Any identified bat roosting sites will not be disturbed. Restoring temporary land use (used for material storage) to pre-construction conditions. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.10 | Interruption to existing utilities | - In order to reduce impacts on existing utilities (such as power outages), the solar plant construction works will be scheduled in a phased manner. All activities will be supervised by PMU Environment and Social Officer. | Implement Traffic and Road Management Plan. Follow planned transportation routes and delivery schedule. Any traffic detours to have danger and clearly visible warning signs as well as flag persons. Compliance with local speed limits vehicle load carrying capacity and other road regulations. Any damage to roads to be borne by Contractor(s). Record of incidents/accidents/near-miss/fatalities/road damage will be maintained. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.11 | Impacts from land clearing and topographical changes | - Land leveling and grading works may alter drainage patterns at the solar plant site that may result in surface erosion, generation of spoil and localized flooding. Impacts will also result from installation of equipment, installation of cable routes (above ground and/or underground) and commissioning of new equipment. | Clear demarcation of work sites, no encroachment outside the demarcated zone; Access to adjacent properties and agricultural land will be maintained, as necessary; Construction camp/ workers accommodation set up at least 400 m from natural streams or any other water source at site. Use of herbicides/pesticides will be prohibited for vegetation clearing to prevent soil contamination; Implement engineering and biological measures to prevent surface erosion such as provision of silt traps or sowing soil-binding grass, as needed; Restore loose soil from foundations through ramming, if required; Implement the following CEMP sub-plans; Spoils Disposal plan; Drainage and Storm-water Management Plan - Also, site rehabilitation and clean-up plan will be implemented after completion of civil works to restore works site to pre-construction conditions, including landscaping along the fenced perimeter, maintain hedges and field margins to reduce visual and dust impact, re-seeding most or all of the site with native plant species (soil binding grass) to stabilize the soil and restore habitat as far as possible. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.12 | Groundwater quality | - There are some potential effluences for groundwater contamination, i.e. contamination of soil and groundwater with oil, grease and hydrocarbon pollution associated with vehicles and heavy machinery and with pathogen and nutrient pollution associated with human activity, accommodation, sewage treatment and putrescible waste. | All chemicals and hydrocarbons will be handled and stored appropriately according to the EMP and associated standard operation procedures, with adequate bunding, hardstand preparation including lining, concreting or compaction, as well as inspection, auditing and contingency measures. Septic tank will be installed to keep wastewater from toilet and kitchen to prevent pollution to groundwater. Groundwater quality is to be monitored using groundwater monitoring bores installed up and down gradient of potential contamination zones. Important quality parameters to be monitored as per applicable guidelines. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.13 | Soil quality | - Vegetation will be cleared in preparation for the construction of project components. Measures are therefore required to minimize soil erosion (by rain and wind) and prevent sediment release. Soil compaction within active construction areas is likely due to heavy traffic passage and construction activities. | The project has been designed to minimize long-term negative impacts to soil in the project site by recovering topsoil stripped from clearing and construction activities and subsequently for progressive and final rehabilitation. Land clearing is to be limited to the immediate project site required for development to the extent practical. Activities involving chemicals or hydrocarbons (including storage of broken and redundant solar panels) are to be restricted to designated areas that have been prepared with appropriate bunding, a concrete or compacted earth base, and fully contained by drainage controls. An appropriate sanitation system is to be provided to collect all waste and sewage for proper storage and disposal; Strict controls on the use of sanitation facilities will be implemented through education and enforcement. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.14 | Local Economic Development | Despite the generally positive impacts the Project will have on economic development and employment, there is a risk of inequitable employment, training and procurement opportunities or inadequate communication leading to community dissatisfaction. | The project will implement the preferential local recruitment and procurement policies in place. Implementation of social management and monitoring as per the EMP and sub-plans: Stakeholder Engagement Plan and Grievance Mechanism. Provide a grievance mechanism for the Project. | Construction Contractor | CSC, PMU | During Construction |  |
|  | 2.15 | Other Risks | - Forest/vegetation catching fire in the dry season are risks that need to be accounted for in the planning, design and management of the Project. | The project will construct a fire path break around the project boundary to prevent forest/vegetation fires from reaching the solar plant and other facilities. The project will maintain equipment ready for fire-fighting (e.g. water trucks with water cannons) and train personnel to respond to forest fires in the project area. Implement and provide inductions to all staff on the Emergency Preparedness and Response Procedures and measures described in the EMP. Ongoing engagement with government authorities and consultation with local communities will form part of a proactive approach to prevent or reduce risk of natural hazards to project activities and communities. | Construction Contractor | CSC, PMU | During Construction |  |
| Operational Phase | 3.1 | Workforce Organization and Orientation | The plant workforce will possess limited capacities in regard to national and ADB specific safeguard requirements. | - The Plant operator will conduct briefing and orientation for all project staff, workers and its turnkey contractor/sub-contractors (facility operators): - EMP, GRM, Information disclosure and meaningful consultation, environmental monitoring and reporting requirements. - Responsibilities of the contractors in implementing SOPs and monitoring environmental compliance with environmental performance indicators and EMP. - Responsibility of the Contractors in engaging with affected persons for project GRM. - Record and maintain briefing and orientation events log with duration and list of attendees. | WASA, Rawalpindi | Env. Expert/ EPAs | During Operations |  |
|  | 3.2 | Water usage for panel cleaning | There could be a potential shortage of water in the project area and/or lowering of water table due to cleaning of solar panels. | - Maintain storm water retention pond for use for PV cleaning. - Periodic cleaning of storm water retention pond to remove debris. - No use of chemicals/detergents for cleaning purposes. | WASA, Rawalpindi | Env. Expert/ EPAs | During Operations |  |
|  | 3.3 | Waste generation | Although not expected, yet certain electrical components might need to be retired and replaced along with potential lack of proper handling might result in breakage of panels etc. along with generation of domestic waste. | - Utilize licensed vendors/suppliers for collection, transportation and disposal of broken/unused PV panels. - Store broken/redundant panels in covered bunded areas. - Implement SOP for Materials Management and Waste Management. | WASA, Rawalpindi | Env. Expert/ EPAs | During Operations |  |
|  | 3.4 | Ecological impacts | - The large area of PV panels in the solar plant may create a ‘lake effect’ which could attract birds that mistake the solar panels for a water body and the hot panels could kill or seriously injure birds, which attempt to land on them especially the young inexperienced ones. | - Visual deterrents and flight diverters will be provided at the solar plant, particularly close to the storm water retention pond. | WASA, Rawalpindi | Env. Expert/ EPAs | During Operations |  |
|  | 3.5 | Occupational Health and Safety | - Contractors will need to comply with the relevant safety measures required by law and as per international good practices. | - Implement SOP for Occupational Health and Safety. - Compliance to relevant national electrical safety standards. - Provide and maintain signage as per Institute of Electrical and Electronics Engineers standards at dangerous places for warning of electrical hazards. - Provide and maintain health assessment by a competent medical practitioner for all workers. - Provide periodic training to all workers with access to electrical and hazardous conditions and workers will be certified to work on site. - Provide and maintain workers appropriate PPE. - Equipment and tools will be inspected before use to ensure proper and safe operation. - Appropriate grounding and deactivation of live power equipment during maintenance work or if working in close proximity to the equipment; provision of lightning arrestors as appropriate. - Record of health assessments, incidents, accidents, near-miss, fatalities will be maintained. - EMF levels expected to be below the limits set by International Commission on Non-Ionizing Radiation Protection which is 4.17 kV/m for electric field and 833 miliGauss for magnetic field; - Periodic EMF monitoring using handheld devices to be conducted, as required. | WASA, Rawalpindi | Env. Expert/ EPAs | During Operations |  |

### Environmental Management Costs for Solar Installation

The Table 7.12 below provides cost estimates for ‘Pre-Construction phase’ monitoring while Table 7.13 provides cost estimates for ‘Construction phase’ monitoring of key environmental parameters.

The costs associated with implementation of the EMP and the necessary mitigation measures are provided as Table 7.14 below. Cost of Capacity Development and Training Programme for Project Contractor(s) is given below in Table 7.15.

Table 7.12: Cost Estimates for ‘Pre-Construction Phase’ – Solar

| Monitoring Component | Parameters | Quantity | Amount PKR | Details |
| --- | --- | --- | --- | --- |
| Air Quality | CO, NO2, SO2, O3, PM10 | 1 (Once only at 1 location) | 50,000 | 1 reading @ PKR 50,000 per sample |
| Noise Levels | dB(A) | 1 (Once only at 1 location) | 50,000 | 1 reading @ PKR 50,000 per sample |
| Ground Water Quality | As per NEQS | 1 (Once only at 1 location) | 50,000 | 1 reading @ PKR 50,000 per sample |
| Total |  |  | 150,000 |  |
| Contingencies |  |  | 15,000 | 10% of monitoring cost |
| Total (PKR) |  |  | 165,000 |  |

Table 7.13: Cost Estimates for ‘Construction Phase’ Environmental Monitoring - Solar

| Monitoring Component | Parameters | Quantity | Amount PKR | Details |
| --- | --- | --- | --- | --- |
| Wastewater Quality | PEQS / NEQS / WHO | 3 (Monthly at 1 location) over 3 month construction period. | 150,000 | 3 readings @ PKR 50,000 per sample |
| Drinking water quality | PEQS / NEQS / WHO | 3 (Monthly at 1 location) over 3 month construction period. | 150,000 | 3 readings @ PKR 50,000 per sample |
| Air Quality | CO, NO2, PM10,PM2.5 | 3 (Monthly at 1 location) over 3 month construction period. | 150,000 | 3 readings @ PKR 50,000 per sample |
| Noise Levels | dB(A) | 3 (Monthly at 1 location) over 3 month construction period. | 150,000 | 3 readings @ PKR 50,000 per sample |
| Total |  |  |  | 600,000 |
| Contingencies |  |  | 60,000 | 10% of monitoring cost |
| Total (PKR) |  |  | 660,000 |  |

Table 7.14: Estimated Costs for EMP Implementation - Solar

| Item | Sub-Item | Estimated Total Cost (PKR) |
| --- | --- | --- |
| Staff, audit and monitoring cost | 1 person for 3 months. (@ 100,000 per month) | 300,000 |
| Monitoring Activities | Provided separately in Tables 7.12 and 7.13. | 825,000 |
| Mitigation Measures as below: | As prescribed under EMP and IEE. | 600,000 |
| (i) Water sprinkling | To suppress dust emissions. | 300,000 |
| (ii) Hazardous and non-hazardous collection & disposal | From construction sites (based on initial estimates). | 300,000 |
| Total |  | 1,725,000 |
| Contingencies | 10% of EMP implementation cost. | 172,500 |
| Total Estimated Cost (PKR) |  | 1,897,500 |

Table 7.15: Cost of Capacity Development and Training Programme for Project Contractor(s) - Solar

| Provided by | Organized by | Contents | No. of training events | Duration | Cost (PKR) |
| --- | --- | --- | --- | --- | --- |
| Pre-construction Phase Monitoring Consultants/Organizations offering specialized services in environmental management and monitoring. | CSC & PMU | Short seminars and courses on: Environmental Management Plan and Environmental Monitoring Plan | Two seminars for Contractor management staff and project staff | 1 day | 200,000/Training |
| Construction Phase Monitoring Consultants/Organizations offering specialized services in environmental management and monitoring. | CSC & PMU | Short seminars on Environmental risks associated with construction phase. Development of Environmental Performance Indicators Occupational Health and Safety (OHS) issues | Two seminars for Contractor management staff and project staff dealing in environment and social issues | 1 day | 200,000/Training |
| Total |  |  | 400,000 (PKR 0.4 million) |  |  |

### Performance Indicators

For evaluating the performance of the environmental management and monitoring plan, performance indicators are identified for the efficient and timely implementation of measures/actions proposed in EMP. The indicators are defined both for the implementation phase and for the operation phase. QA/QC Consultant will be responsible for compiling the information on these indicators and report to WASA/PMU.

To measure the overall environmental performance of the project, a list of performance indicators is given below:

The number of inspections carried out by environmental team of Construction Supervision Consultant (CSC) per month.

The number of non-compliances observed by CSC.

Availability of environmental, and OHS specialists in CSC .

Availability of environmental and OHS specialists with Contractors.

Timely reporting of documents (as defined in EMMP and monitoring plan).

Number of training imparted to stakeholders/other capacity building initiatives.

Number of grievances received.

Number of grievances resolved.

Number of constructions related accidents.

## Public Consultation and Information Disclosure

This section describes the process and outcomes of the consultations carried out with various groups of stakeholders as part of the environmental and social assessment. It includes a brief discussion on the concerns expressed by the stakeholders during the consultation meetings and responses provided in order to address the concerns through necessary mitigation measures.

The specific objectives of the consultation were: (i) obtaining local and indigenous knowledge about the environment and people living in the project area; (ii) interaction with the project affected population and other stakeholders for the collection of primary and secondary data on environment and people; and (iii) engaging stakeholders for maximization of the project benefits.

The disclosure of the enhancement project in advance and subsequent consultation with stakeholders has advantages in the environmental assessment and mitigation of impacts. Public consultation can also provide a conduit for the improvement of the project implementation to better serve the stakeholders.

Information disclosure through public consultation was carried out. Short brief of project activities was developed and shared with stakeholders.

For disclosure of information, PMU will submit the updated IEE report to ADB for uploading the IEE on its website before final approval from ADB.

The implementing agency (RWASA) will prepared brochures related to project containing basic information of the project, construction activities, traffic diversion routes, time of construction etc. for awareness of general public. The Executive Summary of the IEE report will be translated into Urdu for better understanding of people. The translated ES will be placed at different locations including website of PMU, office of PMU & CIU and RWASA.

The PMU / CIU will provide relevant safeguards information in a timely manner, in an accessible place and in a form and language understandable to affected people and other stakeholders. For illiterate people, other suitable communication methods will be used. Disclosure will follow ADB’s Public Communication Policy, 2011.

The environmental assessment process under the Pakistan Environmental Protection Act only requires the disclosure to the public after the statutory IEE / EIA has been accepted by the relevant EPA (in this case Federal EPA and Punjab EPA) to be in strict adherence to the rules. In this IEE the consultation process was performed to satisfy the ADB requirements.

The public consultation process was carried out by the EDCM's Social and Environment teams in December 2022. Mainly key informants were consulted for these meetings which were carried out in an open and frank atmosphere conducive to appreciation of the basic elements of the project and dissemination of information on beneficial and adverse impacts and mitigation for adverse impacts.

The methodology of Focus Group Discussion (FGD) and individual consultation was used for public consultation. Total 10 FGDs were conducted with people of project area. The consultation was carried out along the route of proposed conductance main and water works of KSS. Project Affected Persons (PAP) were highlighted during preliminary visit of the site. People from UC-10, UC-11 and UC-12 were consulted, especially near the proposed construction sites for underground and overhead water reservoirs where population density is little high and the impact due to construction activity is envisaged. As the project will be executed within existing ROW and the number of APs (in terms of displacement and involuntary resettlement) are negligible, therefore, random consultation was carried out. With reference to the consultation of Government Stakeholders, all the relevant departments were invited for consultation meeting at DC Office.

Total 109 men and women participated in these 8 FGDs and individual consultation out of 109 participants 27 (24.8%) are women and 82 (75.2%) are men. Information on positive and negative aspects associated with constructional and operational stage and proper mitigation of adverse impacts were shared at these consultations. Questionnaire used for conducting FGDs and Surveys is attached as Annexure-XIII. The attendance sheets of these FGDs are attached as Annexure-XIV.

### Identification of Stakeholders

Stakeholders are considered to be individuals or organizations which have an interest in the proposed project or knowledge that will provide insight into issues or affect decision making related to the proposed project. On the basis of interest and role criteria there are two types of stakeholders for the proposed project as described below.

#### Primary Stakeholders

The primary stakeholders are primarily the Project Affected Persons (PAPs) and general public including women residing in the project area - for example, people living in the project area particularly those affected by the footprint of proposed project Augmentation of Surface Water Supply for Khayaban-e-Sir Syed from Khanpur Dam and Formation of DMAs. These are the people who are directly exposed to the project’s impacts though in most cases they may not be receiving any direct benefit from the project like people of Sector I-10 because they will not be getting share of water from this project.

#### Secondary Stakeholders

The secondary stakeholders are typically general public including women residing around the COI These also include institutional stakeholders – for instance, related government department/agencies, local government, and organizations that may not be directly affected by the project; however, they may influence the project and its design. They include project proponent, execution agency, other concerned departments that may have a role during various phases of the project, regulatory agencies such as EPA, other relevant departments such as Forest and Wildlife, non-governmental organizations (NGOs), the broader interested communities including academia and journalists, and general public.

### Consultation Process

As part of the present environmental and social assessment, detailed consultations were carried out through meetings and focus group discussions (FGDs) with the communities, including women in the project area. The consultation was carried out along the route of New Conductance Main, replacement of outlived pipelines and existing water works of KSS. Project Affected Persons (PAP) were highlighted during preliminary visit of the site. People from Sector-10, Islamabad, IJP Road, Sector 1, 2 and 3 of KSS were consulted, where population density is high. As the project will be executed within the existing ROW and there are no APs (in terms of displacement and involuntary resettlement), therefore, random consultation was carried out. With reference to the consultation of Government Stakeholders, all the relevant departments were invited for consultation meeting at DC Office, Rawalpindi. Details of this consultation process are described below;

### Consultation with Project Affected Peoples

The consultation with project affected peoples was carried out during the site visit. All data of group discussion, individual discussion and FGDs was recorded. Total 9 FGDs were conducted for the proposed project of Augmentation of Surface Water Supply at KSS including the consultative session with Government Department. Further, public consultation was carried out with individuals from project area who have direct impacts of the project and falls under project affected people. Total 109 participants including 82 (75.2%) men and 27 (24.8%) women were consulted in these FGDs and individual consultations. Details of stakeholder consultations are given in Table 8.1 and Table 8.2. Pictorial evidence of the consultation is given as Figure 8.1.

### Issues, Concerns and Findings of the Focal Group Discussion

The people living in the vicinity of the project area had some concerns related to the water supply system of KSS. The existing water supply is not sufficient to cater the needs of KSS in terms of water demand.

People told that water supply from current water supply system sometimes had foul smell / odor of wastewater and they also have to face stomach problem.

The internal roads of KSS have low traffic capacity and special care shall be given to traffic management during construction phase of the project.

The handling of construction shall be given due care and no storage of construction material on road shall be allowed.

Disposal of solid waste shall be done in scientific manner and no dumping on open spaces and roads shall be allowed

Preference regarding provision of jobs shall be given to local community

Special care shall be taken at school time and prayer time.

Use of low noise producing machinery shall be ensured.

Residents in the site’s vicinity were assured that the Augmentation of water supply in KSS is proposed for the soul purpose to provide enough water supply to the residents of Khyaban-e-Sir Syed so they can fulfil their water needs just enough.

Table 8.1 provide the details about observation/response of individual stakeholder. Table 8.2 provide detail about the responses of FGDs. It is important to mention here that comments and reservations of respondents have been duel taken care while designing mitigation measures of the project.

Table 8.1: Consultation with Project Affected Peoples

| Sr.# | Date | Location/ Venue/ Address |  | Specifics of Participant | Feedback/ Concerns | EDCM and Social Environment Advisors Reply |
| --- | --- | --- | --- | --- | --- | --- |
| Public Consultation |  |  |  |  |  |  |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Naeem ur Rehman [Educationist] |  | I hope the technical staff and project management have sufficiently skills to maintain an orderly workplace both during and after construction. | EDCM and Social Environment Advisors will ensure that skilled labourers are hired in accordance with the correct protocols. |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Maqsood Abbasi [Driver] |  | The drinking water pipelines and sewerage pipeline lines are intertwined; the following project will be helpful to solve this issue. | - |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Ibrahim Javed [Shopkeeper] |  | No issue regarding this project. It is requested that the construction be properly monitored to prevent any disruption. | EDCM and Social Environment Advisors will ensure to follow all of the construction protocols/PEQs. |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Orangzaib [Govt. Employ] |  | As stated, this project will result in an increase in water capacity. You are urged to maintain the water quality. | EDCM and Social Environment Advisors will make sure to maintain the water quality being provided. |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Bilal Muhammad [Fruit Mundi] |  | I have no issue as it seems to be a useful project. Try to fix the traffic problems while the project is in its construction phase. | EDCM and Social Environment Advisors will make sure to reduce the traffic related concerns of locals. |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Ishfaq Ahmad [Shopkeeper] |  | It is requested to handle the construction work properly to avoid any inconvenience in our daily routine. | EDCM and Social Environment Advisors will make sure that the construction work is handled expertly. |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Tariq Zaman [Businessman] |  | Satisfied with the project in provision of water, both in terms of quality and quantity. |  |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mam Palwisha [Housewife] |  | The project appears to be helpful in resolving the water situation. Problems with the flow of traffic shouldn't arise. | EDCM and Social Environment Advisors will make sure to minimize the traffic related concerns of locals. |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Syed Riaz Naqvi [Shopkeeper] |  | No issue. Handle the construction work properly so that it doesn't interfere with our work. | EDCM and Social Environment Advisors will make sure to handle the construction work properly to avoid disturbing the locals working hours. |
|  | 01-12-2022 | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | Mr. Syed Hamid Ali Shah [Retired Police Officer] |  | Please be considerate and control traffic during the construction work. Keep the road clear and finish the construction as soon as you can. | EDCM and Social Environment Advisors will make sure to eliminate the traffic related concerns of locals. |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Aslam Munir [Painter] |  | Water depletion is one of the biggest issue, we are facing in our area. It seems that the stated project will be helpful to solve this problem. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Afzal Munir [Unemployed] |  | We have to face water shortage in summer in our homes. Hopefully this project will be helpful to solve this issue. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Adnan Mubarak [Shopkeeper] |  | I have no issue regarding this project. Construction work should not be a hindrance in our daily lives. | EDCM and Social Environment Advisors will make sure to manage the construction work effectively to prevent interfering with the locals working hours. |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Naveed Shahzad [Karyana Store] |  | No objection. It is requested to handle the construction materials properly and work skilfully. | EDCM and Social Environmental Advisor will ensure that all construction regulations are followed. |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Habib Gul [Scrap Dealer] |  | No objection, the following project will be helpful to provide clean water. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Ahmad Khan [Clerk] |  | It seems a decent project to improve water quality in the area so I have no issue regarding this project. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Saad Ghani Shah [Shopkeeper] |  | No issue, it’s a good initiative for local residents by increasing their employment opportunities and improving the quality of water being provided. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Dil Roz [Mechanic] |  | No objection and satisfied with this project. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Taj Pervaiz [Mechanic] |  | We are experiencing shortage of water in our homes. Hopefully, this initiative will assist in resolving this problem. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Akbar Ibadat [Labour] |  | I have no objection regard this project. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Wajahat Ali [Butcher Shop] |  | It seems a good initiative to fulfil the amount of water required in the area. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Raza Taj [Karyana Store] |  | I have no objections to this initiative as quality of water will increase. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Zulfiqar Ali [Teacher] |  | No issue regarding the project as we are facing both, water quantity and quality issue in our area that is leading us to various diseases. The stated project will be helpful to provide and managing the life better with provision of clean water. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 11, Rawalpindi | Mam. Azra Islam [Housewife] |  | Water amount depleted in summer. Such projects should be implemented, as water is a basic necessity. The project will be helpful to solve water issues in summer. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 11, Rawalpindi | Mam. Mehmoona [Housewife] |  | This is a good project to solve water crisis issues. I have no objection. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 11, Rawalpindi | Mam Shehnaz [Housewife] |  | The project will be helpful to conserve water. Water issues in our household will be resolve so I am satisfied. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 11, Rawalpindi | Mr. Saeed Ahmad [Shopkeeper] |  | The project seems good to save and provide water for coming generation and future so I am satisfied to start this project. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Sajjad Haider [Principle] |  | No issue regarding this project. The project seems beneficial to improve both water quality and quantity. |  |
|  | 02-12-2022 | Khyaban-e-Sir Sayyad, UC 10, Rawalpindi | Mr. Mugheez Abbasi [Shopkeeper] |  | I have no issue as stated the said project will be helpful to increase the source of employment. |  |
|  | 02-12-2022 | PHA Muslim Park, UC 12, Rawalpindi | Mr. M. Afzal Umar [Security guard] |  | I have no issue regarding this project. I hope this project may be helpful in providing clean water in our area. |  |
|  | 02-12-2022 | PHA Muslim Park, UC 12, Rawalpindi. | Mr. Yasir Khan [Dealer] |  | Satisfied with the project. Proper monitoring of pipelines and storage tanks should be carried out on a weekly basis, at least. | EDCM and Social Environmental Consultant will ensure routine maintenance of the pipelines and storage tanks on a regular basis. |
|  | 02-12-2022 | PHA Muslim Park, UC 12, Rawalpindi. | Mr. Naseer Salman [Businessman] |  | Project delays are almost expected in construction so it is requested to work on schedule. | EDCM and Social Environmental Consultant will make sure to regulate the construction activities with appropriate procedures. |
|  | 02-12-2022 | PHA Muslim Park, UC 12, Rawalpindi. | Mr. M. Iqbal [Retired Army Officer] |  | If there are any employment opportunities with the aforementioned project, locals should be prioritized. Every home ought to have access to clean water. | EDCM and Social Environmental Advisor will make sure to give preference to locals for any employment or job facilities. Access to clean water will be available at an affordable price. |
|  | 31-12-2022 | I-10, Mandi morr, Islamabad | Mr. Maqbool Ahmad [Bykea Service Employe] |  | It seems a good project; such projects should be implemented. |  |
|  | 31-12-2022 | Bangish Colony, Rawalpindi | Mr. Abdul Rehman [Bykea Service Employe] |  | With the construction of storage tanks and wide pipelines, the stated project appears to be decent. It seems like water issues in the area will be resolved soon. |  |
|  | 31-12-2022 | Bangish Colony, Rawalpindi | Mr. Ahmad Sarwar [Bykea Service Employe] |  | I have no issue regarding this project; just make sure to manage the construction work properly. | EDCM and Social Environmental Advisor will make sure to skilfully manage the construction activities. |
|  | 31-12-2022 | Bangish Colony, Rawalpindi | Mam Jaweria [Housewife] |  | Timing of transport should be managed during construction in such a manner that it does not affect our earnings. | EDCM and Social Environmental consultant will make sure transit schedules be restricted to prevent interfering with the locals normal schedule. |
|  | 31-12-2022 | Dhok Kasu, Rawalpindi | Mr. Rehan Khan [Labour] |  | No objection and satisfied with the implementation of this project. Locals should be preferred in provision of job opportunities. | EDCM and Social Environmental Advisor will ensure to prefer locals for jobs facilities available, if any. |
|  | 31-12-2022 | Dhok Kasu, Rawalpindi | Mr. Rahat [Bykea Service Employe] |  | As the storage tank enables fast access to water when we need it, so I am very satisfied with the implementation of this project. |  |
|  | 31-12-2022 | Dhok Kasu, Rawalpindi | Mam Dua [Teacher] |  | I have no issue. More water will be available for green belts, parks and houses. |  |
|  | 31-12-2022 | Sector-3, UC 12, Rawalpindi | Mr. M. Aslam [Fruit Shop] |  | Project delays are almost expected in construction so it is requested to work on schedule. | EDCM and Social Environmental Advisor will ensure to complete the construction work on schedule. |
|  | 31-12-2022 | Sector-3, UC 12, Rawalpindi | Mr. Rauf Raja [Labour] |  | I have no issue regarding this project. This project will be helpful to solve the drinking water issues in the area. |  |
|  | 31-12-2022 | Sector-3, UC 12, Rawalpindi | Mam Hafsa [Student] |  | Try to solve construction related issues such as road blockages, dust, and noise of machinery. | EDCM and Social Environmental Advisor will ensure to minimize the traffic related concerns of locals. |
|  | 31-12-2022 | I-10, Service road, Islamabad | Mr. M Awis [Labour] |  | No issue regarding this project. It is requested that the construction must be properly monitored to prevent any disruption. | EDCM and Social Environmental Advisor will make sure that construction work is handled skilfully. |
|  | 31-12-2022 | I-10, Service road, Islamabad | Mam. Sania [Housewife] |  | Satisfied with the implementation of this project. |  |
|  | 31-12-2022 | I-10, Service road, Islamabad | Mam Tabeer [Housewife] |  | The big problem that we faced during construction are the lack of skilled workers, they do not only delay the work but sometimes also responsible to destroy the work so hire skilful person to complete the work without any obstacle. | EDCM and Social Environmental Advisor will ensure to hire skilful workers. |
|  | 31-12-2022 | I-10, Mandi morr, Islamabad | Mr. Aziz ur Rehman [Labour] |  | It seems a good project; such projects should be implemented. Preference should be given to local workforce under this project. |  |

Table 8.2: Response of Stakeholder during FGDs

| Sr. # | Name (FGD) | Location |  | Feedback/Concerns | Reply of EDCM' Environmental and Social Experts |
| --- | --- | --- | --- | --- | --- |
| FGD of KSS - UC 12 |  |  |  |  |  |
| 1 | Miss. Muskan [Teacher] | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi |  | Everyone individual in the FGD has no objections to this project because it will assist provide clean water. They did make a number of suggestions. It was asked that the drinking water and sewage pipelines be handled properly. Leakages should be prevented by routine monitoring. Construction work should be conducted carefully to avoid any potentially hazardous scenarios. To avoid any harmful situations, proper material monitoring should be done during the construction process. Construction projects can fail because of a lack of skilled labourers. Hire knowledgeable individuals to address this problem. In order to prevent pollution-related problems, construction activities must be managed appropriately. Overall, they view the stated project as a decent project for the provision of quality water. They anticipate that the construction work will be handled expertly. They also anticipate that the proposed project will be useful in resolving the problem with the drinking water and sewage pipelines. | The Social Environmental Advisor and EDCM will make sure that personnel are employed in accordance with the necessary regulations and legislation. To prevent any accidents, they will make sure to control the construction activities in standard ways. For any employment facilities, EDCM and Social Environmental Advisor will also make sure to give precedence to locals. In order to resolve construction-related issues with dust and noise, EDCM and Social Environmental Advisor will obey the right procedures. They'll also make sure that clean water is provided through the appropriate channels. |
| 2 | Miss. Nagina [Teacher] |  |  |  |  |
| 3 | Miss. Tahreem [Teacher] |  |  |  |  |
| 4 | Miss. Mahmoona [Teacher] |  |  |  |  |
| 5 | Miss. Hira Fatima [Teacher] |  |  |  |  |
| 6 | Mam Saima [Teacher] |  |  |  |  |
| 7 | Mam Aminah [Teacher] |  |  |  |  |
| 8 | Miss. Minahil [Teacher] |  |  |  |  |
|  | Miss. Sidra [Teacher] |  |  |  |  |
|  | Miss. Maham [Teacher] |  |  |  |  |
|  | Miss. Rabia [Teacher] |  |  |  |  |
| FGD OF KSS Govt Girls High School |  |  |  |  |  |
|  | Mam Robia Arif [Teacher] | Khyaban-e-Sir Sayyad, UC 11, Rawalpindi |  | They approved of the project's implementation as long as the correct procedures were followed to avoid pollution while construction was taking place. They view the expansion of the current project favorably because it will help the community and further tackle its water-related problems. It was suggested that the water tanks and pipelines be adequately inspected. | The EDCM and Social Environmental Assistants will ensure that construction-related difficulties, i.e., dust and noise, are handled. The EDCM and Social Environmental Advisor will ensure that the proper procedures are followed in order to provide clean water. |
|  | Mam Jabeen Akhtar [Teacher] |  |  |  |  |
|  | Mam Shahida Ameer [Teacher] |  |  |  |  |
| FGD of KSS 1 (UC 12) |  |  |  |  |  |
|  | Mr. Syed Khizar Kazmi [Student] | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi |  | There are no objections from anyone in the FGD to this project because it would help deliver clean water. They did offer a few recommendations. They advised making sure the construction activity was appropriately managed. They were happy with the way this job was carried out because the storage tank makes it easy to access water quickly when needed. The drinking water problems in the area will be helped by this project. They believe that the project will help to improve the standard of living. It was asked that the timing of transportation during construction be managed so as not to interfere with their earning hours. Try to find solutions for construction-related problems like traffic jams, dust, and machine noise. They were pleased that there will be more water available for homes, parks, and green belts. They were in Content with the project's provision of clean water. The main issue we encountered during construction was a lack of skilled workers, who not only caused delays but occasionally even caused damage to the work, therefore they suggested to hire skilled people to get the job done without any problems. They anticipate that the construction work will be handled expertly. | EDCM and Social Environmental Advisors ensure the provision of clean water to everyone. To manage the traffic problems, a proper schedule will be in place. Additionally, proper construction-related procedures will be followed. In order to maintain workplace organization, the EDCM, and Social Environmental Advisor will ensure that qualified staff is hired. |
|  | Mr. Raja M. Awais [Student] |  |  |  |  |
|  | Mr. Faheem [Student] |  |  |  |  |
|  | Mr. Abdul Latif [Driver] |  |  |  |  |
|  | Mr. Asim Shah [Student] |  |  |  |  |
|  | Mr. Adnan Shahid [Stationary Shop] |  |  |  |  |
|  | Mr. Shoaib Shah [Businessman] |  |  |  |  |
|  | Mr. Raza Ali [Student] |  |  |  |  |
|  | Mr. M. Zia [Student] |  |  |  |  |
|  | Mr. Chaudhary Amar [Student] |  |  |  |  |
| FGD of CDA Stop |  |  |  |  |  |
|  | Mr. Syed Nazim Hussain [Taxi Driver] | Eid Gah, UC 10, Rawalpindi |  | They seem to be in content with the project as such projects are the need of time. It was advised that the construction activities be properly watched to prevent disturbance, especially during their working hours. They also anticipate that the storage tanks being built, will be useful during the months of drought. During construction, it was advised to handle the materials properly. Follow compliance guidelines to finish the construction on schedule. It was also requested to provide job opportunities to locals. It was also being suggested to keep a close check on the tools being used; if they begin to malfunction suddenly, replace them immediately to stop further damage after construction. | The Social Environmental Advisor and EDCM will make sure to regulate construction operations using the appropriate procedures in order to prevent any mishaps. EDCM and Social Environmental Advisor will make sure to give locals preference whenever an employment opportunity arises. The Social Environmental Advisor and EDCM will ensure that proper protocols are followed during construction activities. |
|  | Mr. M. Shakeel [Tea Stall] | CDA Stop, Rawalpindi |  |  |  |
|  | Mr. Izhan Ullah [Masoon] |  |  |  |  |
|  | Mr. Kalam [Masoon] |  |  |  |  |
|  | Mr. Swab [Masoon] |  |  |  |  |
|  | Mr. Rahab [Masoon] |  |  |  |  |
|  | Mr. Amir [Masoon] |  |  |  |  |
|  | Mr. Imran Khan [Masoon] |  |  |  |  |
| FGD of I-10 |  |  |  |  |  |
|  | Mr. Junaid Ahmed [Student] | I-10 Street Point, Islamabad |  | They were happy with this idea since it will help increase the standard of living. The main issue they encountered during construction was a lack of skilled workers, who not only caused delays but occasionally even caused damage to the work, therefore they suggested to hire skilled people to get the job done without any problems. They don't object because the project seems to be worthwhile. They also suggested to resolve the traffic issues. | EDCM and Social Environmental Advisor will make sure that qualified staff is hired in order to preserve workplace organization. There will be an appropriate schedule in place to handle traffic issues. |
|  | Mr. Abdul Wahid [Masoon] |  |  |  |  |
|  | Mr. Waheed Ullah [Labour] |  |  |  |  |
| FGD of Sabzi Mandi I-10 |  |  |  |  |  |
|  | Mr. M. Anwar [Vegetable Seller] |  | I-10 Sabzi Mandi, Islamabad | For cleaning and bathing, they are using bore water. They must contend with a water shortage in the summer. They have no objections to the project's development and construction because it will help to tackle this problem. As previously said, this project will improve the amount of water available. It was suggested to preserve the water's quality. It was requested to efficiently oversee construction work. Project mentioned shouldn’t have an impact on our daily lives. | EDCM and Social Environmental Advisor will make sure to adhere to the correct processes for resolving construction-related problems and also ensure the provision of high-quality water. |
|  | Mr. Muqallam Shah [Vegetable Seller] |  |  |  |  |
|  | Mr. Sher Muhammad [Vegetable Seller] |  |  |  |  |
|  | Mr. Jamshaid [Vegetable Seller] |  |  |  |  |
|  | Mr. Noor Agha [Vegetable Seller] |  |  |  |  |
| FGD UC-12 |  |  |  |  |  |
|  | Mr. Orangzaib [Govt. Employ] |  | Khyaban-e-Sir Sayyad, UC 12, Rawalpindi | They are satisfied with the project in provision of water, both in terms of quality and quantity. As stated, this project will result to increase in water capacity. They use bore water for bathing and washing. They suffer a water shortage with in summertime. They were happy with the project's development and construction as it will help to fix this problem. It was requested to complete the construction work on schedule. | EDCM and Social Environmental advisors will make sure to complete the construction work on schedule to avoid disturbing the locals. |
|  | Mr. Tariq Zaman [Businessman] |  |  |  |  |
|  | Mr. Raza Zahoor [Property Dealer] |  |  |  |  |
|  | Mr. Seraj [Driver] |  |  |  |  |

Figure 8.1 Photographs of Stakeholder Consultation

| EDCM E&S Safeguard team in consultation with stakeholders at Government Model School, KSS | EDCM E&S Safeguard team in consultation with stakeholders at Private School, KSS |
| --- | --- |
| EDCM E&S Safeguard team in consultation with stakeholders at Government Girls High School, KSS | EDCM E&S Safeguard team in consultation with PEP, KSS |
| EDCM E&S Safeguard team in consultation with stakeholders at KSS | EDCM E&S Safeguard team in consultation with stakeholders at KSS |
| EDCM Social Safeguard team in consultation with stakeholders | EDCM Social Safeguard team in consultation with stakeholders |
| EDCM Social Safeguard team in consultation with stakeholders at Government School in KSS | EDCM Social Safeguard team in consultation with stakeholders at private school in KSS |
| EDCM Social Safeguard team in consultation with at I-10 Islamabad | EDCM Social Safeguard team in consultation with PEP at I-10 Islamabad |

### Consultation with Official Stakeholders

As part of environmental assessment, detailed meetings were held with the institutional stakeholders in the form of one-to-one meetings i.e. Rawalpindi WASA and CIU Rawalpindi and collective consultative session under the chairmanship of Additional Deputy Commissioner (Finance & Planning) Rawalpindi district. Details of this consultation process are described below.

Officials from office of RDA Rawalpindi, RWASA, Irrigation Department, District officer Rawalpindi, Small Dam project Rawalpindi, MC Rawalpindi, Forest Department, Fisheries Department, Program Director PMU, Deputy Program Director PMU and Team Lead of EDCM participated in this meeting held on 3rd December, 2022 at DC office Rawalpindi. Details of consultation meetings with official stake holders are mentioned in Table 8.3 and photographs are attached as Figure 8.2.

Although the project is at its feasibility stage, at some point before or during the construction phase EPA (Federal and Punjab), as well as other government departments will be deeply involved regarding their suggestions, requirements of NOCs from their respective departments and any other specific requirement related to proposed project.

Table 8.3: Finding of Consultation with Government Stakeholders

| Sr. No. | Department | Name (FGD) | Comments | EDCM and Social Environment Advisors Reply |
| --- | --- | --- | --- | --- |
| Venue: PMU Office Date : 30-11-2022 |  |  |  |  |
| 1 | PMU/CIU | Mr. Ali Qunanin [Infrastructure Engineer] | An infrastructure engineer CIU, Rawalpindi gave a description of the project’s objectives. The deputy director development, Rawalpindi also inquired about the project's scope and whether it will have an impact on the supply chains and distribution networking. | The consultant replied that the project will have positive impact on the supply chain and it will have positive impact on the people of Rawalpindi as they will get more water supply. |
| 2 |  | Mam Sehrish Khurshid [Project Officer] |  |  |
| Venue: WASA Office Date : 30-11-2022 |  |  |  |  |
| 1 | WASA | Mr. Azizullah Khan [Deputy Director] | According to the WASA deputy director, "A water distribution network already exists; the project will increase the water's storage capacity, allowing for a continuous supply of water." |  |
| 2 |  | Mr. M.Imran Haider [GM OCL] |  |  |
| 3 |  | Mr. Mujahid [Environmental Consultant] |  |  |
| Venue: Committee Room of Deputy Commissioner (DC), Rawalpindi Date : 03-12-2022 |  |  |  |  |
| 1 | DC Office | Mr. Umer Razzaq [P & D] | People from different departments suggested that plan for this project should be shared with their stakeholders so that they can provide effective input. Also asked that this project is doable for the government and for local residents. infect recommended that new societies are emerging that future population along with this project should also be served with the clean water supply. | The consultant replied that written description of the project along with layouts of project will be shared with all relevant department for their written feedback and inputs. The same was done though CIU Rawalpindi. |
| 2 |  | Mr. Azizullah Khan [Ex. Deputy Director] |  |  |
| 3 |  | Mr. Amir Mumtaz [XEN Small Dam Islamabad] |  |  |
| 4 |  | Mr. Abid Hussain Abid [Forest Conservator Rawalpindi] |  |  |
| 5 |  | Mr. Jawad Arshad [AD (Engineer)] |  |  |
| 6 |  | Mr. M. Haseeb [AD(M & E)] |  |  |
| 7 |  | Mr. Muazzam Mukhatar [Social Safeguard] |  |  |
| 8 |  | Mam Sehrish Khursheed [Project Officer] |  |  |
| 9 |  | Mam Sana Urooj [DDF Rawalpindi] |  |  |
| 10 |  | Mam Rizwana Aziz [ADW Rawalpindi] |  |  |
| 11 |  | Mam Arifa Batool [DDW Rawalpindi] |  |  |
| 12 |  | Mr. M. Farooq [DTL EDCM] |  |  |
| 13 |  | Mr. Aziz Karim [Environmental Specialist] |  |  |
| 14 |  | Mr. Ali Qunain [IE, CIU Rawalpindi] |  |  |
| 15 |  | Mr. Ahmad Afzal [AD (Eng) PRF] |  |  |
| 16 |  | Mr. M. Shahzad [MD reservoir MCR] |  |  |
| 17 |  | Mr. Mujahid [Environmental Consultant] |  |  |
|  | Comments of EPA Rawalpindi Office | Mr. Muhammad Rafiq, Assistant Director, EPA, Rawalpindi | In the written reply, the Assistant Director has mentioned that the project is not located in Sensitive Area. The only sensitive areas, declared by EPA are Murree, Kotli Sattian and Kahuta under Rawalpindi District. The proposed project need Environmental Approval from EPA Punjab | EIA on local format is ready for submission in Punjab EPA to get the environmental approval. |

Figure 8.2 Photographs of Consultations with Institutional Stakeholders

| Consultation Meeting with Deputy Director, Rawalpindi WASA | Consultation Meeting with CIU – Rawalpindi |
| --- | --- |
| Consultative Meeting with Government Departments at DC Office Rawalpindi |  |

### Consultation Plan for Construction and Operation Phase

Consultation plan for construction and operation phase for proposed project will be prepared in order to involve all relevant government departments. Similarly, Social Safeguard teams will be developed to take response of project stakeholders and general public about the project. Periodic consultations and community feedback surveys will be carried out to develop positive perception about the project. Intended stakeholders for such consultations will be all stakeholders that are consulted at the time of IEE preparation and PICIIP-CIU. Record of such consultations will be maintained at PMU/RWASA offices and necessary changes in operational modalities will be introduced in the system in light of the response provided by the consultants.

### Information Disclosure

Consultation is ongoing process that will be carried out with the relevant department and other stakeholders along the alignment of proposed project throughout project implementation period. All future consultations will be documented and record will be maintained properly.

Efforts will be made to maximize the consultations during the project implementation. The consultations will be carried out with the objectives to develop and maintain communication linkages between the project promoters and stakeholders, provide key project information to the stakeholders, and to solicit their views on the project and its potential or perceived impacts, and ensure that views and concerns of the stakeholders are incorporated during the implementation with the objectives of reducing or offsetting negative impacts and enhancing benefits of the proposed project.

After suggesting the possible solutions of the stakeholders’ concerns, the solutions (final IEE report) will be disclosed once again before the stakeholders and general public. IEE report will be accessible to interested parties on request and the version of final report will be available in the office of PMU, CIU and RWASA and its summary will be available in national language (Urdu).

### Consultations for Solar Installations

A consultation session was conducted with city stakeholders on Environmental Assessment, under the chairmanship of Deputy Director (Development) Rawalpindi, dated 5th August 2023 at 2:00 pm in the Committee Room of Deputy Commissioner, Rawalpindi, for the project “Solarization for Sustainable & Resilient Water Supply System in RAWALPINDI “in which the consultant presented their observation as part of preparing the environmental assessment report.

Project Support Officer, City Implementation Unit (CIU) Rawalpindi gave a brief introduction of the Project and priority projects of Rawalpindi for the water sector under DREAMS – I then Engineering Design and Construction Management (EDCM) Consultant’s Environment expert presented the Environmental Assessment on the project. The discussion and decisions are provided in the Table 8.4 below.

Additional consultations were conducted during the public hearing, conducted for this sub-project on 13th of June’24 in Rawalpindi and attended by sixty-eight (68) individuals from both the public and private sectors, consisting of local community members, students, academic faculty, owners of local businesses as well as other stakeholders from the project area. They expressed queries and concerns related to the source of the water supply and its geographical coverage of supply as well as other issues such as expected vibration levels during construction works and potential impacts from high dust and noise levels during the works. Responses were provided to all their comments and queries and these are provided in Table 8.5 below while the photographic evidence is provided as Annexure XV of this IEE study.

Table 8.4: Details of consultations conducted for solar installations

| Sr. No. | Agenda item | Discussion/ Direction | Action By |
| --- | --- | --- | --- |
| 1. | Solarization for sustainable & resilient water supply system in Rawalpindi. | - Mr. Shiekh Mujahid, Environment Expert EDCM enlighten the anticipated positive impacts, and installation/construction environmental impacts of this project. Also, suggested the mitigation measures according to the IEE report, mentioned below: Following are the anticipated POSITIVE impacts from the Projects: - Provision of clean and renewable sources of Energy to make the project energy efficient and sustainable. - Cost Saving in the longer run. - Reduction in Green House Gases - Positive impact on Climate which is very critical to humans and other forms of life. - Reduce water usage (used in energy production) - Help to improve Air Quality Index by reduction in pollutants. - Economic uplift and creation of job opportunities Following are the Installation/Construction Specific Environmental impacts: - Excavation/ digging of soil for base - Cutting of trees/bushes falling within the placement area of Solar PV Panels (Minimum). - Land acquisition - Disturbance created to the public during transportation of solar panels and frames. - Noise during the installation phase. - Solid waste generation during construction. - Surface water body contamination by soil erosion and sediments. Following are the mitigation measures suggested in the IEE report: - Land is already available for the said project (even in already operational buildings the plant can be installed at rooftops which will further reduce the energy cost of cooling) - Negligible civil work, hence negligible fugitive emissions, noise, and vibration. - A tree-cutting plan will be devised and 1:10 will be adopted for tree plantation - Implementation of the Traffic diversion plan - Proper maintenance of vehicle and construction machinery - Development and implementation of Solid Waste Management Plan (SWMP) - Secondary containment for oil and other chemical storage - Provision of septic tanks for construction camps - Development and Implementation of Health and Safety Plans - Environmental consultants of EDCM, further explained the field survey experience and observations for the collection of initial data to forecast a few points regarding the environment and social. - He added that according to the Punjab Environmental Protection (Amendment) Act 2012: Section 12 (1) “No proponent of a project shall commence construction or operation unless he has filed with the Provincial Agency an initial environmental examination or where the project is likely to cause adverse environmental effects an environmental impact assessment and has obtained from the Provincial Agency approval in respect there off”. - Punjab Environmental Protection Agency (Review of IEE and EIA) Regulations 2022: - The proposed project falls under Clause B (4) (Energy) of Schedule I, which requires IEE before the commencement of construction. - Clause B applies to projects related to Energy. - ADB’s Safeguard Policy Statement (SPS), 2009: “All loans and investments are subject to categorization to determine environmental assessment requirements. Categorization is to be undertaken using Rapid Environmental Assessment (REA) checklists, consisting of questions relating to (i) the sensitivity and vulnerability of environmental resources in the project area, and (ii) the potential for the project to cause significant adverse environmental impacts. Projects are classified into one of the following environmental categories:” Category A, B, C & F1, and this project falls in category B. - Mr. Zulqarnain Shah, District Officer (I/S), the District Office, Rawalpindi, highlighted that this project has a life of 30 years therefore what technology/procedure/technique is to recycle/dispose of the waste after 30 years. He suggested that the product-generated waste should be sent back to the supplier country for recycling because in Pakistan currently there is no technology available for it. He also asked about the maintenance measures after installation. - Mrs. Sehrish Khurshid, Project Support Officer, City Implementation Unit, Rawalpindi, said in response that the guidelines for the maintenance services and disposal will be addressed during the feasibility study and detailed engineering design. Mr. Aziz Ullah Khan, DD (PnD), WASA, Rawalpindi added that at this time, it will be difficult to address this point because scientific advancement with the passage of time and technology will be changed after 30 years. - Mr. Maqbool Qureshi, Inspector of Environment Protection Agency, Rawalpindi, highlighted the requirement of the Punjab Environmental Protection Act 1997 (amendment 2012) and the Power of Delegation Rules made under the act. He asked that is this case will be submitted at the Divisional Level or the Head Office of EPA Punjab. The EDCM Environmental Consultant replied that the Delegation of Power Rules were framed almost three years back while the Guidelines has been revised in 2022. Therefore, the report will be submitted to the Head Office of EPA Punjab for environmental approval. Further, Mr. Maqbool Qureshi, Inspector EPA Punjab asked about the requirements of Pak EPA regarding the installation of a Solar Plant.Mr. Sheikh Mujahid Environment Expert EDCM Consultant briefed that IEE/EIA Regulations 2000 of Pak EPA do not give any requirement to obtain Environmental Approval from Pak EPA under Section 12 of Pakistan Environmental Protection Act, 1997. - Mr. Mazhar Nadeem Deputy Director (Development), Rawalpindi asked for the Costing and use of technology for this project. Mr. Sheikh Mujahid Environment Expert EDCM Consultant acknowledged that the feasibility study and detailed engineering design are in progress, as we have the report, it will be shared. - Mr. Muhmmad Sajid Hanif, XEN, IESCO, Rawalpindi inquire about the brands and capacities of batteries, inverters, plates, etc. Mr. Aziz Ullah Khan Dy. Director (PnD) WASA responds that it’s a direct supply of a total of 14.3 MW on 4 sites and we are not using the hybrid technology. - Mr. Muhmmad Sajid Hanif, XEN, IESCO, Rawalpindi asked about the calculation method of the figures mentioned in the table of “Electrical Load & Generation Details”. Mr. Sheikh Mujahid Environment Expert EDCM Consultant acknowledged that in this meeting these factors cannot be defined because the feasibility study and detailed engineering design are in progress, and as we have the report, it will be shared. - Mr. Mazhar Nadeem Deputy Director (Development), Rawalpindi, and Director (Water Supply), WASA, Rawalpindi asked for the cost difference and revenue generation calculations. - Mr. Abid Aleem SDO (E), C/line Cantt, WAPDA added that a new solar system product is introduced by China which consumes low space with maximum output capacity. Mrs. Sehrish Khurshid, Project Support Officer, City Implementation Unit, Rawalpindi requested the Brand name and product specification for a feasibility study. - Mr. Maqbool Qureshi, Inspector Environment Protection Agency, highlighted that 2 project sites are within Rawalpindi and 2 are in Islamabad, so what are the EIA/IEE approval criteria? The Environmental Consultant of EDCM replied that separate reports will be submitted to EPA Punjab and Pak EPA if a requirement arises from Pak EPA. |  |

Table 8.5: Details of public hearing conducted by Pak EPA

| Question / Answer session |  |
| --- | --- |
| Question from Participants | Responses |
| Dr. Saeedullah Khan Advocate, Chairman UC-90 (Sector 1-10) expressed apprehension that the proposed project would damage infrastructure of the area and operation of equipment would disseminate vibrations to the houses of sector 1-10. He inquired about route of the proposed project and source of water. | Mr. M. Mujahid, Environment Consultant (EDCM) responded that the proposed project would not involve road breaking and hence there would not be any vibrations going to sector 1-10 houses. He responded that the proposed project would pass through greenbelt land adjacent to IJP road and service road of sector 1-10. He apprised that source of water would be Khanpur lake/dam. |
| Dr. Ilmas, Assistant Professor AIOU Islamabad, inquired about responsibility of implementing mitigation measures proposed in EMP for the proposed project. She also queried about rehabilitation plan for proposed greenbelt responsible for implementing mitigation measures proposed in EMP for the destruction. | Mr. M. Mujahid, Environment Consultant (EDCM) responded construction contractor would be proposed project and PMU&ADB would supervise it. He informed that excavated material would be used for refilling dugout after laying water supply line. |
| Ms. Saima Zulqarnain, Student AIOU Islamabad, asked about area of Khyaban-e-Sir Syed to be covered in terms of water supply and its futuristic expansion. | Mr. M. Mujahid, Environment Consultant (EDCM) responded that three union councils viz. UC-10, UC- 11 and UC-12 of Khyaban-e-Sir Syed would be covered; it's a pilot project and it would be expanded to other areas depending on its success. He added that there would be 100% water metering and flow meters would be installed on all lines to gauge supply, consumption and loss etc. |
| Mr. M. Asif, resident of 1-10, inquired whether residents of 1-10 might water from this project. | Mr. M. Mujahid, Environment Consultant (EDCM) responded in the negative. |
| .Ms. Ayesha Zakir, Student IIU Islamabad, informed that local community in particular on both sides of IJP road had to face many hardships (traffic congestion, dust, vehicular emissions etc.) during execution of IJP road remodeling project. She inquired about measures that would be taken to address the highlighted issues during execution of the proposed project. She also asked about working hours of the project execution. She also inquired about possibility of mixing of sewage from sewerage line into water supply line. | Mr. M. Mujahid, Environment Consultant (EDCM) responded that mitigation measures suggested in EMP would be implemented to minimize/eliminate adverse impacts. He added that construction work would be undertaken generally from 8am to 5pm. He answered that protection cover would be used to secure water supply line wherever it would be required. |
| Ms. Mehreen Fatima, Student IIU Islamabad, asked about measures to address monsoon flooding during project execution. | Mr. M. Mujahid, Environment Consultant (EDCM) answered that less construction activity and dewatering would be employed to tackle likely monsoon flooding during project execution. |
| Mr. Yaqub Ali Khan, DIG Forests, MOCC&EC, raised the question about the operational life of the project. He also queried about tree plantation plan. He hoped that water wastage would reduce given water metering in the project. He anticipated judicious utilization of ADB loan money for the project. | Mr. M. Mujahid, Environment Consultant (EDCM) responded that as regards HDPE pipe, its life is 50 years and other components 30 years. He apprised that tree plantation plan has been given in the EIA report and would be implemented. |
| Mr. Khurram Shazad, from Cyber Security, raised the question that is What will be the cost for EMP implementation and Bifurcation? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that 1.89 million is the annual cost for the EMP implementation and bifurcation. It includes costs of environmental monitoring of groundwater, air quality, noise levels and wastewater analysis. On the other hand, 1% of total project cost is allocated for trees plantation. |
| Mr. Abdullah Ahmad Awan, from PMD department, raised the question that is How much amount Is allocated for tree plantation and how this plantation will be carried out. | Mr. M. Mujahid, Environment Consultant (EDCM) replied 1% of total project cost is allocated for trees plantation. It will be carried out in collaboration with PMU and CDA. Local species and plantation sites will be identified. |
| Ms. Horiya khan, Director of Student, queried about How will you recover the cost of water if people install their own water pumps? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that it is very costly for people to install private pumps as water table is very deep. On the other hand, it is not permissible for residents to install private pumps. |
| Mr. Farman Ullah Khan, from IMARAT Group of Companies asked about Who will regulate the net metering system? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that WASA will regulate the net metering system. |
| Mr. Muhammad Uman from EPA, asked about How will you control the water leakage and contamination from sewerage pipes? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that testing pit will be dug out to differentiate and identify existing pipelines of gas, sewerage etc. New pipeline will be installed at significant distance from them to avoid water contamination. High quality material and engineering techniques will be utilized to avoid leakage. |
| Mr. Muhammad Zaid ul Hassan from EPA, raised a query i.e. How will you manage traffic during construction? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that project does not involve massive construction activity or use of any heavy machinery. Therefore, traffic will not be disturbed by this project. |
| Mr. Waqas Afzal, Deputy Director Safeguards will asked i.e. How will you manage construction related solid waste? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that Environment Consultant (EDCM) replied that construction related solid waste will be used for filling. There will not be very deep excavations. If any solid waste material is not managed in filling, it will be thrown far away at designated solid waste site by contractor. |
| Ms. Sehrish Kurshaid from PMU- DREAMS-Lahore raised a query i.e. Are there any of tree cuttings involved in this project? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that no trees will be cut in this project as the project is based on very small digging line across the roadside. |
| Mr. Muhammad Umar from PICIP, LG&CDD Punjab asked about is there any road cuttings involved in the project? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that road cuttings are not part of project. Pipeline will be passed by digging tunnel if any road crossings are involved. |
| While concluding the session, the chair appreciated the proponent of the proposed project on their presentation and urged the proponent to take into consideration the suggestions and recommendations that came up during public hearing. It was also stated that the participant/stakeholders could also forward their / observations in writing to PAK-EPA so that the same could be duly while processing the case of environmental approval of the project. |  |
| The meeting ended with a vote of thanks to and from the chair. |  |

## GRIEVANCE REDRESSAL MECHANISM

### General

This section deals with the Grievance Redressal Mechanism (GRM) to be developed for proposed project of Augmentation of Surface Water Supply for Khayaban-e-Sir Syed from Khanpur Dam and Formation of DMAs. The proposed project, at stage of construction and operation may involve disturbance to local community. Therefore, the GRM is necessity for the successful resolution of potential disputes.

The ADB Policy (SPS 2009) requires establishment of a local grievance redress mechanism to receive and facilitate resolution of the Displaced/Affected Persons concerns and grievances regarding the project’s social and environment performance. The measures have been identified to mitigate any potential environmental and social impacts to be caused due to implementation of the project works.

However, despite of best efforts, there is chance that the individuals / households affected by the project or other stakeholders are dissatisfied with measures adopted to address adverse social impacts of the project. To address, such situation an effective Grievance Redressal Mechanism (GRM) will be established at the time of award of contracts to ensure timely and successful implementation of the project. It will also provide a public forum to the aggrieved to raise their objections and the GRM would address such issues adequately. It will receive, evaluate and facilitate the resolution of displaced persons’ concerns, complaints and grievances about the social and environmental performance at the level of the project.

The GRM will aim to investigate charges of irregularities and complaints received from any displaced persons and provide a time-bound early, transparent and fair resolution to voice and resolve social and environmental concerns link to the project.

The PMU shall make the public aware of the GRM through public awareness campaigns. The name of contact person(s) and his/her phone number, PMU contact numbers will serve as a hotline for complaints and shall be publicized through the media and placed on notice boards outside their offices, construction camps of contractors, and at accessible and visible locations in the project area. The project information brochure will include information on the GRM and shall be widely disseminated throughout the project area. Grievances can be filed in writing, via web-based provision or by phone with any member of the PMU.

First tier of GRM: The PMU is the first tier of GRM which offers the fastest and most accessible mechanism for resolution of grievances. The PMU staff for environment and social safeguards will be designated as the key officers for grievance Redressal. Resolution of complaints will be completed within seven (7) working days. Investigation of grievances will involve site visits and consultations with relevant parties (e.g., affected persons, contractors, traffic police, etc.). Grievances will be documented and personal details (name, address, date of complaint, etc.) will be included, unless anonymity is requested. A tracking number will be assigned for each grievance, including the following elements:

Initial grievance sheet (including the description of the grievance), with an acknowledgement of receipt handed back to the complainant when the complaint is registered;

Grievance monitoring sheet, mentioning actions taken (investigation, corrective measures);

Closure sheet, one copy of which will be handed to the complainant after he/she has agreed to the resolution and signed-off.

The updated register of grievances and complaints will be available to the public at the PMU office, construction sites and other key public offices (RWASA) in the project area. Should the grievance remain unresolved, it will be escalated to the second tier.

Second Tier of GRM: The PMU will activate the second tier of GRM by referring the unresolved issue (with written documentation) to the PMU, Rawalpindi who will pass unresolved complaints upward to the Grievance Redress Committee (GRC). The GRC will be established by PMU before start of site works. The GRC will consist of the following persons: (i) Project Director; (ii) representative of District government; (iii) representative of the affected person(s); (iv) representative of the local Deputy Commissioners office (land); and (v) representative of the Punjab EPA and Federal EPA (for environmental-related grievances). A hearing will be called with the GRC, if necessary, where the affected person can present his/her concerns/issues. The process will facilitate resolution through mediation. The local GRC will meet as necessary when there are grievances to be addressed. The local GRC will suggest corrective measures at the field level and assign clear responsibilities for implementing its decision within fifteen (15) working days. The contractor will have observer status on the committee. If unsatisfied with the decision, the existence of the GRC will not impede the complainant’s access to the Government’s judicial or administrative remedies.

The functions of the local GRC are as follows: (i) resolve problems and provide support to affected persons arising from various environmental issues and including dust, noise, utilities, power and water supply, waste disposal, traffic interference and public safety as well as social issues and land acquisition (temporary or permanent if any); asset acquisition; and eligibility for entitlements, compensation and assistance; (ii) reconfirm grievances of displaced persons (if any at any stage of project), categorize and prioritize them and aim to provide solutions within a month; and (iii) report to the aggrieved parties about developments regarding their grievances and decisions of the GRC.

The PMU officers will be responsible for processing and placing all papers before the GRC, maintaining a database of complaints, recording decisions, issuing minutes of the meetings and monitoring to see that formal orders are issued and the decisions carried out.

Third tier of GRM: In the event that a grievance cannot be resolved directly by the PMU (first tier) or GRC (second tier), the affected person can seek alternative Redressal through the district or sub-district committees as appropriate. The PMU or GRC will be kept informed by the district, municipal or national authority. The grievance redress mechanism Process Flow Diagream (PFD) is provided in the Figure below. The monitoring reports of the EMP and RP implementation will include the following aspects pertaining to progress on grievances: (i) Number of cases registered with the GRC, level of jurisdiction (first, second and third tiers), number of hearings held, decisions made, and the status of pending cases; and (ii) lists of cases in process and already decided upon may be prepared with details such as Name, ID with unique serial number, date of notice, date of application, date of hearing, decisions, remarks, actions taken to resolve issues, and status of grievance (i.e. open, closed, pending). In order to provide greater clarity, the pectoral description of the GRM is provided in Figure 9.1 below.

Figure 9.1 Process Flow Diagram of Grievance Redressal Mechanism

## Conclusion and Recommendations

### Conclusion

The proposed project is of high significance considering the urgent need for improving sustainable water supply system of KSS colony of Rawalpindi while the solar PV component will ensure provision of clean and reliable energy throughout the year.

Primary and secondary data has been collected and used to assess the environmental impacts of the project. This IEE report highlights the potential environmental impacts associated with the project and recommends mitigation measures accordingly. Any environmental impacts associated with the project need to be properly mitigated, through the existing institutional arrangements described in this report.

The majority of the environmental impacts are associated with the design and construction phase of the proposed project and majority of these impacts are short term and reversible.

Major impacts during construction phase will be related to deep excavations, traffic congestion and community health and safety issues during laying of Conductance Main and replacement of distribution pipelines in populated areas of Islamabad and KSS, Rawalpindi.

Also, for the solar PV component, it is an environmentally friendly technology which is benign during its operation phase with only water required in limited quantities for cleaning of panels. Any potential impacts are expected only during the project construction phase, which too will be site specific and limited in nature, consisting of impacts such as land clearing and levelling and any temporary impacts due to dust and noise emissions.

The implementation of mitigation measures during construction period will be the responsibility of the Contractor. Therefore, the required environmental mitigation measures will have to be clearly defined in the bidding and contract documents, and appropriately qualified environmental staff retained by the Construction Supervision Consultant (CSC) to supervise the implementation process. The EMP includes measures to minimize project impacts due to traffic, noise, air pollution, waste generation etc.

The EMP prepared for the proposed project under this IEE document is considered sufficient for issuance as part of the contracts to the successful bidder(s) and for subsequent use during the project works.

It should be mentioned that prior to the commencement of works, this EMP must be further updated by the Contractor into site specific EMPs (SSEMPs) for review and approval of ADB. In these SSEMPs, aspects such as a detailed traffic management plan, identification of locations for disposal of debris and spoil and any other details which shall become available later must be included for efficient implementation of all proposed mitigation measures and the subsequent monitoring of these measures.

NOC from concerned department for disposal of mucking material, spoil and municipal solid waste from worker camps will be taken and it will be responsibility of the Contractor.

Based on the above, this report concludes that there are no potential adverse environmental impacts from the proposed project. Impacts of less significance can be mitigated to an acceptable level by adequate implementation of the mitigation measures identified and suggested in EMP, hence, no significant or unacceptable change in the baseline environmental conditions will occur. Similarly, the project will have a visible positive impact on the socio-economic conditions of the local residents in terms of 24/7 uninterrupted water supply, enhancement of water quantity for KSS and provision of clean water along with generation of clean and renewable energy from the solar PV installation on a sustainable basis.

Furthermore, the PMU will ensure that selected construction contractor has contractual obligation with respect to EMP implementation. Also, RWASA will ensure appropriate staffing and budgeting for effective implementation and monitoring of project EMP.

Based on the findings of the IEE, the project is unlikely to cause any significant, irreversible or unprecedented environmental impacts. The potential impacts are localized, temporary in nature and can be addressed through proven mitigation measures. Hence, the classification of the project as "Category B" as per ADB SPS, 2009 is confirmed and the IEE study has been conducted. No further study or assessment is required at this stage.

### Recommendations

Based on the findings of this IEE report, following recommendations have been formalized for smooth functioning of proposed project:

As the project is in feasibility stage, the proponent shall obtain statutory clearances prior to award of contract and ensure conditions/requirements are incorporated in the project design and documents;

Upon mobilization of the contractors, PMU to provide a safeguards orientation as per IEE and project administration manual;

Contractor to appoint environmental and social safeguards, responsible for environmental compliance, occupational health and safety and core labour standards.

The templates for different management plans are attached as Annexures. However, detailed plans shall be developed by the Contractor before mobilization of construction teams.

SSEMP shall be developed and implemented by the Contractor during the construction phase. PMU will supervise the implementation status through the CSC.

Prior approval for setting up of construction camps shall be obtained by the contractor from PMU.

Regular monitoring for leakage shall be carried out to avoid contamination of water supply.

Employment opportunities shall be given to local community as per plans discussed in the IEE report.

# Annexures

# Annexure-I Filled Checklist of REA

Rapid Environmental Assessment (REA) Checklist

Instructions: (i) The project team completes this checklist to support the environmental classification of a project. It is to be attached to the environmental categorization form and submitted to the Environment and Safeguards Division (RSES) for endorsement by the Director, RSES and for approval by the Chief Compliance Officer. (ii) This checklist focuses on environmental issues and concerns. To ensure that social dimensions are adequately considered, refer also to ADB's (a) checklists on involuntary resettlement and Indigenous Peoples; (b) poverty reduction handbook; (c) staff guide to consultation and participation; and (d) gender checklists. (iii) Answer the questions assuming the “without mitigation” case. The purpose is to identify potential impacts. Use the “remarks” section to discuss any anticipated mitigation measures.

Augmentation of Surface Water Supply for Khayaban-e-Sirsyed from Khanpur Dam and Formation of DMAs

Country/Project Title:

- Laying of New Conductance Main (bulk Water Supply Line) from Sector I-10 Islamabad to Khyaban-e-Sir Syed. Rawalpindi

- Replacement of Water Supply Distribution Pipelines

- Formation of DMA

Sector Division:

| Screening Questions | Yes | No | Remarks |
| --- | --- | --- | --- |
| A. Project Siting Is the project area… |  |  |  |
| - Densely populated? |  |  | Khyaban-e-Sir Syed (KSS) is densely populated area located in the WEST-I Zone of Rawalpindi with 1.48 sq. km area. Estimated population of KSS is 70000. KSS consist of 3 Union Councils, UC-10 (Khyaban-e-Sir Syed (N)), UC-11 (Khayaban-e-Sir Syed (N)) and UC-12 (Dhok Najjo). The project area is bounded by three sides, from the North, the IJP road is passing, while on the East and West side, Nallah Lai and Nikki Nullah split the project boundary from the rest of the area. The area of proposed Conductance Main consists of populated area of I-10 of Islamabad and IJP road. I-10 area is densely populated on one side of Service Road West I-10 and on the other side has sparce population for almost 1.5km. At the end of I-10 sector, there is main market for fruites and Vegetables. |
| - Heavy with development activities? |  |  | The area is heavy with development activities and demand of water from PHA is also increasing to maintain greenery of the area. Other than this no heavy development activity has been observed and involved in the proposed project area in conflict with natural resource allocation. |
| - Adjacent to or within any environmentally sensitive areas? |  |  | The proposed project does not fall in any environmentally sensitive area or adjacent to any such area. |
| - Cultural heritage site |  |  | No specific site of cultural value present around the project site |
| - Protected Area |  |  | No protected area is located in or around the proposed project site. |
| - Wetland |  |  | No wetland is present near project site. |
| - Mangrove |  |  | No mangroves exist in and around the project area. |
| - Estuarine |  |  | No estuary exist in and around the project area. |
| - Buffer zone of protected area |  |  | No protected area present near the project site and it is not buffer zone for any protected area. |
| - Special area for protecting biodiversity |  |  | The project area is densely populated with human population. No special area of protecting biodiversity exist near project sire. |
| - Bay |  |  | No bay exist adjacent to or in the project area. |
| B. Potential Environmental Impacts Will the Project cause… |  |  |  |
| - pollution of raw water supply from upstream wastewater discharge from communities, industries, agriculture, and soil erosion runoff? |  |  | There are two sources of water for Khyaban-e-Sir Syed. One source is the treated water supply from Khanpur Dam and the other source is underground water extracted from tube wells. The proposed project has nothing to do with pollution of raw water supply as the New Conductance Main will receive treated water from Khanpur Water Treatment Plant (via Conductance Main of 900mm from Tomar Reservior) |
| - impairment of historical/cultural monuments/areas and loss/damage to these sites? |  |  | No any historical or cultural site/ monument is located in and around the project area, so there will be no impairment of historical/ cultural monuments or any loss/ damage to such sites. |
| - hazard of land subsidence caused by excessive ground water pumping? |  |  | The proposed project will reduce the dependency on ground water by increasing the sources of surface water from KSS. Rawalpindi WASA has calculated capacities for each tube well and to avoid excessive ground water pumping, RWASA is planning to increase the quantity of surface water from Khanpur Dam. No additional load will be on ground water, so no land subsidence is envisaged at all. |
| - social conflicts arising from displacement of communities? |  |  | No displacement of communities is expected by the project, therefore, no social conflicts are envisaged as negative impact of the project. However, some push carts will be temporarily displaced during excavation of trenches for lying of new Conductance Main |
| - conflicts in abstraction of raw water for water supply with other beneficial water uses for surface and ground waters? |  |  | Yes, conflict may arise as the water supply from Khanpur Dam is used for irrigation and water supply to Islamabad as well. The capacity of Khanpur Reservoir is decreasing and this will create conflicting demand between water supply for community in Islamabad, irrigation and for increase in supply to Rawalpindi (KSS). |
| - unsatisfactory raw water supply (e.g. excessive pathogens or mineral constituents)? |  |  | There are two sources of water for Khyaban-e-Sir Syed. One source is the treated water supply from Khanpur Dam (Sangjani Water Treatment Plant) and the other source is underground water extracted from tube wells. The proposed project has nothing to do with pollution of raw water supply as the New Conductance Main will receive treated water from Sangjani Water Treatment Plant (via Conductance Main of 900mm from Tomar Reservior). Further, the analysis of water sampels taken from two localities of KSS shows presence of no pathogens. |
| - inadequate protection of intake works or wells, leading to pollution of water supply? |  |  | The tube wells and water supply from Khanpur Dam is well protected and there are no chances of water pollution at water supply. Further, existing OHRs and GSTs are also well protected to control water pollution. |
| - over pumping of ground water, leading to salinization and ground subsidence? |  |  | The proposed project aims to reduce the load on groundwater by increasing water quantity of surface water from Khanpur Dam. This water will be stored in existing OHRs and GSTs to maintain 24/7 water supply in project area by installation of DMAs to regulate water flow. |
| - excessive algal growth in storage reservoir? |  |  | Storage reservoirs are treated with algaecides to control algal growth. |
| - increase in production of sewage beyond capabilities of community facilities? |  |  | As soon as sewage production will increase, community facilities will also increase accordingly. |
| - inadequate disposal of sludge from water treatment plants? |  |  | No water treatment is involved in this project. |
| - inadequate buffer zone around pumping and treatment plants to alleviate noise and other possible nuisances and protect facilities? |  |  | Noise will be generated due to operation of Installation of one pump having discharge capacity 0.8MG per hour at Sangjani Water Treatment Plant. Further, noise levels will be increased by installation of additional pumps and machinery to be installed on existing OHRs and GSTs to maintain regular 24/7 flow of water in KSS. But the noise level will be curtailed within prescribed limited by installing sound barriers at source. Further, the new pumps will be noise efficient. |
| - impairments associated with transmission lines and access roads? |  |  | Impairments will be associated with the replacement of Conductance Main and replacement of existing distribution network which is 25-33 years old. However, the existing Right of Way (RoW) of road will be utilized for said purposes. Rigid payment and asphalt road cutting is involved during lying of new Conductance Main and Replacement of AC and GI pipelines. Screening for other underground services like transmission lines will be carried out before start of physical work. |
| - health hazards arising from inadequate design of facilities for receiving, storing, and handling of chlorine and other hazardous chemicals. |  |  | The proposed project does not involve treatment of water. Hence, there will be no use of chlorine in the proposed project. |
| - health and safety hazards to workers from handling and management of chlorine used for disinfection, other contaminants, and biological and physical hazards during project construction and operation? |  |  | Typical construction hazards will be involved in installation of pumps, replacement of AC and GI Pipelines and lying of new Conductance Main. The main hazard associated with proposed projects are working at height, falls from height, electrical hazards, mechanical hazards due to machinery and equipment, deep excavation and protection of edges and boundary to avoid boundary collapse. However, all these hazards will be controlled by developing and implementing the Safety procedures and implementation of Health and Safety Management Plan . |
| - dislocation or involuntary resettlement of people? |  |  | The project does not involve permanent dislocation or involuntary resettlement of people. Existing ROW will be used for replacement of Conductance Main and water distribution lines. However, temporary displacement of some push carts will be involved near main Fruit and Vegetable market of I-10, Islamabad. |
| - disproportionate impacts on the poor, women and children, Indigenous Peoples or other vulnerable groups? |  |  | The implementation of the project will help women in their household tasks, hence, positive impact is envisaged due to the project. |
| - noise and dust from construction activities? |  |  | During construction activities, noise and dust from heavy machinery will be generated, especially during excavation of trenches for lying of New Conductance Main and replacement of AC and GI Distribution lines. |
| - increased road traffic due to interference of construction activities? |  |  | During construction activities, additional machinery and heavy vehicles will be mobilized in the project area hence, road traffic will be increased accordingly. Traffic Management Plan will be developed and implemented during construction with proper marking of diversion routes. |
| - continuing soil erosion/silt runoff from construction operations? |  |  | During construction phase, soil erosion may occur, if it is not managed properly. |
| - delivery of unsafe water due to poor O&M treatment processes (especially mud accumulations in filters) and inadequate chlorination due to lack of adequate monitoring of chlorine residuals in distribution systems? |  |  | The proposed project does not involve treatment of water. Treated water from Khanpur Dam (Sangjani Water Treatment Plant) will be received in existing GSTs and OHRs and distribution network. Water from tube wells is also supplied to GST and OHR for distribution to community. |
| - delivery of water to distribution system, which is corrosive due to inadequate attention to feeding of corrective chemicals? |  |  | Water treatment is not part of this proposed project with no chances of delivery of corrosive water. |
| - accidental leakage of chlorine gas? |  |  | The proposed project does not involve handling of Chlorine gas. |
| - excessive abstraction of water affecting downstream water users? |  |  | The main purpose of this project is to reduce pressure on ground water by increasing water from Khanpur Dam (Sangjani Water Treatment Plant). No additional tube wells will be installed. Capacity of already working tube wells is calculated by RWASA. |
| - competing uses of water? |  |  | Increase in water demand from Khanpur Dam will create further competition on water supply from Khanpur Dam as this water is already supplied for irrigation and water supply to other communities. |
| - increased sewage flow due to increased water supply |  |  | Due to increase in water supply, the use of water will also be increased and results in generation of additional sewage flow. |
| - increased volume of sullage (wastewater from cooking and washing) and sludge from water treatment plant |  |  | No wastewater treatment plant is part of this project. However, there will be increase in volume of Sullage and Sludge from activities linked to this project. |
| - large population influx during project construction and operation that causes increased burden on social infrastructure and services (such as water supply and sanitation systems)? |  |  | No large population influx is envisaged therefor, no burden will increase on social infrastructure and services. Local community will be utilized mainly as skilled and un-skilled labour during the construction phase of the project. |
| - social conflicts if workers from other regions or countries are hired? |  |  | Social conflicts may arise if the workers are engaged from other regions. Maximum workforce, skilled and unskilled shall be hired from local community. As the project area is densely populated hence social may arise due to camping at road side and disturbance to privacy of local community. |
| - risks to community health and safety due to the transport, storage, and use and/or disposal of materials such as explosives, fuel and other chemicals during operation and construction? |  |  | Risk to community will occur due to transport, and storage of materials, fuel and other chemicals. However, management of chemicals will be done according to Safety Data Sheets of respective chemicals. No explosives will be used in this project. |
| - community safety risks due to both accidental and natural hazards, especially where the structural elements or components of the project are accessible to members of the affected community or where their failure could result in injury to the community throughout project construction, operation and decommissioning? |  |  | Community safety will be at risk due to accidental and natural hazard, especially near overhead reservoirs. Risk due to excavation of trenches for lying of New Conductance Main and replacement of AC / GI Distribution pipelines will cause risk of slope failure and falling of people in trenches if not properly barricaded. Further, during construction phase, the movement of heavy machinery and dumpers will cause additional risk of accidents to community if not managed properly. Safety Risk due to accidents during operational phase of this project will be negligible as there will be negligible movement of project transport. |

# Annexure-II Environmental Monitoring Reports

# Annexure-III Calibration Certificates of Monitoring Equipment

# Annexure-IV Chance Find Procedure

Chance Find Procedure

Project may involve deep excavation. Therefore, the possibility of chance find is not ignorable. In case of any chance find, the contractor will immediately report through Supervision Consultant to Directorate General (DG) of Archaeological Department, Government of Pakistan to take further suitable action to preserve those antique or sensitive remains. Representative of the DG will visit the site and observed the significance of the antique, artifact and cultural (religious) properties and significance of the project. The report will be prepared by representative and will be given to the DG. The documentation will be completed and if required suitable action will be taken to preserve those antiques and sensitive remains.

In case any artifact, antiques and sensitive remains are discovered, chance find procedures should be adopted by contractor workers as follows:

- Stop the construction activities in the areas of chance find;

- Delineate the discovered site or area;

- Consult with the local community and provincial Archeological Department

- The suggestion of the local communities and the concerned authorities will be suitably incorporated during taking the preventive measures to conserve the antique, artifact and cultural (religious) properties;

- Secure the site to prevent any damage or loss of removable objects. In case of removable antiquities or sensitive remain, a night guard shall be arranged until the responsible local authorities take over; and

- After stopping work, the contractor must immediately report the discovery to the Supervision Engineer.

The contact Address of Archeology Department is given below:

Archeology Department

19 Ataturk Ave, Shakar Parian,

Islamabad, Islamabad Capital Territory, Pakistan

# Annexure-V Framework for Traffic Management Plan

1. Objectives of TMP

One of the prime objectives of this TMP is to ensure the safety of all the road users along the work sites, and to address the following issues:

- Safety of pedestrians, bicyclists, and motorists travelling through the construction sites;

- Protection of workers from hazards associated with moving traffic;

- Mitigation of the adverse impact on road capacity and delays to the road users;

- Maintenance of access to adjoining properties; and

- Addressing issues and concerns that may delay the implementation of the proposed project.

2. Operating Policies for TMP

The following policies will help promote safe and efficient movement for all road users (motorists, bicyclists, and pedestrians, including persons with disabilities) through and around project sites, while reasonably protecting workers and equipment.

- Make traffic safety and temporary traffic control an integral and high-priority element of project from planning through design, construction, and maintenance;

- Inhibit traffic movement as little as possible;

- Provide clear and positive guidance to drivers, bicyclists, and pedestrians as they approach and travel through the temporary traffic routes;

- Inspect traffic control elements routinely, both day and night, and make modifications when necessary;

- Pay increased attention to roadside safety in the vicinity of temporary traffic routes;

- Train all persons that select, place, and maintain temporary traffic control devices;

- Keep the public well informed; and

- Make appropriate accommodation for abutting property owners, residents, businesses, emergency services, railroads, commercial vehicles, and transit operations.

Policy steps for the TMP are shown in Figure 1. However, operating policy for TMP for the construction of the proposed project along various types of roads is shown in Figures 2 to 7.

Figure 1: Policy Steps for the TMP

3. Analyze the Impact Due to Street and Road Closure

Apart from the capacity analysis, final decisions to temporary close a particular street and road and divert the traffic should involve the following steps:

- Approval from the Traffic Police to use the local streets and roads as detours;

- Consultation with businesses, community members, residents, road users and traffic police regarding the mitigation measures necessary at the detours where the road is diverted during the construction;

- determining of the maximum number of days allowed for road closure, and incorporation of such provisions into the contract documents;

- determining if additional traffic control or temporary improvements are needed along the detour route;

- considering how access will be provided to the worksite;

- Contacting emergency service, school officials, and transit authorities to determine if there are impacts to their operations; and

- Publish a notice through print and electronic media about road closure to avoid hindrance to public. As part of this notice, the public should be advised of alternate routes that commuters can take or will have to take as result of the traffic diversion.

If full road-closure of certain routes within the proposed project area is not feasible due to inadequate capacity of the Detour Street or public opposition, the full closure can be restricted to weekends with the construction commencing on Saturday night and ending on Monday morning prior to the morning peak period.

4. Public Awareness and Notifications

As per discussions in the previous sections, there will be travel delays during the construction, as is the case with most construction projects, although on a reduced scale if utilities and traffic management are properly coordinated. There are additional grounds for travel delays in the area, as most of the streets and roads in the proposed project sites lack sufficient capacity to accommodate additional traffic from diverted traffic, as a result of street and road closure to accommodate the construction activities.

The awareness campaign and the prior notification for the public will be a continuous activity which the project will carry out to compensate for the above delays and minimize public claims as result of these problems. These activities will take place sufficiently in advance of the time, when the roadblocks or traffic diversions take place at the particular streets and roads. The reason for this is to allow sufficient time for the public and residents to understand the changes to their travel plans. The project will notify the public about the roadblocks and traffic diversion through public notices, ward level meetings, and city level meeting with the elected representatives.

The Project Management Unit (PMU) comprising staff members of WASA, Supervision Consultant and Contractor will also conduct an awareness campaign to educate the public about the following issues:

- Traffic control devices in place at the construction sites (signs, traffic cones, and barriers);

- Defensive driving behavior along the construction sites; and

- Reduced speeds enforced at the construction sites and traffic diversions

It may be necessary to conduct the awareness programs and campaigns on road safety during construction.

The campaign will cater to all types of target groups i.e. children, adults, disabled and drivers.

Therefore, these campaigns will be conducted in schools and community centers. In addition, the project will publish a brochure for public information. These brochures will be widely circulated around the proposed project area and will also be available at the PIU, and the Contractor's site office. The text of the brochure should be concise to be effective, with a lot of graphics. It will serve the following purpose:

- Explain why the brochure was prepared, along with a brief description of the proposed project;

- Advise the public to expect the unexpected delays or roadblocks;

- Educate the public about the various traffic control devices and safety measures adopted at the construction sites;

- Educate the public about the safe road user behavior to emulate at the construction sites;

- Inform the public how to stay informed or where to inquire about road safety issues at the construction sites (name, telephone and mobile number of the contact person); and

- Indicate the office hours of authorized representatives.

5. Install Traffic Control Devices at the Construction Sites and Traffic Diversion Routes

The purpose of installing traffic control devices at the construction sites is to delineate these areas to warn, inform, and direct the road users about a hazard ahead, and to protect them as well as the workers. As proper delineation is a key to achieve the above objective, it is important to install good traffic signs at the construction sites. The following traffic control devices are used in construction sites:

- Signs;

- Pavement Markings;

- Channelizing Devices;

- Arrow Panels; and

- Warning Lights.

Procedures for installing traffic control devices at any construction sites vary, depending on road configuration, location of the work, construction activity, duration, traffic speed and volume, and pedestrian traffic. Work will take place along major roads, and the minor internal roads. As such, the traffic volume and road geometry vary. The main roads carry considerable traffic; internal the roads are narrow but carry traffic. However, regardless of where the construction takes place, all the construction sites should be cordoned off, and traffic shifted away at least with traffic cones, barricades, and temporary signs (temporary “STOP” and “GO”).

Figures 2 to 7 illustrates a typical set-up for installing traffic control devices at the construction sites of the area, depending on the location of work on the road way, and road geometrics:

- Work on shoulder or parking lane;

- Shoulder or parking lane closed on divided road;

- Work in travel lane;

- Lane closure on road with low volume;

- Lane closure on a two-line road with low volume (with yield sign);

- Lane closure on a two-line road with low volume (one flagger operation);

- Lane closure on a two lane road (two flagger operation);

- Lane closure on a four lane undivided road;

- Lane closure on divided roadway;

- Half road closure on multi-lane roadway; and

- Street closure with detour.

The construction sites should take into consideration, for the space required for a buffer zone between the workers and the traffic (lateral and longitudinal), and the transition space required for delineation, as applicable. For the construction works, a 30 centimeter clearance between the traffic and the temporary STOP and GO signs should be provided. In addition, at least 60 centimeter is necessary to install the temporary traffic signs and cones.

Traffic police should regulate traffic away from the construction sites and enforce the traffic diversion outcome from full street closure in certain areas during construction. Flaggers and personnel should be equipped with reflective jackets at all times and have traffic control batons (preferably the LED type) for regulating the traffic during night time.

In addition to the delineation devices, all the construction workers should wear fluorescent safety, vests and helmets in order to be visible to the motorists at all times. There should be provision for lighting beacons and illumination for night constructions.

# Annexure-VI Framework for Resource Management Plan

1. INTRODUCTION

Most of the resources in this world are finite and non-renewable in nature. We are completely dependent on these resources to fulfill all our daily requirements. Therefore, sustainable development calls for the need to conserve resources in a way that meet our needs of present generation as well as future generation, especially the non-renewable resources.

2. OBJECTIVE OF THE PLAN

The Resource Conservation Plan is intended to make an effort towards achieving sustainable development. The objective of the resource conservation plan is to:

- Minimize the use of natural resources; and

- Mitigate and prevent pollution contaminating the natural resources.

3. PLANNING

Careful estimations of quantities of material, fuel, water and energy required directly or indirectly shall be done to avoid excessive or unnecessary wastage of these materials. In addition to this, pollution prevention strategies shall also be devised to prevent contamination of resources.

The estimations include the following:

- Estimation of construction material required for the project;

- Estimation of fuel consumption for construction machinery, construction vehicles and generators;

- Estimations of the energy requirements during all the stages of the project; and

- Estimations of water consumption for construction activities and construction camp sites.

- Strategies shall be planned to reduce loads on the identified resources to be consumed;

- Best management practices shall be devised to control or reduce pollution resulting from the activities during different stages of the project; and

- An inspector shall be assigned responsibility to oversee the ongoing activities to check the compliance of the planned strategies.

4. EXECUTION OF THE PLAN

The planned strategies shall be implemented to conserve the natural resources including but not limited to the following:

Material

- Material supplied shall be in conformance with the estimated quantities and excess material shall be returned to the supplier;

- Material wastage shall be avoided by using best management practices;

- Waste produced during the project execution shall be disposed of safely to the designated disposal sites through approved contractors; and

- Reuse of the materials shall be appreciated.

Energy

- Reduce trips and optimize routes to and from the construction site for all kinds of activities;

- Regular maintenance of equipment and vehicles to avoid leaks and sustain efficient fuel consumption;

- Switch off idle equipment and vehicles to avoid wastage of fuel;

- Minimize warm up time, unnecessary acceleration and deceleration of the construction equipment and vehicles;

- Avoid unnecessary burning of fuel for cooking in construction camps;

- Avoid unnecessary use of heating and cooling systems during extreme weathers events;

- Construction shall start in early hours of the day to avoid heat in summers and utilization of day light; and

- Alternate energy sources shall be considered for electricity generations during construction and operation to conserve fossil fuel as it is non-renewable resource.

Water

- Avoid using potable water for sprinkling, curing and washing of equipment and vehicles.

- Surface water or treated effluent can be used instead;

- Wastage of water should be controlled through providing proper valves and through controlling pressure of the water;

- Unnecessary equipment washings should be avoided;

- Awareness amongst workers shall be raised to conserve water and immediately report for any leaks detected; and

- Ensure protection of canal water from contamination resulting from construction activities.

Pollution

- Emissions shall be reduced and controlled as far as possible and direct discharges to air shall be avoided by strictly adhering to the mitigation measures outlined in IEE report;

- Waste water shall not be discharged directly into the canal and must be managed as per the recommendations presented in IEE report; and

- Construction waste, and municipal solid waste shall not be dumped and burnt openly, and shall be handled according to the preventative measure given in IEE report.

5. CHECKING AND CORRECTIVE ACTIONS

The R-WASA shall bind the construction contractor through contract agreement to comply with the strategies outlined in the Resource Conservation Plan. The Environmental Committee shall also appoint an Inspector who shall monitor the daily onsite activities and shall report any issues and concerns raised in relation to Resource Conservation Plan. The inspector shall recommend adequate corrective actions to mitigate the issues raised.

# Annexure-VII Framework for Occupational Safety and Health Management Plan

Occupational Health and Safety Management Plan

It is required that all personnel working on the project attend the Health and Safety site induction briefing prior to commencing any work on the site.

The General Site Rules shall act as the main agenda and content of the induction briefing. Additionally, any information specific to the project shall be included in the induction. The induction briefing should be, as far as possible, two way communication with attendees invited to contribute, comment and ask questions regarding health and safety. Other rules are as follows:

- All attendees of the induction briefing will be recorded.

- High visibility jacket/vests, safety helmets and safety footwear (incorporating steel toe- caps and mid-sole) must be worn at all times.

- Safety goggles for protection during all cutting, grinding and drilling operations or where there is risk from impact, dust, chemicals or hot metal.

- Dust masks for protection from dust.

- Ear protection during all operations which produce noise above the level at which you need to raise your voice to be heard. Gloves during concreting work.

- On this site, formal permits must be in place before any of the following operations may be carried out: Permit to Excavate, Hot Work Permit & Confined Spaces Permit. (Where Applicable), thoroughly understand it and get permission from area supervisor (If required).

- Follow the messages and instructions displayed on HSE boards installed on site.

- Be aware of emergency muster (assembly) points and escape routes. In the event of an emergency do not panic, follow the site emergency response procedures.

- Report promptly all accidents to your supervisor and HSE officer at site. Immediately provide first aid for the injured and call for the medic.

- While working alone or in a confined space make sure that your nearby colleague and supervisor are well informed or use a banksman where appropriate.

- Ensure adequate lighting is in place for work on night shifts or for emergency response.

- All Scraps, waste materials and garbage must be disposed of in accordance with the construction waste management Strategy.

- Always clean your work site after completing the job or your shift.

- Maintain appropriate barricades as required.

- Never tamper with electric cables and appliances. Never insert direct cables into sockets, rather use proper plugs.

- Do not enter scaffold that is not tagged safe for access.

- Tools or materials must not be carried while climbing up or down scaffolding or ladders.

- Use pouches or ropes for this purpose.

- Do not smoke or produce naked flame in NO SMOKING area. Use of open fire is prohibited.

- Keep all gangways and aisles clear and clean at work sites.

- Vehicles must be driven at a safe speed, observing speed limits. Drivers must have a valid driving license for the class of vehicle they are operating.

- Vehicles shall only be parked in designated parking areas. Never travel in a vehicle unless in seating equipped with a seatbelt.

- Site HSE Roles and Responsibilities

1.1 Project Manager (PM)

- Develop and disseminate policies for occupational health and safety management systems and environmental management systems

- Provide resources for the management and implementation of occupational health and safety management systems and environmental management systems

- Responsible for ensuring the implementation of occupational health, safety and environmental policies

- Aware of the relevant maters of environment, health and safety.

- Allocate Budget and Provide resources for the implementation of HSE Plan in its true spirit.

- Attend meetings regarding the Health Safety and Environment.

- Report Health Safety and Environment matters to the management of the company as required.

1.2 Construction Manager (CM)

- Responsibilities include, but not limited to:

- Well conversant with international & local, Health, Safety & Environmental rules, regulations and current legislation.

- Ensure that all senior site staff is also conversant with the relevant requirements of current legislations and EPC contractor HSE plan.

- Ensure that site personnel are assigned appropriate duties and responsibilities accordingly to assist in the effective implementation of this HSE Plan.

- Ensure no job is carried on without supervision.

- Approve   work   methods   statements; ensure   safety   procedures   are established and undertaken.

- Funds for essential HSE facilities, equipment and personnel are made available.

- His own personal behavior supports, strengthens and confirms the site’s HSE management program.

1.3 Environmentalist/HSE Officer

- Carry out safety inspection of Work Area, Work Method, Men, Machine & Material and other tools and tackles.

- Facilitate inclusion of safety elements into Work Method Statement.

- Highlight the requirements of safety through Tool-box/other meetings.

- Conduct investigation of all incident/dangerous occurrences & recommend appropriate safety measures.

- Advice & co-ordinate for implementation of HSE plan in its true spirits.

- Convene HSE meeting & minute the proceeding for circulation & follow-up action.

- Plan procurement of PPE & Safety devices and inspect their healthiness.

- Report to Project Manager (PM) and Resident Director EPC contractor on all matters pertaining to status of safety and promotional program at site level.

- Facilitate administration of First Aid

- Facilitate screening of workmen and safety induction.

- Conduct   fire   and   other   emergency   Drills   and   facilitate   emergency preparedness

- Design campaigns, competitions & other special emphasis programs to promote safety in the workplace.

- Recommend to Site In charge, immediate discontinuance of work until rectification, of such situations warranting immediate action in view of imminent danger to life or property or environment.

- To decline acceptance of such PPE / safety equipment that do not conform to specified requirements.

- Encourage raising Near Miss Report on safety along with, improvement initiatives on safety.

- Ensuring that all injuries, accidents, incidents/near misses and hazards are positively and timely reported.

- Assisting Engineers/Area Supervisors in accident/incident investigations, where required Attending and positively contributing in the HSE Committee Meetings.

1.4 Workers

- Use the proper tools and equipment when operating. According to the requirement of operation and different hazard sources using the PPE and protective clothing provided by company, such as, helmets, safety harness, goggles, safety vest, and other personal protective equipment

- Maintain the tool and keep the tool clean after job completion.

- Report to the site supervisor or on-site project manager for the equipment damage and hidden danger.

- Develop personal safety awareness - including workers themselves and others, especially new employees and young people.

- Avoid unnecessary risk generation.

- For known sources of danger, remind new employees to stay away from these hazards.

- It will strictly be forbidden to play in the construction area or to damage the public utilities.

- Provide advice on how to eliminate hazards.

1.5 Sub-Contractor HSE Management

- Sub-contractor’s employees shall immediately correct all unsafe conditions & acts as directed by Contractor Direct supervision. Unsafe acts by any personnel may be grounds for immediate removal and permanent banning from the project site.

- Only properly trained employees shall be authorized by sub-contractor to operate equipment, machinery, vehicles & tools.

- All Sub-contractors are required to follow safe work practices, and meet the requirements clearly identified in field HSE PLAN.

- The Sub-contractor shall erect & maintain safeguards for the protection of workers, any other sub-contractors & the public and eliminate or mitigate HSE hazards created by or otherwise resulting from performance of the work.

2 Communication and HSE Meetings

2.1 HSE Targets and Goals

Pursuant to the Policy Statement, the following HSE Targets and Goals are identified in order to create a positive approach to health, safety, and protection of the environment during all activities of the project; this will be achieved by employing competent and motivated staff:

- To avoid all personal injuries during the execution of the Project,

- To ensure that all personnel employed on the Project are competent to carry out their designated tasks safely.

- To create positive health, safety and environment attitudes and perceptions at all levels of the Project organization, and to raise health safety and environmental awareness in general.

- To implement a training program that supports the achievement of personnel competency in relation to Health, Safety, and the Environment.

- To complete the Project without incurring any significant property damage to permanent equipment, or temporary facilities.

- To complete the Project with minimum avoidable impact upon the surrounding environment.

- To implement a hierarchy of communication forums that ensure that HSE concerns can be raised and addressed at all levels of the organization. To continually monitor and improve HSE performance.

2.2 HSE Score Board

Environmentalist/HSE Officer will arrange a HSE score board and display at the key location of the project site facilities. Information on the HSE Score Board will be updated on daily basis.

2.3 HSE Awareness Sign Boards

HSE awareness signboards shall be displayed at key locations around the site to create and maintain awareness and ownership of HSE issues.

2.4 Internal Site HSE Committee Meeting

Internal HSE Committee Meetings chaired by the Site Manager/Construction Manager, will be held monthly with all Area Supervisors and nominated Engineers, Supervisors and support staff. HSE committee meetings will allow communication of HSE performance and corrective actions. All-important HSE matters of the site as well as the non-compliance reported in the current month will be discussed in these meetings.

1.3 Performance Monitoring

1.3.1 Performance Review in Site HSE Committee Meetings

Performance reviews shall be held on monthly basis in the Internal Site HSE Committee Meetings. The objective of the review is to gather information from monitoring, inspection and site working activities and to assess the effectiveness of the implementation of HSE procedures on site. The key performance indicators are:

a. Compliance with Health, Safety and Environment standards

b. Identification of areas not addressed in the HSE Plan

c. Achievement of specified HSE objectives

d. HSE statistics, root cause and trend analysis of the statistics

1.3.2 HSE Inspections of Equipment and Tools

a. The Site Manager will ensure that no Civil, Electrical or Mechanical equipment will go to the working area without HSE inspection.

b. Environmentalist/HSE Officer   will   co-ordinate   with   Equipment   and   Plant Department for the inspection of all Civil, Electrical and Mechanical equipment or the inspection of Civil, Electrical and mechanical equipment,

c. If during inspection, any equipment is found sub-standard, Manager Site HSE is authorized to reject this equipment and inform the Site Manager.

1.4 Incident / Accident Reporting and Investigation

1.4.1 Objective

The objective of incident reporting, investigation & analysis is to identify the cause(s) of an incident to allow for preparation of recommendations, to avoid recurrence of such incident(s) in future.

1.4.2 Incident Reporting and Investigation

Any work-related incident, accident, injury, illness, exposure, or property loss must be reported to your supervisor, the SS, and within 1 hour. Motor vehicle accidents must also be reported. A report must be filed for the following circumstances:

a)  Accident, injury, illness, or exposure of an employee;

b)  Injury of a subcontractor;

c)  Damage, loss, or theft of property; and/or

d)  Any motor vehicle accident regardless of fault, which involves a company vehicle, rental vehicle, or personal vehicle while the employee is acting in the course of employment.

e)  HSE Officer shall brief the Management about the lost time injury/fatal cases and serious incidents/near-misses.

f) Project Manager and Construction Manager will review the incident report and comments on the recommendations of the Site HSE Staff. They will then assign corrective actions accordingly.

g) Occupational accidents resulting in employee injury or illness will be investigated by the SS. This investigation will focus on determining the cause of the accident and modifying future work activities to eliminate the hazard.

h) All employees have the obligation and right to report unsafe work conditions, previously unrecognized safety hazards, or safety violations of others. If you wish to make such a report, it may be made orally to your supervisor or other member or management, or you may submit your concern in writing, either signed or anonymously.

i) The Site HSE Staff shall carry out a follow up of the recommendations/corrective actions from time to time where he identifies ongoing non-compliance he shall inform the Project Manager Construction Manager.

1.5 Orientation, Site HSE Induction and Other Training Activities

1.5.1 Initial HSE Orientation Program

Each and every person will undergo a HSE orientation program. On completion of orientation he will be issued an Organizational Identity card. Contractor Site HSE Staff shall perform the initial orientation based on but not limited to the following:

- Explaining Organizational HSE Policy and Standards.

- General HSE rules and regulations for working in a Construction Site, Batching plant, Excavation Area, Mechanical and Electrical work including use of Personal Protective Equipment, incident reporting, getting first aid, emergency response, (HSE inspection, housekeeping, etc.

- Hazards at construction site, works, offices, or any miscellaneous work.

- Specific hazards like height, open excavations, electrical, fire, Fumes (including spray painting) and vehicle safety etc.

- Environmental hazard.

1.5.2 Daily Tool Box Talk by Supervisors/HSE Officer

HSE tool Box talk shall be conducted by Supervisor/HSE Officer for specific work groups prior to the start of work. The tool box is a forum for two- way communication between management and the employees. Tool box talk is focused on a specific job. The agenda shall consist of the following:

- Details of the jobs being intended for immediate execution.

- The relevant hazards and risks involved in executing the job and their control and mitigating measures.

- Specific site condition to be considered while executing the job like high temperature, humidity, unfavorable weather etc.

- Recent non-compliances observed.

- Appreciation of good work done by any person.

- Feedback from employees

- Any doubt clearing session at the end.

- Record of Tool box talk shall be maintained as per format attached.

- Incidents, which may occur in the site/works, shall also be discussed in “tool box talks”.

- Daily tool box talk record format is attached as Annex. “A”.

1.5.3 HSE Training During Project Execution

HSE training shall be arranged by contractor as per the need of the project execution and recommendation of HSE committee of site.

1.5.3.1 Site-Specific Training

Prior to working at this site, an initial site-specific training session or briefing shall be conducted prior to commencement of work activities. During this initial training session, employees shall be instructed on the following topics:

- Personnel responsibilities;

- Content and implementation of the HASP;

- Description of assigned tasks/scope of work;

- Site hazards and controls;

- Site-specific hazardous procedures (e.g., lining activities, etc.);

- Coordination of Site activities;

- Training requirements;

- PPE requirements;

- Emergency information, including local emergency response team phone numbers,

- route to nearest hospital, accident reporting procedures, evacuation routes and procedures, location of assembly points, and emergency response procedures;

- Instruction in the completion of required inspections and forms; and

- Location of safety equipment (e.g., portable eyewash, first aid kit, fire extinguishers, etc.).

- The various components of the project HASP will be presented followed by an opportunity to ask questions to ensure that each attendee understands the HASP. Personnel will not be permitted to enter or work in potentially contaminated areas of the Site until they have completed the Site-specific training session.

1.5.3.2 Safety Meeting/Health and Safety Plan Review

"Tailgate" safety meetings will take place each day prior to beginning the day's work. All Site personnel will attend these safety meetings, which may be contracted personnel, subcontractor personnel, or personnel from contractors. The safety meetings will cover specific health and safety issues, Site activities, changes in Site conditions, and a review of topics covered in the Site-specific pre-entry briefing.

1.5.3.3 Management Employees Training Program

- The management employees training program will be conducted during the project to ensure that all management employees are trained.

- An HSE specialist from contractor head office shall conduct the meetings, during his visit to site.

- The duration of this course would be half day.

Annexure Sub-A

| HSE TOOL BOX TALK RECORD |  |  | Quality Records Forms |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Doc. Version 1 |  |
| Project Title: Augmentation of Surface Water for KSS from Khanpur Dam and Creation of DMA Project Section: Topic: Date |  |  |  |  |  |  |
| SR. NO. | NAME OF WORKER / STAFF | CRAFT | AREA | CONTRACTOR |  | REMARKS |
| Delivered By SITE HSE OFFICER |  |  |  |  |  |  |

# Annexure-VIII Framework for Emergency Response Management Plan

Emergency Response Plan

Contractors are committed and obliged to protecting the community, workers, public and concerned company’s property and the surrounding environment; in the emergency situations. These	 situations shall be handled through the implementation of an Emergency Preparedness and Response Plan (EPRP). The construction contractors shall meet with the local emergency service institutions and law enforcement agencies i.e. Fire and rescue 1122, Police, to review and discuss the construction   process, including   unique   construction   equipment(s), the   overall construction process, and schedule/phasing. This plan will be communicated to all the concerned and personnel who will be responsible for managing the emergency situations, and those who will be effected by the emergencies.

1. Purpose

The purpose of the emergency response plan is to identify the actual and potential hazards and their risks, organize responsibilities, identify the resources and plan to utilize them in order to control and minimize impacts of risks in an efficient way. This plan is applicable to all construction processes and sites, machinery/ equipment and materials. It includes preparation to respond in the event of fires, incidents/ accidents involving personnel injuries, property damages etc., hazardous materials emissions, explosions, and natural disasters i.e. flood, earthquake etc. This plan also includes preparation to respond in the event of riots, arsons, terrorist attacks etc.

1.2 Objectives

The objectives of this plan are;

- To develop, maintain, implement, check and improve procedures and practices which will ensure efficient utilization of available resources; in order to protect workers and staff from impacts of hazard’s risks.

- To minimize impacts of emergencies and unplanned happenings.

- The plan includes preparedness and response mechanisms.

1.3 Scope

The emergency management program is applied to all Project elements and intended for use throughout the Project life cycle. The following are some emergencies that may require coordinated response.

- Construction Accident

- Road & Traffic Accident

- Hazardous material spills

- Structure collapse or failure

- Trauma or serious illness

- Sabotage

- Fire

- Environmental Pollution

- Loss of person

- Community Accident

1.4 Types of Emergencies

Following is a list of the hazards and emergency situations, which need planning and proper handling to minimize their risks.

- Fire

- Road Traffic Accidents,

- Materials/ effluents spills i.e. asphalt, POL, etc.

- Natural disasters i.e. torrential rains, floods, earthquakes

- Occupational accidents i.e. fall, electrocutions, cave-ins

- Arsons, vandals

- Terrorist attacks, etc.

1.5 Responsibilities

1.5.1 Alerting

The following procedures will be prescribed for internal reporting of emergencies:

- The Emergency Response   Coordinator (ERC) will   awaken   on-site personnel, including visitors, of the nature of the emergency.

- The ERC will activate and deploy the concerned ERTs, and notify and inform the project person in-charge, emergency medical assistance/ Rescue 1122, fire department, Police etc., whoever and whenever needed.

- Concerned Project Manager (PM) or whosoever is in-charge of the project shall only be authorized to speak on contractors’ behalf to outside agencies (police, fire department, medical services, Media etc.) during an emergency situation.

- The ERC will identify any need for security measures at the Project Area during any emergency and will notify the concerned project or security person in-charge.

- When any person identifies an emergency situation, or the potential for an emergency situation, and reports it to the ERC, the ERC will then activate the Emergency Response Team.

- The supervision consultant shall be notified in writing within 24 hours of any emergency situation.

1.5.1 On Site Person In-charge

The Contractor’s on-site in-charge shall be responsible for handling emergency situations for concerned site. He will act as Emergency Response Coordinator (ERC) and shall be responsible for ensuring that all subcontractors, staff, on-site visitors and others adhere to the appropriate emergency response procedures as stated in this Plan.

1.5.2 Emergency Response Team (ERT)

Emergency Response Team shall be for matted at all camps, each team comprised of 3 or 5 members including and lead by the ER Coordinator or the concerned person in- charge. All the members of the ER Team shall be trained in basic First Aid, search & rescue and firefighting; that will further provide search& rescue, first aid and arrange transportation in case of accidents and emergencies, and extinguish fires, guide workers and staff to assembly points and other safe places. The ERT shall assist the ER Coordinator in accidents investigation. The contact details of the ERTs shall be posted at prominent places so that all workers are aware of them.

| Project Manager | Person Incharge (ERT) |
| --- | --- |
| Environmentalist/HSE Manager | HSE Coordinator (ERT) |
| Admin / Labour Team Lead | ERT Team Member |

1.5.3 Safety Training

On-site induction, Tool Box Talks (TBT), trainings shall be organized by the Emergency Response Coordinator/ HSE Officer/ Engineer regarding the health & Safety measures from potential and existing hazards, environmental protection, etc. New workers and staff shall be orientated regarding their work and the potential hazards, and safety from them. All trainings shall be recorded and communicated to the Supervision Consultant.

1.5.4 Site Inspections

The concerned HSE Officer/Engineer shall visit and inspect all the worksites, plants, machines, workshops, laboratories, working platforms, safety equipment etc., on regular basis, record and notify findings to the concerned Project Manager/ person in- charge. In- case of any observations i.e. unsafe equipment, un-safe acts, violations, absence of safety equipment, etc. required control and mitigation measures must be communicated to the concerned person Incharge and workers/staff must implement those measures immediately.

1.5.5 First Aid Post / Box

First aid post shall be established at all camps & plants or first aid box provided as per the requirement. The concerned ERTs shall be trained in basic first aid. The first aid post/ or the facilities where first aid box are kept shall be indicated prominently.

1.6 Emergency Response Procedures

1.6.1 Fire

Fire is a major cause of deaths, disabilities and deaths at house hold and work-places. Most of ten fires are caused by in appropriate and unsafe use of materials and equipment. All personnel at camp sand plants shall be trained in fire safety and use of fire Extinguishers. Fire extinguishers, no Smoking sings etc. shall be installed at all camps, offices, plants, stores, fueling stations, workshops etc. In case of fire, areas with potential to be affected shall be evacuated immediately and gathered at the assembly point, or any other safe zone. The ERTs shall be activated. The fire if at initial stages shall be extinguished with keeping in view personnel safety of employees/ workers at priority. The emergency services i.e. fire brigade, rescue 1122 shall be called immediately. All the fire safety equipment and signs shall be regularly inspected and maintained. The reasons for accidents shall be recorded; mitigation or control measures designed and communicated to all employees. The situation must be recorded and reported to the Supervision Consultant within 24 hours of the incident.

1.6.2 Road Traffic Accidents (RTA)

All the drivers at site shall be made aware regarding safe driving practices at the active construction site, borrow areas, camps to avoid accidents. All the drivers shall be trained, motivated and enforced in use of seat belt, obeying speed limit and traffic signs i.e. diversion, speed limits, U-turns, etc. which shall be installed at all required locations. In case of any RTA the concerned ERTs shall be activated; first aid shall be provided to injured person (God Forbid) if any on the spot, transported to the nearest emergency medical facility or the first aid post at the nearest concerned camp. The concerned ER Coordinator or the person in-charge shall be informed immediately. The reasons for accidents shall be investigated, recorded; mitigation or control measures designed and communicated to all employees through a series of trainings, orientations. The situation must be recorded and reported to the Assistant to Employer Representative (AER).

1.6.3 Materials / Effluents Spills

Spillage of liquids hazardous to workers’ health, wild life and property shall be avoided.

Concerned workers shall be trained in proper and safe handling of such materials. PPEs shall be provided and their unseen forced on the concerned workers. In case of liquids and effluents spills the spills must be controlled through spill control kit, collected and properly disposed which does not affect human or wild life. Spillage of significant amount shall be recorded and reported. Concerned workers shall be orientated on the findings of the reasons for spillage and control measures.

1.6.4 Natural Disasters

Since the project area in view of the past weather record is not subject and vulnerable to torrential rains, floods, earthquake; however, the camps shall be established at safe places, and in case of torrential rains, floods workers shall be alerted and mobilized to safe places prior to the impact. In case of earthquake all personnel shall evacuate settlements and gather at the assembly point or any other safe place. The ERTs shall be activated. In case of personnel injuries, first aid shall be provided on the spot and causality(s) transported to the nearest emergency medical facility/ hospital. In case of property damage, its causes shall be recorded and reported to the SC.

1.6.5 Occupational Accidents

All the workers and staff at active construction sites, camps, plants, and workshops shall be trained and orientated on occupational health and safety measures. Tool Box Talks (TBT) shall be organized by the concerned HSE Head/Manager, or site in-charge prior to any new activity or activity potentially hazardous. All such activities shall be recorded and reported accordingly. PPEs shall be provided to all workers and they shall be trained in their proper use and maintenance. All measures needed for the health and safety of workers and staff shall be put in place to a safe and healthy working environment. In case of falls from heights, electrocutions, cave-ins, etc., casualty(s); the ERTs shall  be activated; they shall provide first aid and transport causality(s) to the nearest emergency medical facility or the concerned nearest camp/ first aid post. Transportation arrangements must be made immediately and the concerned ER Coordinator or person in-charge must be informed immediately. The happening must be recorded and reported to the Assistant to Employer Representative (AER). The incident must be investigated, findings recorded, control measures devised; and communicated to all concerned, in order to avoid such happenings in future. Daily Tool Box Talk Format and Incident Investigation Format are also formatted for the project.

1.6.6 Arsons / Vandals

All the social disputes among the contractor and workers, subcontractors, and community shall be settled peacefully, with mutual consensus; however, in case of unpleasant situations safety of the workers and staff shall be the priority, and the nearest police station shall be informed to protect employees. Social framework agreements may be signed to sort out the community matters if any.

1.6.7 Terrorist Attacks

Contractor’s own security system will be launched to provide the Security to staff workers and labors and visitors and also security for foreigners, who shall escort them at all construction sites, plants and camps or wherever they move. No foreigners shall be allowed to move outside work areas without Police escort. For security, the contractors have also arranged private security to protect worksites, stores and camps at day and night and prevent, unauthorized trespassing. In case of any terrorist attack (God forbid), safety of the workers and staff shall be the prime priority; however, they shall be trained to protect themselves and seek and move to safe places, moreover the nearest Police/ Elite Force station shall be notified immediately for additional security and further safety.

1.6.8 Emergency Contact Number

Emergency Response personnel/Team will be available during the whole Project and will be utilized to assist during emergency situations or provide first aid as needed.

| Project Manager | Person Incharge (ERT) |
| --- | --- |
| Environmentalist/ HSE Manager | HSE Coordinator (ERT) |
| Admin / Labour Team Lead | ERT Team Member |

In case of any emergency, Safety Manager or designated person will be authorized to Contact the external public safety agencies/emergency services. Following are the contacts of emergency services.

| Fire | 16 |
| --- | --- |
| Police | 15 |
| Edhi Rescuer | 115 |

1.7 Fire Prevention and Fire Fighting

1.7.1 Purpose

The purpose of this procedure is to highlight the fire hazards, precautions and suppression facilities required for the Projects. The responsibilities of various staff for this purpose are stated in the table below:

| Sr. No. | Staff | Responsibilities |
| --- | --- | --- |
| 1 | Construction Manager | Ensures that procedure is understood and being applied appropriately. |
| 2 | Site Health, Safety and Environment Staff | Responsible for the improvement of this procedure and monitoring compliance and introducing whatever training is required for fire teams, fire wardens and personnel. |
| 3 | HSE Supervisors | Office and Camp “FIRE ACTION” notices are displayed at strategic locations, and that sufficient “Emergency Assembly Points” are set up, and regular practice evacuation drills are performed. A number of office/ camp staff are being assigned with the responsibility to act as fire wardens Office/camp staff (in particular new starts) has been briefed and keeps updated on fire and emergency response procedures. Awareness to control flammable materials being used on the premises. Daily checks of the office to ensure that walkways are not blocked, carpet and other fittings are properly secured, fire doors kept closed and that the means of escape are available. Kitchen areas are being monitored to ensure that electrical fittings have been turned off after use and do not present a fire hazard. Training for the use of fire extinguishers has been to staff/workers and signs have been displayed on various locations indicating the emergency procedures. ‘No Smoking’ policy applies in the Meetings/conference rooms. Common social areas, mess halls and toilets, within the camp, are checked before closure of possible ignition sources following each meal time and at end of evening. |
| 4 | E & M (Electrical and Mechanical) Manager | E & M Manager shall ensure that the following control measures are in place: - Workshops are maintained in a neat and tidy manner and that waste oil, rags and other flammable materials are removed at the end of each shift or as necessary. - Maintenance crews are properly trained for the use of fire extinguishers, raising the alarm and fire hazards on the work place. - Battery recharging will be conducted in well ventilated separate areas, with no smoking signs and fire extinguishers in place - Areas around pedestal grinders and other hot work type activities are kept free of combustibles. - Welding and burning shall be screened and controlled to prevent fire risk and exposure to personnel - That flammable liquid such as gasoline, diesel etc. are not used for cleaning purposes and their container is marked as per the containment. - Provision of adequate storage areas that are located in places where exits, passage ways and stairways are not adversely affected - Flammable liquids are stored safely, in a neat and tidy manner with adequate HSE signs provided |

1.7.2 Cutting and Welding

Particular care will be taken while carrying out hot work operations on the locations where combustibles are available. This will include precautions such as; inspection of the surrounding area, removal of any combustible materials, protection by fire blankets and provision of fire extinguishers.

Compressed gas cylinders shall be closed when not in use, and shall be stored, properly secured and used in an upright position at all times. Protective end caps will be fitted when being moved or transported (unless by cylinder trolleys).

Compressed gas cylinders shall be kept clear of electrical equipment and cabling where they are part of an electrical circuit

1.8 Hazard Analysis and Risk Management

Hazard Analysis and Risk management techniques shall be adopted so that potential hazards are identified and evaluated prior to execution, thereby enabling either substitution or adoption of control techniques.

1.8.1 Purpose

To ensure that all steps are taken to control hazards identified in a job and provide a safe working environment.

1.8.2 Typical Hazards

Following are examples of typical hazards that can be encountered:

- Falling objects

- Welding

- Gas cutting

- Grinding

- Erection of steel work

- Installation of pipeline etc.

- Transportation / Lifting of heavy equipment

- Excavation Work

1.8.3 Control Hazard

The hierarchy of Risk/Hazard Control is used to determine risk reduction measure in order of their effectiveness, as follows:

- Elimination or substitution of the task / job step or substance.

- Engineering Control Including guarding and mechanical aids such as scaffolding, extraction ventilation and alike.

- Administrative Controls Including permits, training, signage, reduction in time or personnel exposure personal Protective Equipment

1.9 Personal Protective Equipment

1.9.1 Purpose

This procedure has been developed to ensure that all site personnel on the project are provided with, and wear or hold, appropriate Personal Protective Equipment (PPE) to protect them against work related hazards which may endanger their health, safety and environment.

1.9.2 Scope

This procedure applies to all contractor personnel employed including all of Sub- contractor’s deployed at the project.

1.9.3 Definitions

Personal Protective Equipment is defined as equipment designed to be worn or held by personnel to protect themselves against work related hazards which may endanger their health and safety.

1.9.4 Instructions

The risks posed in any particular work activity shall be assessed, and adequate PPE selected in accordance with the following criteria:

- Gives protection against risk(s) without itself leading to any increased risk is suitable for the personnel involved in the work including correct fitting. Is compatible with the work activity.

1.9.5 Basic Personal Protective Equipment (PPE)

Basic and minimum PPE requirements for the safe execution of proposed project are as:

- Safety helmet of plastic construction, manufactured in accordance with standard.

- Safety footwear with steel toe protection manufactured in accordance with standard.

- The construction and sole materials shall be chosen in accordance with the activity and adequate for the place of work.

- In addition to above, EPC contractor shall also supply coveralls to employees working in workshops.

1.9.5.1 Eye and Face Protection

Suitable protective goggles, face shield or screens shall be worn by personnel involved in, assisting with or adjacent to any activity where there may be a danger of projected debris, dust, sparks or other particles; corrosive fluids or mists; excessive heat, light or other harmful radiation.

1.9.5.2 Respiratory Protection

Respiratory Protective Equipment shall be available to all persons who are exposed to any situation in which there is a possibility of the atmosphere being or becoming deficient in oxygen or containing any harmful substance, whether particle, dust mist, vapors or gas.

1.9.5.3 Hearing Protection

Hearing protection shall be made available to all workers exposed to noise levels of 85 dB (A) or above. The selection of type of protection shall be in accordance with the type of noise hazard and the work being performed. In all work environments where the noise level is at or above 85 dB (A), prominent signs in English and in local language shall be displayed indicating the need for ear protection. A visual sign in the form of a line drawing of ear muffs shall also be displayed.

1.9.5.4 Hand and Arm Protection

Adequate hand and arm protection shall be available for all manual labour. The type of protection worn shall be selected according to the hazard to be protected against. These include but not limited to:

- Impacts, cuts, abrasions and infections.

- Extreme temperatures.

- Chemical, toxic, corrosive and other hazardous substances.

| WORK ACTIVITIES | SUITABLE PPE |
| --- | --- |
| Excavators, breakers, Chippers, drillers. | Protective goggles, hand gloves, Safety helmets and Safety shoes. |
| Mixing cement, concrete, Lime mortar, asphalt material, refractory material. | Safety goggles, gloves and protective foot wears. |

| Electricians | Insulated tools, Rubber hand gloves and electrical resistant shoes, mat. |
| --- | --- |
| Grinders | Protective goggles and leather hand gloves |
| Gas cutters, welder’s helper | Colored goggles, leather hand gloves. |
| Welders | Welding screen, safety shoes with rubber sole, Leather hand gloves. |
| Workers engaged in | Dust mask, hand gloves |
| High noise level area i.e. D.G operator working, near piling work, compressor operator etc. | Ear plug / ear muff |
| Working in dusty | Dust mask, Safety goggles. |

1.9.5.5 Harnesses

Harnesses  shall  be  provided,  worn  and  properly  secured  in  all  work situations, when other safeguards such as nets, planking, or scaffolding  cannot be used. Such situations include, but are not limited to:

- Working on scaffolding.

- Work on any high structure, whether in construction or batching plant site.

1.9.5.6 Transportation

Only vehicles necessary to the construction operation shall be permitted on site.

- It shall be ensured that only authorized and competent personnel are allowed to drive vehicles.

- Loads shall be within the safe weight limit for the vehicle and shall not project beyond the vehicle body in such a manner as to present a hazard to other vehicles, pedestrians or adjacent structures.

- Passengers shall not be carried unless a proper seat is provided.

- Personnel shall not get on or off any vehicle whilst it is in motion.

- All vehicles shall be parked on level ground with the hand brake applied.

- Keys shall be left in the ignition unless at an authorized car park.

- Vehicles shall not block access or emergency points.

- All drivers of vehicles shall be in possession of a valid license for the class of vehicle.

- All drivers shall go through an orientation before entering the site.

- Vehicle Inspection is attached as Annex “A”.

1.9.5.7 Speed Limits

During the project laid down speed limits for the vehicles will be strictly observed.

1.9.5.8 Drivers’ Responsibilities

Drivers’ responsibilities include:

- Take responsibility for the vehicle, the load and passengers being carried.

- Checking of the vehicle prior to the start of the journey.

- Ensuring that all personnel in the vehicle are wearing seat belts.

- Shall not pick up unauthorized persons.

Sub-Annexure-A

Sub-Annex-B

# Annexure-IX Framework for Dust Management Plan

Dust Management Plan

General

This plan is prepared for the project "Augmentation of Surface Water Supply for Khayaban-e-Sir Syed from Khanpur Dam and Formation of DMAs". The purpose of this plan is to describe the measures that the project shall take to ensure that the risk of emissions from dust generated by site operations during construction are minimized and that best practice measures are implemented.

Dust emissions from construction can cause ill health effects to Contractor staff along with nuisance and annoyance to members of the local community. Dust will be controlled through:

- Elimination

- Reduction/Minimization

- Control

This dust management plan shall be implemented based on the measures already provided in the Environmental Management Plan (EMP) relating to controlling dust emissions.

Methodology

The following methodology will be undertaken for each project section:

Step 1 – Identify the dust generating activities

Construction activities that are likely to produce dust will be identified.  The activities that will be taken into account are:

- Haulage Routes, Vehicles and /Concrete Batching Plant

- Roads, surfaces and public highways

- Excavations for lying of new conductance and replacement of distribution pipelines

- Static and mobile combustion plant emissions

- Materials Handling, Storage, Spillage and Disposal

- Storage of material

- Stockpiles

- Spillages

- Storage of Waste

- Site Preparation and Restoration after Completion

- Earthworks, excavation and digging

- Storage of spoil and topsoil

- Demolition

- Construction and Fabrication Processes

Step 2 – Identify Sensitive Receptors

Sensitive receptors have already been identified. The nature and location of the sensitive receptors will be taken into account when implementing control measures.

Step 3 Implement Best Practice Measures to Control

Based on the nature of the activity producing the dust, the likelihood of dust being produced and the possible consequence of dust based on the sensitive receptors, the most effective control measure will be identified and implemented.

Step 4 – Monitor effectiveness of control

Construction Supervision Staff (CSC) will have the responsibility to ensure that dust control measures are being implemented and are effective.

Step 5 – Record and report result of monitoring

All inspections, audits and results of monitoring will be recorded and kept as part of the site filing system.

Method Statements and Risk Assessments

The Contractor’s Risk Assessments and Method Statements will be required to be approved by the CSC prior to commencing work and will be required to contain environmental aspects of the task, including dust control measures where required.

Where dust has been identified within the risk assessment as a significant issue, the method statement will be required to cover the following:

Methods and materials that will be used to ensure that dust generation is minimized.

The use of pre-fabricated materials where possible.

Optimum site layout:

Dust generating activities to be conducted away from sensitive receptors

Supply of water for damping down.

Good housekeeping and management

All employees will be briefed on the Risk Assessment and Method Statement before starting work.

Training

All Contractor staff will be required to attend training seminars as already mentioned in the EMP document. A site-specific induction will also be required before being allowed to work on site. These will include site-specific sensitive receptors and details regarding dust control measures to be taken.

Toolbox talks on air pollution and minimizing dust emissions will be provided on a regular basis to Contractor staff.

Identification of Dust Generating Sources and Control Methods

| Haulage Routes, Vehicles and /Concrete Batching Plant |  |
| --- | --- |
| Dust Source | Dust Control Methods |
| Major haul roads and traffic routes | Haul roads will be dampened down via a mobile bowser, as required. |
| Public Roads | Road sweeper will be used to clean public roads as required. |
| Site traffic management | Site traffic will be restricted to constructed access roads as far as possible. Site speed limit will be set at 10 mph as this will minimize the production of dust. |
| Road Cleaning | A mechanical road sweeper will be readily available and used. |
| Handling, Storage, Stockpiling and Spillage of Dusty materials |  |
| Material handling operations | The number of times a material will have to be handled will be kept to a minimum to prevent double handling and ensure dusty materials are not handled unnecessarily. |
| Transport of fine dusty materials and aggregates. | Closed tankers will be used or sheeted vehicles. |
| Vehicle loading/unloading materials on to vehicles and conveyors. | Dusty materials will be dampened down Drop heights will be kept to a minimum and enclosed where possible. |
| Excavation for Lying new pipelines |  |
| Excavation | Excavation will be carried out by applying green sheets on the edges, especially near Environmental Sensitive Receptors, already highlighted in the IEE report. |
| Handling of Mucking Material | Mucking material will be stored in bonded area to avoid spillage and generation of fugitive dust |
| Loading of material on dumpers | Mucking material will be damped down before loading on dumpers to reduce fugitive dust emission. |
| Storage of Materials |  |
| Bulk cement, bentonite etc. | Bentonite will be delivered in tankers and stored in dedicated enclosed areas. Bulk cement will be transported through tractor trollies or trailers. |
| Fine dry materials | These will be protected from the weather and by storing in appropriate containers and indoors, where necessary. |
| Storage location | Material will be stored in dedicated lay-down areas. |
| Storage of Stockpiles |  |
| Stockpile location | Stockpiles will be placed so as to minimize double handling and facilitate the site restoration. |
| Building stockpiles | Stockpiles, tips and mounds will not be stored at an angle greater than an angle of repose of the material. |
| Small and temporary stockpiles | Where possible, stockpiles will be placed under sheeting. Dusty material will be damped down. Wind barriers (protective fences) of a similar height to the stockpile will be erected, if required. |
| Site Preparation and Restoration |  |
| Earthworks, excavation and digging | These activity areas will be kept damp where required and if possible, will be avoided during dry and windy periods. |
| Completed earthworks | Surfaces will be stabilized by re-vegetation as soon as possible, where applicable. |
| Construction and Fabrication Process |  |
| Cutting, grinding, drilling, sawing, trimming, planning, sanding | These activities will be avoided wherever possible. Equipment and techniques that minimize dust will be implemented. Water will be used to minimize dust. |
| Cutting roadways, pavements, blocks | Water sprinkling to be used. |
| Angle grinders and disk cutters | Best practice measures will be used such as dust extraction. |

Monitoring Arrangements

Monitoring will be conducted at sensitive receptor locations in the project area as provided in the EMP. Furthermore, at locations where PM levels are exceeding applicable guidelines, additional stringent measures will be implemented at the respective location(s) in the project area to ensure dust levels are controlled as far as possible.

# Annexure-X Framework for Noise and Vibration Management Plan

Objective

This management plan defines the measures to control and limit noise emissions and vibration levels, at residential properties and other sensitive receptors in the vicinity of the Project.

General Requirements

Best Practicable Means (BPM) of noise control will be applied during construction works to minimize noise (including vibration) at neighboring residential properties and other sensitive receptors arising from construction activities

The general principles of noise management are given below:

Control at source:

Equipment – noise emissions limit for equipment brought to site.

Equipment – method of directly controlling noise e.g. by retrofitting controls to plant and machinery.

Equipment - indirect method of controlling noise e.g. acoustic screens.

Equipment - indirect method of controlling noise e.g. benefits and practicality of using alternative construction methodology to achieve the objective e.g. vibratory piling techniques or hydro- demolition as opposed to more conventional but noisier techniques; selection of quieter tools/machines; application of quieter processes.

Control across site by:

- Administrative and legislative control,

- Control of working hours,

- Control of delivery areas and times,

- Careful choice of compound location,

- Physically screening site,

- Control of noise via Contract specification of limits,

- Noise Monitoring, to check compliance with noise level limits, cessation of works until alternative method is found.

- Many of the activities which generate noise can be mitigated to some degree by careful operation of machinery and use of tools. This may best be addressed by tool box talks and site inductions.

The maintenance of good community relations is vital. Experience shows that construction noise has the potential to cause disturbance but can be tolerated if prior warning and explanation has been given to residents. In particular advice regarding the nature of construction works, the duration of the works and mitigation measures to be implemented can help to reduce people’s reaction to noise.

Contractors will consult local residents/communities regarding works and to give them details of a responsible appointed person on site who will be able to deal with queries.

Construction working hours should be carefully managed. All works should take place between normal working hours 07:30 and 18:30, Monday to Friday with the exception of restricted works.

Noise and Vibration Control Measures

In addition to specific requirements of the Local Authority, the Contractor will be required to adopt the following more specific measures:

Control measures

Without prejudice to the other requirements of this section, the Contractor shall comply with the recommendations set out in BS5228:2009 and in particular with the following requirements:

- Vehicles and mechanical plant will be maintained in a good and effective working order and operated in a manner to minimise noise emissions. The contractor will ensure that all plant complies with the relevant statutory requirements;

- Site vehicles will be equipped with broadband, non-tonal reversing alarms;

- Compressor, generator and engine compartment doors will be kept closed and plant turned off when not in use;

- All pneumatic tools will be fitted with silencers/mufflers;

- Care would be taken when unloading vehicles to avoid un-necessary noise;

- Maintenance operations will be undertaken at distance from noise-sensitive receptors;

- Reduce the speed of vehicle movements;

- Ensure that operations are designed to be undertaken with any directional noise emissions pointing away from noise-sensitive receptors;

- Vehicles should be prohibited from waiting within the site with their engines running or alternatively, located in waiting areas away from sensitive receptors;

Vibration Mitigation

All vibration producing machinery will be identified by the contractor and vibration producing machinery will be operated at low frequency to reduce the vibration impacts.

- Vibration monitoring will be carried out in accordance with German Standard DIN 4150 (heritage structures) and BS 7385: Part 2 – 1993 (other structures).

- vibration monitoring will be undertaken at the nearest buildings to the construction sites during times of vibration intensive works

- Where exceedances of the criteria are recorded, corrective actions would be implemented where soils at risk of vibration induced settlement are identified.

Noise and vibration monitoring

A regular programme of noise and vibration monitoring shall be implemented as a minimum in accordance with of this document.

The Contractor will submit the proposed method, the frequency and the location of monitoring site to the Planning Authority for agreement prior to commencing works. Noise baseline levels will be agreed prior to commencement of construction.

Reference:

https://proaccionaau.blob.core.windows.net/media/4gbpg4vk/ctp-noise-and-vibration-management-plan.pdf.

# Annexure-XI Framework for Asbestos Management Plan

FRAMEWORK FOR ASBESTOS MANAGEMENT PLAN

The IFC (2007) guidelines for asbestos-containing materials (ACM) requires the management of replaceable AC pipelines which contain asbestos. This need to prepare a written asbestos management plan. The plan should set out how the risks from asbestos are to be managed and the procedure for ensuring that employees or others handle the asbestos-containing materials (ACMs) as per set procedures. The ‘Asbestos Register’ should be used to keep the record of asbestos.

The register for asbestos management shall be maintained according to following format.

| 1. Location / Name of the project |  |  |  |
| --- | --- | --- | --- |
| 2. Responsibilities |  |  |  |
| Who is the site Duty Holder? | Name: |  | Title: |
| Who is the person with day-to-day operational responsibility for managing asbestos material on site? | Name: |  | Title: |
| Who is responsible for undertaking the annual visual re inspection and updates to the asbestos register for the site? | Name: |  | Title: |
| Who is the author of this plan? | Name: |  | Title: |
| 3. Reviewing this Management Plan |  |  |  |
| Date this Management Plan was produced |  |  |  |
| Date of next review |  |  |  |
| 4. How the location and condition of asbestos-containing material is recorded |  |  |  |
| How is information about asbestos in the premises recorded |  |  |  |
| Where are hard copies of the register and plan kept on site? |  |  |  |
| 5. Results of the risk assessments and action required (if any) |  |  |  |
| What was the date of your last Visual Re-inspection |  |  |  |
| What were the concerns and high risk area that required immediate action? |  |  |  |
| 6. Monitoring arrangements for asbestos |  |  |  |
| What are the arrangements for monitoring the known/presumed asbestos-containing materials, to ensure that they remain in good condition and that there is no increased risk of disturbance? |  |  |  |
| 7. How information about asbestos is passed to those that need it |  |  |  |
| How are staff informed and updated on asbestos related issues? Including location of ACM’s. |  |  |  |
| What is the procedure for ensuring that contractors and others check the asbestos register before starting work? |  |  |  |
| How are routine maintenance and construction works monitored during the activity? |  |  |  |
| Who is the nominated person that will act on behalf of the duty holder in the event of absence? | Name: | Title: |  |
| What methods are used to make staff aware of the location of Asbestos Containing materials? |  |  |  |
| 8. Training |  |  |  |
| Asbestos Awareness 1 2. 3. | Date of Training 1 2. 3. |  |  |

| 9. Action Plan |  |  |  |
| --- | --- | --- | --- |
| High risk items / major concerns / training requirements. | Remedial action required | Target date for action | Date completed |

# Annexure-XII Template for Site Specific EMP

GUIDE FOR DEVELOPMENT OF SSEMP

- Step 1: Define Boundaries

- Step 2: Identify Sensitive Receptors

- Step 3: Specify construction activities

- Step 4: Conduct Risk Assessment

- Step 5: Assign Environment Management measures

- Step 6: Prepare Site Plans

- Step 7: Prepare Environment Work Plans (if required)

- Step 8: Monitoring

Step 1: The project area needs to be clearly defined.

Step 2: The mapping of sensitive receptors has already been conducted and needs to be presented clearly in a map.

Step 3: The tentative construction activities to be conducted are as follows:

- Site Surveying and Vegetation (Trees and plants) Clearance

- Establishment of Work Camp and access roads

- Dismantling of existing structures including Utilities

- Preparation of ground for Landscaping

Step 4: The Risk Assessment matrix template is provided in the table below.

Risk is assessed as the likelihood that the activity will have an effect on the environment as well as the consequence of the effect occurring. It is often described like this:

Risk = Likelihood × Consequence

Likelihood Scale

| Likelihood | Definition | Scale |
| --- | --- | --- |
| Certain | Will certainly occur during the activity at a frequency greater than every week if preventative measures are not applied | 5 |
| Likely | Will occur more than once or twice during the activity but less than weekly if preventative measures are not applied | 3 |
| Unlikely | May occur once or twice during the activity if preventative measures are not applied | 2 |
| Rare | Unlikely to occur during the project | 1 |

Consequence Scale

| Consequence | Definition | Score |
| --- | --- | --- |
| Catastrophic | The action will cause unprecedented damage or impacts on the environment or surrounding community e.g. extreme loss of soil and water resources and quality from storm water runoff extreme pollution of soil and water resources including major contamination from hazardous materials widespread effects on ecosystems with deaths of fauna/flora widespread community impacts resulting in illness, injury or inconvenience loss or destruction of archaeological or historical sites Occurrence will almost certainly result in the work being halted and a significant fine. | 5 |
| Major | The action will cause major adverse damage on the environment or surrounding communities’ e.g. major loss of soil and water resources and quality from storm water runoff major pollution of soil and water resources including contamination from hazardous materials significant effects on ecosystems with isolated deaths of non-vulnerable flora and fauna significant annoyance or nuisance to communities major damage to or movement required to archaeological or historical sites occurrence may result in work being halted and a fine | 3 |
| Moderate | No or minimal adverse environmental or social impacts e.g. No measurable or noticeable changes in storm water quality. Water quality remains within tolerable limits little noticeable effect on ecosystems no or isolated community complaints no or unlikely damage to archaeological or historical sites no likelihood of being fined | 2 |
| Minor | No or minimal adverse environmental or social impacts e.g. No measurable or noticeable changes in storm water quality. Water quality remains within tolerable limits little noticeable effect on ecosystems no or isolated community complaints no or unlikely damage to archaeological or historical sites no likelihood of being fined | 1 |

Risk Score Table

| Likelihood | Consequence |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Catastrophic | Major | Moderate | Minor |
|  | Certain | 25 | 15 | 10 | 5 |
|  | Likely | 15 | 9 | 6 | 3 |
|  | Unlikely | 10 | 6 | 4 | 2 |
|  | Rare | 5 | 3 | 2 | 1 |

Risk: 	Significant: 15-25

Medium: 6-10

Low 1-5

Any Medium to Significant risk requires an environmental management measure to manage the potential environmental risk. Judgment will be required concerning the application of an environmental management measure to mitigate low risk situations.

The higher the risk the more intensive the required mitigation measure will need to be; e.g. where site sedimentation is deemed to be low risk, then silt fences may be needed but as the risk increases, then sediment traps may be required. The selection of the appropriate mitigation measure will require judgment based on the level of risk and the specific site parameters.

Step 5: The Environmental Management measures are to be extracted from the IEE study for the project and should be added in the last column of the table below.

| No. | Construction Activity | Hazards to Consider | Likelihood that the site or sensitive receptors will be affected? | Consequence of the site or sensitive receptors being affected? | Risk Score (consequence x likelihood) | Environmental Management Measures |
| --- | --- | --- | --- | --- | --- | --- |
| i | Site Surveying & vegetation clearance | Damage to vegetation beyond project footprint |  |  |  | These can be taken from the EMP provided in the IEE report (If Risk Score is 6 or more) |
|  |  | Erosion of exposed areas and sediment |  |  |  |  |
|  |  | Loss of topsoil |  |  |  |  |
|  |  | Dust generation |  |  |  |  |
|  |  | Noise |  |  |  |  |
| ii | Establishment of Work Camp, Batching plant etc. | Soil deposited onto roads from tires |  |  |  |  |
|  |  | Stockpile erosion |  |  |  |  |
|  |  | Noise & Vibration |  |  |  |  |
|  |  | Traffic congestion |  |  |  |  |
|  |  | Fuel spills |  |  |  |  |
| iii | Dismantling of and existing structures including Utilities | Noise and vibration |  |  |  |  |
|  |  | Dust generation |  |  |  |  |
|  |  | Community safety |  |  |  |  |
|  |  | Worker safety |  |  |  |  |
|  |  | Traffic Congestion |  |  |  |  |
| iv | Preparation of Sub-Base | Noise and vibration |  |  |  |  |
|  |  | Dust generation |  |  |  |  |
|  |  | Traffic Congestion |  |  |  |  |
| v | Camping | Noise and vibration |  |  |  |  |
|  |  | Dust generation |  |  |  |  |
|  |  | Traffic Congestion |  |  |  |  |
|  |  | Community safety |  |  |  |  |
|  |  | Labor safety (PPEs) |  |  |  |  |
| vi | Landscaping | Dust generation |  |  |  |  |
|  |  | Sediment runoff |  |  |  |  |
|  |  | Failure of vegetation to take root |  |  |  |  |

Step 6: The Site plans are a critical part of the SSEMP and will need to be prepared, otherwise the UMDS will consider the document as incomplete.

The site plan will need to provide the following:

- Indication of North and scale

- Existing and planned supporting infrastructure (e.g. access roads, water supplies and electricity supplies)

- Location of planned work

- Contours

- Drainage systems

- Locations of sensitive receptors

Step 7 (if required): The completed SSEMP provides details of all the environmental management requirements for all stages of the construction process. For individual work teams who are responsible for only a small part of the overall construction works it can be confusing as to what is required for their particular work component. For example, the work team responsible for stripping soil for the construction areas are not going to be interested in the requirements for pouring concrete for footings and foundations. However, it is essential that the soil stripping team knows exactly what to clear and what to leave and where to put stockpiles of soil for later use.

In situations where different work activities are required at different times or at different locations, environmental work plans can be prepared. These are similar to the work method statements that are often produced for major construction projects.

Step 8: A detailed monitoring plan will be provided along with frequency and responsibilities to ensure all key environmental parameters are monitored to ensure compliance with both national and ADB requirements.

Template for SSEMP

- Introduction

- Project Overview

- Scope of SSEMP

- Objectives of SSEMP

- Map of Sensitive Receptors

- Construction Activities

- Activities

- Risk Assessment

- Risk Assessment Matrix & Mitigation Measures

- Site Plan(s)

- Environmental Monitoring Plan

- Instrumental Monitoring of Environmental Parameters by Contractor as per EMP

- In-house monitoring

- Third Party environmental monitoring

- Visual monitoring of Environmental Parameters by Contractor as per EMP

- Responsibilities

- Organizational Responsibilities and Communication

- Responsibility of EA

- Responsibility of Construction Supervision Consultant (CSC)

- Responsibility of Contractor

- Responsibility of EPA (Punjab and Federal)

# Annexure-XIII Tool for Socio-economic Survey

SOCIO-ECONOMIC BASELINE SURVEY OF THE PROJECT AREA

Name of Project _ _________________

Name of Interviewer: _____________				Date: _______________

a) Settlement: ___________________		(b) Union Council: _______________________

c) Tehsil: _______________________		(d) District: _____________________________

_____________________________________________________________________________

1.	Name of Respondent		________________________

2.	What is your father’s name 	________________________

3.	What is your age

Age years	15 – 25	________

25 – 35	________

35 – 45	________

Above 45	________

4.	Marital Status.

1.	Married		2.	Unmarried

5.	What is your caste /ethnic group		_____________________

6.	What is your mother tongue?		_____________________

7.	What is your qualification?

1.	Illeterate		2.	Primary		3.	Middle

4.	Metric			5.	Intermediate	6.	Above Inter

8.	What is your profession		____________________________

- Which of the following facilities are available in your house

1.	Electricity		2.	Water Supply

3.	Gas			4.	Telephone

5.	Sewerage

- What are the sources of water for your domestic use

1.	Public Water Supply	2.	Hand Pumps

3.	Channel		4.	Any other

- In your opinion, should this Project be implemented here

1.	Yes			2.	No

If yes, then reasons				If no, then reasons

________________________			________________________

________________________			________________________

________________________			________________________

- In your opinion, what will be possible impacts of the construction of this project area project?

During Construction	__________________________________________________

__________________________________________________

__________________________________________________

After Construction	__________________________________________________

__________________________________________________

__________________________________________________

- What protective measures do you suggest to safeguard your interests

_________________________________________________

_________________________________________________

________________________________________________

- Is there any health institution in this locality

1.	Yes		2.	No

If yes, then what is that	__________________________

__________________________

- Is there any educational institution in this locality

1.	Yes		2.	No

If yes, then what is that		__________________________

__________________________

- Is there any historical /archeological monument in this locality

1.	Yes		2.	No

If yes, then what is that		__________________________

__________________________

- Is there any shrine /mosque /graveyard in this locality

1.	Yes		2.	No

If yes, then what is that?

Shrine		__________________________

Mosque	__________________________

Graveyard	__________________________

# Annexure-XIV Attendance Sheets of FGDs

# Annexure-XV: Public Consultation Minutes for Solar Installation Development & Public Hearing Minutes

GOVERNMENT OF PAKISTAN

PAKISTAN ENVIRONMENTAL PROTECTION AGENCY MINISTRY OF CLIMATE CHANGE & ENVIRONMENTAL COORDINATION

ISLAMABAD

MINUTES OF THE PUBLIC HEARING

Subject: ENVIRONMENTAL IMPACT ASSESSMENT (EIA) REPORT OF AUGMENTATION OF WATER SUPPLY AND FORMATION OF DMAS IN KHYABAN-E-SIR SYED RAWALPINDI.

The Public Hearing of EIA Report of the subject project was held on 13th June 2024 at 11:00 A.M. at Al Makkah Banquet Hall, IJP Road, Sector 1 Khyaban-e-Sir Syed, Rawalpindi under the chairmanship of Director (EIA/Mont), Pak-EPA. List of participants is enclosed as Annexure-A.

2. The Public Hearing commenced with the recitation of the Holy Quran.

3. The chair gave introductory/welcoming remarks.

4.This was followed by a detailed presentation by Mr. Mujahid (Environmental Consultant (EC), EDCM) in which the project details. Environmental & social impacts and their mitigation measures during the construction and operational phase of the project and the proposed EMP were discussed.

5. The floor was thereafter formally opened for questions by the participants/stakeholders.

6.

| Question / Answer session |  |
| --- | --- |
| Question from Participants | Reply |
| Dr. Saeedullah Khan Advocate, Chairman UC-90 (Sector 1-10) expressed apprehension that the proposed project would damage infrastructure of the area and operation of equipment would disseminate vibrations to the houses of sector 1-10. He inquired about route of the proposed project and source of water. | Mr. M. Mujahid, Environment Consultant (EDCM) responded that the proposed project would not involve road breaking and hence there would not be any vibrations going to sector 1-10 houses. He responded that the proposed project would pass through greenbelt land adjacent to IJP road and service road of sector 1-10. He apprised that source of water would be Khanpur lake/dam. |
| Dr. Ilmas, Assistant Professor AIOU Islamabad, inquired about responsibility of implementing mitigation measures proposed in EMP for the proposed project. She also queried about rehabilitation plan for proposed greenbelt responsible for implementing mitigation measures proposed in EMP for the destruction. | Mr. M. Mujahid, Environment Consultant (EDCM) responded construction contractor would be proposed project and PMU&ADB would supervise it. He informed that excavated material would be used for refilling dugout after laying water supply line. |
| Ms. Saima Zulqarnain, Student AIOU Islamabad, asked about area of Khyaban-e-Sir Syed to be covered in terms of water supply and its futuristic expansion. | Mr. M. Mujahid, Environment Consultant (EDCM) responded that three union councils viz. UC-10, UC- 11 and UC-12 of Khyaban-e-Sir Syed would be covered; it's a pilot project and it would be expanded to other areas depending on its success. He added that there would be 100% water metering and flow meters would be installed on all lines to gauge supply, consumption and loss etc. |
| Mr. M. Asif, resident of 1-10, inquired whether residents of 1-10 might water from this project. | Mr. M. Mujahid, Environment Consultant (EDCM) responded in the negative. |
| .Ms. Ayesha Zakir, Student IIU Islamabad, informed that local community in particular on both sides of IJP road had to face many hardships (traffic congestion, dust, vehicular emissions etc.) during execution of IJP road remodeling project. She inquired about measures that would be taken to address the highlighted issues during execution of the proposed project. She also asked about working hours of the project execution. She also inquired about possibility of mixing of sewage from sewerage line into water supply line. | Mr. M. Mujahid, Environment Consultant (EDCM) responded that mitigation measures suggested in EMP would be implemented to minimize/eliminate adverse impacts. He added that construction work would be undertaken generally from 8am to 5pm. He answered that protection cover would be used to secure water supply line wherever it would be required. |
| Ms. Mehreen Fatima, Student IIU Islamabad, asked about measures to address monsoon flooding during project execution. | Mr. M. Mujahid, Environment Consultant (EDCM) answered that less construction activity and dewatering would be employed to tackle likely monsoon flooding during project execution. |
| Mr. Yaqub Ali Khan, DIG Forests, MOCC&EC, raised the question about the operational life of the project. He also queried about tree plantation plan. He hoped that water wastage would reduce given water metering in the project. He anticipated judicious utilization of ADB loan money for the project. | Mr. M. Mujahid, Environment Consultant (EDCM) responded that as regards HDPE pipe, its life is 50 years and other components 30 years. He apprised that tree plantation plan has been given in the EIA report and would be implemented. |
| Mr. Khurram Shazad, from Cyber Security, raised the question that is What will be the cost for EMP implementation and Bifurcation? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that 1.89 million is the annual cost for the EMP implementation and bifurcation. It includes costs of environmental monitoring of groundwater, air quality, noise levels and wastewater analysis. On the other hand, 1% of total project cost is allocated for trees plantation. |
| Mr. Abdullah Ahmad Awan, from PMD department, raised the question that is How much amount Is allocated for tree plantation and how this plantation will be carried out. | Mr. M. Mujahid, Environment Consultant (EDCM) replied 1% of total project cost is allocated for trees plantation. It will be carried out in collaboration with PMU and CDA. Local species and plantation sites will be identified. |
| Ms. Horiya khan, Director of Student, queried about How will you recover the cost of water if people install their own water pumps? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that it is very costly for people to install private pumps as water table is very deep. On the other hand, it is not permissible for residents to install private pumps. |
| Mr. Farman Ullah Khan, from IMARAT Group of Companies asked about Who will regulate the net metering system? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that WASA will regulate the net metering system. |
| Mr. Muhammad Uman from EPA, asked about How will you control the water leakage and contamination from sewerage pipes? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that testing pit will be dug out to differentiate and identify existing pipelines of gas, sewerage etc. New pipeline will be installed at significant distance from them to avoid water contamination. High quality material and engineering techniques will be utilized to avoid leakage. |
| Mr. Muhammad Zaid ul Hassan from EPA, raised a query i.e. How will you manage traffic during construction? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that project does not involve massive construction activity or use of any heavy machinery. Therefore, traffic will not be disturbed by this project. |
| Mr. Waqas Afzal, Deputy Director Safeguards will asked i.e. How will you manage construction related solid waste? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that Environment Consultant (EDCM) replied that construction related solid waste will be used for filling. There will not be very deep excavations. If any solid waste material is not managed in filling, it will be thrown far away at designated solid waste site by contractor.. |
| Ms. Sehrish Kurshaid from PMU- DREAMS-Lahore raised a query i.e. Are there any of tree cuttings involved in this project? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that no trees will be cut in this project as the project is based on very small digging line across the roadside. |
| Mr. Muhammad Umar from PICIP, LG&CDD Punjab asked about is there any road cuttings involved in the project? | Mr. M. Mujahid, Environment Consultant (EDCM) replied that road cuttings are not part of project. Pipeline will be passed by digging tunnel if any road crossings are involved. |
| While concluding the session, the chair appreciated the proponent of the proposed project on their presentation and urged the proponent to take into consideration the suggestions and recommendations that came up during public hearing. It was also stated that the participant/stakeholders could also forward their / observations in writing to PAK-EPA so that the same could be duly while processing the case of environmental approval of the project. |  |
| The meeting ended with a vote of thanks to and from the chair |  |

Attendance Sheet

Pictorial Evidence

# Annexure-XVI: Laboratory Monitoring Results for Solar Installation Site at Sanjrani Water Filtration Plant

https://www.ijsr.net/archive/v10i7/MR21710230015.pdf

https://renewablewatch.in/2020/12/03/waterless-way/#:~:text=The%20amount%20of%20water%20needed,of%20water%20would%20be%20required.

(*Source: Design Criteria of Public Health Engineering for Water Supply, Sewerage and Storm Water Drain)

(*Source: Design Criteria of Public Health Engineering for Water Supply, Sewerage and Storm Water Drain)

(*Source: Design Criteria of Public Health Engineering for Water Supply, Sewerage and Storm Water Drain)

Source: Iqbal M. Sheikh et al., (Environmental Geology of the Islamabad-Rawalpindi Area, Northern Pakistan) by Geological Survey of Pakistan.

*Source: Iqbal M. Sheikh et al., (Environmental Geology of the Islamabad-Rawalpindi Area, Northern Pakistan) by Geological Survey of Pakistan.

Source: TAS for LAI Expressway Project, 2022

*Source: WMO/GWP Associated Programme on Flood Management Pakistan: Lai Nullah basin flood problem, Islamabad – Rawalpindi cities by Ahmed kamal Federal Flood Commission, Government of Pakistan.

Source: https://www.worldbank.org/en/news/press-release/2022/10/28/pakistan-flood-damages-and-economic-losses-over-usd-30-billion-and-reconstruction-needs-over-usd-16-billion-new-assessme

https://www.citypopulation.de/en/pakistan/distr/admin/728__rawalpindi/

https://www.adb.org/sites/default/files/publication/783636/good-practice-management-control-asbestos.pdf

For air quality monitoring: ‘Passive samplers’ such as test tubes can be used or ‘Active samplers’ with sorbent tubes can also be used

For noise monitoring: sampling equipment with duration greater than 1 hour can be used.

ADB, Safeguards Unit for Central & West Asia Department, Environmental Management for Construction Handbook.